Chiara How-To

- Power-up and run

- Make sure the AC-power plug is plugged in or the battery is charged.

- Flip the toggle switch to the RIGHT (toward the power cord) for external power, LEFT (away from the power cord)for battery internal power. The blue LED should light up.

- Press the black I/O button to power on the internal computer. The other blue LED should turn on (indicating power to the computer) and the orange LED should be flashing (indicating disk access). A slight breeze should be coming out of the top exhaust port.

- Allow the Chiara to boot (1 – 2 minutes) until the orange light stops consistently flashing.

- To interface with the Chiara, look at the number taped to your robot’s back. If you want to connect over the wireless, you will connect to chiaraX, where X is the number on your robot. If you want to connect over ethernet, you will connect to chiaraXe, e.g., chiara5e if you’re using Chiara #5.

- On the workstation, open the ControllerGUI from a terminal and give it the name of your robot. If you’re using Chiara #1, you would connect over ethernet like this:

% ControllerGUI chiara1e

A window should open up that says “connecting...”

- Connect to your Chiara using ssh under the username “student”. For Chiara 1 over ethernet:

% ssh student@chiara1e

The password for student is “BugSk00l”, where ‘0’s are zeros and the last character is a lowercase “L”.

This should change the color of the prompt in the terminal to red, and it should say something 	like student@CMU-Chiara1.

- To run Tekkotsu, go to the newly connected terminal to your Chiara (with the red prompt)

% cd ~/project

% ./tekkotsu-CHIARA

Tekkotsu should output a couple lines, and the Chiara should speak “Chiara ready” when 	finished. The ControllerGUI should also come alive with a menu.

- Tekkotsu is now running on your Chiara. To run your behavior, double click “Mode Switch” in the ControllerGUI, then double click your behavior in that list.

- Shutdown and power down

- In the Tekkotsu terminal, type “quit” or press Ctrl+D

- Once you are back to the red prompt, turn off the internal computer by pressing the black I/O button on the back of the Chiara.

- Wait 15 seconds until the second blue LED goes off, this indicates the computer is off.

- Toggle the power switch to the center position to kill power, the first blue LED should go off.

- Compiling and updating the Chiara

- Boot and log in to the Chiara as in “Power-up and run”, but do not run tekkotsu (do not type ./tekkotsu-CHIARA)

- On your workstation (white terminal prompt), go to the project directory

(% cd ~/project). Make sure that your behavior has been added to “UserBehaviors.h”.

- To compile, type “make -j4” in the terminal (% make -j4). This will start compiling Tekkotsu.

- White text is normal

- Green text is information

- Yellow text is a warning. This won’t stop compilation, but could indicate an error

- Red text is a compiler error. You must fix all compiler errors

If everything succeeds, it will tell you that you will probably want to run tekkotsu-CHIARA.

- To copy what you compiled to the Chiara, run getmycode on the Chiara (red prompt):

% getmycode xx@ANDREW.CMU.EDU

Replacing ‘xx’ with your account name. Enter your password and you should see a file transfer take place.

- Once the file transfer has finished, you can now run your newly compiled tekkotsu on the Chiara (red prompt)

% ./tekkotsu-CHIARA

Uh oh, it’s not working...

- I cannot ssh into the Chiara

1. Are you trying to connect to the right Chiara? (check the number)

2. Is the Chiara turned on and fully booted? (both blue LEDs)

3. Are you trying to connect over ethernet but didn’t plug in the ethernet cord?

4. Ask for help.

- getmycode hangs or doesn’t work

1. Are you running it on the Chiara (red prompt)?

2. Did you give it your account name with capitalized domain? 
(% getmycode yourname@ANDREW.CMU.EDU)

3. Wait 2 minutes to see if it wakes up

4. Ask for help.

- I run Tekkotsu and before I do anything with it, it gives an error and stops.

1. Ask for help

- I successfully ran Tekkotsu on the Chiara, but the ControllerGUI doesn’t do anything

1. Did you ssh into the same Chiara you told the ControllerGUI to connect to?

2. In a terminal on the Chiara but not in Tekkotsu (red prompt), does

“% ps aux | grep tekkotsu-CHIARA” come up with more than 1 result? If so, follow that up with “% killall -9 tekkotsu-CHIARA”, then run tekkotsu again.

3. Are you sure that no one else is trying to use your Chiara while you are?

4. Ask for help

- I can’t find my behavior in the ControllerGUI ModeSwitch menu

1. Did you recompile the code and do “getmycode” on the Chiara to pull it over?

2. Did you add your behavior to UserBehaviors.h?

3. Ask for help.

- My behavior was running and then Tekkotsu told me something about “SIGSEGV”, spit out a stack trace, and stopped.

1. This means your code crashed the Chiara. Press Ctrl+C to kill tekkotsu. If this doesn’t bring the red prompt back, press Ctrl+Z and then type “% killall -9 tekkotsu-CHIARA”.

2. Look at the stack trace from the top down to find a line in your code. If there is one, go to that line in your code and debug it so it doesn’t crash, i.e. something is wrong with your code.

3. Ask for help.

- The Chiara is freaking out or making a weird noise and I’m afraid it’s going to break itself.

1. Pick up the Chiara so that someone has physical control of it. (prevent it from falling)

2. STOP TEKKOTSU! Do any of the following until it calms down

- Stop the behavior you ran

- Type “quit” in the tekkotsu command line

- Press Ctrl+D or Ctrl+C in tekkotsu

- Press Ctrl+Z in tekkotsu and run “% killall -9 tekkotsu-CHIARA” in the red 		prompt.

- Turn off the power switch on the robot

3. Ask / yell / scream for help