Chiara How-To

- Power-up and run

- Make sure the AC-power plug is plugged in or the battery is charged.

- Flip the toggle switch to the RIGHT (toward the power cord) for external power, LEFT (away from the power cord) for battery internal power. The blue LED should light up.

- Press the black I/O button to power on the internal computer. The other blue LED should turn on (indicating power to the computer) and the orange LED should be flashing (indicating disk access). A slight breeze should be coming out of the top exhaust port.

- Allow the Chiara to boot (~70 seconds) until the orange light stops consistently flashing.

- To interface with the Chiara, look at the number taped to your robot’s back. If you want to connect over the wireless, you will connect to chiaraX, where X is the number on your robot. If you want to connect over ethernet, you will connect to chiaraXe, e.g. chiara5e if you’re using Chiara #5.

- On the workstation, open the ControllerGUI from a terminal and give it the name of your robot. If you’re using Chiara #1, you would connect over ethernet like this:

% ControllerGUI chiara1e

A window should open up that says “connecting...”

- To run your code on the Chiara, execute the runmycode script on the Chiara you want to use. If you’re using Chiara #1 over ethernet, connect like this:

% runmycode chiara1e

- If all goes well, Tekkotsu is now running on your Chiara, (prompt: HAL:Chiara> )

- To exit Tekkotsu, type “quit” or press Ctrl+D in the tekkotsu command line

( HAL:Chiara> quit )

- Shutdown and power down

- Make sure that no instance of tekkotsu is running on the robot (Step 9 above)

- Turn off the internal computer by pressing the black I/O button on the back of the Chiara.

- Wait 15 seconds until the second blue LED goes off, this indicates the computer is off.

- Toggle the power switch to the center position to kill power, the first blue LED should go off.

- Understanding ‘runmycode’

- It first attempts to compile your code

- White text is normal

- Green text is information

- Yellow text is a warning. This won’t stop compilation, but could indicate an error

- Red text is a compiler error. You must fix all compiler errors

If everything succeeds, it continues to the next step

- Next it attempts to scp (file transfer) the executable onto the Chiara you gave it. This step starts with “scp ...” and will have an increasing percentage counter and decreasing timer

- Next it will run Tekkotsu. If Tekkotsu is up and running, the Chiara will speak “Chiara ready” and the terminal will have a new prompt ( HAL:Chiara> ).

- Once you are done with Tekkotsu, press Ctrl+D or type ‘quit’ on the command line. The script will clean up anything leftover from the executable and then say “DONE”.

Uh oh, it’s not working...

- The scp step of ‘runmycode’ says “No route to host” or “lost connection”

1. Are you trying to connect to the right Chiara? (check the number)

2. Is the Chiara turned on and fully booted? (both blue LEDs)

3. Are you trying to connect over ethernet but didn’t plug in the ethernet cord?

4. Ask for help.

- I run Tekkotsu and before I do anything with it, it gives an error and stops.

1. Ask for help

- It successfully ran Tekkotsu on the Chiara, but the ControllerGUI doesn’t do anything

1. Did you call runmycode with the same Chiara you told the ControllerGUI to connect to?

2. Are you sure that no one else is trying to use your Chiara while you are?

3. Ask for help

- I can’t find my behavior in the ControllerGUI ModeSwitch menu

- 1. Did you re-run ‘runmycode’ on the Chiara to pull it over?

- Did you add your behavior to UserBehaviors.h?

3. Ask for help.

- My behavior was running and then Tekkotsu told me something about “SIGSEGV”, spit out a stack trace, and stopped.

1. This means your code crashed the Chiara. Press Ctrl+C to kill tekkotsu. If this doesn’t kill tekkotsu, press Ctrl+Z and then type “% killall -9 tekkotsu-CHIARA”.

2. Look at the stack trace from the top down to find a line in your code. If there is one, go to that line in your code and debug it so it doesn’t crash, i.e. something is wrong with your code.

3. Ask for help.

- The Chiara is freaking out or making a weird noise and I’m afraid it’s going to break itself.

1. Pick up the Chiara so that someone has physical control of it. (prevent it from falling)

2. STOP TEKKOTSU! Do any of the following until it calms down

- Stop the behavior you ran

- Type “quit” in the tekkotsu command line

- Press Ctrl+D or Ctrl+C in tekkotsu

- Press Ctrl+Z in tekkotsu and run “% killall -9 tekkotsu-CHIARA” in the red 		prompt.

- Turn off the power switch on the robot

3. Ask / yell / scream for help