cd ~/project/

cp ~/Class/Tekkotsu/project/DanceBehavior.h.fsm ~/project

gedit DanceBehavior.h.fsm UserBehaviors.h &

Connect to the robot up to the point that the ControllerGUI is connected.

Open up Background Behaviors, then highlight ‘Relax’, but don’t run it

Press Add in the lower right corner of the ControllerGUI, then hit “OK” in the window that opens

- This will create a script to turn on or turn off the Relax behavior

DO NOT LOAD ANY POSTURES YOU HAVEN’T MADE YOURSELF. THEY MAY BE OLD AND TRY TO MAKE THE CHIARA BREAK ITSELF!!!!!!!!

Generating Postures

+ Double click the “Relax” script in the right list. The command line should say “Relaxing servos”

+ From Root Control, open up File Access, then Posture Editor

+ Move the Chiara’s arm and legs into any position you want.

+ Double click “Save posture...”

+ Type the name of the posture in the text input box in the ControllerGUI, and always end the name in 	“.pos”. For example, “2legsup.pos”

+ Repeat as many times as necessary for all the postures you want

+ Double click “Relax” again to lock the servos

Make the Chiara dance by stringing together a series of “Pose” nodes.

startnode : Pose($, “pos1.pos”, 0.5) =T(2000)=> Pose($, “pos2.pos”, 0.25) =T(1000)=> startnode

Dance Contest

Create a series of postures and string them together to make the Chiara appear to dance. Get fancy and have it flash lights and speak while dancing. You’ll need to be smart about timing things appropriately. Try and make the best dance for the dance-off at the end of class!

Note: Don’t make the Chiara switch between 2 postures very quickly. The legs can gain momentum and cause the Chiara to flip. HAVE SOMEONE READY TO GRAB THE CHIARA WHENEVER IT’S DANCING