QWERK how-to

TURN ON THE ROBOT

- Plug in a USB-wireless adapter into the back of the QWERK

- If you’re running a QWERK on a moving robot, hook up a battery, otherwise plug in a power cord

- Flip the switch to the ON position, lights will start flashing, wait about 45 seconds for it to boot

- If all goes well, LEDs 0, 1, and 2 will be solid and LED 6 will be flashing. Your QWERK is on

CONNECTING TO THE ROBOT

- Make sure the QWERK is up and running

- Make sure your laptop is connected to the “QWERKNET” network

- Click the Connect button in either the Robot Universal Remote, or your compiled program

- Choose “Directly Connect to Peer” and hit OK

- Look for the name on the USB-wireless adapter (reltemp# or relqwerk#)

- Use that name for the hostname and hit Connect

RUNNING CODE

- Click the “Shortcut to MyFirstRobot.jcp” on the desktop to open the IDE

- Open up the file you are working with (usually MyFirstRobot.java)

- Make changes to your code

- Click Build - > Execute File

- Fix any compiler errors that come up

- Once your program is running, connect to your QWERK, then hit the Play button to run your code

If you can’t connect to your QWERK:

- Make sure your QWERK is on and the appropriate LEDs are on or flashing

- Make sure your laptop is connected to the “QWERKNET” network

- Make sure you have the correct name of the USB-wireless adapter