Using the Storyboard Tool

- To run the storyboard, type into the terminal “Storyboard”.

- A window should pop-up named Storyboard Tool.

- Open the ControllerGUI and do runmycode to connect to your Chiara as usual.

- In the Storyboard tool, change where it says host to your Chiara’s name.

- Under mode-switch, decide on a behavior that you’ve made, either TestBehavior or DanceBehavior.

if you don’t have one, copy over the basic one and follow the instructions in “How to create your own behavior” to hook it into the ControllerGUI and reconnect to your Chiara:

-> cp ~/Tekkotsu/project/TestBehavior.h.fsm ~/project/

- Type the name of the behavior you decide in the Storyboard under “Behavior Name”.

- Run that behavior on the Chiara and then hit the “Download Model” button on the Storyboard.

- Say yes if it asks you a question about automatically generating things.

- Once the Storyboard gets a bunch of information, stop your behavior even if it’s doing something.

- This allows you to visualize the behavior.

- Rearrange the boxes and lines in the top panel until they make sense. Try to keep boxes at different vertical levels, this will help later. Notice how you can resize boxes and change elbows of lines.

- Examine the boxes and lines to see what each one corresponds to in your behavior

- Now click “Record New Trace” in the Storyboard, and then re-run your behavior

- You should see the Storyboard tool visualize the states and transitions in the bottom panel

- Once the behavior finishes or repeats, hit the “pause trace” button

- Play around with the contents of the bottom panel to understand what was displayed of your behavior

This gives you a nice visual way to see exactly what states your behavior goes through at what time. It is particularly useful to see if your behavior is going to a state out of order.

You will write a new behavior and debug it using the Storyboard tool. I want you to get practice using the Storyboard so that you can more easily debug your future state machines.

Repeat After Me

Write a state machine that randomly blinks the red, yellow, or green LED and waits for the user to press the button with the same color as the LED that blinked. If the user presses the correct button, reward the user and keep playing. If the user presses an incorrect button, have the robot act sad and then keep playing. Call the behavior “RepeatAfterMe”.

If you have extra time, do Repeat After Me – Reflex

Using the same rules as Repeat After Me, make the state machine take a random amount of time until it blinks an LED. Then make the user press the correct button within half a second of blinking the LED to win, otherwise start over. In the end, a user will wait an unknown amount of time for an LED to blink, and then press the appropriate color button as soon as possible. Call the behavior “RepeatAfterMeReflex”.

Please see the “How to create your own behavior” handout before starting.

How to create your own behavior

Let’s say you want to create a behavior called “MyBehavior”.

The file you will create will be called MyBehavior.h.fsm . The name of the file must be the name of your behavior followed by .h.fsm

In a terminal, go to the project directory, then create the file with gedit.

-> cd ~/project/

-> gedit MyBehavior.h.fsm &

This should open up a blank screen. Here is the most basic file that you should start from:

//-*-c++-*-

#ifndef INCLUDED_MyBehavior_h_

#define INCLUDED_MyBehavior_h_

#include "Behaviors/Leapers.h"

#nodeclass MyBehavior : StateNode

#shortnodemethod setup

#statemachine

startnode: StateNode

#endstatemachine

#endnodeclass

#endif

Note: In the second and third line, you need to rename the part that says “MyBehavior” with the name of your behavior. Lastly, in the line that starts with #nodeclass, you need to change the part that says “MyBehavior” to the name of your behavior.

Hooking your behavior into the ControllerGUI

In gedit, click the Open button and open the file called “UserBehaviors.h”

Around the 10th line, you should see a statement or two that starts with #include.

Add this line right above or below those:

#include “MyBehavior.h”

Note: You must have the name of your behavior followed by .h, all in quotations

Lastly, look around line 25. You should see some lines that say “MENUITEM(something)”

You need to add this line above all those:

MENUITEM(MyBehavior)

Note: That has to be the name of your behavior, with no .h or .h.fsm

Do runmycode again, and if no errors come up, you should see your behavior in the ControllerGUI under Mode Switch!