Open your file by running these commands:

cd ~/project

gedit TestBehavior.h.fsm &

Advanced State Machines

Short Version -

startnode: StateNode =T(5000)=> RedLEDOn =TM(“off”)=> RedLEDOff =T(100)=> startnode

Expanded Version -

startnode: StateNode

Red: RedLEDOn

RedOff: RedLEDOff

startnode =T(5000)=> Red

Red =TM(“off”)=> RedOff

RedOff =T(100)=> startnode

Note: You can define multiple transitions to come from, or go into any node you define

startnode =B(ChiaraInfo::GreenButOffset)=> GreenLEDOn

startnode =B(ChiaraInfo::RedButOffset)=> RedLEDOn

or

RedOff =B(ChiaraInfo::YellowButOffset)=> startnode

Red =T(4000)=> startnode

To define a state:

[NAME]: [TYPE]

Where [NAME] is a name you come up with, and [TYPE] is the kind of node you already know (i.e. RedLEDOn, etc.)

startnode: StateNode

To define a transition:

[NODE A] [TRANSITION] [NODE B]

Where [NODE A] and [NODE B] are states you’ve already defined, and [TRANSITION] is a transition you already know (i.e. =T(4000)=>)

RedOff =T(100)=> startnode

New Transitions

Null Trans, written as =N=> Immediately goes on to the next node

startnode =N=> Red

Can fork to multiple nodes!

startnode =N=> {Red, YellowLEDOn, GreenLEDOn}

This would turn on all 3 LEDs at the same time

Random Trans, written as =RND=> Randomly transitions immediately to any of the nodes you give it

startnode =RND=> {Red, YellowLEDOn, GreenLEDOn}

This would randomly turn on one of the three nodes

Games:

Combination Lock:

Write a state machine that acts like a combination lock. Make a user need to press a particular button sequence of 5 buttons to “unlock” the lock. For example, a user would need to press the red button, then yellow button, then green button, then yellow button, then red button, in that order to win and make the chiara celebrate. If the user ever presses a button out of order, have it nod no and whimper, then start back at the beginning.

Repeat After Me:

Write a state machine that randomly blinks the red, yellow, or green LED and waits for the user to press the button with the same color as the LED that blinked. If the user presses the correct button, reward the user and keep playing. If the user presses an incorrect button, have the robot act sad and then keep playing.

Repeat After Me Reflex:

Using the same rules as Repeat After Me, make the state machine take a random amount of time until it blinks an LED. Then make the user press the correct button within half a second of blinking the LED to win, otherwise start over. In the end, a user will wait an unknown amount of time for an LED to blink, and then press the appropriate color button as soon as possible.