- Run these commands:

cp ~/Class/Tekkotsu/project/TestBehavior.h.fsm ~/project

cd ~/project

gedit TestBehavior.h.fsm

- Try different series of states to make the robot do interesting things. How interesting can you make the transitions between states?

States:

RedLEDOn, RedLEDOff, GreenLEDOn, GreenLEDOff, BlueLEDOn, BlueLEDOff, YellowLEDOn, YellowLEDOff

Speak(string) – Speaks the text – Speak(“Hello, I’m a Chiara!”)

Play(string) – Plays the wav file with the name passed in – Play(“fart.wav”)

Look in File Access ->

Forward( millimeters) – Walks forward – Forward(200)

Sideways( millimeters) – Strafes sideways – Sideways(200)

Turn(angle) – Rotates by the given angle – Turn(M_PI / 2)

Rock(amount) – Lean forward or back. Give value between -30 and 30 – Rock(25)

Sway(amount) – Lean right or left. Give value between -30 and 30 – Sway(25)

Twist(angle) – Rotate about the legs. Give value between M_PI/3 and -M_PI/3 - 	Twist(M_PI/12)

LegsUp – Stands the robot up

LegsDown – Sits the robot on its belly

LookAtPoint(x, y, z) – Points the head at the point relative to the robot – LookAtPoint(0, 100, 100)

LookStraight – Center’s the camera forward

NodYes(number of times) – Nods the head in a “yes” fashion – NodYes(1)

NodNo(number of times) – Nods the head in a “no” fashion - NodNo(1)