Preproposal for the 1998 Continuation of

Automated Materials Transport System

National Robotics Engineering Consortium

The Robotics Institute

Carnegie Mellon University

Ten 40th Street

Pittsburgh, PA 15201

March 10, 1997

Alonzo J. Kelly, Principal Investigator

contact info:

A.J. Kelly

Project Scientist, The Robotics Institute

alonzo@ri.cmu.edu

412-576-9016

# 1. Partners and Facilities

NASA, Ford Motor Company, the United Auto Worker Union, the Hyster Company and the National Robotics Engineering Consortium have organized a product development program to automate materials transport vehicles such as forktrucks and tug-AGV’s. Development and testing work for the robotic forktruck will be carried out at the National Robotics Engineering Consortium facility and at several Ford facilities including Wayne Assembly and Stamping Plant, the Ypsilanti Electrical Components plant, and potentially other facilities. Development work for a forktruck simulator is currently under negotiation with NASA Ames.

Key technologies source from NASA (Map mosaic algorithms, employed by NASA to piece together planetary maps from space probe fly-by, are used in this project to enable high-repeatability vision-based position estimation; and, satellite image tracking technology, used in this project to identify and track the racked and palletized cargo); Carnegie Mellon (planning software expertise); Ford Motor Company (materials flow inventory and control software), and the Hyster Company (coordinated controls technologies).

# 2. Goals and Objectives

This NASA-sponsored project will produce a low-cost hardware component and a software set which may be retro-fitted to existing materials transport vehicles to enable autonomy. Autonomous forktrucks will be able to automatically identify and engage palletized and racked loads, move them from location to location, and load and unload tractor-trailers. Several such autonomous transport vehicles will be coordinated and supervised by an operator located at a central workstation. The project will be deemed a success if the throughput capabilities of the automated forktruck system operating in the Ford test site during the Acceptance Test meet those of current human operated system.

# 3. FY98 deliverables

FY 98 will see the production of a set of extended planners which increase the capability of the autonomous forktruck to perform certain tasks including:

Extended Planner #1

• Identify stacked loads

• Lift stacked loads as a pair

• Destack stacked loads

• Identify the trailer ramp

• unload a double stacked trailer

Extended Planner #2

• Identify and localize dolly

• Place loads on dolly

Extended Planner #3

• Stack pallets

• Load stacked pallets into trailers

Extended Planner #4

• Operate multiple forktrucks in individual, non-overlapping domains.

Extended Planner #5

• Operate multiple forktrucks to cooperate in a single domain.

Each of these planners is tested under an integrated test mutually agreed upon with Ford Motor Company. During FY98, each of the integrated tests is performed as a GO/NO-GO functional test. During FY99, each of these tests will be repeated as part of an overall systems test with a quantified human performance equivalence requirement.

# 4. Justification / User community requirements/ Dual Use

The NREC and NASA designate as Core Technologies those portions of the project deliverables identified as having dual use potential. Each of these technologies is the terrestrial equivalent of a portion of a planetary exploration navigation system. The following technologies under development are identified in the Ford Product Development Plan as Core Technologies:

Target tracking & docking system - A software system that employs vision based methods to guide the vehicle in an approach trajectory to engage the forktruck tines in the rack or pallet. Once inserted, proximity sensors may be used to complete the insertion. This problem has been discussed at length with NASA engineers and appears to be more difficult than the currently addressed ORU insertion task. The production of this system will result in a commercial grade software system.

Data Logger - A software system which records system input and output, for the purpose of performing off-line system debugging. Data loggers are especially important to debugging vision applications due to the volume of data. A commercial grade genralized debugging system will be produced.

Multi-Vehicle Coordination System - A software system which optimizes, against a performance metric, the goals of a set of vehicles operating within a single workspace. A commercial grade coordination system will be produced.

Coordinated Control Software - A software control set that coordinates the motions of low level axes to achieve the bodily motion of the robot. Usually takes the form of a speed controller and a path tracker. This system is a commercial grade    extension of previously developed systems.

# 5. Technical Approach

In this section, the system architecture is presented in the form of grouped subcomponent technologies. Candidate technologies or approaches are identified for each of the component systems. These technologies offer a good starting point for the project, and may help the team to identify underlying issues, but are not meant to be a comprehensive or exhaustive study.

## 5.1    Architecture

The Autonomous Materials Transport System is an extended feedback system in which a low level control system, a planner, a high fidelity position estimation system, and a cargo localizer are the primary components. The low level control system interfaces the software system with the physical machine, provides low level feedback, and coordinates control of individual axes. The planner is comprised of a few speciality modules, each of which performs a planning operation such as docking with a pallet, disengaging from a pallet, and transiting to and from locations. The high fidelity vision-based position estimation system has been developed to enable high repeatability motions necessary to this project. The cargo localizer is a combination vision and non-contact ranging sensor which provide the position and orientation of the cargo load.

## 5.2    Distributed and Coordinated Control

The coordinated control system is a software module that coordinates the control of individual low level axes to produce the bodily motion of the machine. Coordinated control is both straightforward and machine specific. To implement coordinated control, we will develop and emplace a load adaptive servo control algorithm and a path tracking algorithm. This coordinated control software will be developed as a core technology. An REC spin-off company has commercialized distributed control hardware modules for the New Holland harvester project and is willing to cooperatively develop specialized modules that support the functions required to automate materials transport vehicles. In general, the low level control of the electric forktruck is a simpler problem than the electric over hydraulic control of a harvesting machine.

The degree of coordinated control required for this application seems relatively simple and straightforward. There are two axes of motion to be automated - steering and speed. Current indications among corporate sponsors is that the target vehicle will be an electric hi / lo. Speed control of electrical vehicles is straightforward and may be done with linear controllers given the required mass-variable formulation of the control law. Steering control will influence the servoed speed command; however, the steered trajectories are well constrained, indicating that nominal speed profiles may be chosen for each segment, without the need for profile update except in emergency situations.

## 5.3    Vision Based Position Estimation

The project is enabled by the vision-based position estimation technology, all other forms of position estimation having been found unsuitable. This technique creates a large image mosaic of the factory floor, which may be compared to later images to measure relative position. The position estimation system enables robust motion by precisely localizing the autonomous machine within a known work envelope. This system requires the use of a single black and white camera, digitizer, processor and mass storage. The position estimation system must be developed from its current research proto-type stage into a modular, board level system. The development of the vision based position estimation system is a privately funded, non-core technology.

## 5.4    Planning

The vehicle motion planner will compute candidate optimal trajectories to enable loading, unloading, transport, and manipulation of material. The motion planner will consist of several individual specialty modules that are coordinated by a module arbiter. The motion planners are task specific to the project, and will be developed as non-core, privately funded technologies.

The planning system is comprised of a number of individual speciality control modules, each of which performs a single task, such as Transit, which moves the machine from location to location while observing the area for potential obstacles. Modules are selected by the Arbiter, which considers its point in operational cycle, requests from the operator and the state of the physical machine in choosing a control module.

## 5.5    Obstacle Detection

The obstacle detector ensures that the travel area within a specified range of the vehicle is clear of obstructions. This system will be developed as a core technology.  Obstacle detection requires the use of a non-contact sensor in combination with a bumper guard (as required by Ford standards). The type and design of the obstacle detection suite will be dependent upon the manner in which the autonomous forktruck system is integrated into the end-customer’s facility. Specifically, we must consider the frequency and type of obstacle encountered; whether it is dynamic or static; whether it is human or not; etc. Range detectors such as wide beam sonar, or narrow beam radar may be used. We may also choose to use visual techniques. This area is probably the most open-ended from the point of view of design.

## 5.6    Cargo ID and Localizer

The cargo identifier compares a camera image with a portfolio of racks and pallets to recall approximate geometric and mass load data. The identifier also acts as the cargo localizer, possibly in concert with range sensors such as sonar or radar. This system may support the Obstacle Detection function. This system will be developed as a core technology.

During the engage and disengage operations, a visual servo-lock will be acquired and maintained on the cargo pallet or rack. This vision system will perform two functions: (1) identify the type of cargo being manipulated, and (2) localize the cargo. A cross-track localization will be necessary for steering. We are uncertain as to whether a longitudinal measurement must be taken. It may be more efficient to detect low-speed contact.

## 5.7    Operator’s Workstation

The software system resident in the operator’s workstation performs supervisory and coordinated control activities, including the following functions:

• communications to/from individual vehicles.

• individual vehicle status display.

• operator input console.

• hidden debugging and data logging modules.

• planning for multiple vehicle coordination.

• interface to existing materials control or inventory software.

## 5.8    Critical Component Development Sequence

Following a comprehensive system design period, the REC proposes a sequential development of the critical subcomponent technologies with a component-level test and characterization process. The controls hardware will be simultaneously specified and ordered during the first month period. From the system architecture, two critical components may be identified:

• Vision based position estimation - this technology is the enabling technology for the project. We propose to begin the project with a large scale benchmark test of the repeatability and robustness of this method.

• Cargo ID and Localizer - cargo localization is a necessary pre-requisite for cargo handling applications. This technology has been demonstrated in previous projects, both at Ford and at CMU. The REC proposes to develop the localizer to the point where it will support testing, and to benchmark its robustness.

## 5.9    Secondary Components and Availability

The REC will integrate a number of secondary components, many of which are available from current projects, into the overall system. This section details secondary components and their status and availability for integration.

• Distributed Controls Hardware - this system is currently in its third generation. The fourth generation system, under development, is the pre-production prototype of the system which New Holland will employ on its production machines. The REC proposes to adapt this product to the forktruck. Preliminary studies indicate that the process should be straightforward as the electric forktruck is a less complicated control than the hydraulic Harvester.

• Coordinated Control - the forktruck will require both a path tracker and a speed control algorithm. The pure pursuit algorithm is a standard which has been employed in at least half a dozen of our on-road and off-road wheeled machines. The speed controls algorithm may require load adaptive parameters, to be determined during testing. Such adaptation is straightforward.

• Planning Modules - the forktruck cycle will require a few different motion planning modules, each of which will be integrated into the system through a central arbiter. The arbiter uses state information to determine which module to employ throughout the navigation cycle. Each of the modules will be developed during the integration process.

• Obstacle Detection - the obstacle detection suit will be specified and integrated in close consultation with Ford personnel in order to assure that it meets current factory standards.

## 5.10    Integration Sequence

The REC proposes to incorporate the critical components into an integrated system through the following integration sequence:

• Emplace controls hardware on Hyster vehicle.

• Integrate path tracker to enable constant speed trajectory tracking.

• Integrate speed control to enable servo-control trajectory tracking.

• Integrate vision-based position estimation to enable end-point attainment.

• Integrate transit planning module to enable point to point motion.

• Integrate cargo-ID to enable localization of cargo and AGV trams.

• Integrate docking module to enable pallet docking.

• Integrate disengage module to enable pallets to be placed.

The result of this integration sequence is a software system capable of identifying and docking with individual pallets. These pallets may be transited to a new location and placed. The integration sequence represents the embodiment of all of the physical controls necessary to perform robust cargo handling.

# 6. Advances wrt State of the Art.

This commercialization project will advance the commercial state of the art by producing a significantly more cost effective navigation system for factory automation. Vision based position estimation and sensing systems are nearly an order of magnitude less costly than laser-based or inertial systems. The components of the factory automation system, including the two vision systems, will be available as commercial grade software systems, at low cost, for dual use purposes.

# 7. Schedule / Milestones

The year 2 schedule is based upon the attainment of the following goals:

• Implement a software engineering program to achieve commercial grade software standards.

• Expand the capabilities of the system to achieve more complicated activities.

• Expand the system from single to multiple forktruck management capabilities

• Test the system in Ford facilities

• Evaluate the cost effectiveness and performance of the retro-fit design with Hyster.

• Prepare to interface the central system with the Ford inventory control system.

| Table 1: Year 2; Months 13 - 18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 14 | 15 | 16 | 17 | 18 |
| Milestones | Redesign Review; Pick and Place Test | Low Speed Trailer Unloading | Unload trailer onto dolly; Performance Report to Ford | Load a trailer | High Speed Trailer Unloading | High Speed Trailer Loading; Base System Demonstration. |

| Table 2: Year 2; Months 19 - 24 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 20 | 21 | 22 | 23 | 24 |
| Milestones | Base Test #1 @Ford Facility | Unload single trailer using two cooperative forktrucks | Unload multiple trailers using multiple non-cooperative forktrucks | Base Test #2 @Ford Facility |  | System Demonstration |

Extended Planners

The system will be made more capable through the integration of a series of sequentially more sophisticated planning modules, each of which is designated an Extended Planner. The capabilities of each of these planners will be tested in the context of the integrated system.

Base Tests

In addition to these tests, a series of two benchmark tests occur at the Ford facility. The first benchmark test has the following goals:

• Test the capabilities of the EP#2 system in a Ford facility

• Expect the system to fail when confronted with skewed, or force constrained loads.

• Gather statistical data, regarding failure, of sufficient base for interpretation.

• Describe and quantify contextual differences between Ford site and REC site.

• Gather and report on site-specific integration needs.

• Catalogue failure modes and compare with expectations.

• Identify potential long term operational changes for Ford.

• Author a first draft of an operations guide for the system.

The second benchmark test has the following similar goals:

• Test the capabilities of the EP#3 system in a Ford facility.

• Utilize and / or provide the same criteria, basis, and results as the previous base test.

# 8. Budget - All years:

## 8.1    Budget Summary:

| Table 3: Budget Summary |  |  |  |
| --- | --- | --- | --- |
| NASA | $1M | $1M | $1M |
| Industry | $1M | $1M | $1M |
| Total | $2.0M | $2.0M | $2.0M |