Remote Reconnaissance For Environmental Restoration

Tim J. Denmeade

Sr. Product Manager

RedZone Robotics, Inc.

1.  Application Overview

There is a need domestically and internationally for field robotics

technology for deconstruction in support of environmental restoration.

The key resource is a set of compatible component technologies -

mobility systems, communication and control technologies, sensing and

modeling methods, tooling and work packages - that can be quickly

configured into purpose-built, low-risk, high impact work systems.  The

availability of these systems will enable major environmental

restoration projects around the world: dismantlement and cleanup of

government weapons complex sites, remediation of ordnance stores,

nuclear power plant decommissioning, and environmental accident

response.

Deconstruction of nuclear or hazardous facilities, whether accident

damaged or not, requires extensive remote capabilities including:

*	characterization and monitoring of contamination levels, structural

integrity, and other features of the environment for planning

*	sampling and removal of hazardous or radioactive materials

*	obstruction removal to enable facility access for remediation

activities.

*	demolition and retrieval of materials

*	waste packaging and transport.

To signal the maturation of these technologies, we will address one of

the most visible and dramatic environmental restoration problems in the

world - the stabilization of the Chornobyl Unit-4 accident site.  The

threat of a criticality event within the Chornobyl Unit-4 "Ukritiye" or

shelter is a constant priority.  Recent elevated neutron measurements

within the shelter have raised concern to a heightened level and

prompted a renewed sense of urgency.  Criticality issues are compounded

by the frequent application of thousands of gallons of water to control

dust emissions from the shelter.  As such, the situation at Unit-4

requires swift, definitive action.

Part of the fuel, having melted from a combination of decay heat and

chemical interactions with surrounding materials, formed peculiar

lava-like flows or fuel containing masses (FCMs) that entered various

areas of the reactor building beneath the core.  This NASA-sponsored

project will augment work conducted in 1997 and produce a second

radiation hardened telerobotic system for nuclear facility

reconnaissance  (nuclear accident response and Chornobyl-type

remediation) with particular focus on maturation of positioning

technology to support environment mapping.

2.  Justification

The challenges posed by radiological conditions and accident debris at

the damaged Chornobyl Unit-4 reactor require remote technology.  In

response to this need, robots will be used at Chornobyl to perform

dismantling activities and extensive characterization of the facility

to ultimately facilitate the removal and packaging of the "lava like"

fuel containing masses (FCMs) which have flowed into lower regions of

the reactor building.  Inherent in any deployment mission into the

Unit-4 Shelter is the need for characterizing the work environment to

allow efficient task planning and operations.

This program team will provide an efficient and effective response to

this application:

*	Our team is already doing substantial Chornobyl-specific work with

the Ukrainians.

*	Our team has available technology requiring minor maturation for

deployment at Chornobyl Unit-4.

*	Our team integrates best-in-class organizations and provides a fast,

capable, committed resource.

An extensive agenda of cleanup and dismantling activities exists at the

damaged Unit-4 reactor.  The respective governments of the G-7

Countries and the Commission of the European Communities have developed

a cooperative approach to support the decision of Ukraine to close the

Chornobyl Nuclear Power Plant by the year 2000.  As part of this

international commitment, the Ukraine and the G-7 will continue to

cooperate in the development of a cost effective and environmentally

sound approach to stabilization and eventual remediation of the shelter

for Unit-4.

3.  Objectives and Goals

We propose to evolve for ultimate deployment two vital robotic

technologies for remote operation and position mapping in the harsh

environment of Chornobyl Unit-4.  These vital pieces address the needs

of remote physical access (locomotion), and remote presence

(location/environment sensing).

We propose to integrate an existing, tested locomotor, RedZone's

Houdini system to suit the needs of Chornobyl.  The particular

requirements of compactness, ruggedness and low mass which are required

for ease of deployment.  The Houdini platform will be used to deliver

the positioning sensor payload.

Remote technology for environmental restoration require the delivery of

various tooling payloads over rigorous terrain.  Such reconnaissance

and workhorse machines require accurate pose awareness (position and

orientation) to support environmental mapping techniques and real time

work tasks.  Accurate pose awareness is critical in the development of

mapping tools in support of future work tasks.  However, conventional

pose estimation aids impose severe limitations.  The best self

contained pose sensors such as landmark recognition, dead-reckoning and

IMU (Inertial Measurement Unit) fall short in long duration missions

such as Chornobyl-like reconnaissance.  Integrating these positioning

methodologies can solve the limitations of either sensor alone.  We

propose the development of an integrated sensor that will fuse data

form inertial gyros and accelerometers with other positioning schemes

in a tightly coupled feedforward Kalman filter.

4.  Technical Approach

Technical success of the integrated sensor depends on applying the well

understood Kalman filtering approach to the special case of the

Chornobyl reconnaissance  mission.  This requires an understanding of

the physics and inner workings of existing positioning technology like

IMU and how they are impacted by the harsh conditions inside the Unit-4

shelter.  Initial work will determine the behavior of known sensors

under these conditions and evolve the structure and optimize the

parameters for the Kalman filter to take this into account.  Later work

will reduce these parameters into a functional module for testing on

the Houdini vehicle platform.

5.  Technology Maturation

A vital aspect of this project is the next step in navigation aids and

mapping for various mobile systems.  Ultimately, accurate pose

estimation can be used in systems providing skid control, navigation,

assistance in road following, and aids in car safety.  Mobile robot

types for military and commercial uses such as advanced scouts, mine

identifying and clearing, munitions deployment, security, civil

safeguarding and convoying are also direct application areas.

Likewise, this project will have an evolutionary path to an integrated

remote work module which has significant uses in nuclear dismantling,

and in NASA's ISRU (In Situ Resource Utilization) vision to lower

planetary human exploration costs, leading to self sustaining planetary

bases.

6.  Partners/Deliverables

RedZone Robotics, Inc. together with collaborators from Carnegie Mellon

University, National Robotics Engineering Consortium , NASA and

Ukrainian Institutes will develop an innovative integrated sensor which

will deliver sub-meter accuracy positioning for long duration missions

in harsh Chornobyl-like operating conditions.

7.  Schedule

Remote Reconnaissance For Environmental Restoration - Program Tasks

Description                Month:

| Description | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Needs Assessment Plan | X |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Specification |  | X | X | X |  |  |  |  |  |  |  |  |
| Hardware Suitability |  | X | X | X |  |  |  |  |  |  |  |  |
| Fabrication and Integration |  |  |  |  | X | X | X | X | X | X |  |  |
| System Demonstration |  |  |  |  |  |  |  |  |  |  | X |  |
| Documentation |  |  |  |  |  |  |  |  |  |  |  | X |

Task Definitions:

Needs Assessment:  Project team to define and develop methods and

application scenarios for remote deployment within Chornobyl shelter

consistent with G-7 working group implementation plans.  Cooperatively

refine technical objectives to meet application requirements.

Equipment Specification:  Project team will develop a complete

specification of the requirements for the system architecture.  This

activity culminates in a report explaining the complete system

framework.

Hardware Suitability:  Project team will evaluate the suitability of

existing RedZone Houdini vehicle and establish a sensor technology

baseline.

Fabrication and Sensor Development:  Project team will complete

fabrication of a Radiation Hardened Remote Reconnaissance System.

System Demonstration:  Project team will conduct mock-up demonstration

of system features and functions.

Documentation:  This activity culminates in a report explaining the

complete design including system diagrams, test results, and

recommended modifications for future work.

8.  Budget

NASA Cash		NASA In-Kind		Match

1997			      $700 K     	             $300 K			$1.2 M

1998		 	      $700 K	            $300 K			$1.0 M