Precision Surface Operations

Principal Investigators................................. Tony Stentz

..............................................................Kerien Fitzpatrick

................................................................... Takeo Kanade

Objectives and Goals

Goal: 	Increase cost-effectiveness of farming operations

Objective:	Selectively apply spray materials using field database, satellite imagery, and local imaging to minimize costs. Field database provides coarse goals, satellite imagery looks for trouble spots, and local imaging locates trouble spots for precision spraying.

Objective:	Automate driving functions to reduce overall labor costs. Intelligent agents would minimize the number of operators, focusing more on humans for maintenance than operation.

Objective:	Increase daily productivity by increasing operations window. Automated equipment does not tire allowing productive operations during low-light or no-light conditions.

Objective:	Improve farmer understanding of daily operations by improving ability for remote monitoring and command. Farm operator would check status of all intelligent agents through the Virtual Farm Interface.

Objective:	Reduce supervisory cost by enabling intelligent agents to coordinate amongst themselves. Intelligent harvester would schedule rake after it completes the harvesting task.

Goal:		Precision surface operations

Objective:		Intelligent agents interact with supervisor via virtual interface.

Objective:		Inter-agent coordination and teaming

Objective:		Reallocation of agents upon equipment failure

End-User Needs

Farm operations seek to minimize the cost of inputs for a given yield performance for their acreage. In addition, federal regulations for agricultural chemicals are strengthening to minimize contamination of local ground water. In the near term, developing increased resolution field models will enable computer-controlled spray systems to further minimize overall amounts of applied chemicals. Next, integrate processing of satellite imagery for guiding the evaluation of local imaging to locate small hot spots for specific treatment. Integrate this visual identification to spray systems capable of fine control of each spray nozzle to further limit chemical application.

Due to the inability of farming to stabilize its environment (temperature, sunshine, precipitation), this process will need to be tested over multiple years to obtain the necessary data to completely evaluate the performance.

Technical Approach

Intelligent Implement

Work cooperatively with the CMU Spray Systems Center, New Holland, and ADM to determine the necessary performance specifications for the intelligent sprayer. CMU would build upon an existing computer control sprayer to add the necessary processing for it to become an intelligent agent. The sprayer would know its capabilities, its requirements from other equipment (tractors), and its costs (chemicals, etc.).

Autonomous Tractor

CMU and New Holland will move part of the Demeter system to the tractor. Equipment-specific modules would be developed and validated by CMU. New Holland would integrate the new technology into its tractor production line.

Precision Spray System

Combine the tractor and intelligent implement into a precision spray system. The precision spray system would be able to spray very fine profiles with independent control of spray nozzles. Surgical spraying or selective application would be possible using a visual recognition system to identify local hot spots for increased or decreased application. CMU would develop the visual recognition system. ADM would provide expert advice regarding chemical applications.

Telepresence for Precision Surface Operations

NASA Ames would refine and restructure their VEHI technology to develop an efficient interface for monitoring and command of numerous intelligent agents with varying capabilities and costs. First generation proof of concept on an SGI. Second generation would move to a Pentium/NT platform for actual use by the end-users.

Intelligent Field Operations

INEL would implement their transfer of command technology to a simulator of the intelligent agents being developed under this funding (tractor and spray implement). This would demonstrate a future capability for the intelligent agents to determine that a cooperative agent had failed, request a replacement, and transfer task knowledge to the replacement. INEL would follow by expanding this scenario to encompass additional intelligent farm agents such as combines, bale wagons, and additional implements.

Advances in the State-of-Art

Increased resolution for spray control by examining the local area for specific visual phenomena. While GPS allows current operations to construct spray profiles for a one meter grid, sampling currently is much coarser or looks a localized results (yield monitoring). Visual examination of local phenomena would catch problems that could be fixed and increase the resolution in the future to a per plant basis for some crops.

Telepresence would improve the understanding and acceptance of complex planning and monitoring systems by putting known images in front of the supervisor. In addition, farm operators typically spend their day traveling from field to field to monitor their operations. Remote monitoring would allow them to conduct business in their office which would be more productive and less costly.

Inter-agent coordination and teaming would represent a significant milestone for capable robotic systems. Intelligent equipment would be capable of thinking like supervisors - understanding the larger operation than just their task.

FY98 Deliverables

Intelligent implement (spray) docks with automated tractor. Agents communicate with each to determine operational specifications and needs.

Define the classes of agents for farming operations and specify the communications protocol and medium for their interaction.

Construct a model of the farm operation and implement two of the intelligent agents (tractor and spray implement)

Partners & Facilities

National Robotics Engineering Consortium, The Robotics Institute

NASA Ames

New Holland North America, Inc.

Spray Systems Center, Carnegie Mellon University

Archer Daniels Midland

Idaho National Engineering Laboratory

Dual-Use Potential

Precision surface operations for terrestrial applications would share similar requirements for class definitions as would extra-terrestrial surface operations. This earth-analog would allow testing of ideas and software more fully than with just simulation.

Schedule & Milestones

Process satellite images for crop conditions						June 1998

Intelligent Spray Implement					July 1998

Demeter Tractor					August 1998

Interface precision mapping to implement & tractor					September 1998

Virtual farm developed with classes of agents					October 1998

Agents and capabilities define for field operations					October 1998

Precision towing of intelligent implement					June 1999

Selective Application using intelligent implement					August 1999

Selective application using satellite & local imagery					August 1999

Virtual farm running on Pentium/NT					September 1999

Command transfer when equipment fails					September 1999

Budget

FY1998

NASA		1000 K

Others		2150 K

FY1999

NASA		1000 K

Others		2500 K