# Technologies for Automating Continuous Mining Operations for Entry Development

Anthony J. Stentz

1.0	Objectives and Goals

Carnegie Mellon University is teaming with Joy Mining, Inc. and NASA to develop technologies for automating underground continuous mining equipment. The primary objectives are to increase machine productivity, reduce operational costs such as labor and maintenance, and increase overall safety of mining operations. In the program, the team will develop sensing, planning, control, and mechanism building blocks for automating a wide range of equipment. These building blocks will be integrated, tested, and validated on a Joy continuous miner resulting in a series of commercializable, fieldworthy products.

2.0	Justification, User Community Requirements, and Dual Use

The proposed program will develop technologies important for the colonization and development of planetary habitats. Autonomous underground mining and material handling would allow human operators to oversee the operations from the planetary surface or even from Earth, thus removing humans from the work face where forceful terrain engagement can pose a safety hazard.

3.0	Technical Approach

The technical approach is to develop a stream of mining products that incrementally automates the functions of the miner. Product A will be safeguarded control. The miner will be equipped with a scanning range device to detect obstacles in the vicinity of the machine. The system vetoes any commands issued by a remote operator that would result in a collision or unsafe condition. Product B will be configuration control. Rather than actuating the machine via remote lever commands, the operator specifies higher-level directives based on local mine geometry, such as cut material at a given heading or tram down a particular open corridor. Product C will be task control. The operator remotely specifies parameters to configure specific mining tasks, and then the miner executes the task automatically. For example, cutting a cross cut or a pair of box/slab cuts are two tasks performed by mining machines. Product D will be site control. The remote operator specifies a volume of material to be extracted, and the system plans the operations required to extract it. The operator can edit the plans and oversee the operation via a graphical depiction of material cut against the desired volume.

Each product in the series calls for less frequent human interaction at increasingly higher levels. With the first product, the operator interacts with the machine via the same controls that are found on the machine, but remotely located. With the last product, the operator issues high-level mining directives and observes the results via a three-dimensional display. The products layer on each other in the sense that each new product can fully utilize the capabilities of all products before it.

The problem of autonomous underground mining requires several technical advances. For example, the location of the machine makes GPS positioning impossible. Inertial solutions can help but drift over time. Our proposed solution is to use an on-board rangefinder to determine the machine’s position with respect to local mine geometry. These local mine maps can be stitched together to build a map of the entire mine as it is developed for site-level reasoning. The range data can also be used to detect obstacles and rendezvous with other mining machines. Hardening this scanner to adverse environmental conditions is another difficult problem. The machine vibrates severely while cutting material. Dust that is produced during cutting can obscure the scanner’s field of view. We propose to use a combination of mechanical protection and electronic processing to safeguard the sensor and eliminate noise due to dust. Planning machine motion at the site level is yet another problem, since the machines operate in highly constrained environments, in close proximity to one another, and dragging tethers for power. We propose to follow normal procedure for coordinating the machines, check the plans for interference, and request operator invention (via plan editing) when a difficult problem is encountered that cannot be resolved through simple plan variations.

4.0	Advances with Respect to State-of-Art

The state of the art is that CAD models of enclosed areas have been constructed from scanner data assuming the position of the scanner for each image is known. Two-dimensional maps of mines have been constructed without scanner location knowledge--we propose to extend this approach to three dimensions. With a three-dimensional model, we can register the miner’s position at any time by taking a new scan.

Algorithms exist for detecting obstacles and recognizing objects in rangefinder images. The challenge for the proposed program is to detect/recognize them quickly, reliably, and accurately in order to perform mining operations in real time.

Scanning rangefinders available today are mostly laboratory prototypes--few have found their way into actual application. To our knowledge, no scanner is rugged enough to withstand vibration, dust, dirt, and water, and still be good enough to reject these effects in their data.

5.0	FY98 Deliverables

The following core technologies will developed and delivered as part of the NASA-funded FY98 activity:

•	Prototype two-axis scanning laser rangefinder for use in underground mines.

•	Basic heavy equipment simulator and visualization tool.

•	Obstacle detection software.

•	Heading tracking software for cutting operations.

•	Corridor navigation software including turning capability.

6.0	Facilities and Partners

Joy Mining, Inc. is the largest manufacturer of underground continuous mining equipment in the United States. Headquartered near Pittsburgh, Joy Mining is a short drive from Carnegie Mellon. Joy offers matching dollars to adapt the proposed core technologies to mining-specific applications. The company has extensive experience in mining equipment design, development, testing, and field support. Joy develops electronic and software controls for their machines, and is very knowledgeable in mining procedures and component ruggedization.

Carnegie Mellon University has extensive experience in developing fieldworthy, mobile robotic systems. In the area of underground mining, CMU developed two-dimensional mapping and position registration software for mining operations and task planning software for producing mine plans. CMU successfully demonstrated some of this technology on a Joy mining machine in a West Virginia coal mine. In the area of ruggedized sensing, CMU has experience with the technologies needed for dust penetrating range sensing as part of an automated excavation program.

We expect the NASA Centers to contribute heavily to this program. Two primary areas we have identified for contribution are:

•	Operator interface: method for operator to issue higher-level commands to the miner and the representation of three-dimensional mine and machine data to the operator.

•	Inertial sensing: rugged, inexpensive heading and roll/pitch sensors to assist in map building via Kalman-filter integration.

The team will be sited at the National Robotics Engineering Consortium. Joy Mining has already emplaced personnel and a continuous mining machine on site, and we expect the NASA Centers to site personnel on the premises.

7.0	Schedule and Milestones

The schedule and milestones for the full three-year program are given below. Each block in the time line represents a quarter. Each product will be demonstrated on a Joy continuous mining machine (i.e., CM-12) cutting coalcrete at the National Robotics Engineering Consortium.

| NASA Fiscal Year | 97 | 97 | 98 | 98 | 98 | 98 | 99 | 99 | 99 | 99 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Item | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 |
| Upgrade OCU for safeguarding interaction (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop on-board computing box (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop explosion-proof container for computing (Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop reconfig sensor mounting hardware (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop two-axis surrogate laser rangefinder (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop hardened enclosure for rangefinder (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop basic simulator capability (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to detect obstacles in range data (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Integrate Product A in sim and test on CM-12 (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop mine-hardened laser rangefinder |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software for tracking a heading (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software for following a corridor (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to execute turns (Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Integrate Product B in sim and test on CM-12 (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Study and evaluate existing CID sensors (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to automate sump/shear cycle (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to automate box/slab cutting (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to automate cutting a crosscut (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to recognize haulage system (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Integrate Product C in sim and test on CM-12 (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to merge multiple scans into map (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to register miner to map data (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop site control unit (SCU) w/ graphical display (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to load and display mine model (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to load and display mine plan (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to plan mining ops from mine plan (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to allow supervisor to edit ops (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Develop software to overlay mining status on plan (CMU) |  |  |  |  |  |  |  |  |  |  |  |  |
| Integrate Product D in sim and test on CM-12 (CMU/Joy) |  |  |  |  |  |  |  |  |  |  |  |  |

8.0	Full Program Budget

8.1	FY97

•	Joy Cash and Inkind: $1000K

•	NASA Cash and Inkind: $750K

8.2	FY98

•	Joy Cash and Inkind: $1400K

•	NASA Cash and Inkind: $1000K

8.3	FY99

•	Joy Cash and Inkind: $1400K

•	NASA Cash and Inkind: $850K

8.4	FY00

•	Joy Cash and Inkind: $500K

•	NASA Cash and Inkind: $0K