Lightweight Survivable Rover (LSR)

Jet Propulsion Laboratory

California Institute of Technology

Pasadena, California

Paul S. Schenker, Ph. D. (PI)

schenker@telerobotics.jpl.nasa.gov

(818) 354-2681 (voice)

(818) 393-5011 (FAX)

Objectives

The objectives of this task are to develop, demonstrate and quantify an enabling base of technology for lightweight, low volume, strong, agile, and thermally robust rover architectures.    This will allow better use of launch resources in future missions, optimize rover science payload relative to mobility mass, and expand mission science site planning options. We demonstrate progress by conceiving, designing, fabricating and operating in simulated Mars environments new vehicles that address needed functions of the Mars Surveyor Program (MSP). As seen below, FY96 start-up work of the task developed “Lightweight Survivable Rover - 1 (LSR-1),” a general prototype for highly mobile (VL1/2) rover science. Per FY96-end report to the TRIWG (Oct. 16-18, 1996, TRIWG Proceedings, JPL, P. Schenker, and accompanying JPL videotape AVC-97-005), we demonstrated the vehicle in a MPF/Sojourner-like autonomous sortie for “find-rock” wherein an on-board multi-spectral imager acquired mineralogically and geologically relevant data. The recent findings of fossil rock evidence for early life on Mars have heightened interest in a sample return, with Code S taking earliest initiative to implement the required mobile science sample selection & acquisition and sample cache return functions. Another Telerobotics Program task, Long Range Science Rover (S. Hayati) addresses the mobile science function; the LSR task addresses the sample return, and provides flight-relevant mechanical and materials technologies to both.

Photo: LSR-1 (left): a 7 kilogram (Kg) rover R&D prototype having 20 centimeter (cm) diameter wheels, 97 cm length, 70 cm width, and 29 cm ground clearance (note: LSR-1 is shown prior to bonding of composite helical growsers to wheel rims, and the picture inset shows the wheel in stowable configuration). Sojourner (right): the recently launched 11+ Kg '96 Mars Pathfinder microrover flight experiment (13 cm wheels, 63 cm length, 45 cm width, and 15 cm ground clearance). LSR-1, which incorporates significant advances in composite and thermal materials, as well as a new integrated thermal-structural chassis, is conceived to operate over larger obstacles (~.4 meters), broader thermal latitudes (equatorial to near polar regions), longer distances (multiple kilometers), and extended duration (multiple months). LSR-1 introduces a novel spot pushbroom sensor, enabling the rapid detection-and-avoidance of hazards in variably featured terrain, while using less power and computation. A central element of LSR design is "Volume Efficient Mobility," wherein wheels and mobility running gear collapse for transport, thus allowing rover stowage during flight to as little as 25% operational field volume. The LSR mobility design scales to larger/heavier vehicles, and a related mobility platform is being developed by the LSR task for use in a next generation 40+ Kg LRSR mobile science platform.

To set a brief perspective for the proposal, we outline system functional objectives of LSR work for the planned task duration of FY95-99. See also the later listing of Level 1 yearly quantified milestone deliverables

FY96: develop an enabling technology concept for a low mass, highly stowable rover, expanding the range of thermal operations – innovate as needed in areas of mobility mechanization, structural and thermal materials, low-mass and computation sensing (completed 9/96, seen above, and later summarized in greater detail). Demonstrate in a concept vehicle bench-marked against Mars Pathfinder/Sojourner (MFEX) mass, volume, mobility, thermal isolation, etc.

FY97: develop a first concept Sample Retrieval Rover (SRR) for the planned MSP ’05 sample return mission, with the objectives of minimal mass, maximal cache payload, and short range operations as facilitated by precision landing. The mission model -- per early Code S/MESG/MEPO input -- is landing, fast cache retrieval from a single known location, and sample return in less than one day, i.e., no diurnal cycling. (NOTE: sample cache/handling tools in this and subsequent SRR systems are developed/provided by the TR Program “Planetary Dexterous Manipulators”    task.)

FY98: develop a more capable sample return vehicle, one that can search a local area for (possibly multiple) caches, carry a higher payload to mobility ratio (target, 50%), has thermal and power resources for multiple diurnal cycle operation, and as desired by users, carry piggyback science.    As one important example, show the that SRR vehicle could, as requested on contingency, use its assets to collect ad hoc samples/science (e.g., a “sample grab,” due aborted sample cache return or motivated by opportunity).

FY99: develop a light, agile SRR (3-5 kg class) in which the warm electronics enclosure is eliminated. The rover will run ambient electronics, reduce actuator m/v/p requirements, minimize lander support equipment, and continue to carry sample retrieval/handling mechanization.    As such, this system will offer practical opportunity to fly multiple capable rovers on a single landed mission.

In the above context, and further mission rationale given below, some of our specific technical objectives are:

- Advance 5-10 Kg-class rover capability from local operations (10-100 meters) to 10 kilometers or more over variable terrain (Mars nominal, VL1, VL2), carrying 20-50% science mass, at speeds of 1-10 cm/sec (applications dependent). At this scale or larger, demonstrate a viable thermal isolation/controls approach for equatorial to near polar sites (~+-70 degrees). Push the rover m/v/p design curve down to 3-5 Kg vehicles carrying sample/cache acquisition tools, initially for short range operations (about lander/ascent vehicle), and later for extended distance (local areal operations of cumulative Km’s)

- Develop a “volume-efficient” mobility concept (stowage to 25%) for VL1/2 terrainability that is consistent with MSP-class (‘01/’03) lander constraints, scales with science mass and rover size (10-50 Kg+), and benchmark with respect to existing rover designs (e.g., metrics of distance/time, distance/energy, obstacle-clearance/mass, etc.). Make self-deployable, and target reduction of conventional lander support mass (tie-downs, ramps, etc.) by 50%

- Through innovative use of new materials, structures, and related engineering processes, reduce mass of suspended, stressed and actuated vehicle components by up to 50% (target areas include 2-D/3-D composite frames, thermal control designs integral with same, solid state high output motors), and validate critical components in realistic environmental tests (stress, temperature cycling, etc.).

Justification

With a return to Mars for detailed study phases of MSP ’98, ’01, ’03, ‘05, NASA/Code S has undertaken an aggressive science program to investigate evolutionary climate, life, and resources. These studies will include frequent observational and physical interactions with the planetary surface. Among the instrumentation strategies considered to date by MarsSWG members are: macro and micro imaging, visual and infrared (near to thermal) spectroscopy, APX/Neutron/Mossbauer/NMR spectroscopy, capability to freshly expose rock/outcropping surfaces, as well as robotically place instruments against rocks or move rocks (for better viewing, access to under-surface areas that are less weathered, or possible sampling of frost formations). Longer term possibilities/interests include DSC/EGA, mass spectrometers, geophysical sounding, X-ray diffraction, rock-splitting, and multi-function manipulation. The Mars’98 effort is now underway (Mars Volatiles and Climate Surveyor, D. Paige/UCLA), preparing a near southern Polar lander mission, and utilizing a TR Program robotic sampling arm concept developed by this proposal’s PI and participating team members. Mars Surveyor ’01/’03 are planned as rover missions, in which landing ellipse and science diversity considerations motivate development of longer range, high payload science rover (10’s of kilometers, 10’s of kilograms).    The ‘01/’03 rover will go significant distances, needing to locate, identify, analyze in-situ, and potentially cache (and carry) samples of interest. The deployment of these science capabilities requires lower mass, stronger, more thermally robust rover structural-thermal architectures than now exist, or enabling technology supports. A sample return rover, to be used for the planned MSP ’05 (and later Mars, as well as possible related small body) mission, will necessarily be small, fast, and powerful, if shorter ranged and less articulated, so as maximize ascent vehicle mass/volume and minimize visitation. Indeed, the structural, thermal, actuation, and design requirements scale more severely to smaller rovers, where it is desired to minimize mass, maximize mobility, and optimize science payload ratio, while not losing significant baseline capability (computing, communications, navigation, power, and thermal). There is thus needed for both the mobile science acquisition and sample return vehicle functions a new enabling base of mechanical design, materials, and integrated mobility-thermal architectures. This task, Lightweight Survivable Rover (LSR), provides the technology. Our work demonstrates in FY97-98 a resulting Sample Retrieval Rover paradigm for MSP’05. Through development/delivery of a long range mobility platform (FY97) and    thermal chassis (FY98),    for end-FY98 integration as a new flight-relevant long range sampling rover (LRSR task), we also enable major progress toward the Mars’01/’03 mission objectives. Finally, with continuing innovations of later work in FY99, we open the door to much more productive multiple rover and low mass/volume launch access planetary science.

Approach

We implement our task with a R&D team drawn from JPL line organizations in mechanical systems engineering, mechanical science & technology development, thermal & materials engineering, and automation & control. The principal mechanical engineer (Sword) and materials & thermal structures engineer (Hickey) both have developed related Sojourner products; the mobility chassis designer (Bickler) innovated the six-wheel rocker-bogie in now common use and has under development new mass-saving approaches to actuation of multiple rover functions by a single motor; the task robotics specialists (Baumgartner, Matthies, Schenker, Wilcox) are experienced in NASA Code X and related ARPA technology developments.    The system engineer (Baumgartner) brings a rich background of applied robot and rover mechanical design experience, including structural and dynamic analysis. As noted above, the task PI, Schenker, led the robotics arm proposal into Mars Surveyor ’98/MVACS in coordination with Professor David A. Paige/UCLA, and Wilcox played a similar role in proposal of a lightweight rover for the Polaris/JPL-L. Soderblom (USGS) effort (cf. also, Nanorover/MUSES-CN) In related TR and MET Program tasks, Schenker/Baumgartner lead the development of rover manipulator technologies for sampling, cache retrieval, and their integration to the future LSR and LRSR based vehicle designs. Thus, we undertake the effort with a good skill base and representative experience in current Code X/Code S rover developments. With this team come a similar diversity of supporting facilities: 1)    MarsYard simulated VL1/2 and Mars nominal short range traverse; 2)    Bldg. 113 adjustable incline sand/mars simulant (Basalt) testing; 3) Bldg. 158 composites processing & thermal test facilities; 4) Bldg. 107 support electronics/sensor hardware development, and calibrated rover test-board (cf. Rocky series/Go-For); 5) and a new 2400 sq. ft. Planetary Robotics Laboratory scheduled for summer ’97 completion and enabling significant indoor work in cooperative rover-lander tasking.

The task is in its second year of activity, and we outline a plan that will conclude efforts in FY99, approximately in alignment with earliest projected MSP ‘03/’05 technology “freeze” points. We present the task sequentially, briefly recapping relevant prior FY96 outcomes, detailing our ongoing FY97 work as motivation for further advances, next describing the proposed FY98 main thrusts, and briefly summarizing further directions for FY99 developments. In the later section, “Deliverables,” we list corresponding yearly Level 1 Milestones, FY95-99. We break our attack on enabling task technology into mobility design & mechanization, thermal & structural design, low mass computing,    and perception & navigation, and provide supporting development rationales. There is considerable cross leverage with other Telerobotics Program tasks: 1) LSR ‘97/’98 mobility/chassis deliverables to LRSR; 2) Nanorover/LSR cooperating on low-mass, ambient computing development; and 3) the navigation work being led jointly for LSR/LRSR by L. Matthies with common software development/use; 4) co-development of low m/v/p rover sampling/caching manipulators in the JPL TR/MET program tasks; 5) and integration of related MET-funded work on planetary protection (Sample Containment & Quarantine task, J. Barengoltz) into rover/manipulator mission paradigm concepts.    As we later note, our cross disciplinary JPL team has initiated a number of productive interactions with the Mars science community (universities and JPL), NASA TR Program university grantee institutions, and visiting scholars.

FY96 work initiated the LSR task and yielded enabling technology developments, design, fabrication, integration, and first science-based demonstration of the LSR-1 rover architecture. Work in the area of 2D and 3D composites (incl. a new enabling machinable 3D composite process from which major LSR-1 strut joints and wheel hubs were fabricated, and design of the collapsible wheel rims realized) was leveraged from the TR Program Planetary Dexterous Manipulators (PDM) task. As further shown below, the PDM task is a source of compatible lightweight sampling manipulators for LSR (and future LSR-derived mechanizations of the Rocky series long range science rovers)

- Developed and demonstrated a novel rover mobility platform having a collapsible rover wheel of 20 cm dia. (260 g composite/Al structure) stowable to 30% its operational volume, all-composite running gear (2D tubes, and new 3D process joints & pivots); provides a basis for next generation low-mass, high payload sample retrieval and long range science vehicle LSR designs

- Developed and demonstrated a mass-saving approach to rover integrated thermal-structural design wherein the Warm Electronics Enclosure (WEE) serves dual functions of thermal control and load-bearing chassis. Improved thermal isolation (via a new opacified aerogel) and stabilization (by new Phase Change Materials) respectively reducing WEE thermal losses up to 30% and temperature variations by 50% (-40-to-0 °C range) compared to Sojourner

- Developed and demonstrated a fast rover obstacle avoidance scheme based in diffracted, highly collimated multi-spot projection from miniature lasers. CCD imaging of the geometrically calibrated, time sequenced spot array -- a moving "pushbroom" -- enables temporal 3D reconstruction of obstacles and near terrain. Obtained 97-98% positive spot detection with no false alarms, operating in direct daylight conditions over variable surfaces without difference imaging.

FY97 work develops, implements and demonstrates our baseline concept for a Mars sample return vehicle, which we call Sample Retrieval Rover (SRR). This vehicle, depicted in early CAD configuration below, consists of a volume efficient, collapsible structure which includes a hinged running gear and collapsible 20 cm wheels (drawings are to scale) wheels. SRR is configured as a 4-wheel, skid-steering vehicle and includes a three degree-of-freedom robot arm (MicroArmII, cf. TRIWG98 PDM task proposal) that is utilized to collect the sample cache from a science rover/store.

<= Left: deployed SRR (posable hinge, low posture)

=> Right: SRR in stowed configuration

The computer board set for SRR prototyping is based on the PC104 architecture which is centered around a 486 processor. The PC104 architecture also provides off-the-shelf solutions to motion control, video frame-grabbing, and sensor data sampling. Communication with the rover is handled by an ethernet RF modem link. The software used to control the rover is based on the December, 1996 Rocky-7 code with appropriate modifications to conduct SRR-specific functions. The navigation and localization of the SRR is based on various sensory inputs including a pair of stereo cameras, a goal-recognition camera, a sun sensing camera, a set of three single-axis accelerometers, gyroscope, and wheel encoders. The gross localization and navigation of the rover is carried out through the sun sensor, accelerometers, gyroscopes and wheel encoders. Local navigation and reactive behavior is conducted via the forward-looking stereo camera pair for obstacle avoidance function. Finally, the goal-recognition camera is used to identify the science rover from a given distance and to identify the location of the sample cache on the science rover.

The operation of the rover during the sample cache retrieval event is broken down into the following phases:

(I)	Long Range Traverse (greater than 20 meters from science rover)

(II) 	Guidance Traverse (2-20 meters from science rover)

(III)	Sample Cache Localization Maneuver (0.5-2 meters from science rover)

(IV) 	Sample Cache Retrieval and Storage (<0.5 meters from sample cache)

(V) 	Return to Ascent Vehicle

In Phase I, the rover travels from the ascent lander to within 20 meters of the science rover based on dead-reckoned localization. It is assumed that the relative heading from the ascent vehicle to the rover is known from a science rover beacon. Phase II begins when SRR visually locates the science rover via the goal recognition camera mounted on the semi-deployed MicroArm-II. This visual localization provides an updated relative heading from SRR to the science rover. The SRR then navigates along this updated heading until it is within 2 meters from the science rover.

In Phase III, the location of the sample cache on the science rover is determined again using visual means by detecting visual cues mounted on the science rover. Once the location of the sample cache on the science rover is determined, a rover alignment maneuver is conducted to bring the rover to the correct approach trajectory to the sample cache. The rover then maneuvers along the approach trajectory and parks within 0.5 meters of the sample cache container. Then, in Phase IV, MicroArm-II is commanded to retrieve the sample cache where the location of the sample cache relative to the rover is computed via the stereo-pair navigation cameras. Phase IV concludes with the stowage of the sample cache container on SRR. Finally, the rover returns to the ascent vehicle in Phase V. The location of the ascent vehicle relative to SRR is known approximately through the stored state information. The traverse to the ascent vehicle is similar to Phase I and Phase II where the long-range navigation code is used to bring the rover within a range in which the goal recognition camera visually acquires the ascent vehicle. The terminal guidance traverse then brings the rover into close proximity with the ascent vehicle. Progress allowing, we will incorporate simple bearing-only beacon guidance for the early Phases.

Mobility Design & Mechanization (FY97 - forward)

From a mobility concepts and mechanization perspective, four FY97 LSR task design objectives have been identified: volume efficiency, low mass, variable stance, and evolution of LSR-1 running gear for 40-50 Kg class rovers. The enabling technology development associated with LSR volume efficiency is an innovative self deploying hinge. This hinge, combined with the VEDRW (Volume Efficient Deployable Rigid Wheel ) developed by this task in FY96    will allow stowage of the SRR in less than 25% of its deployed volume. The main body of the hinge will be fabricated from the new 3-D composite material (cf. TRIWG FY98 PDM task proposal, “high-density 3D composite” development) and will be designed with sufficient stored energy to deploy even in Earth gravity. The design objective of low mass will be accomplished through the extensive use of 2-D and 3-D composites, implementation of Vespel bushings (rather than the traditional all steel ball bearing sets) at most rotating interfaces, designing with an endoskeleton type structure (as opposed to the exoskeleton used on LSR-1), and utilizing a four-wheeled architecture    instead of the JPL standard six-wheeled rocker-bogey design.    The FY 97 SRR will introduce an ability to vary the angle between the wheel struts. This functionality is primarily included to enable volume efficient stowage, however, the ability to adaptively pose itself (vary geometry of ground clearance, wheel-base, center of gravity) has potentially huge benefits for an SRR and possibly for rovers in general. In order to deliver this functionality, an actuator similar in design to that used for the joints on the SRR sample cache pick-up arm(cf. TRIWG FY98 PDM task proposal, “MicroArmII” development) will be imbedded in each of the shoulder joints. These actuators will be connected together, through the chassis, with sets of bevel gears that will become the internal differential controlling chassis tilt. An ability to change the angle between wheel struts by a total of 180 degrees will allow SRR to achieve a ground clearance of    up to three wheel radii, and at the other extreme maximize stowage volume. At the intermediate position of one wheel radius of ground clearance, SRR is expected to be sufficiently stable to pick up the target cache at full extension of the arm.

Running gear derived from LSR-1 technologies will be designed and built for compatibility with 40 to 50 Kg class rovers and will interface with the existing FY96-97 LRSR metal chassis. This mobility gear will be adaptable to a next generation larger science volume chassis (for end-FY98 integration of a new LRSR mechanical prototype). All components of the VEDRW have been analyzed, and where needed, redesigned to handle the five-fold increase in rover mass. The cleat design has been modified to account for the mass increase and change in steering approach. The running gear structure (rockers, bogies, and steering struts) will be fabricated from a combination of 2-D and 3-D composites with rotating interfaces supported by Vespel bushings. Drive and steering actuators will be sized to provide reasonable margins at a total vehicle mass of 50 Kg.

The primary objective for FY98 is to develop and demonstrate a volume-efficient, high strength, thermally robust SRR mobility platform capable of supporting science and/or engineering mass equal to its own mass. Utilizing many components developed for LSR, LRSR, and SRR,    we will develop a six-wheeled rocker-bogie running gear and chassis combination    to stow in a minimum volume and yet deploy to a vehicle one meter in length with 20 cm diameter wheels. The entire vehicle, including electronics, navigation hardware, power, telecomm, thermal, and a four d.o.f. arm, will be 10 to 12 Kg and be capable of supporting its own mass in additional engineering and science. The rover will be designed to operate for multiple kilometers and days in mid to high latitude Martian environments, as consistent with the FY98 Level 1 milestone for extended search of    local areas for caches. Component level development will focus on life testing of actuators , bushing/bearing wear tests, CTE effects on 3-D / 2-D bonded joints.

FY99 development will refine the above FY97/98 mobility design concepts, leveraging additional volume stowage compression as may be derived from elimination of the WEB and concomitant use of ambient electronics (below). This LSR generation will be targeted as a potentially smaller, self deploying, volume-efficient system needing little or no lander-mounted equipment to safely reach the Martian surface (for a small rover, realizing a sizable further fractional mass/volume savings!). Other issues we will investigate include ability to sever an actuator output shaft coupling in event of seizure (allow that wheel to "free-wheel"), dust removal from solar panels, expandable/retractable solar panels for peak power needs,    sensor-based techniques for the self-detection and optimal adaptation of vehicular pose, and self righting capability (cf. ongoing work for smaller, simpler Nanorover architectures).

Thermal & Structural Materials

Future lightweight rovers for Mars have serious constraints on mass, volume and power. Our FY96 activity focused on developing -- and improving the performance of – a very light integrated rover thermal structure (a subsystem chassis acting as both thermal protection and mechanical support). This led to a LSR-1 WEB design having nearly twice the science volume as Sojourner’s, at comparable mass, and demonstrating 10-30% lower thermal losses in Mars relevant test environments (cf. FY96-end TRIWG report for details). An enabling product was development of a new thermally isolating opacified aerogel; we also incorporated a thermally capacitive Phase Change Material developed in cooperation with the MET program (cf. JPL/R. Manvi) for diurnal cycle temperature stabilization.    As mission scenarios evolve with separate designs for science and sample return, each class of supporting vehicle will have different needs for its structural and electronics chassis. There is also general need to capitalize on the development of low temperature electronics that do not require separate thermal control – a mass and complexity savings. There will be continued need for thermal control for battery systems, sensitive electronics, and science packages, but the requirements will be minimized.    For rovers with a surface duration lifetime of less than a day, power may be more efficiently provided by high density primary batteries at a lower mass. With the absence of a solar array, the rover system architecture changes significantly.    This allows innovative chassis designs that leads towards a “WEB-less”    configuration. As conceived, a WEB-less architecture will be an optimized structural backbone with a shell to support the electronics, power and other subsystems. Structural rigidity    will only be required at load bearing interfaces for the mobility system and the deployable    camera mast/microarm. By utilizing lightweight, high stiffness 2D and 3D composites, mass can be minimized. This WEB-less concept can be extended for both small rovers to larger science rovers, and indeed is already the spirit of    the FY97 SRR vehicle mission paradigm, given the MEP/MESG developmental starting point of one-day, one-sortie, short range, sunlit operations. The primary materials-structural objective for FY97 is to develop a novel integrated rover structure for a collapsible four wheel system architecture that includes a multifunctional manipulator and navigation arm.    Specific objectives for FY 97 are:

- Design and implement a composite structural/mechanical design for the SRR chassis that minimizes mass by providing primary structural rigidity and strength only at (endoskeletal) interfaces for the mobility system and cache retrieval MicroArm, with a simple shell structure to support electronics and power subsystems

- Design such attachment interfaces using novel 3D graphite/polycycanate composite materials, including integration of    the dual function deployable Microarm/Camera Mast (per earlier, the arm carries a camera for cache goal recognition)

- Develop and evaluate new composite materials for deployable wheels that improve durability and enhance mobility under extended load/use, and apply same to the LRSR task evolved Rocky vehicle

The main objectives for FY98 are to combine the integrated thermal structure concept of FY96 with the WEB-less architecture advances ongoing in FY97. This will lead to a FY98/99 system utilizing ambient temperature electronics and provide distributed thermal controls for temperature-sensitive electronics, batteries, and science packages. This will be a compartmentalized architecture minimizing mass and volume.    Specific objectives for FY98 are:

- Definition, design, fabrication and preliminary experimental tests of the above distributed thermal controls

- Design and implement a LSR-1 type integrated thermal/structural chassis for a LRSR-class rover, and delivery of    same for FY98-end integration with LSR-derived FY97 mobility gear (see above)

- Develop finite element models (FEM) to characterize thermal performance of the above systems and conduct testing in a simulated Mars environment, thereby validating the models and new design concepts

Technology design and simulation of the LRSR WEB will be performed by this task; fabrication and test costs for the LRSR WEB (as a model targeted to Mars’01) are estimated at $100K, and this is an    item not previously planned.

Low-Mass Computing

A major limitation in demonstrating the operational feasibility of advanced structures and mechanisms to traverse a significant distance, localize/acquire a sample cache, and return it in a short period of time is need for an on-board computer which matches the mass, power, I/O, and speed capabilities of the expected flight system. In the first year of this task an engineering model (“EM brain”) of the Sojourner flight electronics was used to provide all computing and power functions for LSR-1, where the mass of these electronics was about 1.5 Kg. The computing and software environment of Sojourner is not sufficiently capable to enable the functions required of the SRR, some further described below. As a result, in FY97, a commercially-available PC-104 computing environment has been adopted to give the necessary infrastructure to support the navigation software developed under the Long Range Science Rover task.    This software provides the autonomous navigation capability necessary for extended traverses to specific targets.

In FY '98 it is planned to develop, in parallel with the use of the commercial computer used in '97, a new electronics package which will drastically reduce the mass devoted to computing and power functions.    This electronics will be derived from the technology developed under the Nanorover Technology program, which has demonstrated a flight-like

electronics package in FY97 with a mass of order 100 grams. The central processor in this electronics package will be upgraded from an 8-bit computer (an 8051) to a 32-bit computer (e.g. an R3000). Standard I/O modules developed under the Nanorover program (e.g. camera interfaces, PWM motor controllers, etc.) will be replicated as needed for the SRR hardware needs, and a new printed circuit board will be produced. The resulting technology milestone is to demonstrate and deliver an electronics package which will provide all the necessary computing, I/O, and power management functions needed for the SRR in FY99.    It will be compatible with the software development environment used in LRSR and in the SRR FY97/98 commercial computer (e.g. VxWorks/ControlShell), and composed of flight-like components packaged in a way which is straightforward to qualify for future missions. The cost of this activity in FY '98 is $250K, and this is an item not previously planned.

Perception & Navigation

The objective of this work is to develop the perception & navigation capabilities required for position estimation, sample canister manipulation, and obstacle avoidance in a rover sample retrieval mission covering several kilometers between the lander and the sample cache, in terrain characteristic of the Viking 2 landing site. We provide here a technology plan that looks to FY2000+    -- consistent with the PBS 4.1.1 “Planetary Telerobotics Technology” guidelines -- and meets the LSR mission paradigm Level 1 task delivery needs for FY97-99. We discuss the plan in some depth, reflecting a careful coordination established with the TR LRSR task, and synergistic involvement the co-investigator L. Matthies has related MET Precision Landing task (JPL/D. Farless).

The perception and navigation capabilities required for such a mission break down into three major categories:

- Position estimation of the rover relative to the sample cache and the lander.

- Terminal guidance for maneuvering the rover near the sample cache and picking up the sample canister, as well as for maneuvering the rover near the lander and dropping off the sample canister on the return trip.

- Obstacle avoidance en route from the lander to the cache and back.

Position estimation and terminal guidance have significant requirements that are unique to the sample retrieval mission and distinct from the requirements for a long range rover sample acquisition mission. On the other hand, the requirements for obstacle avoidance have more common with those for a long range sample acquisition rover.    Activity in FY97 is developing rudimentary capabilities for all three items in order to demonstrate a basic level of functionality in a fully integrated sample retrieval rover system. Proposed activity in FY98 through FY00 will conduct more in-depth development in each of the above three areas in turn; this will leverage work on obstacle avoidance done in the interim by the Long Range Science Rover (LRSR) task.    The result will be a rigorously evaluated perception and navigation capability that enables demonstration of a multi-kilometer sample retrieval mission by the end of FY00, in terrain characteristic of the Viking 2 landing site.

The FY97 level 1 milestone for this year is to traverse 100 meters of mild terrain and pick up a sample cache from a "dead" long range rover. This requires a modest obstacle avoidance capability plus abilities to home in on the dead rover from a 100 meter distance, find the sample canister on the dead rover, and pick up the canister. For obstacle detection, we are porting algorithms for stereo vision, obstacle detection, and reactive obstacle avoidance from Rocky 7's 68060/VME computing environment to a smaller, 486/PC104 computing environment.    This is a cost-effective solution that enables us to leverage results from the LRSR task while focusing new effort on other sub-problems that are distinct to the sample retrieval mission.

For homing in on the dead rover (i. e. position estimation), we are developing a bearing-only radio navigation aid which will use a small, low-power transmitter on the dead rover and a receiver on the sample retrieval rover to provide an estimate of the direction to the cache. In general, knowledge of the location of the cache relative to the sample retrieval lander will be poor and line-of-sight contact between the lander and the cache will be unavailable.    These circumstances dictate the use of a radio homing beacon on the cache.    For FY97, we will rely on the bearing-only beacon to get the rover within a few meters of the cache, then use visual terminal guidance to accomplish the rest of the mission.

Finding the sample canister on the dead rover and picking it up (both lumped under "terminal guidance") will be achieved by putting visually distinctive markings on the dead rover and on the sample canister itself.    These markings will enable the sample retrieval rover to easily recognize the dead rover, drive around to the side of the dead rover known to hold the sample canister, and plan the motion of a manipulator to grasp the canister.    For F'97 we will assume that the dead rover came to rest in flat, obstacle-free terrain.

As described above, finding the cache involves following a radio homing beacon until the dead rover is within visual range, then using visual terminal guidance to find the sample canister and grasp it. This process will be much more efficient if the radio beacon provides an estimate of the range to the cache, in addition to the bearing, because otherwise the visual recognition routines must be operating for the entire traverse.    Therefore, work in FY98 will survey methods to extend the radio beacon to give bearing plus range.    The simplest approach appears to be to use signal strength as a coarse range estimate; other methods will also be explored, such as deploying a second receiver from the lander to enable triangulation.    The maximum working range of the system will be extended to greater than one kilometer.    The beacon system will also be extended to support return to the lander, for example by using a second transmitter on a different frequency on the lander.

Although radio homing beacons suffice in principle to solve the sample retrieval problem, efficient navigation of the rover also requires accurate short-term position estimation for such things as building a local obstacle map.    However, the position and heading estimation systems of Mars rovers developed to date perform poorly. Therefore, work in FY98 will also begin to develop rigorous error models for the other navigation sensors (gyro, sun sensor, wheel encoders, and inclinometers), a Kalman filter for position and heading estimation, and a visual egomotion sensor that enables detection of wheel slip.    Overall performance of the position and heading estimation system will be evaluated by extensive experiments compared against GPS-derived ground truth.    This work will provide the first high-quality, well-characterized Mars rover position and heading estimation system.

The rudimentary terminal guidance capability developed in FY97 will assume that the cache lies in flat, obstacle-free terrain.    In general, the long range rover may come to rest in uneven terrain with nearby obstacles.    Clearly, a sample retrieval rover must be able to succeed in such conditions.    Although much work has been done on the separate pieces of this problem, an efficient, integrated system does not exist for the combined problem of mobility and manipulation in the presence of obstacles.    Therefore, work in FY99 will develop perception algorithms to map the terrain around the dead rover and motion planning algorithms to generate trajectories to the sample canister.    A similar problem exists on the return to the lander, when the rover must hand over the canister.    Work in FY99 will enable both the cache pick-up and the cache drop-off to succeed in uneven terrain with obstacles.    The ability to work in terrain characteristic of the Viking 2 landing site is a logical target for this effort.

Up to this point, we will work with obstacle detection and avoidance algorithms transferred from the LRSR task.    At present, these are very simple, behavioral algorithms appropriate only for terrain with relatively large, sparse obstacles.    To complete a realistic rover system for sample retrieval, it is essential that the obstacle avoidance subsystem be capable of fast navigation through more challenging terrain.    Therefore, work in FY'00 and beyond will complete the system by developing a fast perception system to detect untraversable terrain, local mapping algorithms to remember recent obstacles, and fast path planning algorithms to chart an efficient path through terrain up to the Viking 2 level of difficulty.    This work will leverage progress on obstacle avoidance up to that time in the LRSR task, as well as past work in our own LSR task on fast obstacle detection (Matthies et al., IEEE ICRA’97 to appear).    It will then extend this work by using maps constructed on the trip from the lander to the cache to plan more efficient traverses from the cache back to the lander.    This will increase the probability of mission success by reducing the overall duration of the mission.

In conclusion of this section we note that LSR Perception & Navigation work will contribute several technologies that are critical to a sample retrieval mission, including homing to the cache and back to the lander, precise short-range position estimation for mapping en route, terminal guidance for approaching and grasping a sample canister in rough terrain, and efficient return to the lander using map knowledge gained on the way to the cache. This will culminate in a demonstration of the first Mars rover system capable of executing an entire sample retrieval mission in realistic terrain.    By achieving this prior to 2001, we will enable such a robust mission to be flown by 2005.

Deliverables

FY96 Level 1: 	Develop, demonstrate, and evaluate for local area science sorties (10 meter dead-reckoning, MarsYard) in various VL1/2 class terrain, a new volume-efficient rover mobility design – 20 cm wheel diameter, the collapsible wheel stowing to ~30% active volume – capable of traverses at 1-to-3 cm/sec under visual hazard detection/guidance by a novel spot-pushbroom hybrid laser/CCD-stereo sensor. Statistically characterize navigational system performance relative to the MFEX baseline [Sep 96]

FY97 Level 1: 	Design, implement and demonstrate a Sample Retrieval Rover weighing less than 8 kilograms (cache and cache handling mechanization < 3 Kg) in simulated near-field operations in benign terrain (10-100 meters) about an ascent vehicle; this assumes bearing-guided navigation, cache recognition & localization, and robotic sample container pick-up    [Sep 97]

FY98 Level 1: 	Demonstrate an end-to-end "find-cache" sample return sequence (10-12 Kg class rover, broad area/terrain operations, fully collapsible vehicle, capable of 50% payload mass) -- surveying a nominal 100x100 square meters area, navigating under bearing-and-range guidance, and localizing a sample cache (s) from a previous missions,    and returning/transferring this package to a simulated ascent vehicle [Sep 98]

FY99 Level 1:    	Develop and environmentally test a low-mass/volume, thermally survivable, collapsible rover having flight-relevant ambient electronics and “WEB-less (distributed thermal control) architecture.    Target this LSR design at 3-5 Kg and maximum terrainability. Also, demonstrate sample return and sample grab functions (opportunistic science) in non-benign terrain, addressing required local coordination of mobility and manipulation [Sep 99]

Resource Requirements

Year		FY96		FY97		FY98		FY99

Budget 		800(800)	940 (1000)	1200 (800)	1000 (800)

Wkyrs		3.9		3.6		5.0		3.6

*    Rationale for budget augmentation request: LSR has taken on FY98/99 development of an enabling computing architecture and beaconing system to realize the MSP ’05 low-mass/volume sample return function requested by MEPo users and the TAP Office effective FY97.    Per above, the estimated cost for SRR computing (looked at with some care by B. Wilcox) is $250K, and for simulated SRR beacons is $50K. Further, JPL/TAP-MEPO-SESP advocates LSR mobility-thermal-materials technology as the evolution path for larger science rovers, and the LSR task is now asked to provide same into a reference design for Mars’01 (FY97 development of LRSR mobility gear for LRSR within plan, and FY98 development of an integrated LRSR structural-thermal chassis at unplanned $100K cost). TOTAL: +$400K

Facilities, Partnerships and Additional Participants

Facilities:     summary provided page 3 bottom in introduction to “Approach”

Participants (JPL): Mechanical Systems Engineering & Research Division; Avionics Systems & Technology Division

Science:    collaborations ongoing or to date with Mars’98/UCLA (Prof. David A. Paige, PI for Mars’98/MVACS science payload; Mars ’01/Rover Design Team (cf. Lynn Lowry,/J. Plescia for sampling science requirements & architecture definition); JPL In Situ Exploration and Sample Return Center (J. Cutts)

Grants/Contracts: MIT/Mech. Engrg. -- S. Dubowsky (task-level mobility planning); MIT/AI-Lab & Mech. Engrg. -- J. K. Salisbury (vision and force-based perception & control of rover sample acquisition/handling); USC/Elec. & Cptr. Engrg. – G. A. Bekey (sensor fusion for rover perception & navigation)

Dual Use Potential

Collapsible mobility, lightweight composite design/materials, and sensing technologies developed in this task are basis for our ongoing proposal of a significant military field robotics effort.    We have filed related NPO reports.

Schedule (Level 1 Milestones in Bold)

Collapsible composite/Al wheel								FY96, Q2

Integrated thermal-structural WEB								FY96, Q3

Spot-pushbroom hazard sensor								FY96, Q3

Lightweight Survivable Rover-1 (LSR-1)							FY96, Q4

Four wheel SRR-1 running gear w/ articulated struts						FY97, Q3

Cache localization/recognition in benign terrain (bearing only)					FY97, Q3

LRSR’98 system lightweight mobility platform development 						FY97, Q3

Sample Retrieval Rover-1 (SRR-1)								FY97, Q4

Six wheel collapsible SRR mobility for 50% payload						FY98, Q3

Cache area navigation by beacon bearing-range & Kalman update					FY98, Q3

Distributed thermal controls and supporting FEM 							FY98, Q3

Ambient electronics referenced to low-mass SRR-2/3 design						FY98, Q3

SRR-2 wide area (100x100 m^2) search and return						FY98, Q4

LSR endokeletal frame/compartmentalized thermal control						FY99, Q3

Cache acquisition/cooperative mobility-manipulation in non-benign terrain				FY99, Q3

SRR cache acquisition and opportunistic sampling in adverse terrain				FY99, Q4

References

See P. Schenker, end-FY96 task report to TRIWG, Oct. 16-18, 1996, JPL/Caltech, Pasadena, CA and FY97 proposal.