02/19/97        13:25:5410:47 AM

TELEROBOTICS CONTINUATION PROPOSAL FOR FY98

Autonomous Mobility, Navigation, and Control for Venus Aerobots

Dr. Jonathan M. Cameron	Dr. J. Balaram

Jet Propulsion Laboratory	Jet Propulsion Laboratory

Phone:	818-354-1189		Phone:	818-354-6770

FAX:	818-393-4440		FAX:	818-393-5007

Email:	Jonathan.M.Cameron@JPL.NASA.Gov		Email:	J.Balaram@JPL.NASA.Gov

Figure 1:    Aerobot at Venus

Abstract

Balloons systems which operate under robotic control, called aerobots, offer rich potential for exploration of the planets.    This task will develop position sensing techniques, motion control algorithms, and path planning methods for a Venus-applicable aerobot.    Computer simulations will be used to verify algorithms and to perform sensitivity analyses.      Earth-based experiments will be performed to validate the sensing, motion control, and path planning algorithms.      This technology will enable an aerobot to control its vertical motion and exploit prevailing winds to autonomously move to different planetary locations to explore Venus.

# 1.	Background

Planetary exploration has evolved from reconnaissance missions using “fly-by” spacecraft, to survey missions involving orbiters and non-mobile vehicles such as landers and probes.    Planetary exploration is evolving towards detailed in-situ exploration using mobile vehicles such as rovers and aerial vehicles. Aerobots are a balloon-based class of robotic aerovehicle systems which provide planetary scientists with a powerful new exploration capability [Cutts, Nock].    Current concepts for aerobots involve using balloons as the principal mobility mechanism, requiring little on-board energy for their operation as they are carried along by planetary winds.      For planets such as the Earth and Venus—which have atmospheres that cool with increasing altitude—aerobots can be designed with control over vertical mobility by using a renewable non-ballasting buoyancy control technique [Jones].    Coupling this vertical mobility control into horizontal motion provided by planetary winds allows a sub-orbital planet-wide exploration capability.

Figure 2: Nominal Vertical Oscillation

The vertical motion of an aerobot is driven by a renewable buoyancy control system based on fluids (called phase change fluids) which are gaseous low in the atmosphere and condense readily at some point higher and cooler in the atmosphere.    The aerobot's vertical motion, an oscillatory sinusoidal waveform with periods of several hours, is a natural by-product of the buoyancy change produced by the condensation and evaporation of the phase change fluid.    Error: Reference source not found shows an artist’s concept of what an aerobot might look like in the atmosphere of Venus.      Error: Reference source not found shows an example ascent and oscillation.      The condensation altitude for a typical phase change fluid at Venus is approximately 50km.

The atmosphere and environment of Venus present unique and difficult challenges to scientific missions of any significant duration.    The temperature at the surface of Venus is approximately 460° C at a pressure of 92 atmospheres. Constructing a probe which can survive such extreme temperature and pressure for a short time is difficult, but has been done in previous Venus missions such as the Russian Venera landers. Higher in the atmosphere, the temperature is relatively cool.      Aerobots offer the revolutionary capability to repeatedly explore the deep atmosphere of Venus as well as visit the surface of Venus for several hours at a time.        At the end of each deep atmosphere excursion, the aerobot will    rise high enough in the atmosphere to cool off.    No previous technology (such as passive balloons, probes, or landers) can provide such capability for repeated ascents and descents.    A Venus aerobot science mission will allow extended exploration of the deep atmosphere as well as repeated visits to the surface which will help scientists answer many of the outstanding science questions about Venus such as resolving questions of cloud chemistry, the structure of the deep atmosphere winds, driving mechanisms for the global circulation, and the nature of some anomalies in the Magellan radar data.

This proposal represents a continuation of an initial Venus simulation task undertaken in FY97 to validate, in simulation, the end-to-end robotic capability in sensing, mobility control, and planning necessary for Venus aerobot operations.    A funding augmentation from $300K to $950K is requested for FY98 to experimentally validate, using appropriate test platforms, the vertical motion sensing, control and planning techniques necessary to autonomously achieve a rich family of down-wind flight profiles of the aerobot in a complex 3D atmosphere.      A similar level of effort in FY99, extends the mobility to lateral motions utilizing cross-winds, and provides the aerobot with sensor-activated emergency terrain collision avoidance behaviors. Together, this two-year experimentation and validation activity serves to demonstrate the feasibility of planetary scale aerobot operations for targeted science objectives.    The task culminates in the fourth year with demonstration of extended global autonomous operations and    landed in-situ surface operations.

# 2.	Technical objectives

This research focuses on three capabilities which are necessary to support autonomous mobility and science operations of an aerobot in a remote and complex planetary atmosphere such as that which exists at Venus.        This research will not resolve all the technical challenges for aerobots, but only those with significant telerobotic components.      The following areas describe the three key technical objectives that will be addressed in this research.

Autonomous state/location determination:    In order for science data collection to be most useful, it must be correlated with specific locations on the planet.    Thus it is critical to know the location of the aerobot at all times.      Unfortunately, the opaque upper cloud deck on Venus makes many conventional location determination technologies untenable. To determine its location,    an aerobot on Venus will be forced to use a variety to techniques such as crude radio-frequency (RF) sensing of the sun, camera images of the surface, radar altimeter data, and beacon signals from orbiters.    In all cases, it will be critical to know the attitude of the aerobot’s swinging instrument platform    to do the required location calculations.      The coupling of attitude estimation and global position determination is more complex and demanding than that for typical interplanetary spacecraft or rovers.

Vertical mobility modeling and autonomous control:    Many desired science objectives for an aerobot science mission will require that the aerobot be able to control its vertical motion.      The types of motions that will be possible include: passive vertical oscillation, controlled descent, hovering, controlled ascent, and safe landing.    Landings will require a landing snake (a slender flexible device hanging off the bottom of the aerobot) to gently slow down the aerobot as it contacts the surface.    Landing snakes may also host soil-sensing and collection hardware.

Autonomous path planning: Scientists want to investigate specific features of Venus.      For an aerobot to move to a desired location, it must use its vertical motion control to take advantage of the structure of the winds to move appropriately.    This leads to a complex motion planning problem.    There are several distinct types of motion planning that will be needed:    short-term path prediction and monitoring to initiate obstacle avoidance,    path planning to increase probability of the over-flight of a selected area, path planning to maximize over-flight of a set of selected targets (at specified altitudes), and path planning to fly as close to a specific target or land near it.

# 3.	Justification and requirements

Aerobots are strongly represented in the Mission to the Solar System Road Map commissioned late last year by NASA, with inclusion of aerobot missions to Venus, Mars and Titan.    In order for an aerobot to be maximally useful for science data collection, it must be able to do three things (1) determine where it is, (2) plan its motions, and (3) robustly execute its motions in an uncertain and changing 3D atmosphere.

Aerobots are aerovehicles whose motions happen on a relatively fast time scale.    Rovers can rest on the ground safely for hours (or days) while decisions are made on the Earth.    Probes that fly for months or years in the predictable environment of space have ample time for Earth-based operators to construct operational sequences.    An aerobot does not have that luxury; it must actively control its own motions even while it is out of radio contact with Earth.    For these reasons, aerobot position determination, path planning, and motion control must be largely autonomous.

The purpose of this proposal to develop and validate the algorithms, techniques, and software necessary to autonomously perform the position determination, motion planning, and control necessary for aerobots to be practical scientific exploration vehicles.      A comprehensive experimental validation approach, including flight demonstrations of key technologies, is planned to confirm necessary telerobotic capabilities.      The following table sketches the phases of a Venus aerobot science mission and evaluates the science relevance and technology content of each phase.    The goal of this chart is to make it clear how the technologies being developed by this research relate to a potential future mission.        A detailed study of potential science objectives for a Venus aerobot mission can be found in a study from Brown University [Head].

| Phases of a Venus Aerobot Science Mission | Science Relevance | Telerobotic Content Sensing Control Planning |  |  |
| --- | --- | --- | --- | --- |
| Arrival at Venus and insertion burn | enable | none | none | none |
| Entry into high atmosphere | enable | low | none | none |
| High atmospheric supersonic parachute | low | none | none | none |
| Mid-atmosphere balloon deployment and inflation | low | low | low | low |
| Detailed mapping and atmospheric sampling (passive vertical oscillation) | high | high | none | none |
| Short visits to near-surface (descent/landing) | high | high | very high | high |
| Safe surface sensing/sampling (landing snake) | high | moderate | high | moderate |
| Avoiding grounding/obstacle avoidance (mid/short term planning) | enable | high | high | moderate |
| Low atmosphere hovering observations | moderate | high | high | low |
| Optimized tour of science target set | high | high | high | high |
| Fly near specific science target | high | high | high | very high |
| Cool-off and scientific data relay (through orbiter) | enable | moderate | moderate | low |

# 4.	Approach

The first year of this effort has concentrated on an end-to-end simulation of many of the telerobotics technologies necessary for a Venus aerobot mission.    With this baseline established, key experiments using appropriate experimental platforms are proposed to validate the telerobotic concepts developed in this first years simulation effort.    While some of these experiments require a balloon system such as the Planetary Aerobot Testbed (PAT) as an experimental platform, others can be conducted using simpler stand-alone component experiments.      For more information about PAT and how this activity fits into the larger aerobot program, see section .

The following sections describe telerobotic technologies required for Venus aerobot autonomy and how to validate them.      Each technology includes a summary table which identifies the resources required for developing and testing the item, the model that require validation, possible test platforms (including laboratory platforms, airborne piggyback payloads, small low-cost balloons as well as Planetary Aerobot Testbed, PAT), measured data and ground truth, and quantities that will be analyzed as well as how the results will be extrapolated to improve the model.    The resource estimates are identified with each fiscal year and culminate in the Level 1 milestones that are described in Section .

## 4.1	Atmospheric vertical motion control

Planetary winds provide the natural east-to-west motion for a Venus aerobot along a latitude.    Vertical motion sensing, planning, and control is critical to achieving descent, hover, or landing near downwind targets along this wind-driven path.    Vertical motion control is also a necessary precursor to utilizing the lateral winds at different altitudes to adjust the lateral ground track motion of the aerobot.    Here we focus on the vertical motion capabilities of the aerobot as they relate to the downwind motion of the aerobot.

A key quantity that needs to be sensed is the downwind motion of the aerobot along the latitude band.      As most of the nominal oscillatory motion of the aerobot will be in the clouds, this can be achieved through a combination of crude RF sun sensing to locate terminator crossings and inertial dead-reckoning.      Dead-reckoning drift can be minimized by azimuthal RF sun sensing.    Special estimator models that encode the natural pendulum-style behavior of the aerobot allows the position to be optimally propagated with low errors.    These estimator algorithms which have been refined in FY97 will undergo experimental validation in FY98.

Figure 3: Trajectories

The vertical motion control has unique features in that the control authority is one-sided—the rate of descent can only be slowed by releasing condensed phase change fluid into the evaporator.      Furthermore, this control is only possible during that portion of the aerobot’s vertical oscillation that is below the condensation altitude.    A family of vertical motion primitives has been developed during the FY97 task including ascent, oscillation, descent, and hover primitives.    When concatenated in an appropriate manner, arbitrary feasible vertical motion profiles can be achieved subject to the constraints on the control.    These are shown in Error: Reference source not found. The thermodynamic models needed to support the simulation of the complex physics of reversible-fluid phase changes during vertical ascent and    descent have already received extensive validation in the Earth's atmosphere. Representative models for the Venusian atmosphere are being developed in the    FY97 effort and are being exercised for vertical motion control of a    simulated Venus aerobot. The FY98 activities will serve to validate the planning and control functions using actual balloon flights with active control capabilities.

Aerobot motions require significant amounts of time, and once initiated, often cannot be changed easily.    As the control is unidirectional, and only valid during half    of the natural oscillation cycle, committing to a control sequence requires careful planning. Planning consists of concatenating several vertical motion primitives and modifying the parameterization of the primitives to achieve desired motions.      Search-based planning techniques explored in FY97 will be further refined and experimentally confirmed in FY98.

| Technology Item | Model | Experimental Platform | Measure/ ground truth | Analyze Extrapolate |
| --- | --- | --- | --- | --- |
| Inertial dead- reckoning / RF sun sensing (FY98, $150K) | pendulum | lab hanging gondola | delta-x, y simulated sun sensor | covariance analysis, extrapolate to flight sensors |
| Calibrate vertical motion primitives (FY98, $200K) | kinematic templates | balloon flights | Altitude/GPS, valve flow | Least-squares fit. Kinematic parameter adjustment. Scale to Venus |
| vertical motion planning & execution (FY98, $300K) | kinematic template concatenation | balloon flight, analyze previous flights | planned path vs actual path | Reachability analysis, stochastic downwind wind variations |

## 4.2	Lateral ground-track motion adjustment / path planning

While an aerobot moves through the atmosphere of Venus its predominant ground-track is from east-to-west as it is carried by the zonal winds within a latitude band. As    we have seen with vertical motion technology, the aerobot can be commanded to achieve various descents, hovers, landings and ascents along this down-wind trajectory. If the aerobot is required to achieve trajectories that are not simply described in terms of a    down-wind motion, but are instead tied to actual ground-tracks described in planetary coordinates, then motion control must be exercised both along zonal and meridonial directions, i.e. both east-west and north-south. We have already seen the zonal wind mechanism for east-to-west motions, north-south meridonial motions are possible utilizing atmospheric circulation winds - Hadley cells that transport air between the equator and the poles. These Hadley cells can be found at various altitudes in the atmosphere.    They are expected to provide both north-to-south winds as well as south-to-north winds depending on the circulation patterns of the cell. However, the speed of these meridonial motions is expected to be less than the zonal winds. Thus 3D aerobot motion on Venus is not a generic point-to-point motion capability, but instead a control of    a dominating east-to-west motion superimposed by a lateral north-to-south and south-to-north adjustments. Achieving this capability requires new technologies in sensing, wind modeling,    and motion planning that have been the focus of the FY97 simulation task.

It is no longer sufficient to only sense the    down-range motion, but lateral motions must also be determined. This is possible by using inertial propagation dead-reckoning techniques from an initial position obtained, for example, by a orbiter    beacon Doppler measurement. However, in comparison with the extent of the motions involved, the relative accuracy in estimating lateral motion will be less than that for estimating the zonal    motion. An independent and direct measurement of the actual ground-track motion is useful in reducing    the size of estimation errors. The most promising technique is to use refinements to the image-based frame-to-frame motion estimation methods developed in FY97 to directly determine    the ground-track motion. As we have discussed, this is quite feasible at night-time on Venus as the surface becomes visible when the aerobot dips below the clouds, or in day-time at lower altitudes.    Ray-tracing simulations of the Venusian surface appearance in FY97 are    expected to confirm the feasibility of navigation imaging and establish error bounds on the motion estimates. A direct measurement via Doppler analysis of a radar altimeter echo would also be feasible and will be experimentally confirmed in FY98.

Sensing is also required of the local wind-structure to assist in the planning of the motions. As the aerobot is carried in the winds, it is sufficient to estimate the    velocity of the aerobot using the inertial and frame-to-frame imaging methods already discussed. Modeling and dealing with uncertain and changing global and local wind patterns is a serious technical problem that must be solved in order to fly the most effective aerobot science missions.    Current models of the circulation of the Venus atmosphere are very limited and uncertain.    Therefore, an aerobot must be able to update its wind models on the fly.    This is a difficult problem both in terms of how to adapt to new or changing conditions as well as how to do it efficiently enough to be done in an on-board computer.    Wavelets are a promising technique which should allow explicit control of spatial resolution, controlled updating, and compact representations. Initial investigations undertaken in FY97 appear promising and will be    refined and experimentally validated in FY99.

Motion planning to achieve the    desired ground-tracks requires the Integration of vertical and lateral motion planning. If the atmospheric circulation patterns are known, it is possible to use the vertical motion control to exploit the winds and move laterally.    The amount of control over lateral motions depends on many factors including the extent of the vertical motion control and the knowledge and stability of wind patterns.      This is a very interesting technical problem reminiscent of path planning for wheeled vehicles.    Several types of maneuvers are anticipated—all designed to optimize the “dwell” time in a favorable lateral wind.    The on-board controller will have to execute the plan with a significant amount of freedom to take advantages of favorable winds, when encountered.      An adaptive circulation model will be a key technology for this function. In addition to more autonomous methods, we also expect to investigate heuristic methods of motion planning by allowing multiple operators to    "fly" the Venus aerobot simulation built in FY97. These experiments, in addition to providing a heuristic motion planner that could be used on the aerobot, also serve to establish a performance baseline.    Data from manned earth flights undertaken as part of various balloon global circumnavigation expeditions will also be analyzed to bound    motion planning performance.

| Technology Item | Model | Platform | Measure/ ground truth | Analyze Extrapolate |
| --- | --- | --- | --- | --- |
| Image-Based Motion Sensing (FY98, $100K) | frame-to-frame registration | high-altitude balloon payload | delta-x, y GPS | Velocity error, extrapolate performance of Venus simulated images. |
| Doppler Ground-Track Velocity (FY98, $200K) | Doppler altimeter | high-altitude balloon payload | Doppler profiles, sensor attitude | Velocity error, SNR terrain dependence. Optimize low-mass designs. |
| Adaptive wind model (FY99, $150K) | multiresolution with time tagging and forgetting factors | Multiple weather balloon launches | Successive 3D flight trajectories | Correlate actual with prediction, Examine improvement for second/third flight, Scale adaptation parameters to Venus |
| Heuristic lateral motion planning (FY99, $100K) | Heuristic motion planning | Simulation with variety of time-varying, noisy wind patterns. Human operator trials. Data from manned balloon flights. | Simulated flight paths. Global balloon trajectories. | Targeting error, Capture operator heuristics into flight planning rules. Establish planned path vs. actual performance for manned flights. |
| Autonomous lateral motion planning (FY99, $200K) | vertical kinematic primitives projected through wind patterns | Planetary Aerobot Testbed | Flight path | Cross-track reachability analysis, location uncertainty growth analysis. Scale to Venus |

## 4.3	Global science target coverage optimization

The primary goal of an aerobot mission to Venus will be to address numerous scientific questions about the nature of the surface and geology of    Venus.    A variety of science targets will be selected beforehand and the aerobot will attempt to overfly or land as near to as many targets as possible. Given the uncertain knowledge of the Venusian atmospheric circulation and the unique nature of the aerobot vertical control, it    will probably not be possible to fly directly to a specific target in general.    On the contrary, the aerobot must be able to recognize and respond to targets of opportunity as is shown in Error: Reference source not found.    This implies constant flight path prediction and mission target reachability analysis.      When an opportunity arises, the aerobot must decide what to do, plan the    appropriate descent trajectory, and execute it robustly. Basic feasibility of such an approach is being established by means of the    global Venus aerobot simulation effort in FY97.

Figure 4:    Possible Aerobot Path at Venus

Two key technologies are involved: global position estimation in planetary coordinates and motion planning and reachability analysis on a spherical planetary body. Whereas dead-reckoning propagation and sensing of local motion sufficed for short time-scale down-wind and lateral trajectory control,      global position determination requires the addition of map-based sensing, sun sensing of terminator crossings and sun elevation angles, and orbiter    assisted beacon-based radio-metric sensing. Map-based methods can be image-based whereby a map of the Venusian surface is correlated to a global map, or it can be profile-based where a cross-sectional profile of the surface as revealed by the on-board altimeter is matched to the surface topography. On Venus, night-time surface images in the 1 micron band directly provide a measure of the topography because of    high lapse rate in the atmosphere which causes higher surfaces to appear cooler (i.e. darker) in the image when compared to lower hotter (i.e. brighter) surfaces. As Magellan data provides high-resolution topography data, correlation is straightforward in principle. Practical implementations, however, require that direct correlation not be attempted and instead simpler correlations in “feature space” be used instead. Similar techniques can also be used with the altimeter profile data.    Sun-sensing also provides a method for planet-wide localization. The sun-elevation angle constrains the latitude of the aerobot, and the sensing of the terminator crossing constrains the longitude of the aerobot at that time. Radio-metric measurements of    a relay orbiter’s beacon signal also provides    similar    periodic constraints as derived from measurements of either the    received 1-way Doppler profile or the signal acquisition and loss times. In FY97, the simulation task established performance bounds on these techniques for use in global position determination. The actual algorithmic development of the correlation methods is proposed to be undertaken in FY99.

Given adequate position determination, desired science target locations, and reasonable vertical motion control, it will be useful to plan paths that maximize overflights or landings    near desired targets, avoid obvious problems such as striking one of    Venus’ high peaks, and conform to the limited deep atmosphere dwell time imposed by the hot atmosphere of Venus.    This on-going planning will focus on how to safely optimize the science return for    a limited look-ahead window (up to one or two revolutions of the planet).    Using well-known search techniques, a plan for the limited look-ahead window will be constructed and set in motion.      Executing the path will not be simple, since nothing will happen exactly as planned.    Every descent into the lower atmosphere to shift latitude will not achieve precisely the desired latitude shift.      Sometimes path error growth will simply mean small changes in the dwell time for the next descent; sometimes it will completely alter what is best to do from that point on.    So the plan will have to be repeatedly constructed and modified on the fly.      Having a clear measure of science return value makes this constant replanning maximally useful and forms an important part of this effort. The FY97 simulation effort establishes basic feasibility for a set of test cases using simplifying assumptions and one-time plans to bound the scope    of the problem.      Developing and testing the more comprehensive autonomous on-the-fly path planning and execution will be done in FY99 with participation of interested planetary scientists.

| Technology Item | Model | Experimental Platform | Measure/ ground truth | Analyze Extrapolate |
| --- | --- | --- | --- | --- |
| Science target coverage optimization (FY99, $250K) | Flight monitoring, mission-value driven path planning | Simulation, Planetary Aerobot Testbed | flight trajectories, minimum target distances | Monte Carlo analysis of mission-value robustness of simulated plans. Comparison of simulated vs actual flight data. Evaluation by scientists. Mission value metric improvement. |

## 4.4	Aerobot hazard avoidance / survival

Balloon hazard detection requires that imminent collision of the balloon with high terrain be detected sufficiently in advance to take evasive action.    For a Venus aerobot, encountering hazardous terrain is a definite possibility when the system descends down to the surface for landing.      The challenge is to distinguish the nominal, non-hazardous landing terrain below the aerobot from hazardous terrain relief in the horizontal path of the aerobot as it descends towards the landing point. The first class of terrain is benign and easily handled by the landing snake of the aerobot system whereas the second class of terrain could cause serious damage to the balloon envelope and systems.

It is proposed to use a radar altimeter ranging sensor to perform the    terrain detection. This sensor utilizes radar signals and on-board signal processing to classify the return echoes from the terrain into a series of range intervals. Pattern recognition of the range intervals as they evolve with time allows the aerobot to distinguish between benign "flat" terrain that is suitable for landing from hazardous "craggy" terrain that constitutes a flight hazard. The severity of the hazard is also a function of the horizontal velocity of the aerobot since otherwise mild hazards could become more dangerous with increased horizontal velocity of the aerobot system.

To avoid    endangering the mission, the aerobot must be prepared to ascend rapidly when impending impact is suspected.    In addition to the radar altimeter sounding mentioned previously, the aerobot must constantly predict its trajectory as if it had just dumped all its condensed phase-change fluid into the evaporator and started to ascend as rapidly as possible.    If the actual surface ahead of it approaches this predicted profile, the aerobot should take immediate action to ascend.

Another activity necessary for aerobot survival is estimating maximum deep-atmosphere dwell times given the thermodynamic limitations of the aerobot.      Critical electronics and hardware of the aerobot will be contained in a highly insulated gondola.      Unfortunately, on-board heat generation and extremely high temperatures near the surface of    Venus will not allow more than an hour or two of dwell time at low altitudes.    At that point it will be necessary to ascend to the upper clouds to cool off.      In the previous discussion, the necessity for continual flight path projection was described.    In a similar way, projections with a simplified thermal model will be necessary to determine if the planned flight profile will the thermally satisfactory.    There is a body of research on “homeostatic” self-monitoring and control which can be applied to the general question of how an aerobot can monitor its own conditions and plan its actions in order to fulfill its mission goals as well as protect itself [Arkin].      This research methodology will be applied to develop an integrated system to ensure the survival and performance of a Venus aerobot.

| Technology Item | Model | Experimental Platform | Measure/ ground truth | Analyze Extrapolate |
| --- | --- | --- | --- | --- |
| Hazard detection test (FY99, $250K) | Hazard profile model | Helicopter with radar ranger | altitude and range gates | False alarm probability, miss probabilities, extrapolate to multiple terrain types |
| Emergency ascent tests (FY99, $150K) | vertical motion model | PAT | actual ascent (via GPS) | Compare actual with plan, extrapolate to Venus |

## 4.5	Surface in-situ science collection

When an aerobot descends to the surface, some mechanism is required to prevent a hard landing.    It is likely that this device will be a long thin landing snake that hangs below the aerobot.      As the aerobot nears the surface, the lower end of the snake touches the surface, and that weight is no longer borne by the aerobot.      As progressively more of the snake rests on the surface, the aerobot will slow down quickly and smoothly before the gondola strikes the surface.        Designing the landing snake will involve careful mechanical design and testing.    Sample units will be constructed and tested in the laboratory.    If possible, landing snakes will be flown on aerobot flights, such as PAT.      Other ideas which will be investigated is the possibility of collecting samples in the snake and then retracting samples into an analysis unit.

Since the snake would be in contact with the surface, it would be highly desirable to collect science information about the surface through the snake.    One desired objective is to embed electrical surveying instrumentation    into the snake to allow compositional determination of the low emissivity snow-lines found in Magellan radar data.     It will be difficult to develop sensors which can operate inside the snake in the harsh environment at the surface of Venus.      Separation of sensing elements and support electronics will be essential.      High-temperature sensing heads and data return cabling will be developed and tested.    An adjunct to the snake will be optical sensors that can operate through a quartz window in the bottom of the gondola.      Images and limited spectroscopic readings can be performed.

| Technology Item | Model | Experimental Platform | Measure/ ground truth | Analyze Extrapolate |
| --- | --- | --- | --- | --- |
| Basic landing snake mechanical design (FY00, $200K) | kinematic / dynamic simulation model. | lab sandbox, multi-scale mockups, | force sensing, kinematic history through joint sensors | analyze model fidelity, improve model parameters, scale to Venus |
| Landing snake performance (FY00, $150K) | integrated aerobot/snake kinematic / dynamic simulation model. | small balloon flight with landing snake and landing | kinematic history through videotape reconstruction | verify expected performance, improve model, scale to Venus |
| Landing snake high-temperature sensors (FY00, $200K) | separation of minimal-mass sensing head from rest | small sandbox in test oven | temperatures, sensor output | verify concept, check performance, improve design |
| Landing snake retraction/sample transfer mechanism (FY00, $150K) | Mechanical design | lab sandbox, various test mechanisms | transferred material flow | monitor performance, iterate design |

# 5.	Schedule and cost breakdown

## 5.1	Technical Milestones

The technical performance goals as represented in the milestones below are based upon nominal sensor characteristics and models of atmospheric uncertainties. Significant improvement in sensor technology (e.g. low drift inertial sensors) has the potential for improvements in performance.    Similarly, better understanding of the Venusian wind structure will also lead    to improvements in the global motion accuracies. The FY97 simulation effort serves to benchmark the nominal expected performance with    experimental validation in the continuing tasks serving to give credibility to the performance numbers. The simulation tool also serves to benchmark human motion planning capabilities in comparison with the autonomous methods flown on-board the aerobot.

FY97

Simulate end-to-end Venus robotic mission technology in Planning, Sensing and    Control to achieve landing of aerobot    within 30 km of 500 km distant target.

(Establishes in simulation, fundamental feasibility of autonomous balloon operation including planet-wide localization and vertical/horizontal motion planning and control.)

FY98

Demo Venus-applicable vertical motion technology to land within 10 km of down-wind target and to achieve short duration 15 min hover motions (to within 0.5 km), including ability to sense down-range motion to within 10 km for each hour of wind-driven motion.

(Experimental validation in a test vehicle, of the ability to achieve autonomous vertical motion control of a balloon, a complex thermophysical system,    in a real planetary atmosphere.      Experimental validation and calibration of vertical motion planning and control algorithms developed in computer simulation in FY97.)

FY99

Demo Venus-applicable 3D motion planning, sensing    and control technology to show horizontal path change of 5 km using local winds to overfly science targets, and sensing of absolute position using    Map/Image    correlation to within    30 km.      Achieve targeted landing (within 25 km of nearby target) with radar-based descent control (3 m/s soft landing).

(Experimental validation of lateral motion planning and control developed in computer simulation in FY97.)

.

FY00

Design and fabricate preliminary landing snake concepts.    Test landing snake sensing and sample collection techniques.

(Validation of in-situ science collection tasks.)

## 5.2	Task Costs

| NASA Telerobotics Funding Profile ($K) | FY97 | FY98 | FY99 | FY00 |
| --- | --- | --- | --- | --- |
| FY97 End-to-end Venus aerobot feasibility simulation | 300 |  |  |  |
| FY98 Inertial dead-reckoning / RF sun sensing |  | 150 |  |  |
| Calibrate vertical motion primitives |  | 200 |  |  |
| Vertical motion planning |  | 300 |  |  |
| Image-based motion sensing |  | 100 |  |  |
| Doppler ground-track velocity |  | 200 |  |  |
| FY99 Adaptive wind model |  |  | 150 |  |
| Heuristic lateral motion planning |  |  | 100 |  |
| Autonomous lateral motion planning |  |  | 200 |  |
| Science target coverage optimization |  |  | 250 |  |
| Hazard detection test |  |  | 250 |  |
| Emergency ascent testing |  |  | 150 |  |
| FY00 Landing snake design |  |  |  | 200 |
| Landing snake performance |  |  |  | 150 |
| Landing snake high temperature sensors |  |  |  | 200 |
| Landing snake sample collection |  |  |  | 150 |
| Totals for each FY: | 300 | 950 | 1100 | 750 |
| Full Time Equivalents: | 1.6 | 5 | 5 | 4 |

# 6.	Relationship to other programs

The Planetary Aerobot Testbed is a small aerobot Earth-based balloon flight demonstration system that is being developed at JPL as part of the larger Aerobot Program    [See reference 1 for the PAT web site].    PAT provides the necessary infrastructure, in terms of power, communication, gondola, cutdown systems, FAA transponders, embedded computers, launch facility, and command and control stations, etc. that is necessary to fly various buoyancy control, sensing, planning and science instrument experiments.      To support the Venus aerobot effort, PAT will be configured with a phase change fluid buoyancy control system, Venus-applicable sensors, and Venus-specific vertical and lateral motion planning primitives developed in this telerobotic research effort.    This arrangement, in which the engineering infrastructure is provided by PAT allows the telerobotics resources to concentrated on the experiment definition, technology experiments, and data reduction.      While PAT is expected to be integrated and flown in FY97, thereby providing an opportunity for some of the telerobotics experiments to be run in FY97, the nominal plan described here assumes one PAT telerobotic-related flight per year commencing in FY98.      If PAT is not available, other simpler balloon systems will be developed based on the experience of the Aerobot team at JPL.      If the early flights (in FY97) prove fruitful, then the technology development schedule presented here can be significantly accelerated.      Other aerobot activities, funded largely with JPL internal research funding, include the design of a gondola which can survive the high temperatures and pressures at Venus and development of specialized materials for a Venus balloon.

The technologies developed here are also candidates for a New Millennium (NM) Deep Space flight program.      Members of the aerobot team have been accepted in both the NM Autonomy IPDT and the Modular and Multifunctional Structures (MAMS) IPDT.    The telerobotics technology developed in this Telerobotics research will be advocated by these IPDT members for flight validation in a FY04 Venus DS-5 Mission.

# 7.	Participants

The JPL participants in this activity are primarily from the Automation and Control Section and the Navigation and Flight Mechanics Section.    Some expertise will be provided by personnel from Mechanical Systems Engineering and Research Division.    Occasional support will be provided by NASA and JPL atmospheric science experts, operations researchers at the Naval Postgraduate School and atmospheric scientists at the University of California at Los Angeles and at the Goddard Institute of Space Sciences.

# 8.	Principal investigators

Jonathan M. Cameron received his B.S. degree in Mechanical Engineering in 1979 and his M.S. in Engineering Science and Mechanics in 1980, both at Georgia Institute of Technology.    In 1981, he was employed by Jet Propulsion Laboratory (JPL) in Pasadena, California, where he was involved in spacecraft and ground vehicle control research including path planning for robotic vehicles.    In 1988, he left JPL on an educational leave of absence to pursue a Ph.D. in Mechanical Engineering at Georgia Institute of Technology under the direction of Professor Wayne J. Book.    He completed his Ph.D in December, 1993.    From then until 1995, he was employed by the College of Computing at Georgia Tech to work with Prof. Ronald Arkin as a research scientist investigating multiagent robotics.    In July of 1995, he returned to JPL and is now a member of the Automation and Control section where is working on several advanced space exploration concepts.    His research interests include robotics, dynamics, kinematics, controls, and software development.

J. Balaram received the B. Tech degree in Mechanical Engineering from the Indian Institute of Technology in 1980, and the MS and Ph. D. degrees in Computer and Systems Engineering from Rensselaer Polytechnic Institute in 1982, and 1985, respectively. He has been with NASA's Jet Propulsion Laboratory since 1985 and has worked in the area of task planning and machine vision for several JPL projects in telerobotics. He is currently involved in the design and implementation of perception and navigation systems for future Mars rovers and Venus/Mars balloons.    His interests include machine vision, real-time architectures, and task-level robotic control.

References

- “Planetary Aerobot Program (JPL)'',    World-Wide-Web    http://robotics.jpl.nasa.gov/tasks/aerobot

- Cutts J. A., et. al, “Planetary Exploration by Robotic Aerovehicles'', Journal of Autonomous Robots, January 1996.

- Nock K. T., Jones J. A., and Rodriguez G., “Planetary Aerobots: A Program for Robotics Balloon Exploration'', AIAA 34th Aerospace Sciences Meeting & Exhibit, January 15-18, 1996, Reno, Nevada.

- Jones, J.A., “Reversible Fluid Balloon Altitude Control Concepts,”    11th Lighter-Than-Air Systems Technology Conference, Clearwater    Beach, FL, 16-18 May, 1995.

- Arkin, R.C., “Homeostatic Control for a Mobile Robot: Dynamic Replanning in Hazardous Environments,” Journal of Robotic Systems, 9(2), p. 197-214, 1992.

- James W. Head, III, et-al,    Investigation of the application of aerobot technology at Venus: Scientific goals and objectives of the proposed balloon experiment at Venus (BEV) and Venus Flyer Robot (VFR). Final Report. Brown University. June 1, 1996.