Principal Investigator:

Donald R. Sevilla

1.0 Objectives

Future exploration of interplanetary small bodies, such as comets and asteroids, requires technology development in a variety of areas.    Landing and operation in the low gravity environment of small bodies (10-4 to 10-2 meter per second squared) is an extremely challenging problem.    The Exploration of Small Bodies task will focus on developing the enabling technologies to accomplish in-situ scientific studies of these interplanetary objects.    The primary objectives of this task are to develop the mechanisms and control strategies to perform landing, anchoring, surface/subsurface sampling and sample manipulation for a complement of science instruments.

The low gravity of small bodies allows unpowered descent to the surface with a landing system to absorb the impact forces and limit the loads imparted to the science payload.    An attachment system is required to secure the lander to the surface, enabling it to accommodate any "spring-back" of the landing mechanisms (preventing rebound off the surface) and retain the forces generated during sampling.    Acquisition of surface and subsurface materials in a microgravity environment will require reliable containment.    Manipulation of material samples and transportation to a complement of science instruments will demand an understanding of the type of instruments under consideration, the range of interfaces to receive samples that are possible and the form of sample material required.    Development of the control algorithms to allow autonomous operation of all of these mechanisms is not a trivial effort, and so this task is planned to be integral with the mechanical work from the onset.

Return of samples to Earth is a requirement that is now being baselined in proposed missions to comets and Mars.    Transfer of samples, separation of multiple samples, and containerization technologies are needed to meet this challenge.      Meeting the stringent planetary protection requirements for the samples returned to Earth require new and unique methods of double sealing the containers brought back from these other worlds.    The samples to be returned must not only be sealed from contaminants from Earth, but the returned sample containers also must not expose any extraterrestrial materials to an uncontrolled environment on Earth.      Required are automated sealing methods that are hermetic to insure that important volatiles are not lost as well as meeting these planetary protection requirements.

One key technical challenge for this effort is centered around the minimally available resources from the proposed spacecraft that will perform these missions.    All proposed craft are small (to be launched with "small" launch vehicles such as the Delta). The comet/asteroid landers are planned to be powered by batteries only, while the Mars rovers will use solar cells without power to spare.      A further challenge is that the nature of the comet surface is unknown, requiring the ability to sample a wide variety of material compositions and strengths that range over seven orders of magnitude.    Reliable anchoring in this wide range of material properties is a formidable task. The sample acquisition process must be as reactionless as possible, since the anchoring of the lander must not be disturbed from high thrust loads during drilling.    Demands are placed on the control system that stem from potential compliance in the anchoring system.    Another key technical challenge involves the fundamental nature of comets.    They are very cold and do not melt, but sublimate.    Therefore, the comet will actually "draw away" from any lander structure or anchor that is too warm, and could actually eject it.    This is a problem that past landers have not had to deal with, in that for comet missions the structure and mechanisms that contact the comet must operate at cryogenic temperatures.

The first two years of this effort has been dedicated to the production of a drilling and sampling mechanism, and a landing system suitable for the unique requirements of a cometary mission.    The second two years will focus on meeting the needs of planned sample return missions to small bodies and Mars exploration.

2.0    Justification

A common theme of proposed missions to small bodies in the Solar System such as comets and asteroids is the acquisition of surface and subsurface material samples.    To perform these proposed missions, there is a definite need to develop the required landing and anchoring technologies.    Subsurface sample acquisition by drilling (or coring) and the handling of material samples in a cryogenic microgravity environment are technical challenges that are made more difficult by the minimally allocated resources to perform them.    This task will spearhead the development of the required technologies to achieve these goals.

Furthermore, revived interest in the potential for life on Mars has brought sample return missions to the forefront of mission planning.    Both Champollion and the current Mars mission plans are now baselining sample return.    Champollion's "wish list" of needed technological developments have included small lightweight penetrator firing mechanisms, high speed cable deployment systems, motor and gear systems for cryogenic operation, drilling control feedback systems and low rebound impact absorption.    All of these items have been in development within this ESB task, with the efforts under continuous review by the Champollion project.    Since Champollion has recently incorporated sample return in the mission, this task is now including the development of the important enabling technologies to meet the requirements.    While ESB originally planned to investigate mobility systems for multiple site exploration in the FY98 and FY99 thrusts, there was a consensus within the JPL science community that mobility technologies for comets and asteroids was a lower priority compared to the needs of sample return missions.    In addition, the majority of the sample return, containerization and sealing requirements are common to Champollion and Mars exploration.    The New Millennium technology workshop at the University of Arizona and the Mars Drilling Workshop at JPL illustrated the current anticipated needs for sample return and containerization technologies for use by both the New Millennium and the Mars Exploration Programs.    The Exploration of Small Bodies task will coordinate its development of these technologies with both the Champollion and Mars Exploration projects.

3.0 Approach

The approach for Phase One has been to develop a single-site scientific explorer, originally utilizing the Champollion Lander as a mission requirements context.    While the Champollion mission team has subsequently selected a different lander configuration, key component technologies developed by this ESB task have been deemed very relevant by the Project, as discussed below.

The completion of Phase One in the first two years will allow Champollion to utilize technology which will have been demonstrated in a cryogenic environment, and then further develop it towards its specific flight program requirements in 1999.    Phase Two will move on to concentrate on the requirements of sample return missions for small bodies and Mars Exploration.    In both Phases, the efforts will consist of a tightly coordinated development of three technology areas:    Landing and Attachment; Subsurface Sampling and Transport; Sample Return and Containerization.

Landing and Attachment

| The approach of this task during the FY96 and FY97 efforts has been to utilize a deployable leg configuration similar to the Surveyor and Viking landers to provide a large footprint that will react against the overturning moment from landing horizontal velocities and to compensate for high relief in the terrain. This configuration is also needed to achieve the low operational temperature requirements of the structure that will contact the comet by isolating it from the lander body and providing large radiating areas to space. |  |
| --- | --- |

The ESB task has developed a new low rebound damping strut that utilizes the shearing of polyurethane foam while collapsing to absorb energy.    These struts also exhibit retention of large tension and compression forces after collapsing (which will react the drilling loads), and were demonstrated at cryogenic operating temperatures.

The anchoring techniques in development involve launching a tethered harpoon from each leg into the comet surface, and winching the lander against the surface.    During FY96, this task developed a sealed pyrotechnic mechanism to accelerate the harpoon and demonstrated that effective penetration is possible with 80 g harpoons fired as low as 90 m/s.    Champollion science goals include instrumenting the anchoring components to directly obtain science data such as temperature and to infer material properties by obtaining the depth of penetration.    Collaboration with a Principal Investigator for Champollion science is on-going, with the hope of integrating an instrumentation package within the ESB anchor during FY97.    A particular challenging task is to provide the needed electrical interfaces to the anchor.    The FY97 efforts include developing the necessary high speed cable deployment system as well as the winching mechanism.    To minimize the force that each anchor must retain for stabilizing the landing, a controller can receive the inputs from accelerometers in the feet and basebody, as well as gyro and contact sensor information, to control the winch motors and arrest lander tip-over.    A single axis system-level demonstration will be performed in FY97 using a horizontal sled test fixture to perform a simulated "landing"    of an mass with integrated leg mechanisms.    The completed three axis system will be demonstrated in early FY98 on a full size lander testbed.    An "End-to-end" test demonstration may be performed by suspending the lander base with integrated mechanisms "side down" like a pendulum, and simulating a microgravity landing by swinging it towards a cometary simulated material landing base.

Subsurface Sampling and Transport

| The Exploration of Small Bodies task has developed a unique Compact Drill Mechanism that has eliminated the drill tower normally required for a automatic drilling machine, and includes the percussion capabilities required for drilling into hard materials. The drill mechanism has a removable drill head with a sampling chamber. Closed loop control has been integrated in the mechanism to allow this "smart" drill to choose the correct drilling parameters for the entire range of substrate properties and to limit the forces that must be resisted by the anchoring system. The FY97 efforts are centered on the development of the data archiving to calibrate the "smart drill" against a complement of various substrates so that an accurate determination can be made of the properties of the material being sampled. In this way, the sensors already required for autonomous |  |
| --- | --- |

control of the mechanisms may also provide scientific information such as surface hardness, density and porosity data.    The motor and gearing, already demonstrated in a nitrogen atmosphere at -165°C, will be life tested in a cryo-vacuum environment.    The mechanisms will drill and sample cometary simulation materials at cryogenic temperatures.    Further mass reductions of approximately 20% will also be accomplished.

The efforts in FY98 will be to integrate the needs of sample return missions with the Drill.    The Drill has the capability for removable drill heads, therefore different types of sampling (such as coring) and sampling transfer to return containers will be developed.    Partitioning or other ways of separating the individual samples will be investigated.    Collaborative work will be performed with the on-going MEMS Motors task to integrate new technology motors.    These electronically commutated brushless motors will provide a longer running life, more reliability and a higher power density.

The drill stem passes right through the Compact Drill Mechanism, driven and percussed anywhere along its length.    It is possible to robotically attach drill stem segments to allow deeper drilling.      Work will be initiated to expand the capabilities of the Compact Drill Mechanism to robotically drill to multiple meter depths.    The capabilities are currently unknown, as the increasing mass of additional drill segments will eventually overcome the effectiveness of the percussion mechanism.    This task will ascertain the drilling limits of the Compact Drill Mechanism and investigate ways to improve the drilling depth performance.    By the end of FY99, ESB will demonstrate robotic drilling and sampling at least three times deeper than the length of the launch envelope.

Sample Return and Containerization

There is currently high interest within NASA (again) for sample return missions.    Champollion, since breaking away from ESA's Rosetta Mission, is now being planned as a New Millennium mission to return a comet sample to Earth as well as perform in situ scientific investigations.    The Mars Exploration Program is now designing three missions that all involve a number of sample return scenarios.    These scenarios center around various proposed methods to acquire, select, cache, transport and containerize different types of rock and soil samples.    Key to these scenarios is the required development of the mechanisms to perform these tasks.    A primary reason these mission designs remain "scenarios" rather than baselined mission designs is the unknown capabilities and characteristics of the required mechanisms, as well as resource requirements such as mass, volume and power.

The Exploration of Small Bodies task will investigate some of the most promising sample return designs and container mechanisms, using Champollion and Mars Exploration mission requirements.    Each require the acquisition of subsurface samples by drilling, each require sealed containerization, and each involve one (or multiple) transports of the samples across separation/docking interfaces.    ESB will build on the work of the Sample Acquisition, Analysis and Preservation task (SAAP), performed in 1989 for NASA's Office of Aeronautics and Space Technology.    This task was a NASA multiple agency thrust to "develop critical enhancing technologies for remote selection, collection, analysis and preservation of materials for in-situ study and Earth return."    The SAAP task was a five year effort that was terminated after the first year for funding reasons.    However, in this first year the task had compiled the overall existing technology capabilities and needs in the areas of system design, site and sample selection, sample acquisition, preparation and analysis, and container design.    The ESB task will extend the truncated effort by performing a demonstration of a mechanism that will acquire a subsurface sample, transfer it to a storage container, seal this container, and then transfer the container from one docked berth to another.      ESB will develop the containerization, sealing and berthing mechanisms, and the system demonstration will be performed in collaboration with the Mars Sample Return Rover task and the Mars Manipulator Arm task.

Champollion's sample volume expectations range from 15 cc of a high strength substrate to 100 cc of a low strength substrate.    Champollion requires a hermetically sealed and temperature controlled container.    The 2004 Mars Sample Return mission (MSR) sampling requirements are to hermetically seal a container that is limited to one kilogram of sample plus container.    For the Mars mission, temperature control is currently not required.    However, for planetary protection purposes there is a requirement for "double sealing" the returned sample to insure that no Mars material is exposed to an uncontrolled Earth environment.    While these planetary protection requirements are not currently levied on comet sample return missions, there is a high probability that they will be in the future.    Therefore, a solution to the problem of acceptably sealing a sample container, and insuring that the sample return container meets planetary protection requirements, will be relevant to both Mars and cometary sample return missions.

The approach will be to design an outer canister assembly with docking latches mounted to the inside walls.    The sample return container, transferred from a Champollion lander or a Mars rover, would be docked reliably into the outer canister for the return to Earth.    The sample return container would be coated with material from the landing site, as would the inside walls of the outer canister.    Once inside the outer canister, the double sealing requirements will be met by having the outer canister mounted inside of a conventionally sealed box (o-ring or gasket), with only the end diameter exposed.    The end closure of the outer canister would be commanded closed and then actively sealed.      This end closure would have a contamination cover    mounted over it (a bioshield), conventionally sealed over 100% of the outside surface area.      Prior to the transfer of the canister to the return vehicle, the contamination cover would be commanded to be released and ejected while in orbit.    The outer canister will itself be docked into the box (any enclosed volume of the lander that can be sealed from the environment).    Unlatching and transferring the outer canister into a docking berth in the Earth return vehicle completes the sample return mechanization.

| Two methods of sealing the outer canister will be investigated, with the best (or most affordable) design being implemented. Automating the Apollo style sample canister is one method, in which a knife-edged open can is sealed when a lid is clamped to it under high force. The sealing media used is a soft metal such as indium. (Ref. "Catalog of Apollo Lunar Surface Geological Sampling Tools and Containers", Judith Haley Allton, report for NASA/JSC Solar System Exploration Division; JSC-23454). This design has been measured to achieve a near hermetic seal, but requires methods of automating the closure and clamping of the lid as well |  |
| --- | --- |

as dust protection for the seal.    Another promising method is to utilize a small sealed ribbon pyrotechnic to perform explosive seam welding for a true hermetic seal.    Joining thin materials by explosive cladding has been demonstrated in the past, in which linear ribbon RDX explosive, sealed in a lead sheath, accelerates a "flyer plate" (the container lid) across a small gap to accomplish a high velocity collision to a base (the container body).    Surface oxide films are effaced and interatomic linkups result in a true metal-to-metal bond.    (Ref. "Practical Small-Scale Explosive Seam Welding", Laurence J. Bement, Proceedings, 17th Aerospace Mechanisms Symposium, May 5 & 6, 1983; NASA CP-2273).    Methods to keep the pre-welded interfacing surfaces dust free must be implemented, such as the use of removable shields or covers.    Pyrotechnic shock and sample container temperature rise will have to be characterized, and shock    attenuation interface bracketry may have to be incorporated.    The sealing leak rates under positive and negative pressurization, weld structural integrity and the effects of entry/descent/landing dynamics will be measured.    This work will be performed in collaboration with the Planetary Protection task under the Mars Exploration Program.

Comet sample return missions face a particular challenge in that the sample container must be maintained at cryogenic temperatures throughout the return to Earth.    Space facing radiators with good thermal conduction to the docked sample canister will keep it cool for the cruise back to Earth, but a thermal storage system must be incorporated in the design to maintain the canister as low as 110°K during Earth reentry and recovery.    Champollion has work on-going in the use of phase change materials that are expected to meet these thermal storage requirements.    ESB will insure that compatible interfaces exist in the design and incorporate the thermal control hardware in the FY99 demonstrations.

4.0 Participants

The participants at JPL will consists of members of the technical staff of the Mechanical Engineering and the Control and Automation Sections.    Members of the Champollion and Mars Sample Return mission teams will be actively involved in an interface role to confirm the design directions and assist in the refinement of the requirements and goals. Collaboration is ongoing in the instrumentation of the landing anchors with the principle investigator of Champollion's Physical Properties Probe, Dr. Thomas Ahrens of Caltech.    A coordinated effort will be performed with JPL's Mars Exploration Planetary Protection task, the Manipulator Arm task and the Sample Return Rover task to result in a working system using elements from all of the tasks.    The ESB task is also aligned with the MEMS Motors task as a recipient of prototype motors for integration in the Sampling Drill.    Motors and gearing capable of cryogenic operation are being developed in association with Advanced Technology Consortium, a small business in Los Angeles.

5.0 Deliverables

Deliverables will consist of report documentation and lab demonstration (videotaped) of the milestones below.    The major efforts in the past two years were focused on the comet sampling requirements of Champollion.    The next two year's efforts will be directed towards the sample return requirements of Champollion as well as the Mars Sample Return Mission.    Technology developed and associated Level 1 (indicated) and Level 2 (all others) milestones are as follows:

Fiscal 1996 Accomplishments

•Developed a unique Compact Drill Mechanism with sampling drill head.

•Developed autonomous Drill control algorithms using embedded sensors.

•Developed low rebound damping struts and tested at -175°C.

•Developed sealed pyrotechnic anchoring mechanism and tested at -165°C.

•Demonstrated motor/gearbox operation at -165°C.

•Performed full-up lander testbed drop testing, demonstrating functional operation of integrated landing leg force attenuation system.    (Level 1 Milestone).

Fiscal 1997 (Efforts in Progress)

•Rework Drill Mechanisms to decrease mass 20% (Level 1 Milestone).

•Test mechanisms in the relevant cryogenic environment.

•Implement software archiving and perform testing to derive science properties of substrate drilled.

•Develop integrated pyro-fired, tethered anchor and winch system.

•Perform "end-to-end" system level test (single axis) demonstrating smart landing (controlling anchoring forces) by using integrated sensors (Level 1 Milestone).

Fiscal 1998

•Perform "end-to-end" system level test (three axis) demonstrating dynamic stability through "smart landing" including impact absorption, anchoring and autonomous winch control. (Level 1 Milestone).

•Develop sample container docking and grappling systems.

•Develop container sealing mechanisms and measure planetary protection performance.

•Demonstrate methods of automating the hermetic sealing of a sample return container (Level 1 Milestone).

Fiscal 1999

•Demonstrate robotic deep drilling system using a segmented drill stem (Level 1 Milestone).

•Demonstrate "end-to-end" sample return system including hermetic sealing, transfer to return vehicle and temperature control    (Level 1 Milestone).

6.0 Commercialization Potential

Potential spin-off of the sub-surface sample acquisition system exists in the area of remote sensing and sample gathering such as a hazardous material site.    The low mass/low power system could easily be carried into a dangerous environment by a small autonomous vehicle and used to gather samples in areas suspected of containing surface or buried toxic material.    The    robotic sample manipulator system has a variety of applications including a manipulator for a small rover system doing inspection and/or reconnaissance tasks.    Motors developed for operation in a cryogenic environment may fulfill a niche market for actuators used in high altitudes and/or cold climates.

7.0 Funding

The intent of this effort is to accomplish the enabling technology to perform Discovery class missions for the next two decades.    This task is intended as a significant new JPL thrust in the realm of robotic, autonomous spacecraft with capabilities directed at science return, but with the modern constraints of very low mass and power allocations.    Continuously reviewed during the duration of this effort will be that the design directions include attention towards low cost manufacture and testing.

| Fiscal Year | Workforce | Services | Procurements | Industry Partnerships | Fiscal Year Total |
| --- | --- | --- | --- | --- | --- |
| FY96 | $500K | $270K | $80K | $150K | $1000K |
| FY97 | $500K | $205K | $175K | $120K | $1000K |
| FY98 | $600K | $250K | $200K | $150K | $1200K |
| FY99 | $450K | $200K | $50K | $100K | $800K |

8.0 Facilities

The majority of the effort for this task will be performed at the Jet Propulsion Laboratory in Pasadena, California.    A dedicated lab has been set up for the development testing and research for this effort.    In addition to the computer workstands and cryogenic mixing equipment, this lab has access to a high bay within the building which will allow drop testing and gravity compensation fixturing.    Caltech is currently doing research in firing small penetrators into comet analogs using sophisticated air guns, and some collaboration is planned.    Some of the other resources available at JPL that are planned to be utilized are the temperature/atmosphere chambers and thermal vacuum chambers to verify the designs in a relevant environment.

9.0 Principal Investigator

Donald R. Sevilla is a Member of Technical Staff in the Mechanical Engineering Section at JPL.    In the past 19 years at JPL, he has been the Technical Manager for the Mechanical Devices Subsystem for both Cassini and Galileo spacecrafts as well as a lead mechanisms expert for numerous flight projects.    Mr. Sevilla was a recipient of the NASA Exceptional Service Medal in 1992.

(Phone:818-354-2136;    Fax:818-393-1324;    mail:Donald.R.Sevilla@jpl.nasa.gov;

JPL Mail Stop 301-320)

The mailing address of the principal investigator is:

Jet Propulsion Laboratory

4800 Oak Grove Drive

Pasadena, California        91109-8099