Planetary Dexterous Manipulators (PDM)

Jet Propulsion Laboratory

California Institute of Technology

Pasadena, California

Paul S. Schenker, Ph. D. (PI)

paul.s.schenker@jpl.nasa.gov

(818) 354-2681 (voice)

(818) 393-4057 (FAX)

Objectives

The task defines, develops, and quantifies – with guidance of MarsSWG/Mars Expedition Strategy Group and Mars Exploration Program (Code S) mission planners – robotic sampling devices for in situ Mars surface/near-surface science. Task technology products include sample acquisition robots for both landers and rovers, as illustrated in the photograph below. Technology challenges include reduction of sampling system mass, volume, power, and cost consistent with the planned Mars Surveyor Program mission set (‘98-‘01-‘03-‘05, the last a sample return to earth). In FY98 we focus on small, light rover manipulators and effectors enabling new classes of high dexterity, mobile science operations.

MarsArmII:  (top) a 4 kilogram R&D prototype of all-composite, graphite-epoxy construction. MarsArmII is 2 meters long, employs a torso-shoulder-elbow-wrist 4 degrees of freedom design, and carries a multi-function 1 kg science end-effector that can dexterously scoop, grasp, expose, and closely view surface and sub-surface rocks and soils (The effector incorporates a micro-camera). MarsArmII utilizes built-in position, force, & vision sensors to monitor its motions, which proceed autonomously under remote science commands of earth-based operators. An earlier JPL MarsArmI R&D concept is the basis for lander-based planetary sampling on the upcoming NASA 1998 Mars Volatiles and Climate  Surveyor mission; (bottom): LSR-1, with MicroArmI, a robot about 25% the scale of MarsArmII and including the same science end-effector functions – in development, FY97.

The target robotic operations are diverse, requiring advances in both robot mechanization and sensor-based robot control in unstructured, uncertain environments (for the latter, force contact interactions with highly variable media, limited precision visual feedback, etc.). We develop sample acquisition automation & control strategies in a companion task of the Mars Exploration Technology Program (Code X). In this task, we focus sharply on mechanization issues. Our earlier FY95-96 task efforts addressed lander based sampling. We developed and successfully proposed for use in the NASA Mars’98 mission (Mars Volatiles and Climate Surveyor/Integrated Science Payload PI, Prof. David A. Paige) a 3-d.o.f. arm. FY97 work and beyond emphasizes development of rover-based sampling and sample return functions.

Specific, quantified technical objectives and advances over state-of-the-art include:

- Conceive, prototype and evaluate in relevant sampling operations a dexterous (3-dof or greater) serial manipulator architecture that reduces conventional design mass (referenced to metal-metal, or composite-metal joint-link architectures) up to 50% -- include within this framework a means for (stowed) volume reduction of  40-60% the deployed value. As nominal design targets (referenced to prior Viking lander experience and reachability to more pristine near-field materials) assume 2-to-3 meter full extent, maximum 6 kilogram mass (with effector), 2 kg science payload of which effector is less than 50%, and 50 Newtons or better tip down-force

- Concomitant with Objective 1, define, develop, and quantify as needed, enabling new low mass, environmentally robust robotic structures (characterizing engineering process, structural and thermal parameters) that can be formed and machined in conventional ways. As a goal, seek composites 30-50% lighter and stronger than aluminum, having good 3-D load and impact-bearing properties in critical robotic applications such as arm joints and rover chassis members, and affording improved DTCE and chemical interface and environmental properties. Target design to Mars near-polar temperatures and diurnal cycles, including analysis of behavior under prolonged space ambient exposures (atmosphere/radiation).  Within this materials process framework – and as motivated by improved closed-loop servo and arm flight safety issues (as communicated by MSP’98/MVACS) -- develop and demonstrate a (“smart structures/materials”) basis for integral sensing of mechanical deformation/forces, e.g., in critical load bearing links and joints, and also effector contact areas.

- Toward overall mass, volume, power, and cost reduction goals, develop and quantify alternative high-torque density, minimal gearing, low complexity actuators (major drive elements formable in batch process) survivable in the full range of anticipated Mars ambient surface science conditions. As design targets, develop and demonstrate a rotary ultrasonic motor concept (initially in the 1-to-20 in-lb. range) having 5-to-10x DC brushed motor torque-density, 10x reduced no-load speed, and comparable volume. As part of this effort, characterize motor performance and critical design parameters under salient Mars operating conditions (as to material, structural, and electrical issues, e.g., DTCE effect on close-tolerance interfaces, and discharge-arcing-breakdown phenomena in reduced atmospheres)

- Develop and demonstrate scaleability/extensibility of these low-mass, high-power-density component technologies to: a) rover-borne 3-d.o.f.-or-greater light, strong dexterous micro-arm operations (avoiding often iterative and time-consuming “macro-micro” rover-arm positioning issues that arise in 2-dof implementations); and later, b) lander and/or rover based sub-surface sampling devices. As a specific design targets: define, develop, and demonstrate in cooperation with related Telerobotics Program tasks on Lightweight Survivable Rover (LSR) and Long Range Science Rover (LRSR) two appropriate, technically related serial “MicroArms” for sample cache collection and sample selection/acquisition respectively (nominal 0.5-1.0 meters, 1-2 kg, 2:1 or better payload-to-mass -- we discuss design details below in “Technical Approach”). Finally, as an out-year objective, through innovative concept development and quantitative lab/field experimentation, demonstrate possibility of a rover subsurface coring mechanism (0.5- 1.0 meter penetrability) based in all-composite design, including, as for the first lander-arm above, a means of self-inclusive volume stowage efficiency (nested linkage, flexural segments, etc.). Benchmark with respect to penetrability versus mass, wear characteristics, controllability (modal/flexural issues).

Justification

With its return to the Martian surface for detailed study phases by Mars Surveyor  (’98, ’01, ’03, ’05), NASA/Code S and the international science community has undertaken an aggressive program to investigate evolutionary climate, life, and geological resources. Activities will include frequent observational/physical interactions with the planetary surface:

- close-up viewing (micro-camera and/or imaging spectroscopy)

- analytic probing (point spectrometer or meteorology sensor)

- instrument emplacement (APX, NMR, Mossbauer spectroscopy)

- material extraction (trenching, abrasion, coring and granulation)

- sample-exposure-acquisition-manipulation-and-containment

- (grasp-and-turn-for-view, pick-and-place, expose and collect)

- As shaped by interests of the Mars science community  (e.g., Mars Science Working Group - MarsSWG), the range of Mars Surveyor Program (MSP) mission activity spans the upcoming Mars ’98 polar lander (MVACS) to a 2005 sample return – with rover based sample collection & caching activities part of the planned ’01 and ’03 mission sets. The landed mission budgets are tightly capped, respecting “FCB” and frequent launch access. Robotics, essential to enabling envisioned lander and mobile science sample collection and analysis, will confront fundamental challenges in mass/volume/power reduction, while also addressing requirements for dexterity, survivability, operational range (rover) – all relative to nominal daily communication downlinks from unstructured, highly variable physical environments. There does not at present exist enabling technology solutions to span envisioned MSP requirements.

- The goal of this task is to provide breakthrough robotic solutions for known Mars community surface/near-sub-surface sampling interests, working within projected MSP mission design constraints. Beyond this MSP-driven goal, we more generally seek to develop a core of reference robotic component technologies – robotic concepts, actuators, material processes, and benchmarks – that may provide a baseline for mass/volume/power (m/v/p) efficient sampling on other solar system bodies. Mars Surveyor ’98 landed science package constraints are illustrative of challenges that will drive these developments – the NASA MSP’98 Science AO/PIP guidelined a nominal mass/volume/power/cost budget of 20 kilograms/70 liters/25 watts (day average)/$20M. Of the proposing science teams, most allocated 20-40% resources to robotics. It was generally held that lander deck robotic science operations should be 5Kg/10L(stowed)/10+ W at 20-30% of science cost with two or better meter reach to the near field. The ’01 mission m/v allocation is potentially greater (under AO/PIP specification process at this time), with far greater expectations of the long range sampling rover facility it is intended to host. Thus, there are significant pressures to develop cost efficient, very lightweight, highly volume efficient robotic mechanization, as this task has already done for Mars’98. NASA has now committed aggressively to rover-based missions for ‘01-05 (and after) Mars explorations. It is thus imperative maximize the strength, dexterity, and function of rover sampling assets relative to mass/volume allocable to on-board science instrumentation and cached samples. Further, MarsSWG interests in climate, life, and resource  in situ analysis span the equatorial to polar (> -70 S) regions over both changing seasons and full diurnal cycles -- potentially subjecting robotic components to extreme temperatures (<-100 C) and thermal shock, as well as atmospheric, chemical and radiative/UV degradation considerations. We note that recent Mars ’01 science planning more strongly emphasizes fresh rock exposure, and we are accordingly including appropriate sampling arm effector concepts/devices are explored in our FY98 task work.

- Approach

- We have broken our attack on the task objectives into three closely linked technical thrusts -- robot design & mechanization, advanced robotic materials, and advanced actuation – each strongly supported through an accountable JPL disciplinary line organization and their facilities. As we subsequently note, this cross disciplinary JPL team has initiated a number of productive interactions with the Mars science community (universities and JPL), NASA educational programs, and visiting scholars. The task is in its third year of activity, and we outline a plan that will conclude efforts in FY99, approximately in alignment with earliest projected MSP ‘03/’05 technology “freeze” points. We present the task sequentially, briefly recapping relevant prior FY95-96 outcomes, detailing our ongoing FY97 work as motivation for further advances, next describing the proposed FY98 main thrusts, and finally summarizing further directions and rationale for FY99 developments. In the later section, “Deliverables,” we list corresponding yearly Level 1 Milestones.

- FY95 work developed a lightweight, stowable lander-manipulator, MarsArmI -- a 3-dof lab gas-deployable segmented (telescopic) composite link robot. MarsArmI uses brushed DC terrestrial motors, highly geared planetary/harmonic metal joints, and a one-dof actuated “back-hoe” style effector for open face trenching. We demonstrated (coordinated motion) open loop joint position control of trenching, end-of-arm viewing, and point spectroscopy.

- FY96 work made major advances in arm (and rover!) enabling component technologies.  We developed MarsArmII, similar to MarsArmI, but 40% lighter and having a pitchable wrist d.o.f. and powered effector, the entire assembly of all-composite construction.  The multi-function effector includes an opposable jaw gripper (>10 in-lb at tip), micro-viewing camera, and active abrader (DC-motor driven) for shallow exposure of fresh rock. The enabling composite, developed within this task, and also utilized to create the LSR-1 vehicle (Lightweight Survivable Rover task), utilizes an air lay-up post-impregnated 3D carbon fiber and resin transfer mold process, tunable in thermal/mechanical properties by choice of matrix (epoxy, polycyanate, etc.). This “black Aluminum (JPL/Caltech NPO filed)” composite is machinable through conventional metal shop practice, enabling formation and integration of complex, mass-versus-stiffness optimized parts.

- Recent & ongoing technology developments of the Planetary Dexterous Manipulators task

[technology illustrations include related MET/Sample Acquisition user interfaces developed in support of the PDM task]

Above, center: MarsArmII (FY96), the four degrees of freedom, four kilogram, two meter sampling robot prototype of all-composite construction developed in FY96 as a Level 1 task objective (cf. October 18, 1996, task report to TRIWG, and NASA Telerobotics Program 4Q96 Web page quarterly report). From upper left, counter-clockwise: science user interface for visually designating/selecting a particular task action (e.g., "trench along this line & angle, grasp this designated object, locate the arm camera here, etc."). A pseudocolor rendition of the 3-D terrain profile in the arm's workspace, as acquired by stereo cameras. The MarsArmII all-composite end-effector (FY96). Thermal testing of a low mass, high-torque density ultrasonic motor (FY95-forward) for which servo and closed loop inverse kinematics positioning controls were developed. MicroArmI joint assembly utilizing USM, in conjunction with machined 3D composite parts (FY96-97). Screen display of a finite element modeling tool for analysis and prediction of motor properties (FY96-forward), essential to design of the servo controls and drive electronics. A precursor MarsArmI (FY95) hybrid 2D composite-link/aluminum-joint robot prototype, performing simulated science operations. The MarsArmI concept was selected for NASA's Mars Surveyor '98 science sampling operations, as sketched upper right (ref: Mars Volatiles and Climate Surveyor; PI, Professor David A. Paige/UCLA)

A major feature of the FY96 MarsArmII deliverable was first integrated use and precision control of the ultrasonic motor (USM) rotary actuators being developed for space-qualified applications within this task. We employed three such 6 in-lb USMs in MarsArmII (a fourth 1 in-lb device being used in the wrist-effector pitch joint), and through the companion MET/Sample Acquisition task developed accompanying servo and inverse kinematics positioning controls. These controls included visually referenced operator designation in a stereo workspace of target sampling objectives, and a new higher-level task control architecture enabling dexterous, adaptive actions: This "Goal Oriented Behavior Synthesis (GOBS)" intelligent control architecture is a task-extensible approach to programming critical sampling skills such as trenching, rock sampling, close-up viewing, rock/soil probing, etc., wherein the robot in real time can sense and correct for its errors/uncertainties in position and force (see references). This provides task robustness, enabling the task to proceed autonomously, adjusting to a priori unknown environmental circumstances. As a specific example, MarsArmII was utilized under GOBS-based control to demonstrate adaptive impedance control for trenching, wherein the robot, discovering ground plane mis-positioning relative to visually designated location, changing soil simulant conditions, and/or hard sub-surface obstacles, recalibrated its local workspace and computed an adaptive trajectory to complete the trenching task. As later discussed, we are generalizing this capability for use in rover based dexterous sampling in FY97, to be first implemented on the FY97 Level 1 deliverable, MicroArmI (shown above in very early mock-up, mounted to LSR-1).  Relative to the FY96 USM developmental work itself there were several important advances achieved by a joint JPL/MIT (N. Hagood)/Quality Materials Inspection, Inc. (Costa Mesa, CA, based TCA partner) team: 1) created and characterized various rotary ultrasonic motors (USM) as potential mass/volume/power-optimal solutions for a wide class of planetary robotic (and small spacecraft) applications -- this entailed significant design, fabrication, and laboratory  test/trade studies of materials, emphasizing the critical rotor-stator interface design for high-efficiency wave transduction and thermal DTCE match; 2) conducted first cryo-vacuum tests of a USM, with successful operations to ~ -50°C; 3) developed a highly accurate FEM design tool (modal analysis, drive frequency, etc.), and validated in parametric experimental studies and in situ analytic/diagnostic electronic speckle pattern interferometry (ESPI); 4) demonstrated, with MIT, two new high-power motor designs, one reaching 16 in-lb in preliminary tests, a best result reported to date.

FY97 shifts the task focus from large lander arms to scaled-down rover-based sampling arms. As general goals, we seek to develop robots of 1-2 Kg class, .5-1.0 meter extent, and capable of as much lift/force-to-mass as possible. The general arm architectures remain like those successful to date in our MarsArmI&II/Mars’98 concept – three or four degree-of-freedom, serial link, independently actuated manipulators, incorporating as much end effector functionality as consistent with rover m/v/p constraints. There are two applications & integration foci for these small arm developments:  rover sample acquisition and rover sample return.  In the first area we are initiating development of an arm intended to become a prototype of a light sample selection & caching arm for the Mars ’01 and ’03 rovers. As such, members of this team are currently participating in a Mars’01 facility Rover Design Team (D. Shirley/MEPO). As discussed in more detail in the LSR (Lightweight Survivable Rover) FY98 task proposal, JPL is developing a next-generation “Rocky” series vehicle, to integrate end-FY98, utilizing a rover mechanical architecture derived from LSR efforts, and an arm deliverable from this PDM task. We demonstrate first concept with MicroArmI, the FY97 Level 1 task milestone deliverable. MicroArmI is a 4 d.o.f., all composite, commercial USM (1 in-lb Shinsei motor) driven manipulator of t/s/e configuration, pitchable wrist and multi-purpose, gripping end-effector. This arm – .7 meter full extent, 1.25 in link diameters, and 1.25 Kg mass -- will be mounted and demonstrated on the LSR-1 platform to perform the following science functions:

- sample acquisition and on-board storage (sample caching concept)

- trenching and digging of rover soils surrounding the rover

- sample manipulation including in-situ rock manipulation (flipping,

- turning,  etc.) and transfer of rock to on-board science instrument

- fresh rock exposure using an end-effector mounted Dremel-type

- tool (initial investigation into the feasibility of tool and method)

- close-up visual inspection of with end-effector mounted micro-camera

- integrated force feedback operations for sample handling tasks

The remote operator visually designates target samples within the MicroArm-I workspace through the stereo camera pair nominally used for the obstacle-avoidance aspect of rover navigation (we are investigating other more novel visual designation/servo/ranging concepts in the MET/Sample Acquisition task, including use of a novel JPL-developed arm-borne 3D imaging sensor).  Note that MicroArmI implements (computationally practical) inverse kinematics and force & task adaptive controls (via integral arm strain gauges),  by comparison to the prior simple forward joint controls of MarsArmI or Rocky7 sampling. We note also that the JPL/QMI first low temperature USM prototype (FY97 Level 2 deliverable) will be integrated into a joint of MicroArmI for a system operational FY97 demonstration/evaluation.

MicroArmI is a first-cut late-FY96/FY97 PDM technology development path for an all-composite small arm. We cited above two small arm applications, the second of which is Mars sample return. For this second function, the LSR task is developing a new concept light rover (FY97 Level 1 deliverable) requiring a very light and strong cache retrieval arm, one which can grow to perform basic opportunistic science. We are using this target application to drive next generation design of MicroArm technology, MicroArmII -- a development that leverages knowledge gained from the design and fabrication of MarsArm II, MicroArm I, and the MVACS flight arm for Mars’98 (which has provided useful guidance on DCmotor life cycle issues, tribilogy, and materials & structures limitations ). MicroArmII is like MicroArm I in that it has a t/s/e configuration (to initially incorporate a simple single d.o.f. end effector for sample cache grasping), but will be “in-line,” having no offset in the pitch joint.  The motivation here is to improve stiffness of the arm and address issues associated with cabling. The elbow joint and the shoulder joint will not be restricted severely in rotational range, with each approaching 270 degrees of range. MicroArmII is to be made of a new superior, stronger 3D composite substrate from SEP, for which we are currently developing a baseline process.  MarsArmII construction will introduce the use of “Flexprint” cabling instead of traditional round wire cables. Flexprint cables are made by directly depositing and etching copper on a substrate of Kapton (a high temperature polyimide plastic from Dupont extensively used in space) and then covering with another layer of Kapton. Rather than creating an external wire harness and routing it around and through the arm, the light and compliant flexprint cables (or circuits) will be bonded internally to the tubes and joint structure. This will result in up to 75 % reduction in mass of the cabling harness and connectors (which can be 25% or more of system mass), and provides for a safe integration of the flexprint to the arm structure. Beyond this integration of power and signal lines, we will explore flexprint as an R&D paradigm for implementing “smart sensors” (viz., integral and potentially distributed temperature, strain, vibration, etc., sensors to be used in conjunction with closed loop compensating controls) as well as distributed thermofoil heaters.  Such MultiFunctional Structures (MFS) will potentially create a new architecture for flight robots. This year our effort will be simple, integrating only cabling and force sensors into the MFS, but the experience base should help us in FY98-99 to expand the MFS emphasis into both arms and rovers. MicroArm II will be actuated by flight-relevant DC permanent magnet brushed motors that come assembled with a planetary gearbox and magnetic encoders. The output torque of the actuator assembly is almost 2 in-lb, driving directly into the same harmonic gears used in MicroArm I, and potentially providing payload:mass > 3:1. The MarsArmII design addresses the dusty nature of Mars, incorporating intrinsic mechanical seals into all rotating joints.  We note in summa ry that MicroArmII will provides the following capabilities on the forthcoming LSR sample retrieval rover (SRR/FY97): 1) retrieval of sample cache from an expired long-range science rover; 2) storage of the sample cache container on-board SRR; and 3) a stable mechanical platform for wrist mounting of the SRR goal-recognition camera.

We next outline our planned FY98 work, describing first the continued development of robotic sampling arms/effectors, then multiyear perspectives for the enabling technology thrust work in composite materials and high performance motors.

MicroArm architectures:  Two thrust areas within mechanism design will be addressed. The first area is design and fabrication of a flight-targeted, low mass, all-composite rover science arm and end-effector. The second area is exploratory development of synergistic arm-deployed, rock exposure mechanisms. Recall that the above-discussed MicroArm-II has only a simple gripping mechanism for purpose of acquiring a sample cache container. Extending this concept, we will develop a flight-relevant, multi-functional end-effector. This design will use integrated cabling/sensing technology developed for MicroArmII, will incorporate a means to generically grip (clam shell scoops), and offer a means of securely grasping/aligning mechanical interfaces of a deployed tool, instrument, cache container, etc. The related arm architecture -- to be finalized in cooperation with the end-user Long Range Science Rover task -- will be based in strengthened hybrid-2/3D composite designs and include trade studies on use of the new JPL high-torque, low-temperature USM actuators versus conventional DC motors (more below under these topical technology areas).  We will evaluate arm assemblies in relevant thermal environments. The integrated arm-effector will be called MicroArm-III, to be integrated with the LSR-derived Rocky 8 FY98 mechanical rover architecture.   The second thrust addresses the yet unsolved problem of exposing fresh rock for science investigation and sample collection. To address this problem, we will develop a high-dexterity, rock exposure mechanism (REM) that is deployed by a rover-mounted arm to the rock surface of interest. The mechanism will be mounted on the rover frame and power to the REM will be provided through a spring-loaded, retractable tether. The operational scenario for this mechanism is as follows: the robot arm grips the REM and guides the mechanism from its stowed location; the REM (now within the grasp of the robot arm's end-effector) is positioned against the rock sample of interest; once the rock exposure procedure is completed the REM is placed back in its stowed position by the robot arm. The actual mechanism for completing this rock exposure may possibly be a cutting tool (saw), a chipping mechanism, or an explosive (pyro) mechanism. There are several advantages to this operational approach. First, the robot arm is not continually burdened with the task of carrying around the REM and the cabling of the REM is kept separate from the arm cabling. Thus the effective functionality of the robot arm's end-effector is enhanced not by adding the REM to the end-effector itself, but instead by providing the ability to deploy the REM using the existing end-effector design. Secondly, the dexterity of the REM is enhanced. As opposed to rover-fixed designs, the REM can be positioned anywhere within the operating workspace of the rover-mounted manipulator. Finally, the feedback sensors that alread exist within the robot arm can be used to control directly the dynamics associated with the rock exposure event (sawing, chipping, etc.). The design of the REM will also include a collection basket that will collect the chips that fall from the cutting tool of the REM while sifting out any dust produced by the cutting process. These chips can then be transferred via the manipulator to a sample collection device located on the rover body for further science investigation. Personnel carrying out this work have participated/led in the seminal SAAP task study for Mars sample extraction, and are well-informed in the salient issues of tool fatigue & abrasives wear, cutting/impact design, actuation-power cycle optimization, etc. In addition to the design and fabrication of the REM, a study into the effect of the dynamics of the rock exposure event on the manipulator/rover system needs to be conducted. This study would need to insure that the forces and vibration environment experienced by the manipulator/rover system do not adversely effect the performance of the entire system. In particular we note our collaboration with the MIT/J. K. Salisbury task on force-referenced perception and control of sampling  device  interactions with hard/variable media including the fresh rock exposure; given the known difficulty of efficient coring/chipping extractions and challenges of power delivery from light, potentially compliant platforms/arms, the MIT mechanical design and controls expertise will be very valuable.

REM: corer, 1.0/.8 cm OD/ID, SiC or diamond tip, ~5.-1.0 Kg. Device is grasped by preloaded cones within the powered effector jaws.

MicroArm

CAD Rendering

Advanced materials: This work concentrates on development of novel environmentally robust 2 and 3-dimensional composite materials from which to realize the above discussed lightweight sampling arm, effector, and rover designs. There is an established technology base of high strength 2D composite processes which underlie the fabrication of arm links and rover struts in typical buckling and torsional loads. In FY96 we investigated and to some degree optimized such material systems as a function of different resins, forming processes, and cure cycles. Our testing to date using epoxy and polycyanate resins at both cryogenic and room temperatures revealed a modest (~10-20%) difference in relevant mechanical/thermal properties.  This minimal difference, along with task budget constraints, lead to the conclusion that this is a secondary line of research for FY97. Work in this area will re-intensify in FY98 as we evaluate flight-relevant environmental exposure effects  (primarily UV light and radiation). A much more central line of work is our development of a new class of 3D composite begun in FY96 – a machinable, tappable material that we believe will enable creation of new robots and rovers of higher structural rigidity at 20-50% lower overall mass. In FY96 we invented a first such prototype process based in resin transfer molding (RTM) of a B. F. Goodrich-furnished lower density 3D random air lay-up carbon fiber preform. Using this JPL developed material, we constructed many primary load-bearing components of both MarsArmII and LSR-1. During processing of the materials, we observed in some bulk process material samples lower than expected strength, and cracking/delamination/celling (cf. TRIWG 10/96 report for specifics). We believe, based on FY96 simple coupon tests, that there is a much better alternative in SEP "Novoltex (SPC)" high density material. We are developing and will characterize in FY97 -- for both initial mechanical and thermal environmental performance -- a new RTM process for this material.  If this material proves suitable, we will undertake a thorough FY98 mechanical characterization and process optimization of the material, also characterizing its salient UV/radiation effects. We emphasize that this 3D composites work is fundamental, in both the materials R&D sense and applications importance to robot and rover designs; we have found few external reports of related work. In passing we note that we have identified several new possibilities for 3D preforms, should the SEP process prove suboptimal. Another planned and potentially major advance in our 3D material development, is selective stiffening/strengthening by a directionally hybridized lamination of 1D/2D composites onto load paths of  3D orthotropic composite components. As a final note, we will characterize design trades between  more conventional aluminum (or titanium) versus 3D composite part forming.  This will demonstrate potential mass, interface DCTE matching, structural rigidity, and environmental stability gains.

Advanced actuator development: We are developing rotary ultrasonic motors (USM) as a hypothetically superior alternative to conventional DC brushless actuators. Rotary USMs are high-torque density piezoelectric devices of low fabrication complexity; major drive elements are formable in batch process; and, the underlying mechanization is conceivably survivable at extended life cycles over a full range of anticipated Mars ambient surface science conditions (operating in physical principle to near 0°K!). USMs have additional advantages of self braking (non-backdrivability), and inherently low operating speeds at high torques, thereby facilitating reduced gearing mass and making them very attractive for rover wheel actuation. As design targets, we seek to develop and demonstrate a piezoelectric rotary motor having 5-to-10x DC brushed motor torque-density, 10x reduced no-load speed, at comparable volume. As part of this effort, we will characterize motor performance and critical design parameters under salient Mars operating conditions (as to material, structural, and electrical issues, e.g., DTCE effect on close-tolerance mechanical/electrical interfaces, and discharge-arcing-breakdown phenomena in reduced atmospheres). We note that at this time, there are few design tools for the systematic analysis and synthesis USMs -- an important accompanying development of this task. Work is being performed as a JPL-led tripartite collaboration between Dr. Yoseph Bar-Cohen (JPL),  Prof. Nesbitt A. Hagood IV (MIT/Aero &Astro)., and JPL TCA partner Quality Materials Inspection, Inc. (Costa Mesa, CA). It is significant that QMI has begun the first US commercialized development of USM technology  (currently centralized in Japan for OEM electronics/appliance applications, and limited to nominal 1-to-6 in-lb motors of purely terrestrial relevance and simple open-loop controls). The collective JPL/MIT/QMI effort targets a rotary motor concept for space environmental application, in capabilities ranging 1-50+ in-lb (the highest reported result to date being ~15 in-lb, with no prior literature in space applications): technical challenges include conceiving new transduction architectures to increase motor torque density (JPL/MIT), developing a stator-rotor interface robust to DTCE and cryo-vac degradation effects (JPL/MIT), creating motor designs that reliably scale to larger size, e.g. 3” dia. and 15-30 in-lb upward (MIT), providing first analytic design framework for design of such motors (JPL/MIT), development of experimental tools for run-time in situ dynamic characterization of such devices (MIT, cf. electronic pattern speckle interferometry--EPSI) and innovation of batch manufacturing processes and supporting driver electronics that will carry higher-power commercial USMs to broad-scope, inexpensive application (QMI/JPL).  JPL has conceived a novel, patent-pending reverse segmented and stacked USM design for ferroelectric poling of the USM stator; this concept has potential to greatly increase torque-density (up to x10) and largely alleviate differential temperature effects on motor interface mechanical stability (thought to be the primary degradation/failure mode for experimentally observed low temperature efficiency loss and seizure). Further, the new JPL motor design is amenable to batch process in readily available low cost commercial piezoelectrics (compare Japanese manufacturing processes using more costly single re-poled piezoelectric rings). JPL has now a patent pending for this invention, which we have recently reduced to practice in one first-cut prototype. Further, JPL/MIT has developed a dynamic finite element model (FEM) of a USM – enabling simulation of the piezo-mechanical structure upon which flexural traveling wave modes are electrically transduced to interact frictively with the rotor, exciting bi-directionally controllable rotation at variable speeds.  This enables a significant characterization/design/visualization of modeled motor operating frequencies, modal properties, relative conversion efficiency, etc. (cf. the earlier photo of “technology development”). In FY95, we developed the experimental foundations for space ultrasonic motors, including the design of a cryo-vac test system and we started to develop a finite element analytical model for interactive design of USMs; with MIT we demonstrated/evaluated a novel larger dual-rotor USM design concept.  In FY96, we completed the development of the Mars relevant cryo-vac system and its programming to allow measuring torque vs. speed as a function of temperature down to 120°K and vacuum down to 2x10-2 torr.  A commercial 1-in-lb Shinsei motor (USR-30E3) was tested and was found to maintain a relatively uniform operation characteristics to stall at -48 deg. C  under vacuum.  This is a surprising, important result; Shinsei does not rate motors for use in vacuum or below -10 °C, thus giving preliminary evidence that the inherent DTCE mechanical stability of USM designs may be adequate to achieve very low temperature operation. Further, we developed the above-noted key analytical design tools for USM and showed excellent agreement with EPSI interferometric measurements that were made at MIT, thus showing an important closure between our new theoretical design tools and experimental validation/optimization.  In FY97 the emphasis is on making a first baseline space-targeted 1-in-lb USM that will be installed on the MarsArmI and testing its operation at low temperatures and vacuum.  Efforts are made to employ stack actuators while overcoming frequency limitations that are associated with the inherently increased electric capacitance. In FY98 our USM development program focuses sharply to analytically and experimentally characterizing the low temperature failure modes of USMs, and those mechanical/electrical factors that would be causal in limiting motor life cycle in particular temperature/environmental regimes (including ambient atmosphere). Work on small low-temperature USMs targeted to robot/rover application will include objectives of: 1) 50% torque increase over the FY97 small arm motor design (through model-based design optimization and experimental trials); 2) development and demonstration (with QMI, Inc. TCA partner) of miniaturized drive electronics consistent with flight m/v/p constraints; 3) definition of a survivable USM design concepts for a Rocky8-long range rover/Mars’01, and LSR-sample retrieval rover; and 4) same, for applications for rover-borne rock exposure and subsurface coring/drilling applications (for which USM have potentially very attractive operating curves). FY99 work will continue this effort to fabrication, thermally validated testing, and demonstrations of USM items “3” and “4”.

At task level, the FY99 work will concentrate on extending the successful base of arm/effector lightweight, survivable component technologies in this task to broader sampling science objectives. A pivotal development, much emphasized in recent science working group discussions (cf. Mars Drilling Workshop, JPL/Caltech, Jan 28, 1997, Org.: J. Cutts) would be creation of means for sub-surface sampling from a rover. We will develop a low mass (1.5-3.0 kg) all-composite USM actuated drilling/coring architecture for (.5-2.0 meter stroke) applications, designing new USM-based force-velocity servo controls, and characterizing control structure interactions as can be expected from such a low mass, flexural body, high torque, low speed system. If successful, this architecture (given its component environmental robustness as proven in prior years of task activity) should be of broad space science applicability. Working in a complementary direction, we will continue the “REM” corer/chipper system miniaturization (.5-1.0 kg). Finally, based on component level concept work to be initiated in FY98, we will investigate approaches to mechanizing more agile underground sample recovery technique. Such dexterity strategies -- e.g., a segmental borer (viz., individually actuated inch-worm articulated links as one reference concept) -- wherein, given unpredictable subsurface soil densities, rock inclusions, porosity, permafrost characteristics, etc., sampling can proceed unhindered to significant depths (1-2 meters). This highly articulated manipulator design, as deployable from a lander or larger rover, would have inherently good stowage characteristics, and offers intriguing opportunity for both onboard science instrumentation and multiple sample containments, and offers a functionally complementary direction of R&D from Subsurface Explorer tethered underground vehicles.

Deliverables

FY95 Level 1: develop a three degree of freedom arm [composite links, metal joints] capable of two meter full extent reach and stowage volume reduction into a nominal 10 liter space (via gas deployable segmented links). Demonstrate  the arm in simulated Mars lander operations such as sample acquisition [May 95]

FY96 Level 1: conceive, design, fabricate, and experimentally test a new lightweight, high-strength all-composite three degree of freedom robotic arm. Demonstrate the arm in representative lander-based dexterous sampling tasks, and quantify mechanical and material properties of the new enabling composite technologies. As a design goal, reduce arm mass approximately 30% from a similar hybrid composite-body/metal-joint arm architecture [Aug 96]

FY97 Level 1: develop, integrate, and evaluate under task-level controls an all-composite arm, utilizing new JPL/QMI-MIT high-torque ultrasonic motor designs, advancing 3D RTM composite strength and reliability, and characterizing the enabling motor and material technologies under realistic space environmental conditions. Demonstrate this arm in representative dexterous sampling and sample-return processing tasks [Sep 97]

FY98 Level 1: develop, demonstrate, and experimentally evaluate a lightweight, flight-targeted micro-sampling arm (1.0+ kg/.6 m class) based in low-temperature (- 60-100 degrees C) actuation and 2-D/3-D hybrid high-strength composite architecture, and characterizing functional elements (e.g., an operative all-composite arm joint/link complex) in relevant thermal environments. This development will include: 1) design, implementation, and test of a multi-function science end-effector with active tooling for fresh rock exposure; 2) integration of a high-fidelity task positioning control (visually referenced); 3) instrumentation for closed loop force response (via embedded strain sensors); 4) realistic demonstration in science-advised scenarios for fresh rock exposure/extraction from a light rover platform [Sep 98]

FY99 Level 1: conceive, develop, and demonstrate in relevant simulated media a low mass (1.5-3.0 kg), composite body, USM-actuated subsurface drill/corer architecture for rover-borne sub-surface sampling applications (0.5-2.0 meters), innovating as needed new USM-based force-velocity servo controls, and characterizing control structure interactions as can be expected from such a low mass, flexural body, high torque, low geared system [Sep 99]

Resource Requirements

Year		FY95		FY96		FY97		FY98		FY99

Budget 		600		550 (600)	700 (400)	750* (600)	600 (600)

Wkyrs		2.7		2.2		1.5		3.0		2.0

*  actual (prior TRIWG plan):  the increased FY98 request over plan is to support this task in its previously unplanned role as sole source developer of robotic arms for both the LRSR-Rocky8 and sample LSR-Sample Retrieval Rove) vehicles during FY98-99, the first as enabling  technology staging  for a MSP’01 rover design.

Facilities, Partnerships and Additional Participants

Facilities: currently, Bldg. 277 Mars Robotics Lab as pictured page 1; now under development for early summer ’97 completion, the Planetary Robotics Lab., a new 2400 sq. ft. facility for coordinated robotic sampling and rover R&D

Participants (JPL): Mechanical Systems Engineering and Research Division; Avionics Systems and Technology Division

Science:  collaborations ongoing or to date with Mars’98/UCLA (Prof. David A. Paige, PI for Mars’98/MVACS science payload); R. Singer-UAZ/D. Blaney-JPL/S. Kim-JPL (arm-based science imaging and spectroscopic instrumentation) Mars ’01/Rover Design Team (cf. Lynn Lowry,/J. Plescia for sampling science requirements & architecture definition); JPL In Situ Exploration and Sample Return Center (J. Cutts)

Grants/Contracts: MIT/Aero-Astro: Professor N. Hagood IV (solid state actuators); MIT/AI-Lab & Mech. Engrg. - J. K. Salisbury (vision and force-based perception & control of sampling)

Educational Outreach/Programs: we have hosted two university co-ops (C. Voorhees/RPI and Emily Norman/T&AM-JSC). Dr. Eric Baumgartner (Michigan Tech), NASA-ASEE Summer Faculty Fellow in CY’95 developed our early visually based sample acquisition techniques (cf. Baumgartner & Schenker, IEEE ICRA’96) and joined JPL full-time summer ’96 as system engineering lead for this task. Dr. Gerard McKee (Univ. Reading) continues his yearly visitations with us to work on task-driven sensor fusion and controls.

Dual Use Potential

We have ongoing collaborations with QMI, Inc. (actuators) and B. F. Goodrich Co. (composites), as noted above. Five patent submissions are pending/in-process.  Seagate and Teleflex have approached us for further collaborations on USM-derived actuators. Composites technology developed in this task have become the adopted JPL basis for rover mechanical architectures of both the Long Range Science Rover and Lightweight Survivable Rover tasks. These collective technologies are basis for our ongoing proposal of a significant military field robotics effort.

Schedule (Level 1 Milestones in Bold)

Collapsible MarsArmI concept	FY95, Q2

Hybrid 2D composite-link/Al-joint arm								FY95, Q3

JPL/MTU Visually servoed sampling (CSM)							FY95, Q4

MarsArmI lander demo (joint control)								FY95, Q2 (MSWG) & Q4

MIT dual rotor USM										FY96, Q1 & Q3 (rev1)

JPL 3D low density RTM composite (BFG-“ALC”)							FY96, Q2

USM (Shinsei) low temperature tests								FY96, Q2

JPL/MIT  USM FEM & parametric experimental studies						FY96, Q3

All composite joint and link fabrication								FY96, Q3

MarsArmII demo (IK controls/stereo sample designation)						FY96, Q4

JPL high density RTM composite (SEP-“SPC”)							FY97, Q2

MicroArmI low density composite build								FY97, Q2

JPL/QMI reverse segmented USM & low temperature tests						FY97, Q3

MicroArmII high density build for LSR-SRR cache retrieval						FY97, Q4

MicroArmI rover-based dexterous sampling demo						FY97, Q4

Hybrid 2D/3D high strength composites								FY98, Q2

Characterization of composite environmental exposure factors						FY98, Q3

Rock Exposure Mechanism (REM) concepts & development						FY98, Q3

MicroArmIII flight concept arm build (“flexprint”/sensors)						FY98, Q3

Low temperature JPL segment/stacked USM & integrated arm test					FY98, Q4

MicroArmIII effector/REM integration and rover science demo					FY98, Q4

USM-based actuators for rover wheel and sub-surface applications					FY99, Q3

USM extended life cycle operations/optimization							FY99, Q3

Collapsible composite body corer/drill architecture definition						FY99, Q3

Rover based subsurface sampling/control demo							FY99, Q4

References

See P. Schenker, end-FY96 task report to TRIWG, October 16-18, 1996, JPL/Caltech, Pasadena, CA and FY97 proposal.