A Continuing Baseline University Research Program

The Physics-Based Design, Planning and Control of Robotic Systems for Space

Professor Steven Dubowsky, PI

Room 3-469a

Department of Mechanical Engineering

Massachusetts Institute of Technology

Cambridge, MA 02139

Tel: 617-253-2144

Fax: 617-258-7881

email: dubowsky@mit.edu

I. INTRODUCTION

Future robotic systems will perform important tasks in space, in orbit and in planetary exploration, see Figure 1.    Based on our interactions with the technical staff members of several NASA laboratories and centers we have identified several physical characteristics inherent in space robotic systems that would limit their capabilities.    In this continuing baseline university research program we are developing new design methods and control and planning algorithms to enable future space robotic systems to overcome these limitations and meet their future mission objectives.    The underlying intellectual focus of the program is to construct a set of integrated design, planning and control techniques based on an understanding of the fundamental mechanics of space robotic systems.

a. An Orbital System	b. A Planetary Exploration System

Figure 1.    Two Space Robotic System Concepts

The work has two main thrusts.    The first is the development of new design and planning algorithms for planetary exploration robotic systems.    The second is the development of very high performance control algorithms for space manipulators for orbital and planetary applications.    In both cases our work tries to build upon a fundamental understanding of the physics of these systems.    This document will briefly describes the current and future directions of our research.

II.    TASK OBJECTIVES AND GOALS AND JUSTIFICATION

The objectives of our research is to develop techniques that will enable space robotic systems to meet their mission objectives.    Specifically, our work in physics based design, planning and control methods for mobile robots, such as planetary rovers and for the high-performance control of manipulators is described below.    It should also be noted that because this is a fundamental research program, new concepts and directions that offer promise to contribute to the technical development of robotic systems for space will also be pursued as they arise.

The Physics-Based Design, Planning and Control of Planetary Robotic Systems

Based on our interactions with JPL and other NASA centers, we have identified several important areas that require fundamental study to permit the long-term development of space robotics technology.    To be very effective, rovers will need to be designed to:

a.	 Be very small, lightweight and power-efficient.

b.	Be able to function in very poorly characterized and very physically challenging environments, such as in dense fields of large obstacles, ultra-soft soil, deep crevasses, or very low gravity.    The difficulties to be encountered      may not be well known until well into the mission.

c.	Have the capability to reconfigure themselves to meet unexpected obstacles or to extract themselves from unexpected traps or hang-ups.    These systems will need to be smart, agile, and maybe even athletic.    In our research we have shown that even nominally similar environments can present quite different difficulties to a given system configuration. (See Figure 2.)

d.	Adapt or recover from component or subsystem failure or degradation in capabilities.

e.	Develop model-based methods to enable the ability to plan their behavior through, over, and around large and complex obstacle fields.

f.	Develop new configurations and concepts, such as articulated systems with or without legs or combinations of legs, wheels, skids, and tracks, the use of “finite mechanical state machines,” and bearingless parallel kinematic structures with distributed actuation.

To meet these challenges, our work will perform the following specific tasks:

Task 1. 	Modeling:    Develop analytical physics-based models for on-board use to enable robots to more effectively plan their own actions based on on-board models. (See Task 3.)

1.1	Develop models to enable systems to “predict” what they can do and cannot do successfully in a given situation based on the fundamental mechanics of the situation.

1.2	Apply modeling methodologies to a class of robots to be selected in cooperation with JPL, and evaluate these models using experimentally available test results.

a. A Trench Following Task	b.    A Step Task

Figure 2.    A Robot Configuration Attempting to Negotiate Two Somewhat Similar Terrains

1.3	Refine the modeling method to meet the limitations of anticipated on-board computational resources.

Task 2. 	The Design Methodology Planetary Robots, including On-Site Reconfigurable Robots.

2.1	Development of design methodologies that will allow planetary rovers to physically adapt themselves so as to be appropriate for the terrain/situation they face. (Self repair, self re-planning, trap recovery, physical adaptability).    This work would be done in support of a program in reconfigurable rovers being proposed by JPL.

2.2	The exploration of new hardware configurations and subsystems concepts for very lightweight, yet effective designs.    The application to on-site reconfigurablity will be a major consideration in this work.

2.3	Develop design techniques that will reduce planetary system’s development times and costs.

Task 3.    Planning Algorithms for Planetary Robots.

3.1	Development of model-based planning algorithms, for pre-planning and design, that will allow rovers to fully utilize their mobility capabilities in challenging tasks and environments and enable them to aggressively perform their tasks.

3.2	Development of algorithms that will permit systems to self-construct navigation plans for physically challenging environments and tasks, based on local sensory data and on-board models.    A typical mission such as a planetary sample return mission will be used to evaluate these methods.

3.3	Extend planning algorithms to compensate for system component failures or degradation.

The High-Performance Control of Space Manipulators

In our previous work we have developed a number of control algorithms that can significantly improve the performance of robotic manipulators.    These algorithms can compensate for physical effects that can seriously degrade the performance of space robotic systems.

Two of the most important of these effects for space manipulators are the dynamic interactions between space manipulator and its spacecraft or supporting structures (see Figure 1a) and/or between a mobile manipulator and its vehicle (see Figure 1b), and the friction and nonlinear characteristics in the actuators and transmissions of the manipulators joints.    The vehicle and the supporting structure dynamic interactions are very important in space systems because the use of light-weight structures and vehicles greatly enhances their degrading effects.    The same need for light-weight systems forces the use of high gear ratio transmissions that increases friction and nonlinear effects.    This problem is further exacerbated by the limited lubrication and/or lubrication seal friction demanded by the vacuum of space.

We have recently developed a very effective method for compensating for nonlinear actuator and transmission effects in position controlled manipulators called BAST Control, as discussed in section III.    Here we would extend this to a manipulator in contact tasks with the environment.    An important example would be to apply this to the Master Arm of telerobotic system to achieve a “virtually frictionless” master, a highly desirable system for bilateral force feedback applications.    We would then consider combining this method with our previously developed methods for the dynamic interaction problem.    These methods include the Virtual Manipulator, the Coupling Map and Inferred End Control (ICE): See Appendix A.    An example of a mission requiring such capabilities would be a planetary mobile manipulator performing a delicate scientific task (such as grasping/ flipping a rock) autonomously, in unstructured environments--while operating from highly flexible vehicles (See Figure 1b.    In connection with this task we will study the manipulation of highly irregularly shaped objects in contact with the environment.    Such control function would be required for such tasks as packing of rocks for a sample return mission.    We believe that the methodology to be developed in this task would be quite promising for this problem.    The specific tasks for this phase of the work would be:

Task 4.	Development of Friction Compensation Control Algorithm for Force Control Tasks.

4.1	Develop BAST Control algorithm for force control tasks.

4.2	Apply BAST Force Control to produce a “Virtually Frictionless” telerobotic master arm.

4.3	Study the fundamental character of force control from compliant bases.

4.4	Develop force control algorithms for compliant bases.

4.5	Apply BAST force control to the manipulation of irregular objects in contact with stiff environments.

4.6	The experimental Demonstration of BAST Control using a vehicle mounted manipulator.

III. TECHNICAL APPROACH

The Physics Based Design, Planning and Control of Planetary Robotic Systems

This work will be very closely coordinated with a proposed research program in reconfigurable robots proposed by Paul Schenker, and his team at JPL.    Our research approach is based on our hierarchical search methodology for modular system design []..    (See Figure 3)    Modular design has been suggested by several researchers for conventional industrial manipulators [, , ].    We have extended it to field robots and provided an analytical framework for this application.

In this method systems are assumed to be assembled from a finite set of hardware components.    In a similar manner their plans are assembled from a finite set of “action modules” to make an action plan [].    In the method an inventory of physical modules and realizable actions, called action modules, are assembled to produce a physical configuration and an action plan [].    The sequencing of modules is accomplished using a hierarchical selection process which includes a genetic algorithm.    This selection process exploits the fundamental physics of the task to effectively search for the proper configuration and action plan.    The method has been successfully demonstrated in the context of a number of example tasks [].    For reconfigurable systems the configurations that can be considered by the selection algorithms must be obtained without the use of an external agent.    This can be achieved by having the system contain additional means of actuation to perform this reconfiguration.    In the simplest sense one might envision a system that lowered wheels to run on relatively flat terrain.    These would be raised and legs lowered to climb obstacles.    A more ambitious design might be able to achieve a radical change in form, almost like children's “transformer” toys of a few years ago.    The system might be viewed as a mixed finite state and conventional mechanical system [, ].

Figure 3.    Modular Design and Planning Concept

The High-Performance Control of Space Manipulators

The basis of the high-performance control that will be developed in this research is the BAST control (BAse Sensor Torque control) method that we have been developing [, , ].    The method is able to eliminate the effects of joint friction to yield very accurate position control, even when the manipulator needs to perform very delicate (small amplitude and slow motion) tasks.    Previous methods have suggested the use of either complex modeling and identification techniques, or expensive and delicate torque sensors that must be integrated into the manipulator. This paper proposes a simple, cost-effective method for compensating the effect of joint friction, which utilizes a six-axis force/torque sensor mounted on the manipulator’s base. (See Figures 1a and 1b and Figure 4.)

From base wrench measurements, the joint torques are estimated and fed back through a torque controller, that virtually eliminates friction and gravity effects.    (See Figures 5 and 6.)    With such high-quality torque control, a simple PD position controller is sufficient to provide high precision motion control even at very low speed and small motions.    Our experimental results, for an industrial manipulator with high Coulomb friction in its gear trains, show the effectiveness of the method. The achieved precision is substantially greater than for conventional methods and approaches the resolution of the system’s encoders.    In this program the method would be applied to mobile systems where the vehicle motions may effect the sensor signal.    It would also be extended to force control task and the manipulation of highly irregularly shaped objects in contact with the environment.

| Fig. 4.A mobile manipulator with Base Sensor. | Fig. 5. BAST Control Block Diagram |
| --- | --- |

Figure 6.    A Schematic Diagram of the BAST Control Algorithm Computation

IV. DELIVERABLES

The results of the work will be documented in semiannual progress reports.    The specific results to be obtained are:

The Physics-Based Design, Planning and Control of Planetary Robotic Systems

1.    Design methods for generating task-driven configurations from sets of possible configurations.

2.    Metrics for evaluating planetary rover performance in a given class of task.

3.    Analytical prediction models and simulations of rover robots in representative planetary environments and tasks.

4.    Measures of improved performance for rovers using on-board performance prediction models.

5.    A quantitative evaluation of modular robot designs performed by 3-D quasi-static and dynamic simulations on representative planetary exploration missions.

6.    A quantitative evaluation of re-configurable robot designs performed by 3-D quasi-static and dynamic simulations on representative planetary exploration missions.

7.    A comparison of analytical performance prediction with available experimental and field data.    Ideally, data from actual missions would be available for model and method assessment.

The High-Performance Control of Space Manipulators

1.    Design methods for BAST force control.

2.    Metrics for evaluation of BAST force control for stationary base and mobile manipulators.

3.    Measures of improved performance for BAST control over conventional force control based on experimental data.

4.    Experimental evaluation of BAST control for mobile systems using the VES II experimental system.

V.    FACILITIES, PARTICIPANTS AND ADDITIONAL PARTNERS

In this work the research will be done in close collaboration with the staff of JPL.    The partnership has grown substantially during the past year.    A graduate student from our group has spent a summer period at JPL.    We have had visits and seminars at MIT by our JPL partners.    In addition there have been several meetings at JPL during the past year for detailed technical discussions and plans for new work.    One result of these discussions is the JPL proposal on reconfigurable robots that will, to some degree, implement our technical approach to this problem.

In addition we will maintain our active intellectual interchanges with researchers at Tohoku and Tokyo Institute of Technology in Japan and Robotics Laboratory of Paris and several other countries.    Finally we will continue our close relationship with the Salisbury and Brooks NASA-sponsored activities at MIT.    In this regard we have established a “Mini-Triwg” series where the students from the three groups make informal presentations at a meeting between the students and faculty of the three groups.

In terms of facilities, this work will involve substantial experimental work in our laboratory.    The unique VES II system will be used to emulate the mobile manipulator experimental studies required by the program.

VI.    BUDGET

The projected cost of the program for the following four years are:

| FY ‘98 | FY ‘99 | FY ‘00 | FY 01 |
| --- | --- | --- | --- |
| 175,000 | 190,000 | 205,000 | 220,000 |

VII.    DUAL USE POTENTIAL

The work on reconfigurability and modular robotic systems has important dual use potential.    The work would make the design and deployment of field robots for important civilian applications much more cost-effective.    These non-space applications include: decommissioning facilities and equipment from out-of-date nuclear facilities, performing inspection repair and maintenance of roadways and bridges, and the preservation of historic sites and monuments, such as the USS Constitution.    The work in ultra-high performance manipulations has many industrial applications.    We are currently applying such techniques to a world-class cancer treatment facility being built at Harvard University’s Massachusetts General Hospital.

VIII.    MILESTONES AND SCHEDULE

The schedule for the ‘98 FY of the program is as follows:

| Task | Quarter I | Quarter II | Quarter III | Quarter III |
| --- | --- | --- | --- | --- |
| Task 1. Develop analytical physics-based models. | ____________ | ______________ | ______________ |  |
| Task 2. Design Methodology for the Design and On-Site Reconfigurable Rovers . |  |  | ____________ | ______________ Continuing |
| Task 4.1 Develop BAST Control for force control. |  | __________ | ______________ | ______________ Continuing |
| Task 4.2. Demonstrate a “Virtually Frictionless” telerobotic master arm. |  |  |  | ____________ Continuing |
| Plan Activities for FY ‘99 |  |  | --------------------- |  |

APPENDIX A

Papers, Invited Lectures, Visiting Scholars and Student Theses for the Past Three Years

This program is an academic research program.    To some measure the success of the program should be measured by the scholarly papers produced, the students educated and international scholars trained by, and who have contributed to, the program.    The impact of our program in these areas is documented in this appendix.    It includes papers published, student theses and invited lectures from the past three years that are based on, or based in part on, our NASA-sponsored research.

Scholarly and Professional Papers Published during the Past Three Years

Dubowsky, S., Durfee, W., Kuklinski, A., Muller, U., Paul, I., and Pennington, J., “The Design and Implementation of a Laboratory Test Bed for Space Robotics: The VES MOD II,” Proceedings of the 23rd ASME Mechanisms Conference, Minneapolis, September 11-14, 1994. Also accepted for publication in ASME Journal of Mechanical Design, 1994.

Torres, M.A., Dubowsky, S., and Pisoni, A.C., “Path Planning for Elastically-Mounted Space Manipulator’s Experimental Evaluation of the Coupling Map,” IEEE International Conf. on Robotics and Automation, San Diego, CA, May, 1994.

Sunada, C., Argaez, D., Dubowsky, S. and Mavroidis, C., “A Coordinated Jacobian Transpose Control for Mobile Multi-Limbed Robotic Systems,” IEEE International Conference on Robotics and Automation, San Diego, CA, May 8-13, 1994.

Dubowsky, S., Durfee, W., Corrigan, T., Kuklinski, A. and Müller, U., “A Laboratory Test Bed For Space Robotics: The VES Mod II,” IROS ‘94 IEEE/RSJ/GI International Conference on Intelligent Robots and Systems-Advanced Robotic Systems and the Real World, Seeped, 1994, Müchen, Germany.

Corrigan, T. and Dubowsky, S., “Emulating Micro-Gravity in Laboratory Studies of Space Robotics,” Proceedings of the 23rd ASME Mechanisms Conference, MN, Sept. 1994. Also accepted for ASME Journal of Mechanical Design,1994.

Dubowsky, S., Moore, C., and Sunada, C., “On the Design and Task Planning of Power Efficient Field Robotic Systems,” Proceedings of ANS 6th Topical Meeting on Robotics and Remote Systems and ANS Trains, Monterey, CA, Feb., 1995.

Dubowsky, S., Cole, J., Rutman, N. and Sunada, C., “Mobile Autonomous Robotics Systems for Unstructured Environments - With Applications to the USS Constitution,” Proceedings of the NSF Design and Manufacturing Grantees Conference, San Diego, CA, January 4-6, 1995. Invited.

Cole, J., Dubowsky, S., Rutman, N. and Sunada, C., “The Application of Advanced Robotics and Sensor Technologies to the Preservation of the USS Constitution,” Proceedings of the Conference on the Technical Aspects of Maintaining, Repairing and Preserving Historically Significant Ships, September 12-14, 1994, Boston, MA. Invited.

Pisoni, A.C., Santolini, C., Haug, D. and Dubowsky, S. “Displacements in a Vibrating Body by Strain Gauge Measurements,” Proceedings of the 13rh International Modal Analysis Conference, February 13-16, 1995, Nashville, TN.

Dubowsky, S. “Dealing With Vibrations in the Deployment Structures of Space Robotic Systems,” Proceedings of the 5th International Conference on Adaptive Structures, in Sendai, Japan, January 1995. Invited.

Mavroidis, C., Dubowsky, S. and Raju, V., “End Point Control of Long Reach Manipulators Systems,” Proceedings of the Ninth World Congress on the Theory of Machines and Mechanisms, September 1-3, 1995, Milan, Italy.

Yoshida, K., Mavroidis, C. and Dubowsky, S. "Experimental Research on Impact Dynamics of Spaceborne Manipulator Systems," Proceeding of the 4th International Symposium on Experimental Robotics, June 30-July 2, 1995, Stanford, CA.

Mavroidis, C., Rowe, P. and Dubowsky, S., “Inferred End Point Control of Long Reach Manipulators,” Proceedings of IEEE International Conference on Intelligent Robots and Systems, IROS 95, Pittsburgh, PA, August 1995.

Torres, M., Dubowsky, S., Corrigian, T., and Pisoni, A. C., “The Coupling Map Method for Path Planning of Long Reach Space Robotics Systems With Experimental Evaluation,” accepted for the IEEE Transactions on Robotics and Automation.

Yoshida K., Mavroidis C. and Dubowsky S., "Dynamics and Control of a Space Robot Capturing a Floating Target (Experiments of Impact Estimation)," Proceedings of the 1995 ISAS 5th Workshop on Astrodynamics and Flight Mechanics, August 1995, pp.108-114.

Yoshida K., Mavroidis C. and Dubowsky S., "Experimental Estimation of Impact Dynamics of a Manipulator using Extended-Inverse Inertia Tensor," Proc. of the 13th Annual Conf. of Robotics Society of Japan, Nov., 1995 (in Japanese).

Farritor, S., Dubowsky, S. Rutman, N. and Cole, J., “A Systems Level Modular Design Approach To Field Robotics,” Proceedings of the 1996 IEEE International Conference on Robotics and Automation, Minn., MN, April 24-27, 1996.

Morel, G. and Dubowsky, S., “The Precise Control Of Manipulators With Joint Friction: A Base Force/Torque Sensor Method,” Proceedings of the 1996 IEEE International Conf. on Robotics and Automation, Minn., MN, April 24-27, 1996.

Torres, M. A., Dubowsky, S., Pisoni, A. C., “Vibration Control of Deployment Structures of Long-Reach Space Manipulators: The P-PED Method,” Proceedings of the 1996 IEEE International Conference on Robotics and Automation, Minneapolis, MN, April 24-27, 1996. pp. 117-132.

Yoshida, K., Mavroidis, C., and Dubowsky, S., “Impact Dynamics of Space Manipulators Mounted on a Flexible Structure,” Proc. of the 3rd Inter. Conf. on Dynamics and Control of Structures in Space, London, England, May, 1996.

Morel. G. and Dubowsky, S., "High Performance Control of Manipulators Using Base Force/Torque Sensing," Proceedings of XI CISM-IFToMM Symposium on Theory and Practice of Robots and Manipulators, Udine, Italy, July, 1996.

Mavroidis, C., Dubowsky, S. and Thomas, K., “Optimal Sensor Location in the Motion Control of Flexibly Supported Long Reach Manipulators,” Proceedings of the 1996 ASME Mechanisms Conference, Irvine, CA, August 19-22, 1996.

Farritor, S. and Dubowsky, S., “On the Design of Rapidly Deployable Field Robotic Systems,” Proceedings of the 1996 ASME Conference on Design Methodology, Irvine, CA, August 19-22, 1996.

Flanz, J., et al., “Design Approach for a Highly Accurate Patient Positioning System for NPTC,” Proceedings of the PTOOG XXV and Hadrontherapy Symposium, Belgium, September, 1996.

Mavroidis, C., Dubowsky, S., Drouet, P., Hintersteiner, J., and Flanz, J., “A Systematic Error Analysis of Robotic Manipulators: Application to a High Performance Medical Robot,” accepted for the Proceedings of the 1997 International Conference in Robotics and Automation, Albuquerque, NM, April, 1997.

Farritor, S. and Dubowsky, S., “A Genetic Algorithm Based Navigation and Planning Methodology for Planetary Robotic Exploration,” accepted for the Proceedings of the American Nuclear Society Sixth Topical Meeting on Robotics and Remote Systems, Augusta, GA, April, 27- May 1, 1997.

Iagnemma, K., Morel, G. , and Dubowsky, S., "A Model-Free Fine Position Control Method Using the Base-Sensor: With Application to a Hydraulic Manipulator," accepted for Proc. of 1997 IFAC Symp. on Robot Control, France, Sept., 1997.

Invited Lectures

Given as part of our technology transfer activities outside of NASA, the following are invited lectures based on our NASA research given during the past three years:

September 1994, "The Application of Advanced Robotics and Sensor Technologies to the Preservation of the USS Constitution," Boston National Historical Park, Charlestown Navy Yard, Charlestown, MA.

October 1994, "Space Robotics Technology and its Applications to the Preservation of the USS Constitution," University of Wisconsin-Madison, Dept. of Mechanical Engineering, Madison, WI.

November 1994, "The Control for Mobile Multi-Limbed Robotic Systems," University of B.C., Vancouver, Canada.

November 1994, "On the Dynamics and Control of Space Robotic Systems," Simon Fraser University, School of Engineering Science, Burabay, B.C., Canada.

November 1994, "On the Dynamics and Control of Long Reach Space Robotics Systems," University of Victoria, Vancouver, Canada.

December, 1994, "Dealing with Vibrations in the Deployment Structures of Space Robotic Systems," Sendai, Japan.

December 1994, "On the Dynamics and Control of Space Robotic Systems," Tokyo Institute of Technology, Japan.

December 1994, "On the Dynamics and Control of Space Robotic Systems," Kawasaki Heavy Industries, Japan.

January 1995, "On the Dynamics and Control of Long Reach Robotic Systems, with Application to Space Station Alpha and the USS Constitution," UCLA, Dept. of Mechanical, Aerospace and Nuclear Engineering, Los Angeles, CA.

February, 1995, "On the Design of Mobile Telerobotic Systems in Unstructured Environments, with Application to the Restoration and Preservation of Historic and Artistic Treasures and Sites," American Nuclear Society Sixth Topical Meeting on Robotics and Remote Systems, Monterey, CA, February 5-10, 1995 (Keynote Address).

October, 1995, “The Dynamics and Control of Long Reach Manipulators,” Laboratoire de Mécanique des Solides, Université de Poitiers, Poitiers, France.

November 1995, "The Dynamics and Control of Robotic Manipulators," Robotics Laboratory of Paris, Université Pierre et Marie Curie (Paris VI).

January, 1996, "The Modular Design of Robotic Systems Using Genetic Algorithms,” Laboratory of Fundamental Computer Science and Artificial Intelligence, INRIA Rhône Alps, Grenoble, France.

March, 1996, "The Design of Rapidly Deployable Systems Using Modular Methodology," The Engineering Design Center, Engineering Department, Cambridge University, Cambridge, England.

June 1996, "Some Observations on the State-of-the Art of Robotic Systems," Sandia National Labs., Albuquerque, NM.

July 1996, "The Control of Flexible Structure/Robot Interactions," Sandia National Laboratories, Albuquerque, NM.

July 1996, "The Ultra-High Performance of Robotic Manipulators Using Base Force/Torque Sensing," Sandia National Laboratories, Albuquerque, NM.

September 1996, "Research in the Field and Space Robotics Laboratory," d’Arbeloff Laboratory, MIT, Cambridge, MA.

December 1996, "The Manipulation of Highly Irregularly Shaped Objects, With Application to Semiconductor Wafer Manufacture," SEH Corporate Research and Development Center, Isobe, Japan.

December 1996, "The Modular Design of Planetary Rover Systems," Department of Mechanical Engineering Science, Tokyo Institute of Technology, Tokyo, Japan

December 1996, "Methods for the Rapid Design, Programming and Fabrication of Field Robots," NTT Corporate Research and Development Center, Tokyo, Japan.

December, 1996, "Control Systems for Very High Performance Long Research Manipulators," Department of Aeronautics and Space Engineering, Tohoku University, Sendai, Japan.

December, 1996, "Ultra-Precise Control of Industrial Robotic Manipulators," Honda Corp. Research Labs., Wako, Japan.

December, 1996, "The Base Force Sensor Control of Manipulators," Mitsubishi Development Dept., Nagoya, Japan.

December, 1996, "Sensor Control of Long Reach Manipulators," Toyota R and D Center, Nagoya, Japan.

January, 1997, "The High-Precision Control of Robotic Manipulators Using Indirect Sensing," Department of Mechanical and Aerospace Engineering, Rutgers State University, New Brunswick, NJ.

Student Theses

Following is a list of student theses that have resulted from NASA-sponsored research during the past three years.    In addition approximately 12 undergraduates have had research experiences in, and contributed to, this NASA program under the MIT UROP (Undergraduate Research Opportunity Program).

Sunada, C., "A Unified Control Approach for Multi-Limbed Mobile Robots: Coordinated Jacobian Transpose Control, " MS Thesis, 1994.

Corrigan, T., "The Development and Application of Methods for Emulating Micro-Gravity, " MS Thesis, 1994.

Morel, G., "Progammation et adaptation de l’impédance de manipulateur au contact," Ph.D. thesis, University of Paris (VI), 1994.

Rowe, P.S., "Endpoint Control of Elastically Mounted Robotic Systems: Simulation and Experimentation," BS thesis, 1995.

Thomas, K., "Design of the Supporting Structure for a Laboratory Long Reach Manipulator System Using Finite Element Analysis," BS thesis, 1995.

Ford, S. , "An In Situ Keel deformation Metrology System for the USS Constitution," BS thesis, 1995.

Rutman, N. , "Automated Design of Modular Field Robots," MS thesis, 1995.

Cole, J., "Rapid Generation of Motion Plans for Modular Robotic Systems," MS thesis, 1995.

Raju, V., “Design of an Experimental Systems for Modular Robotic Systems," BS thesis, 1996.

Cassenti, L., “A Computer Simulation of a Mars Rover System," BS thesis, expected 1997.

Garcia-Rubio, P., “The Design of a Gripper for Irregularly shaped Objects," BS thesis, expected 1997.

Bevly, D. , The Control of Modular Robotic Systems," MS thesis, expected 1997.

Iagnemma, K. , “Ultra-Fine Manipulator Force Control," MS thesis, expected 1997.

Farritor, S., The Design of Modular Field Robotic Systems," Ph.D. thesis, expected 1997.

Visiting Scholars

During the past three years the following international visiting scholars have contributed to this NASA program.    For almost all, the support for these scholars was provided by their own institutions.

Pisoni, Attilio Carlo

Ancona University, Italy

Prof. Yoshida, Kazuya

Tokyo Institute of Technology, Japan

Dr. S. Rudolph

University of Stuttgart, Germany

Dr. Guillaume Morel, EDF

Chatou, France

Dr. Phillipe Bidaud

The CNRS Robotics Laboratory of Paris, France

Phillipe Drouet

Ecole Nationale Supérieure d’Arts et Métiers, France

Dr. Guang Jun Liu

University of Toronto, Canada

EFERENCES

.Farritor, S., Dubowsky, S., Rutman, N., Cole, J.,   "A Systems Level Modular Design Approach to Field Robotic Systems," IEEE International Conference on Robotics and Automation, 1996.

Paredis, C. J. J., Brown, H. B., Khosla, P. K., "A Rapidly Deployable Manipulator System," in Proceedings of 1996 IEEE International Conf. on Robotics and Automation, April 22-28, Minn., MN, pp. 1434-39, 1996.

Paredis, C. and Khosla, P., "Synthesis Methodology for Task-Based Reconfiguration of Modular Manipulator Systems," Proceedings of the International Symp. on Robotics Research, Hidden Valley, PA, Oct. 2-5, 1993.

Tesar, D., and Butler, M., "A Generalized Modular Architecture for Robot Structures," Manufacturing Review, 1989. vol. 2, no 2.

Brooks, R., "A Robust Layered Control System for a Mobile Robot," IEEE Transactions on Robotics and Automation, vol. 2, no. 1, 1986.

Farritor, S., Dubowsky, S., "A Genetic Algorithm Based Navigation and Planning  Methodology for Planetary Robotic Exploration," ANS Conference on Robotics and Remote Systems.

Farritor, S., Dubowsky, S., Rutman, N.,  "On the Design of Rapidly Deployable Field Robotic Systems," ASME Design Engineering Technical Conference, 1996.

Pamecha, A. Chirikjian, G.,  "Design and Implementation of Metamorphic Robots," IEEE International Conference on Robotic and Automation, 1996

I.-M. Chen and J.W. Burdick, "Enumerating Non-Isomorphic Assembly Configurations of Modular Robotic Systems," Proceedings of IROS, Yokohama, Japan, pp. 1985-1992, 1993.

Morel , G. and Dubowsky, S., “The Precise Control of Manipulators With Joint Friction: A Base Force/Torque Sensor Method,” Proceedings of the 1996 IEEE International Conference on Robotics and Automation, Minneapolis, MN, April 24-27, 1996.

Morel. G. and Dubowsky, S., "High Performance Control of Manipulators Using Base Force/Torque Sensing," Proceedings of XI CISM-IFToMM Symposium on Theory and Practice of Robots and Manipulators (ROMANSY ‘96), Udine, Italy, July 1-4, 1996.

Iagnemma, K., Morel, G., and Dubowsky, S., "A Model-Free Fine Position Control Method Using the Base-Sensor: With Application to a Hydraulic Manipulator", accepted for the Proceedings of the 1997 IFAC Symposium on Robot Control, Nantes, France, September, 1997.