Space Operations

FY98 Continuation Proposal

University of Maryland
Space Systems Laboratory

NASA Telerobotics Intercenter Working Group
March 1997

Objectives:

The goal of this program is to investigate the roles and limitations of telerobotics for future space operations. This is predicated on the development of complete telerobotic systems (incorporating manipulation, mobility, vision, sensors, communications, and interfaces), and the operation of these systems on realistic space tasks in a meaningful simulation environment (neutral buoyancy). This approach also facilitates a second overall goal, which is to perform research into possible productive roles for cooperative interactions between robotics and astronauts, both EVA and IVA. Within these goals, specific objectives for this research program include

•	developing a data base on telerobotic performance in a simulated space environment that will be of use to planners of future space missions;

•	exploring paradigms of telerobotic operations beyond those currently envisioned, including integrated human/telerobot worksites and novel telerobot system configurations;

•	advancing the state of the art in space operations simulation, with particular emphasis on neutral buoyancy, and including both analytical modeling techniques and neutral buoyancy -specific instrumentation.

Background:

The need for better and more capable ways to operate in space will lead to a number of future systems, which will ultimately range from completely manual activities to completely automated ones, with room for every feasible mixture of humans and machines in between those extremes. The SSL has undertaken the development of a number of telerobotic devices capable of a variety of space operational tasks, in order to extend the experimental data base into the realm of automation and robotics. The current research mixture of the SSL emphasizes telerobotic operations, both in comparison to and in cooperation with human operations on the same tasks.

A number of key decisions were made in the process of designing and developing the current SSL telerobot systems. These decisions profoundly affected the experimental approach of this research, and in turn have led to the capabilities and limitations of the present systems. These design decisions included:

•	The use of    integrated systems – to obtain the most complete and comprehensive data possible, the decision was made to develop complete telerobotic systems, including the capability for unlimited 6DOF motion control, manipulation (as required), and all necessary auxiliary systems, such as vision and other sensing. This decision grew out of prior experience in the development of research versions of the manned maneuvering unit, and reflected a serious interest in understanding the human control implications of full telerobotic systems in a high-fidelity simulation of space operations.

•	Neutral buoyancy simulation – for meaningful simulations of manipulative space tasks, it was necessary to model the weightless dynamics of the space environment. The choice of neutral buoyancy simulation offered the potential for more meaningful correlation of the telerobotic operations results with that of pressure-suited subjects in the same environment, allowing direct comparison of humans and machines performing identical tasks. A good deal of high-fidelity hardware exists from satellite development efforts and crew training in neutral buoyancy; this hardware forms a ready-made set of future applications for the study of telerobotic satellite servicing.

•	Design for initial teleoperation, with provisions for system upgrades – It was recognized immediately that one of the prime reasons for the existence of these research telerobot systems was to gain knowledge about the operational use of telerobots, rather than just how to design and build such a system. Since the operational implications were not apparent a priori, the decision was made to make the hardware as capable as possible initially, but with the potential to upgrade the software for increasing levels of system autonomy as experience indicated such a capability would be useful.

The Beam Assembly Teleoperator (BAT) was designed to assemble the same structure used by the Space Systems Laboratory for the Experimental Assembly of Structures in EVA (EASE) program. EASE involved two pressure-suited subjects repeatedly assembling a six-element tetrahedral truss, and included both neutral buoyancy simulation and a shuttle flight experiment flown on STS 61-B in late 1985. By choosing as a design case to assemble this same structure, direct comparisons could be made between EVA and telerobotic assembly, as well as correlation to the flight experience. This structure was designed to be challenging for EVA assembly; no major modifications in the structure were allowed for simplifying the task for robotic assembly. Thus, BAT was designed from the outset to be as capable as EVA for this one specific assembly task, and generically capable of a variety of other EVA tasks as well.

Although the Beam Assembly Teleoperator has a fully capable free-flight mobility unit, it was primarily designed to investigate teleoperated manipulation tasks. Since the initial research objective for MPOD was to investigate the role of different types and fidelity of sensory feedback to the human operator, it was designed around a cockpit for internal flight control. Thus, the same task (such as docking to a satellite mockup) can be performed onboard and teleoperated for direct comparison of task performance in the different control modes. The only external payload on MPOD in its reference configuration is an instrumented docking probe, used for rigidly docking with a matching drogue which may either be fixed, or mounted to a 4000 pound satellite mockup for unrestrained docking tests. MPOD has also been used as a simulated Astronaut Support Vehicle: in this configuration, the docking probe is replaced by an EVA work station with foot restraints, and the MPOD mobility system is used to provide positioning for the EVA subject.

Many telerobotic activities require multiple views to give the remote operator a true picture of the current state of the work site. To investigate the possible use of supplementary free-flying telerobots to provide such views, the SSL developed the Secondary Camera And Maneuvering Platform, or SCAMP. SCAMP is much smaller than the existing SSL telerobots, and is optimized for the sole purpose of positioning a video camera at arbitrary points around the neutral buoyancy work site. As part of SCAMP development, much work took place on innovative thruster concepts, and on an integrated package design that promotes serviceability and quick turn-around for routine use. SCAMP has been used intensively during the past year, both for data collection on multiple telerobots and as a standard source of high-quality video for both vehicle operators and visitors to the NBRF. It has fulfilled its goal of being an excellent training device for new telerobot operators, and its low maintenance and quick test preparation protocol have resulted in its continual use for all NBRF operations.

Although all of the SSL telerobotic vehicles have free-flight capability, there are many tasks in space operations which are better done from a more fixed work site. In order to investigate the roles of restrained positioning, the SSL has developed a modular positioning manipulator. This system uses three rotary manipulator joints and interchangeable variable length linkages to provide three-degree-of-freedom positioning up to fifty feet away from the base fixture. In addition, a six-DOF Stewart platform is used in place of a classical wrist, to provide both three rotational degrees of freedom and three translational degrees of freedom for fine position control. This unit is called the Stewart Platform Augmented Manipulator, or SPAM. SPAM was removed from active research during FY97.

The Space Systems Laboratory has a long history of research into space human factors. Maintaining basic capabilities in human factors testing is important, in both that it provides a control for telerobotic capabilities, and since many applications of robotics assume the use of a microgravity operator control station. In recent years, SSL human factors research has concentrated on the development of a data base for the design of microgravity control stations, based on neutral buoyancy simulation. A system of three 6 DOF force-torque sensors was developed to measure input and resultant loads to a human operator in neutral buoyancy simulations. This system has now been applied to both humans and telerobotic systems in neutral buoyancy, and will be the basis for future human factors data base collection.

Justification:

It is important to develop advanced telerobotic technology. It is equally important, if the telerobotics program is to have a real and lasting effect on the nation’s space program, to understand and quantify the efficacy of telerobotic systems in performing necessary tasks in space. While some of this research can be performed in a laboratory setting, the maximum degree of knowledge can only be obtained by fully replicating the future space telerobotics worksite. This requires a complete telerobotic system, operating on a high-fidelity space task in a medium which realistically simulates the dynamics of the space environment. Neutral buoyancy does have drawbacks related to both the water drag dynamics and the difficulty of designing and operating hardware in that environment; however, no other simulation approach allows the same degree of fidelity, or the complexity of the task environment to approach that commonplace in space.

Approach:

The approach adopted for this program is to develop integrated telerobotic systems for the neutral buoyancy simulation environment, and test them against both fiduciary tasks and high-fidelity simulations of future space operational requirements. Primary emphasis in FY98 will be on application of the existing telerobotic systems to obtaining publishable data on operations performance, and on preliminary planning and research to facilitate the next generation of neutral buoyancy vehicles. Specific activity areas are planned to include:

Modification of Base Grant Research Directions: Over the past year, some significant changes have occurred to the overall research funding of the Space Systems Laboratory. These are associated with additional support which significantly leverages base grant funding to existing research topics, including Telerobotics Program specific support to Advanced Controls Systems Development and SCAMP Development programs. Details of planned activities under these spin-off activities are contained in their renewal proposals. Activities in FY98 in these areas under the base grant will be limited to targets of opportunity for applications of the advanced controls systems technology, and the utilization of SCAMP for activities both at the NBRF and at other remote sites to enhance test operations.

Advanced Development Activities: A significant effort will be made in the coming year to develop advanced technologies to improve the program's ability to perform advanced space operations.    In actuality, advanced technology development under the Space Operations topic is restricted to a few particular areas of SSL expertise, such as advanced servo-level control, human-in-the-loop control, and advanced instrumentation systems for human factors research and for underwater simulation. These technologies are developed in theory and benchtop proof-of-concept systems; they are then ported onto the neutral buoyancy vehicles for understanding their impact on system performance in realistic tasks and environments. As part of this research area, advanced instrumentation, navigation, and flight control systems are under development to increase the fidelity of the neutral buoyancy environment, and to actively overcome inaccuracies in the simulation medium, such as the effects of hydrodynamic drag. This section of the Space Operations topic also includes a significant effort in space human factors, both to understand human capabilities as the ultimate comparison metric for robotics, as well as to better understand the design of human interfaces for telerobotic control, both ground-based and in microgravity.

Significant among the planned technology programs is an effort to reduce the size of Ranger-type vehicle without significant reduction of capabilities. The goal of this effort is a vehicle capable of performing all of Ranger's task set, with a physical size no more than half that of the current vehicle, and a target goal of 1/3 the size of Ranger NBV. Immediate tasks in this area will be the design, prototyping, and bench testing of a high-torque, low volume manipulator actuator. While the eventual goal of this development is an integrated vehicle, the technology will also be directly applicable to research opportunities such as adding small manipulators to SCAMP.

A second technology effort will be aimed at the next generation positioning manipulator system, to replace SPAM. This will include the development and testing of a long-reach underwater linear actuator, along with the design of a high-torque rotary actuator capable of modular use throughout the length of the coarse positioning manipulator. These systems will use electromagnetic actuators, replacing the water hydraulic systems of SPAM which led to the extreme maintenance requirements of the older system.

Development and Evaluation of Integrated Systems Concepts: This segment of the research topic deals with the development and testing of integrated telerobotic systems. Rather than studying manipulation by stacking blocks on a laboratory bench, this research topic addresses specifics of how to perform realistic tasks, such as changing out Hubble Space Telescope components, in a simulated weightless environment (neutral buoyancy). This approach provides a sharp focus for the technology research, as well as providing valuable lessons on what it takes to bring disparate technologies (manipulators, mobility systems, control interfaces, software architectures) together into a functional system to perform operational tasks. In some cases, the proof-of-concept systems developed in this task (SCAMP, Ranger preliminary designs) have led to focused research topics designed above. In other cases, results from system testing have demonstrated significant limitations to the concept, and the test vehicle was quickly retired or recycled into a subsequent development effort. Another unique feature of this research section is a significant focus on human-robotic cooperation. Results from the past decade of research has unequivocally indicated that the most productive system for performing space operations is a mixture of on-site humans and telerobots, each performing the tasks to which they are best suited.

Cooperative Human-Telerobotic Interactions: With the completion of Ranger NBV, a significant capability now exists to examine cooperative roles for humans and machines in a unified work site. In FY98, the SSL will utilize Ranger and SCAMP to examine cooperative roles for humans and machines, both EVA and IVA. IVA research will focus on simulated science operations from Spacelab or Space Station, and will be performed at the University of Maryland Neutral Buoyancy Research Facility. EVA cooperative tasks will draw from both existing structural assembly tasks in the SSL and future spacecraft servicing tasks. This research will be performed at the NBRF utilizing the Maryland Advanced Research/Simulation (MARS) Suit (developed under internal funding over the last two years), with correlative activities using shuttle Extravehicular Mobility Units (EMUs) at the NASA Marshall Space Flight Center and the NASA Johnson Space Center.

Development of Performance Metrics: This ongoing research area deals with understanding the significant parameters affecting the applicability of telerobotic systems to space operations. It has included the development of work station instrumented to measure force-torque interactions, along with the design of tasks which provide basic data on telerobotic performance. These approaches will be combined with more traditional timeline studies in the performance of high-fidelity space operations tasks, to measure system performance at a variety of levels simultaneously. This same system will continue to also be used for measuring human performance, to better understand the relative performance metrics between humans and machines.

Data Base Development and Documentation: Over its two decades of existence, the Space Systems Laboratory has performed hundreds of test operations researching the capabilities and limitations of humans and telerobots for performing space operations tasks. While the most significant results have been documented in technical journals and conference proceedings, there is a plethora of useful data for someone interested in better understanding performance of space operations. During FY96 and FY97, the Space Systems Laboratory made a concerted effort to document this extremely large data base on space operations, using an HTML format to allow the development of extensive hypertext links throughout the body of data. By the summer of 1997, the first version of this data base will be available to the outside world via the World-Wide Web. In FY98, this data documentation process will continue to expand the percentage of past SSL research results available, as well as to incorporate new research results in a timely manner. In addition, in FY98 the first effort will be made to incorporate non-SSL data (either directly or through active links) to drive towards the ultimate goal of a useful comprehensive data base on space operations.

Participants:

The primary participant in this research is the University of Maryland Space Systems Laboratory. Shared activities are planned with the NASA Johnson Space Center (NBL operations) and the NASA Marshall Space Flight Center (NBS operations.)

Milestones and Deliverables:

FY 1997:

•	Development and testing of a neutral buoyancy flight control system with full six-axis compensation for hydrodynamic effects

•	Publication (via the World Wide Web and CD-ROM) of the first version of a hypertext-based data base on human and telerobotic performance in space operations tasks

•	Lab-based demonstration of advanced technologies for robotic augmentation of specific body joints of EVA subjects

•	Continued testing of Ranger NBV to quantify effects of human-telerobot cooperation in space operations tasks

FY 1998:

•	Development of a prototype system for virtual reality simulation of EVA operations including neutral buoyancy simulation of microgravity

•	Advanced technology testing for creating a Ranger-class servicing vehicle at 1/3 the current size and 20% the current mass

•	Release of version 2.0 of the hypertext data base on space operations experience

•	Testing of microgravity work site design parameters based on an underwater force reflecting hand controller

Commercialization Potential:

In FY94, a test series was conducted at the SSL Neutral Buoyancy Research Facility, supported by Global Outpost, Incorporated. This test series demonstrated the use of SSL-developed technologies, including autonomous flight control of MPOD and teleoperated control of SPAM, in the manipulation and berthing of a DOE-supported space nuclear reactor concept. Global Outpost is interested in developing a commercial space facility for contracted space operations, using many of the technologies developed by the SSL. (Global Outpost is a participant in the Technology Assistance Program of the State of Maryland, and is located on the University of Maryland campus.) Talks also took place in FY94 with companies involved in undersea technologies and operations, including Oceaneering, Subtechnics, and Schilling.

Funding:

Base funding for this project in FY96 is set at $500,000. A top-level breakdown of this cost is as follows:

Cost Element	Cost ($K)

Labor	213

Overhead	147

Fabrication and Materials	101

Equipment Procurement	56

Travel	29

Administrative	4

Facilities:

Principal location for this effort will be the Neutral Buoyancy Research Facility of the Space Systems Laboratory at the University of Maryland. Additional tests will be performed at the NASA Marshall Neutral Buoyancy Simulator.

Principal Investigator:

Dr. David L. Akin
Director, Space Systems Laboratory
Associate Professor, Department of Aerospace Engineering and Institute for Systems Research
Building 382
University of Maryland
College Park, MD 20742

phone 301-405-1138
fax: 301-314-9738
email: dakin@ssl.umd.edu