Real-time Learning Controllers for Enhanced

Autonomy and Reliability of Space Robots

FY98 Continuation Proposal

University of Maryland

Space Systems Laboratory

NASA Telerobotic Intercenter Working Group

March, 1997

Objectives:

The primary objective of this joint NASA/NSF program is to evaluate and further develop new technology enabling free-flying and free-floating space robotic systems to maintain accurate, high performance operation despite large uncertainty about the physical characteristics of the robot and its payload, or about the structure of the environment in which they interact. This will be accomplished by using a new class of adaptive nonlinear control algorithms which use neural network and nonparametric wavelet estimation techniques to significantly extend conventional adaptive solutions for uncertain robotic systems. The specific top-level goals for this program include:

• Development of a complete, provably effective methodology for using real-time wavelet network modeling techniques to allow spacecraft and space robots to autonomously maintain peak performance in all operating modes and space environments.

• Demonstration and quantification of the performance of the new technology on simulated space operations using prototype spacecraft and space robots operating in neutral buoyancy.

• Improvement of neutral buoyancy simulation of space operation by using the new controllers to offset the poorly modeled hydrodynamic forces which hinder accurate correlation of space telerobotic operations with terrestrial simulations.

Background:

Reliable, high performance control of free-flying and free-floating space robots (and their underwater counterparts) presents challenges well beyond those encountered in more traditional space vehicles.    The complexity of these devices, and the necessity of ensuring accurate operation across their entire operating range, requires addressing directly the inherently coupled, nonlinear nature of the dynamics of these systems. While satisfactory high gain linear feedback controllers can be designed for a given performance envelope, treating the nonlinear coupling as disturbances to be rejected, such a strategy inherently requires conservatively large control authority, thus requiring large amounts of electric power and reaction mass, hence increasing the overall launch mass and cost and/or decreasing the useful lifespan of the system. Robust nonlinear control strategies, using conservative bounds on the effects of the nonlinearities on system performance, suffer similar drawbacks.

On the other hand, due largely to previous efforts of the NASA telerobotics community, the general structure of the coupled dynamics of these systems is by now well understood, as is the structure of exact nonlinear control laws which will permit precise tracking of any user-specified trajectory for the manipulator and its mobile base without excessive control authority. As model-based algorithms, however, these controllers can be sensitive to the exact mass properties of the manipulator and its mobile base; errors in the estimates of these parameters may produce degraded operational performance.    Careful prelaunch measurements can reduce the amount of initial uncertainty, however the mass properties of the system will change each time it grapples a new object or expends fuel mass to maneuver. To move beyond carefully scripted experiments, in which the mass parameters of a serviced satellite are known precisely beforehand and all fuel used is carefully preplanned, the control algorithms of space robots must at least be capable of reconfiguring themselves to accommodate arbitrary, perhaps unexpected, changes in the mass properties of the system.

Such an algorithm has been available for almost ten years in fixed-base robotic applications, and has been extensively utilized in numerous academic and industrial applications. Unfortunately, these standard adaptive robotic algorithms require a highly structured characterization of the uncertainty in the dynamics of the system. This characterization is naturally available when the mass properties of a fixed-based manipulator are poorly known;    however for free-floating space robots, in which the attitude and position of the manipulator base is left uncontrolled, the dynamic uncertainty arising from poorly known mass properties does not conform to the required structure, thus precluding the use of fixed-base adaptive methods. When the orientation and position of the base of the robot are also actively controlled, a structured uncertainty model can again be obtained, but since uncertainty on the manipulator mass properties will also produce uncertainty in the base attitude dynamics, an adaptive manipulator controller must in this situation be appropriately coupled with an adaptive attitude control algorithm, which is itself an important current research topic.

Perhaps more significantly, even in fixed-based robotics there are additional sources of uncertainty in the dynamics which cannot be appropriately characterized for the standard adaptive control techniques, especially when the physical source of the uncertainty is itself variable or poorly modeled, such as friction or cogging.    Space and planetary robots, which by design are operated in hostile and potentially unknown environments, inherently contain a high percentage of this kind of weakly structured    dynamic uncertainty.    Earth orbiting robots, for example, must contend with dynamic perturbations due to thermal cycling, gravity gradients, atmospheric drag, solar pressure, and geomagnetic torques, all of which are at best approximately modeled.    Planetary and interplanetary robots will contend with similarly complex operational environments for which even approximate models may not exist.    Finally, damage to the system will cause substantial, unpredictable changes to the structure of the dynamic response of the robot which cannot be accommodated by standard adaptive methods.

In order to increase the reliability and autonomy of space robots, it is thus necessary to develop control algorithms capable of accommodating these kinds of relatively unstructured (and unexpected) dynamic uncertainty. By appropriately merging new techniques from artificial intelligence and nonparametric estimation with both the adaptive nonlinear controllers used in fixed- base robotics and recently developed algorithms for adaptive attitude control, a new class of algorithms has been developed which provide space robots with just such enhanced adaptive capability.    These new algorithms provide an unprecedented level of adaptive operation, capable of automatically reconfiguring themselves to eliminate the effects of each of the uncertainty sources discussed above, and possess the same theoretical guarantees of accuracy and robust stability as the standard adaptive robotic controllers.

Just as a human experimenter might fit a curve through data collected about a poorly modeled physical phenomenon, so too these new controllers utilize highly flexible, online function approximation techniques to autonomously model unknown components of the system dynamics.    Similar to “neural networks” or “fuzzy logic” techniques in this regard, the algorithms contain however none of the ad hoc, trial-and-error prescriptions which often accompany the learning algorithms used in artificial intelligence. Instead, a precise mathematical description of the approximator used by the controller is included in the closed-loop equations of motion, and nonlinear stability theory is used to derive adaptation algorithms which will ensure globally stable closed-loop operation, and asymptotic elimination of any tracking errors.    These new controllers thus achieve the claims of neuro- and fuzzy control advocates, but retain the rigor, and hence formal guarantees of stability and convergence, found in conventional control law designs.

A variety of different approximation techniques can be used in these new control laws.    Indeed, standard neural network or fuzzy logic designs can be employed, replacing their usual training algorithms with those determined by nonlinear stability theory. Unfortunately, many of these designs encounter the well known “curse of dimensionality” of approximation theory, producing controller specifications which would be impractical to implement for a typical seven degree of freedom manipulator on a six degree of freedom mobile spacecraft base.    This drawback has been mentioned in both the scientific literature as well as previous TRIWG reviews.

The new control strategies developed for this task eliminate this defect by incorporating techniques from multivariate wavelet analysis and nonparametric estimation theory into the operation of an adaptive neural network. The resulting network employs a time varying number of hidden layer nodes, each of which dynamically changes the number and strengths of its connections to the signals in the input layer.    The controller thus adaptively varies the amount of computation it performs, attempting to simultaneously achieve high performance with minimum complexity. Although significantly more sophisticated than previous (fixed structure) neurocontrol designs, formal analysis shows that these designs provide the same theoretical guarantees of stability and effectiveness as previous algorithms, and can provide at least an order of magnitude reduction in the required computations.

Justification:

While the new technology has been proven effective, both mathematically and through detailed simulation study, its true potential can only be evaluated through comprehensive testing on actual free-flying systems. The major thrust of this task is thus to implement these new adaptive control algorithms on existing robots at the Space Systems Laboratory (SSL), and to determine precisely the performance which can be gained as compared to more traditional, nonadaptive approaches.    By studying the impact of uncertainty on current Ranger control algorithms, and by evaluating the ability of the new controllers to adaptively maintain nominal performance, this task will explicitly quantify the benefits of the new technology for NASA, and will give a “seeing-is-believing” confidence in the algorithm and its capabilities.

A successful demonstration of these new control techniques will potentially enable a new generation of “smart” spacecraft and robots, which can easily recover from partial failures and accommodate unexpected changes in their operating environments.    The high degree of reliable onboard intelligence afforded by the new controllers will be instrumental in creating the extremely robust, autonomous spacecraft envisioned for the 21st century. Somewhat more conservatively, the technology described can also be used in a passive mode, identifying unexpected changes in the behavior of the system and providing an updated dynamic model to ground personnel for evaluation and further action.

Finally, note that while neutral buoyancy simulation can be extremely useful in rehearsals of orbital operations, high fidelity correlation of orbital and neutral buoyancy data is impeded by the significant hydrodynamic effects present in neutral buoyancy operations. From the viewpoint of the new adaptive controllers which have been developed, however, the (poorly modeled) hydrodynamic effects appear simply as additional environmental perturbations which must be offset to ensure accurate operation.    Thus, exactly the same control technology which will provide more robust space robotic operations will also provide more accurate neutral buoyancy rehearsal and simulation of these operations, by automatically compensating for the interaction of the fluid dynamics with the robot equations of motion.

Approach:

The overall approach for the NASA funded component of this task is to collect data quantifying the “real-world” potential of these new adaptive control techniques.    This will be accomplished by implementing the new controllers in software on existing Space System Laboratory robots, specifically MPOD and Ranger/NBV (Neutral Buoyancy Vehicle).    Through experiments conducted in the SSL's Neutral Buoyancy Research Facility, MPOD will be used to evaluate adaptive attitude and positioning control, while Ranger/NBV will be used to demonstrate adaptive free-floating manipulator joint control as well as adaptive free-flying coordinated control.

During FY98, implementation of the new controllers will continue on both vehicles,    gradually integrating software and hardware to provide increasing degrees of autonomous performance; the following section provides an overview of the progress to date on this task. Data collection will continue quantifying the performance on baseline rendezvous and satellite servicing tasks. These data will be compared with the performance of current baseline control laws on the same tasks, contrasting both the accuracy, computational requirements, operator workload, and efficiency (required control authority) of the different approaches. Simulation results suggest that order of magnitude improvements over previous control technology is possible for the first two metrics, and a factor of two improvement is possible for the second two metrics; the experiments will attempt to verify this performance with the existing SSL robotic systems.

FY97 Results To Date:

FY97 has seen the completion of most of the (NSF funded) theoretical components of the research required to begin implementation in the flight control software of SSL’s robotic systems.    One preliminary finding from these initial implementations is that the achievable system performance is currently limited by the (relatively low) quality of the feedback available from the system’s sensors.    Part of the FY97 effort to date has thus been directed to improving both MPOD and Ranger’s sensor systems, via hardware redesign and improved digital signal processing techniques.

Even with these limitations, the initial field implementations of the new technology appear to live up to the potential illustrated in the (previously reported) simulation studies.    Figure 1 illustrates the benefits of using the new controllers for Ranger joint control: shown is the tracking of a “typical” rapid joint trajectory using first the baseline    PD control strategy, then using the adaptive neurocontrol strategy.    Use of the adaptive neurocontroller ultimately reduces the worst case tracking errors from 300 mrad to approx 25 mrad.    As shown in Figure 2, this reduction is possible because the network provides an adaptive estimate of the (substantial) friction characteristics of the joint actuator, which is used to modify the torque commands required to follow the trajectory.    Comparing the network estimate of friction as a function of joint velocity with independently obtained measurements of the friction torques shows that the network has autonomously determined an accurate model of the joint’s characteristics.

When Ranger is on orbit, its manipulator arms will experience thermal cycles as the spacecraft circles the Earth.    To simulate the effects of these changing conditions, the above experiment was repeated while the joint was heated from room temperature to 115 deg C, then cooled to -20 deg C.    Figure 3 summarizes the results of these experiments for the cooling phase; notice that, despite the varying environmental conditions, the adaptive neurocontroller maintains about 30 mrad worst case tracking for almost the entire experiment. As shown in Figure 4, this performance was obtained by tracking the evolution of the joint friction characteristics with operating temperature; as would be expected, the lower temperatures produced signficantly larger friction torques as a function of joint velocity.    Results similar to these were obtained for the heating phase of the experiment.

Note that, as a side effect of improving the system’s performance when executing a user-specified task, the new neurocontrol designs also provide a model of the velocity dependent torques (friction et. al.), due to the actuator design, which influence the motion of the system.    As a result, this new software is currently also being used in the SSL to model and refine joint actuator designs for the flight version of the Ranger experiment.

The above results were obtained using a fixed-structure neurocontroller consisting of 40 nodes for a single manipulator joint; extending this design to seven or eight degrees of freedom might prove computationally infeasible.    Thus, the performance of the new, structure-adaptive, wavelet neurocontrollers was explored during a (constant temperature) tracking experiment, to explore the possibility of maintaining performance while autonomously reducing computation. Figures 5 and 6 summarize the result of this experiment; as the controller learns, the tracking errors are gradually reduced again to a worst case 25 mrad, but additionally the number of computational elements employed to achieve this performance is now autonomously varied until a steady state of only six nodes is achieved.    Simulation studies with a full seven degree of freedom model    of Ranger’s arm dynamics indicates that even greater computational savings can be obtained using these techniques in higher dimensional state spaces: 2-3 orders of magnitude reduction in the number of computations required to accurately control the system have been obtained.

Finally, Figure 7 shows preliminary results obtained implementing the new control algorithms in the Ranger/NBV attitude control system.    The figure shows the error tracking a rapid “tumbling” maneuver which, while perhaps not characteristic of the maneuvers a free-flying robot would be required to execute, nonetheless presents a severe challenge to conventional flight control designs.    As the controller learns, the hydrodynamic and inertia properties become better modeled, allowing a more accurate estimation of the torques required to follow the trajectory, and as a result the tracking error decreases asymptotically toward a minimum (representing about 5 deg. of time-varying misalignment).    This asymptotic level of tracking error is currently near the best which can be obtained given the noise and inaccuracy in Ranger/NBV’s sensors (note, however, that near perfect performance is obtained for the less challenging task of attitude regulation, as opposed to tracking); the new hardware and signal processing software should greatly lower this achievable tracking limit.

In each of these studies, representing realistic aspects of the operation of the SSL robotic systems, the unstructured nature of the dynamic uncertainty which arises cannot be adequately addressed with previous adaptive robotic technology.    The new algorithms, however, provide the means to reliably compensate for unanticipated changes in the behavior of the system, enabling a significant extension in the capabilities of autonomous operation.    Moreover, with the new wavelet designs, the controller automatically evolves “just enough” complexity to provide satisfactory performance, minimizing the total amount of computation required to solve the control problem.

FY98 Deliverables:

•          Demonstrations of 6DOF control in neutral buoyancy using the new

control techniques.

•          Demonstrations of free-floating joint control using the new control

techniques.

•	Report quantifying the performance improvement using the new adaptive controllers for 6DOF base control and free-floating robotic operation.

•	Report comparing the performance of conventional control algorithms with the new controllers on these tasks in the presence of varying levels of uncertainty.

Task Milestones:

Level I Milestones (FY97-FY99):

June 97	MPOD and Ranger/NBV demonstrate new adaptive attitude controllers.

Demonstration of neuro/wavelet controllers for Ranger/NBV arm joints.

Jan 98		Ranger demonstrates adaptive joint control while free-floating.

June 98	MPOD and Ranger/NBV demonstrate adaptive coordinated 6DOF

position and attitude control

Jan 99 		Ranger/NBV demonstrates adaptive coordinated control of base

and manipulators.

June 99	MPOD demonstrates adaptive autonomous docking.

Participants:

All coding, fabrication, and testing for the task will be performed at the University of Maryland Space Systems Laboratory.    The work will be supervised by the PI, Dr. Robert Sanner, who will be assisted by two full-time graduate research assistants and two part-time undergraduate students.

The National Science Foundation (Robotics and Machine Intelligence Program; Dr. Howard Moraff, Director) is cooperating with NASA to fund this proposal. NSF is particularly interested in the theoretical aspects of the program, and to this end is providing approximately 1/3 of the project budget for its three year duration. The funding request below is for NASA's contribution to the FY98 budget.

Funding:

The funding requested for this project in FY98 is $65,000.    A top level breakdown of this cost is as follows:

Cost Element	Cost ($K)

Labor:	39

Overhead:	17

Fabrication and Materials:	5

Travel:	4

Administrative:	1

Facilities:

The activities associated with this task require use of the SSL robots MPOD and Ranger/NBV, and the testing of these systems in the SSL's Neutral Buoyancy Research Facility at the University of Maryland.

Commercialization/Dual Use Potential:

The extreme flexibility of the controllers described extends their potential application far beyond space robotics. Virtually any robotic system can benefit from the new technology, and a successful demonstration thus has significant application also in the commercial robotics industry.    Autonomous airborne and underwater vehicles, whose dynamics are governed by complex and only approximately modeled fluid flow, can similarly exploit the new control methodology; preliminary interest in commercializing the technology for undersea manipulators has already been expressed. Similarly, the algorithms described are applicable to the control of any conventional space vehicle, with possible application to the commercial and military satellite market, especially in the passive (fault diagnostic) operating mode. The PI is working with staff from the Naval Research Labs to further explore these future applications.

Principal Investigator:

Dr. Robert M. Sanner

Associate Director for Research, Space Systems Laboratory

Assistant Professor, Department of Aerospace Engineering

University of Maryland

Department of Aerospace Engineering

ENGR 3186

College Park, MD 20742

phone: (301) 405-1928

fax: (301) 314-9738

email: rmsanner@eng.umd.edu

<NB: The following two pages contain postscript figures; please use Print Preview>

Figure 1:    PD and neurocontrol tracking

of    specified trajectory (dashed lines) for a

Ranger arm joint.

Figure 3: Neurocontroller tracking performance

while manipulator is being chilled to -20 deg C.

The steady-state tracking errors, with one

exception, remain below 30 mrad.

Figure 2:    Neural network estimate of friction-velocity relation compared with experimental friction data.

Figure 4:    Internal estimate of friction model computed by the network at different times during the cooling experiment.    Note the changes in the friction model as temperature decreases.

Figure 5: Neurocontrol tracking of the specified Ranger joint trajectory using a structure-adaptive wavelet network. The asymptotic worst case tracking error is

identical to that seen above.

Figure 7: Preliminary data showing attitude tracking performance with Ranger/NBV using the new adaptive controller; shown is the norm of the vector component of the error quaternion.    At t=4700 sec, the adaptation mechanism was engaged; reduction past .05 error should be possible with the improved sensors.

Figure 6:    Evolution of the number of computing elements used in the wavelet network over time. Notice that the network evolves a structure which achieves the same performance observed above, but with significantly fewer computing elements.