Title: Photo-realistic Virtual Reality Modeling

Institution: NASA Ames Research Center

Principal Investigators: Michael Sims (ARC), Daryl Rasmussen (ARC), Ted Blackmon (ARC)

Task Objectives and Goals:

The goal of this task is to extend the capability of NASA to rapidly produce photo-realistic virtual reality terrain models during remote exploration missions.    Previous research has shown the benefits of using virtual environment technology as part of operator interfaces and scientific visualization systems for telerobotics.    However, developing the 3D virtual reality models for unstructured terrain environments, such as those found on Mars, still requires many hours of skilled labor. Furthermore, current state of the art modeling techniques for 3D panoramic terrain scenes have not yet reached the level of image resolution and visual fidelity contained in the original camera images from which the model was calculated.

Recent developments at the Intelligent Mechanisms Group of the NASA Ames Research Center have shown the potential to produce fully photo-realistic, 3D panoramic terrain models suitable for real-time rendering in a virtual environment.    A key component in generating virtual reality terrain models with full photo-realism involves the combination of 3D image processing (depth form stereo) with 2D image processing techniques (model driven edge detection).    The proposed task involves the enhancement of the current stereo camera pipeline technology to allow the rapid (i.e., 3 minutes or less) generation of fully photo-realistic virtual environment terrain models. This enhancement will open up the accessibility of this VR technology to the suite of NASA space missions.

Justification:

NASA telerobotic missions such as Pathfinder require an accurate description of the remote environment both for vehicle operation and also for scientific understanding. Current methods have limitations in registration of the images to the terrain models and in the time needed to provide the models during mission operation. Logistical steps in developing an automated approach to accurate terrain modeling for telerobotic vehicles could eventually enable real-world modeling on-board autonomous robotic vehicles.

Technical Approach:

The overall approach to produce a photo-realistic virtual reality terrain model from panoramic stereo video images utilizes bottom-up image processing algorithms actively controlled by a top-down computer model.    The bottom-up image processing algorithms include the computation of 3D range maps from calibrated stereo camera images and the use of 2D image processing to aid with terrain feature identification as well as accurately capture object boundaries in the camera images. A top-down computer model template incorporates the a-priori knowledge of an environmental setting (telerobotic vehicle on a terrain) to actively guide smart image filtering (i.e. edge detection, blur, wavelet analysis, multiple matched filters) in order to isolate potential regions-of-interest in the 3D model utilizing a-priori knowledge of geological features, such as potential rocks and stratification.

The selected regions-of-interest intelligently control a multi-resolution, adaptive meshing algorithm to provide the necessary level of detail in the topographical terrain map.    A single image of the original panoramic stereo video pair is then digitally warped onto the 3D model to provide a photo-realistic rendered scene.    This process is supervised by a human operator who can direct and adjust the top-down computer model to actively map the bottom-up data streams into a 3D virtual reality model. A novel concept for automating this intelligent modeling technique is to utilize the photo-realistic rendered stereo images as a feedback comparison with the original stereo video images from the on-board cameras, thus quantifying and producing a information entropy map of the 3D model accuracy.

The virtual environment modeling system involves a number of modular components in the process pipeline. These components involve a broad arena of leading-edge research investigations, including internal research within NASA, as well as collaboration and integration of algorithms and modules produced at universities such as CMU, Stanford, U.C. Berkeley, and industrial partners.    Some modular components of the virtual environment modeling system include:

1) Virtual reality calibration for augmented reality. 2) Stereo video correlation algorithms to produce dense range maps. 3) Monocular edge detection for image segmentation. 4) Top-down computer model actively controlling the bottom-up image processing. 5) Smart image filtering to assist with the top-down modeling process. 6) Adaptive multi-resolution triangulation and meshing. 7) Engraving of video image onto 3D model for photo-realistic texture.

Quantified Technical Advances:

Advances of this project over current state-of-the-art entail three main components:

1) Full photo-realism of the virtual reality terrain models. -- The aim of this project is to produce virtual environment models of panoramic terrain scenes with full photo-realism.    Previous work in related areas provide little quantitative analysis of the overall virtual environment model accuracy.    By comparing the rendered stereo images against the original stereo video, a quantitative measure of the information entropy resulting from the modeling process will be produced.

2) Reduction in the overall time of the modeling process. -- Current techniques to produce nearly photo-realistic panoramic terrain models involves considerable skilled labor and is typically on the order of hours to days.    An advance of this project is to reduce the virtual reality modeling process to near real-time (on the order of minutes).

3) Real-time rendering performance of the model. -- Another limitation of current methodologies is that the meshing of range data sets from high-resolution stereo images can easily produce an excessive number of polygons (over a half million polygons for an 512x512 pixel stereo video pair).    Thus, high fidelity virtual reality terrain models are typically not suitable for real-time rendering.    Incorporation of adaptive, multi-resolution meshing of the terrain model will enable real-time rendering of the terrain models on moderately priced graphics computers.

Deliverables, Milestones and Schedule:

1) Report on photo-realistic virtual reality modeling system for panoramic terrain scenes. 2) System demonstration and application in field tests of the Intelligent Mechanisms Group. 3) Development of the system as a generic tool for telerobotic interfaces and visualization systems.

9/98 -- Field test demonstration of photo-realistic virtual reality modeling system for 3D panoramic scenes.    Have photo-realistic VR modeling chosen as the preferred representation of image data (over 2-d images) by a majority of scientist in field test.

9/99 -- Stereo pipelining to photo-realistic VR models (<3 minutes from data receipt)

9/00 -- Development of photo-realistic virtual reality modeling system for 3D panoramic scenes as generic tool for operator interfaces and scientific visualization systems.    Publish tool under Cosmic.

Budget:

FY 98: $325K ($250K - ARC; $75K - Berkeley)

FY 99: $325K ($250K - ARC; $75K - Berkeley)

FY 00: $325K ($250K - ARC; $75K - Berkeley)

Facilities, Partnerships and Additional Participants:

The Telerobotics and Neurology Units at the University of California at Berkeley, directed by Professor Lawrence W. Stark, will collaborate directly with the Intelligent Mechanisms Group at NASA Ames on the development of the photo-realistic virtual reality modeling system for panoramic terrain scenes.

Additional participants from industry include Sense8,

SiliconGraphics, Intergraph and Stereographics.

Dual Use Potential:

This capability has far-reaching applications in a variety of fields such as military planning, hazardous waste characterization, non-destructive inspection, geographical surveying, cartography, and entertainment.