- TASK TITLE

Highly Accurate Positioning in Natural Terrains

- INSTITUTION

ARC Intelligent Mechanisms Lab CMU Stanford

- PRINCIPAL INVESTIGATORS

Michael Sims (ARC;overall task)

Visual Servoing - David Wettergreen (ARC)

Landmark Navigation - Eric Krotkov (CMU, DARPA) & Martial Hebert 	(CMU)

GPS Pseudolites Beacon Based Navigation - Jonathan How (Stanford),                                  	Robert Cannon (Stanford)

- TASK OBJECTIVE AND GOALS

Rover missions on Mars are severely hindered by the limitation of very few uplinks per day, perhaps even only one per day.    How do we get the most out of a rover during its lifetime?    If the rover were able to reliably execute multiple commands per uplink, the rover will be able to accomplish more before it succombs to the harsh planetary environment.    One of the major road blocks to executing multiple commands per uplink is rover positioning.    Dead reckoning is inherently a loosing proposition, even in very well controlled conditions.    If the rover is able to move from location to location accurately, scientists will be able to designate a number of tasks that the rover will be able to carry out.    Without improved navigation abilities, the rover will have position information that continually degrades.    After several moves, a rover tasked with deploying an instrument has very little chance of reaching small targets. Scientists need to be able to accurately place instruments on targets as small as several cm across.

Current technologies are not able to provide continuous rover position measurements to the accuracy needed by these applications (approximately 3 cm).    Current technology to overcome these problems is to view the rover from a lander base station (e.g. Pathfinder/Sojourner).    This is an excellent solution, but forces the rover to remain in view of the lander.    Unfortunely, this precludes many types of critical scientific exploration which require a rover to travel kilometers to get to the next science site.

We propose a combination of three techniques that will provide the required positioning data: visual servoing, natural feature recognition, and rover placed pseudolites.    Together, these three systems will be able to provide accurate and robust position information enabling sequences of tasks to be carried out.

The current state-of-the-art in navigation within a science site is the Mars Pathfinder rover flight experiment, which guides rover motions based on images acquired by lander cameras.    This approach is not viable for science sites out of sight of the lander cameras. Future missions call for extended traverses.

Visual Servoing:

A visual servoing system is a feedback controller that uses the error between the location of an object (or texture) in an input image and the desired location of that object as a control signal.    The visual servoing subsystem will allow an operator to designate a target in a rover's local area.    Using a correlation based algorithm, the rover will track the target's location and range and drive to within manipulating range of the target.

Natural Landmark Navigation:

The landmark navigation system is based on the tracking of multiple targets as the rover moves through its environment.    As the rover moves through its environment, it keeps track of the direction to each target.    Combining landmark direction with rover dead reckoning, the system is able to decrease error in the known location of landmarks and rover.

The natural landmark navigation subsystem will demonstrate the capability to navigate to targets of scientific interest, without imagery from lander cameras, and without requiring three-dimensional modeling.    This technology will enable robotic vehicles to traverse a science site, visiting rocks or other objects designated as targets, despite poor dead reckoning and without range sensors.

The technical objective of this task is to enable rovers to travel from designated target to designated target, arriving within 3 cm of the targets, close enough that a manipulator can easily reach them.

Rover Deployable Planetary Surface Pseudolite Array:

Stanford has been a major if not the principle player in the development of GPS technology.    This role has included the invention of PseudoLite technology which consists of an array of locally distributed GPS stations and allows highly accurate positioning (sub cm accuracy) within a site of up to a couple of hundred square meters.

- JUSTIFICATION, including user community and responsiveness to requirements

Many of the problems associated with navigating within natural terrains become immensely simpler if one knows accurately one's position.    We are taking a three pronged approach to a solution.

Proposed scenarios for planetary rover science missions are the drivers behind the development of highly accurate positioning. Careful examination of these missions leads to the conclusion that rovers will be active in one of two modes.    The first mode is an extended scientific investigation of a site (e.g., exploring an ejecta field from a 2 km. impact crater).    This type of activity will typically last on the order of weeks and cover an area of tens to hundreds of square meters.    A second mode, traverse, used to travel between science sites.    Each traverse will typically last a few weeks.

For the traverse between science sites, reasonable efficiency can be obtained by simply uplinking corrections to the rover's bearing. Since precise positioning is not as important, over head imaging can support rover navigation.

However, for extended stays at a science site, the scientific productivity is a function of how efficiently the rover operates within that fairly well understood local terrain.    The ability to do high precision localization allows the rover to execute multiple commands during a single uplink.    This will increase the science return during the rovers limited lifespan.

- TECHNICAL APPROACH

Visual Servoing:

Visual servoing is based apon an operator designating a target to the tracking system.    The rover will visual servo to that target based on correlation tracking of the given target, periodic update of correlation filters, and stereo tracking of the distance to the target.    The tracker will possition the rover so that the target is in the manipulation envelope of the arm and end-effector.

Natural Landmark Navigation:

This approach will locate and track features from the rover's on-board cameras, and determine the rover position through triangulation and filtering against dead-reckoned estimates of vehicle motion.    Relative navigation about a science site will be achieved by tracking distinct image features, such as prominent rocks or ledges, and computing vehicle position in a local coordinate frame.

The components of the system will include the following. 1/    Landmark detection, based on color and texture. 2/    Landmark tracking, using correlation and sum-of-squared-difference operators. 3/    Relative position estimation, using the Approximate Transformation approach pioneered by P. Cheeseman and implemented by G. Thomas, and using classical maximum likelihood estimation.

The system will be tested in extensive field experiments conducted at the CMU Moonyard and at selected Ames sites.

Pseudolites:

Pseudolites will be acquired through a collaborative effort with IntegriNautics, a spinoff company from the Stanford GPS Lab.    Stanford will develop the algorithms necessary for calibration after deployment; and ARC and Stanford will jointly integrate the local GPS algorithms into the Marsokhod vehicle for field demonstrations of the capability in natural, outdoor terrains.

There are two issues in the pseudolite approach.    The first is rover placement of pseudolite transmitters.    The second is development of the neccessary callibration algorithms and procedures.

- A brief description of the technical advance over the current SOA:

The ability to know highly accurately a rovers position in a natural terrain is a major advance over the state of the art and will greatly simplify many of the most difficult traditional problems associated with rover mobility.

- QUANTIFIED description of advances over current state of the art

Visual servoing allows accurate single command movement to a target in natural, uninstrumented terrains.    With a single command, the rover will be able to bring a target into its manipulation envelope.

Landmark navigation will allow navigation of a wide range of areas providing accuracy for moving to specific targets and for long rover traverses.

A rover deployable pseudolite array will be able to be deployed and calibrated for local rover navigation.    When calibrated, a pseudolite array is capable of sub cm accuracy without drift.

- DELIVERABLES (remember - they must be QUANTIFIED and MEASURABLE!)

See Milestones and Schedule

- BUDGET for all years of the task

FY98:      $550K ($175K - Stanford; $175K - CMU)

FY99:      $550K ($175K - Stanford; $175K - CMU)

FY00:      $550K ($175K - Stanford; $175K - CMU)

- FACILITIES, PARTNERSHIPS and ADDITIONAL PARTICIPANTS

Natural Landmark Navigation

This task will use the facilities of the Intelligent Mechanisms Lab at ARC, the Field Robotics Center at CMU and the ARL lab at Stanford.    The component technologies will be integrated onto the Marsokhod testbed at ARC and field tested.

- DUAL-USE POTENTIAL

Visual Servoing:

This research is relevant to application on any missions with mobility or an arm which are instrumented with cameras.    This visual servoing work has relevance to on-orbit robotics as well as planetary surface rovers.    This work has numerous applications to Earth based robotics and we expect these techniques to be integrated into REC projects.

Pseudolites:

This will be applicable to rover missions to planetary surfaces where the rover will do extended activities at a particular site. Pseudolites will be applicable for use by humans in similar circumstances.    In particular, it will be widely used at human habitat sites, both for tracking of robots and for tracking of astronauts.    In such high precision activities around orbital structures such as the Space Station or Shuttles, pseudolite navigation will be of immense importance.    In addition the deployable and field calibration of the pseudolites should have important applications to military and civilian commercial applications.

- MILESTONES AND SCHEDULE

9/97: Demonstrate relative position estimation using landmark navigation with an accuracy of 30 cm, based on automatic tracking targets of geological interest in a sequence of at least 100 video images acquired at two natural sites.

1/98: Visual servoing of mobility and arm movement to instrument placement on a designated science target in a natural terrain without lander positioning.

2/98: Demonstrate relative position estimation with an accuracy of 3 cm, based on automatic detection and tracking of targets of geological interest.

9/98: Demonstrate automatic selection of geological targets around a natural site 20 meters in diameter that enables the accuracy of relative position estimation to improve over time while traversing the site. Selection criteria to include the distribution of targets (for example, around the site boundary), visibility while traversing the site, and target discriminability.

9/98: Successful demonstration of deployment and calibration of Pseudolites in outdoor terrain.

1/99: Accurate visual servoing to multiple targets under above constraints.

9/99: Successful demonstration of successful vehicle mobility over natural terrains with sub cm positioning accuracy.

1/00: Accurate visual servoing demonstration for prototype orbital spacecraft.