# Unified Lander Rover for Planetary Exploration

Principal Investigator:

Red Whittaker (412-268-6559, red@ri.cmu.edu)

The Robotics Institute
Carnegie Mellon University

For the period 10/1/97 to 9/30/98

Amount requested:    $250,000

# 1.0 Objectives

Carnegie Mellon University (CMU) proposes a four year program to research, simulate and demonstrate the merits of a unified lander rover (ULR) paradigm for planetary exploration. The program will:

1. Research, design, develop, and demonstrate a ULR archetype.

The ULR will combine the capabilities of a lander and mobile robot through complementary use of components, such as power, structure, computing, communications, and sensors. This configuration will reduce duplication of componentry, as well as auxiliary hardware such as ramps, inherent to the traditional rover-on-lander (ROL) paradigm.

2. Research the tradeoffs between surviving impact with absorbers versus avoiding impact by landing with finesse.

The program will trade off the need for shock absorbing structures, such as collapsible legs, locomotion suspensions and airbags versus landing finesse such as zero energy contact and obstacle avoidance in final descent. The ability to safeguard against landing hazards and land with control and precision will allow a ULR to land in rugged terrain.

3. Quantify the distinctions of the ULR paradigm versus the conventional ROL.

The program will bring to bear research activities that quantitatively compare the two mission archetypes. Anticipated metrics include landing velocity, terrainability for both landing and roving, component duplication, risk and complexity. Of utmost importance will be the ratio of scientific payload mass to total mass.

By combining technologies inherent to high performance mobile robots with structures and systems required for landing, the ULR will provide the functionality of a lander with the versatility of a highly mobile robot. This new class of spacecraft could revolutionize the way we explore planetary surfaces.

# 2.0 Justification

Whether robotic or human, mobility is crucial to planetary exploration. It is key to robotic missions chartered to return comprehensive scientific data or scout as precursors to manned missions, and it is essential for future human exploration of the planets (manned ULR’s are a possibility). Mobile exploration, however, presents the troubling dilemma of how to deliver a mobile robot to the planet surface. The role of a conventional ROL lander is final descent, shock absorption, deployment of rover to surface and in some cases as a stationary relay. The possibility of ULR is to accomplish these functions in a spacecraft that becomes a surface explorer after landing.

Setting a precedent in robotic rover and lander design, ULR spacecraft will enable high-yield missions, and carry the added potential of superior performance of both landing and exploring mission phases compared to what is now possible. Dual use of components promises reduced cost, savings in mass, and maximal use of payload. Further, the possibility of refueling from planetary ice resources, could allow the ULR to lift off the surface again.

# 3.0 ULR Scenario

During descent of a ULR, onboard imaging sensors survey the landing zone for a safe landing spot. Servoing from the imaging sensor’s view of the surface, a ULR controls thrusters for deceleration, attitude control, and touchdown. Other sensors such as inertial motion units (IMUs) stabilize control.

Touchdown occurs with little or no velocity relative to the ground. A ULR then transitions from lander to rover. Ground navigation software is downloaded over landing software. Mechanical changes, such as the repositioning of sensors and the discarding of landing components such as thrusters, are performed as the mission requires. Now a wheeled robot powered by solar panels and batteries, a ULR drives off to perform its mission.

A Unified Lander Rover concept

# 4.0 Technical Approach

The technical approach for the ULR program consists of three principle thrusts:

- ULR configuration

- Tradeoff of shock absorption structures versus landing with finesse

- First principles of ULR's and metrics to compare them with existing systems

## 4.1 Configuration

While performing different tasks, landing vehicles and robots share fundamental attributes: structure, power, computing, communications, inertial navigation and imaging systems. The proposed program will research, develop and evaluate a novel, efficient, and cost-effective spacecraft that combines the lander and mobile robot into one vehicle. This may lead to significant parts reduction, mission development simplicity, cost and mass savings for a broad class of missions.

The landing and roving phases of the mission occur sequentially with no temporal overlap. Since no subsystem is needed for landing and roving concurrently, design will be driven by the phase requiring the higher performance. The program will examine peak requirements for the two mission phases for power, computing, communications, and inertial navigation systems and isolate differences that may impact the feasibility of combining specific subsystems.

The dual use of imaging systems and other navigational sensors presents several challenges. First, the velocity of the ULR and sensor range are quite different for the two mission phases, a variation which would require a sensor suite with a fast response time and a broad detection range. Secondly, the sensors must be positioned downward while the ULR is landing but forward when roving. Further, to provide adequate coverage and generate sufficient information during roving, the sensors must be mounted high on the robot. A third potential problem is interference with the sensors from the deceleration thruster exhaust while landing. The proposed program will address these imaging and navigational problems through proper selection of sensors and development of methods which maximize the dual use potential without compromising mission capability.

Another challenge lies in design of the mechanical structure. During landing, a rigid structure is preferred to provide a stable platform for thrusters and to reduce the amount of low frequency oscillations. This approach, however, conflicts with the requirements for touchdown and roving, where a rocker bogie or suspension is needed for compliance. The ULR is also constrained in mass and volume.

The research program will investigate potential configurations for an instance of the ULR class of spacecraft. Complete simulations, including functional models of all subsystems and the environment during descent, landing, and roving will be created and used to evaluate possible configurations. Based on simulation results, a prototype ULR will be developed. A series of hop tests will be performed, culminating in a fully powered test of the ULR using thrusters to demonstrate viability of design. The fully powered tests will be performed at a remote site, close to CMU. The program will commission consultants and sub-contractors, such as Lockheed-Martin to oversee these tests and ensure that proper safety precautions are implemented.

## 4.2 Shock Absorption Structures vs. Landing with Finesse

To reduce the role of energy absorbing structures the ULR must be capable of landing directly on its locomotion system with little or no velocity. This zero energy landing will allow the ULR to alight on the surface and smoothly transition to planetary explorer without the complexity of jettisoning energy absorbers.

A consequence of reducing or eliminating energy absorbers is that the ULR will require more fuel during the descent. Another consequence is that the ULR can no longer land blindly, but must rather actively detect the surface and identify potential landing hazards.

The research program will examine the feasibility of landing with finesse for the ULR. This will include zero velocity landing combined with object detection and avoidance during descent. The benefit of eliminating shock absorption structures will be weighed against the increases in fuel consumption and complexity to evaluate the best level of landing with finesse and shock absorption structures.

## 4.3 ULR Basic Principles and Evaluation Metrics

Exploration missions from Mars life to Mercury ice would benefit from some form of ULR. The conditions encountered on such missions vary greatly with gravity ranging from 1/10g to 1/3g and surface compositions from dust to rock. Since the prototype ULR developed in this program will not be ideal for all missions, the program will seek to define certain basic principles of ULR's which are applicable to all ULR systems and to vary these across the range of possible destinations and missions.

Of equal importance are the metrics which allow the quantitative comparison of a particular ULR system to a standard ROL system, or two competing ULR designs. Anticipated metrics include landing velocity, terrainability for both landing and roving, component duplication, risk, complexity, gravity, lighting and surface conditions. To quantify the efficiency of the mission the ratio of scientific payload to total mission mass can be examined. Other metrics will be considered and included as needed.

# 5.0 Advances Over Current State of the Art

The combined landing and roving functionality of the ULR is an unprecedented spacecraft capability. Landing velocity, ratio of science payload to total mass, terrainability and mobility will exceed those of current missions.

The ULR program will leverage on current state of the art in air and ground vehicle sensing, control and autonomy.

# 6.0 Milestones

FY 98	Functional simulation demonstrated with a ULR. Preliminary ULR design with sensor placement and mechanical constraints. Initial definition of ULR evaluation metrics.

FY 99	Descent object avoidance completed and tested in simulation. ULR design finalized. Quantified results of the trade off of shock absorption structures versus landing with finesse.

FY 00	Working prototype of ULR mechanicals. Working prototype of landing obstacle avoidance. Testing program for integrated system designed.

FY 01	ULR with landing obstacle avoidance tested and demonstrated. Document containing the basic principles of the ULR paradigm, metrics for evaluation, and a comparison of ULR design to current landing technology.

# 7.0 Dual Use Potential

The ULR archetype will revolutionize planetary exploration and contribute technology appropriate to both robotic and manned planetary missions. By enabling the maximum use of existing payloads, larger missions at lower cost will be possible using current rocket technology.

The technology developed for landing with finesse is applicable to autonomous aircraft and conventional planetary landers. The control concepts will also prove invaluable to other areas of mobile robotics, particularly unconstrained such as underwater, airborne, and orbiting robots.

# 8.0 Participants

The Unified Lander Rover program will be undertaken by Carnegie Mellon Univeristy in cooperation with Lockheed-Martin.

The Robotics Institute at Carnegie Mellon University has grown into an interdisciplinary research organization with a staff of over 250 including 70 graduate students enrolled in the Robotics Ph.D. program, and 50 graduate students enrolled in affiliated programs.

The Robotics Institute will build on its existing programs for planetary exploration robots and autonomous helicopters to complete the Unified Lander Rover program.

Lockheed-Martin will provide their expertise in planetary lander technology and propulsion components.

## 8.1 CMU TRIWG Strategy

CMU TRIWG programs are building complementary technologies that stand on their own merit, and also synergize to produce a whole that is greater than the sum of its parts. The Meteorite Search Technology program provides the basis for perception and automatic search and exploration strategies. The Meteorite Search Demonstration program allows for these technologies to be fielded and hardened for flight scenarios. The Unified Lander Rover is a spacecraft paradigm that could deliver a higher fraction of scientific payload to planetary surfaces while still permitting the mobility required for exploration. Finally, Operational Autonomy promises the ability for such rovers to autonomously perform ambitious high-return missions when teleoperation is infeasible due to task complexity and communications limitations. But CMU goes beyond creation and promulgation of knowledge, using REC to carry development to a commercial standard by productizing and hardening NASA technologies for industry.

# 9.0 Deliverables

FY 98	Produce metrics for general comparison and evaluation of the ULR and ROL paradigms. Preliminary design of a ULR instance.

FY 99	Blueprints for a ULR design. Software for landing obstacle avoidance.

FY 00	Preliminary prototypes of a ULR and landing obstacle avoidance.

FY 01	ULR demonstration. Document containing the basic principles of the ULR paradigm, metrics for evaluation, and a comparison of ULR design to current landing technology.

# 10.0 Schedule and Funding

| Fiscal Year | Budget ($k) | Activities |
| --- | --- | --- |
| 98 | 250 | Formulation of the basic principles of ULR's. Develop metrics for comparison of ULR's and ROL's. Perform trade studies on current lander and robot configurations for conversion to a ULR. Develop an initial design of a ULR and test it in simulation. Initial investigations into landing with finesse issues started. |
| 99 | 500 | Algorithms for descent object avoidance are formulated, tested in simulation, and completed. Refinement of initial ULR design into a working design for prototype construction. |
| 00 | 1000 | Construction of a prototype ULR mechanical subsystem. Uncontrolled drop tests will be performed to test the structure's ability to absorb residual velocity at impact. A prototype of the landing obstacle detection subsystem will also be built and tested. Details of complete tests will be finalized. |
| 01 | 1000 | Integrate ULR with descent obstacle avoidance. Demonstrate capability of ULR by performing controlled hop tests where the vehicle will perform a landing with minimal (ideally zero) landing velocity and then drive away. |
| Total | 2750 |  |

The budget for FY 98 will consist of (in thousands of dollars):

| Personnel | 123 |
| --- | --- |
| Travel | 15 |
| Supplies | 14 |
| Equipment | 14 |
| Overhead | 84 |
| Total | 250 |

Personnel will consist of 10% of William Whittaker, 2 graduate students and one undergrad student.