Rover-Mounted Rock Internal Inspection

and Selection System

Greg R. Gillis-Smith, Samad Hayati, and Donald R. Sevilla

1.0    Objectives

With the growing interest in the geology and evidence of past life held within the rocks of Mars, there is an increased desire to study and bring back rock samples to Earth.    Simply looking at the exterior rind of the rocks is like looking at a geode, dusty and indistinguishable on the outside.    If one wanted to find geodes without breaking every rock, one could use sensors or scales to improve the odds of choosing them.    When looking for evidence of life on Mars, instead of packing up the ten nearest rocks and hoping one contains evidence of life or is otherwise scientifically valuable, one could drill into the rock for in-situ optical and multispectral examination.    This requires that a hole be cored or drilled into the prospective rock and then viewed with a fiber optic microscope.    The core or drill tailings would need to be captured and stored for later return to Earth.    Alternately, the rock may be fractured and the inside inspected, such as what a field geologist would do.

Since the time available to the sampling rover will be limited, it is important that the rock be sampled at the same time it is inspected.    Thus, the inspection mechanism must be compatible with the capability to discard or cache the samples.      Since the sample collection rover is planned to have a manipulator arm and imaging system, these resources could be used to enable a greater overall capability in the inspection mechanism.    The key technology needed is to design a mechanism that allows imaging of the interior of a rock, so the scientists may determine what lies under the surface weathering rind.    The two methods the planetary scientists are most interested in to accomplish this task are to either produce core samples or to fracture the rock into pieces suitable for return to Earth.

This task will spearhead the development of the technologies required to inspect and select the most scientifically significant rock samples.    The inspection and selection mechanism would be rover mounted.    A drilling/coring mechanism would deploy against the surface of the rock for stability while drilling and inspecting.    When the desired depth is achieved, the drill would back off and allow the optical instruments to be indexed into place.    If the rock was found to be scientifically valuable, the sample core would be cached.      A rock fracturing mechanism would deploy to break pieces off of larger rocks.    If the pieces remain too large for caching, or for the smaller rocks that may be picked up from the surface, the manipulator arm would transport them to a fracturing chamber for further downsizing.    An imaging system will allow scientists identify, select and cache the best samples.

Although drilling, coring, fracturing, viewing and storing a sample may sound simple, the mechanism and control issues pose interesting challenges.    Both the rock and rover are on random and non-solid terrain.    Once the rock is selected for drilling the mechanism must drill a hole, pull away to allow indexing to the viewer without losing the hole position.    Each process must be controlled such that it autonomously drills the proper amount, finds the same hole in the rock and have autonomous contingency plans for possible errors.    Not only does the mechanism have to survive the dusty, cold, low atmosphere environment in order to select rocks of interest, but the drilling and imaging must be done onboard a small rover.    By reducing the mass and power requirements of a drill, one may effectively core or drill a 0.5 cm hole 0.5 to 2 cm deep.    The sample acquisition process, whether by coring or drilling, will be only part of a complex task of acquiring, transporting, and storing each discrete sample.    The sample must be extracted and moved to a storage container without mixing or contamination.    Particularly important will be failsafing provisions in the mechanism design to insure that a failure in the sampling/inspection mechanism will not cause complete loss of rover mobility.    This issue was one of the primary design drivers to the APXS deployment mechanism supplied to Sojourner.

There are several control problems associated with this operation.    The rover must drive to a correct location so that the drill mechanism when deployed would be perpendicular to the rock surface, otherwise the drill bit will slip when first contacting the rock surface. While drilling, the torque of the drill must be monitored and controlled.    Control strategies might be required to vary the drilling speed or reverse the direction for successful drilling.    A drill bit exchange mechanism and control system must be designed to take drilled cores out.    This is required both for sample acquisition as well as drilling into many rocks during long range missions.    Drilling into one single rock is not a viable solution.    After the drilling process, any small motion in the rover will cause the rover to loose its position.    Therefore, reacquiring the hole for subsequent visual and spectral imaging must be done using the onboard vision system.    Finally, the core samples must be picked up and placed in the storage container.

2.0    Justification

JPL’s Center for In-situ Exploration and Sample Return and the Mars Exploration Program Directorate have expressed the strong desire of NASA to bring back rock samples from Mars that have evidence of past climatic fluctuations or life.    These samples could provide information about early conditions with warm and wet climates and open lakes.    These lakes may have deposited carbonates, sulfates, and other salts.    Warm, wet conditions would be conducive to generating and maintaining life or at least prebiotic compounds.    Scientists require sampling the rocks in the areas that may be surface expressions (hot springs) of former hydrothermal sites and make measurements of rocks in-situ, both without disturbing them and by getting beneath the rock coatings and weathering rinds or patinas.    Drilling, coring, and fracturing are all good ways of getting to the unweathered surface.    This would be followed by close up multispectral imaging from 0.4 to 2.5 micrometers and by using a Raman point spectrometer.    Raman spectroscopy does mineralogy and should show organic compounds as distinct features.    One would get textural and mineralogical constraints on mineralogy and overall makeup of the rock (with organics) and how they vary as a function of depth.

Upon further investigation into the technology requirements of future Mars rovers, it was found that there is a great need for this type of integrated mechanism and instrument.    Figure 1 depicts the technology development on-going by JPL for the Mars Sample Return Missions.    A hole exists in the current technology development that this proposal will fill.      Three missions have been defined, culminating in sample return to Earth.    Two rover missions in 2001 and 2003    are to send long-range rovers to Mars for in-situ science experiments, sample selection, and caching. In the third mission, a small rover will transfer the cached samples from one of the previously flown rovers to an ascent vehicle so that samples can be brought to Earth for further laboratory experiments.    An important element for the 2001 and 2003 missions is the ability to acquire samples from the interior of rocks, examine them, and acquire small samples for return to Earth. Currently such a drilling mechanism does not exist which satisfies the power and mass requirements. Furthermore, this type of technology has not been demonstrated on a rover platform performing autonomous drilling, inspection, and acquisition of samples. This task will require the development of several key component technologies as well as one of the most complex autonomous control systems ever to be loaded into a rover.    The link between the rover controls group (i.e., Rocky 7 and 8 task members) and the hardware development has already been established for the purpose of scoping this task.

Figure 1 - Technology Development for Mars Sample Return

Besides being project relevant, this task will develop hardware and a control system that will not only perform a mechanical function of drilling rocks and storing samples, but also provide scientific information about the rock’s exterior, interior, and strata.    This information will enable the scientists to make decisions about which samples are most interesting and valuable to return to Earth.    Many rocks could be sampled in a relatively short period of time allowing for greater scientific study and selection.    By performing in-situ optical examination on rocks via an onboard drill, rocks may be selected for further study and possible sample return.    This reduces the lift-off mass of the sample return vehicle or allows for more samples of known scientific worth with low cost and mass impact relative to return vehicles and rock samples. This mechanism will be integrated to carry the optical instruments or other science-related hardware which would further reduce the overall mass of the system.    After spending hundreds of millions of dollars to develop, fabricate, launch, cruise, land, relaunch, cruise back to Earth, and land a Mars sample return vehicle, the reward for the investment to develop and fly this mechanism will be to increase the odds of bringing back valuable scientific data and possible evidence of life.    The concept of this mechanism could be used in different forms for in-situ or sample return missions to Mars, asteroids, or any planetary or lunar locations.

3.0    Approach

The approach will first be to determine the configuration designs for both a drilling/coring inspection system and a rock fracturing inspection system, compatible with the existing sample return requirements.    The mass, resource requirements, risks and reliability will be compared.    After consensus is reached as to the risk/benefits of each approach, a baseline design will be fabricated and demonstrated.

The approach to be pursued for a drilling inspection system is to use the motors and drilling technology developed by the Exploration of Small Bodies Task and Honeybee Robotics and design a new small, lightweight mechanism.    If drilling and coring are chosen as the best method to obtain samples, we will work with Honeybee Robotics to develop that technology under their SBIR.    Constant drilling force would be generated by a spring-loaded deployment mechanism in the mechanism arm.    The challenge of locating the drilled hole will be accomplished through the use of a mechanical guide that will stabilize the rover and drilling mechanism.    By integrating these operations of the rotating components, one indexing mechanism, one thrust mechanism, viewing instruments, and storage system the result will be a lightweight, multifunctional, and robust mechanism.

The alternative approach for a rock fracturing inspection system will consist of designing two component assemblies.    One would be a deployable rock fracture mechanism, using either a spring-loaded linear piston to cleave pieces off of larger rocks, or a rotational hammer approach similar to a geologist's field tool.    The manipulator arm would transport these pieces to an inspection platform, and arrange the sample as required for inspection of the interior surfaces.    Rock fragments too large for sample return, or rocks selected directly from the surface, would be dropped into the chamber of the other component assembly.    This would be a rock splitting mechanism, which would consist of a motor driven ram to fracture the selected rock into smaller pieces for inspection, selection and sample return.    The fractured pieces would be dropped onto a "washboard" type of separator for manipulation and inspection using the robotic arm.    The washboard could be commanded to tip up and dump the residual pieces prior to splitting additional rocks.

The types of instruments being considered are optical near IR and visible microscopy and Raman point spectroscopy.    This would provide visual, mineralogical, and organic analysis.    Another valuable tool would be a laser microprobe which would vaporize a 20 to 50 micron surface sample for mass spectroscopy to determine elemental composition. The Long Range Rover task is currently developing a miniature fiberoptic IR spectrometer, so it is planned to incorporate this device in the full-up system demonstration.

The controls required for drilling, inspection, and sample acquisition will use rover’s already developed computing, science instrument pointing and data reduction, and manipulation capability.    This will reduce the overall mass required to integrate such instrumentation on the rover.    The capability to select a rock from the panoramic imagery and traverse to a specific rock is currently being developed in the Long Range Science Rover Task, and it will be ready in time for drilling operations.    Also, rock surface normal calculation using onboard vision system is currently being developed.    These two capabilities are required to drive the rover from a location where panoramic images are taken to the target location.

The rover must then deploy the drill mechanism, and autonomously execute the drilling operation by controlling its speed via monitoring the drill torque and pressure on the surface to ensure that the drill bit does not get stuck.    Control system will be developed to back off and if necessary reacquire the hole for the purpose of rock internal examination. Rock internal inspection will be done via close up multispectral imaging from 0.4 to 2.5 micrometers and perhaps by using Raman point spectrometer. Raman spectroscopy does mineralogy and should show organic compounds as distinct features.

After drilling, an automated system is required to take out the drill bit and put a new one in.    The core must then be stored if the examination shows that the rock is of scientific value for return to Earth.    This will be done by using the manipulation arm on Rocky 8 which will have at least four degrees of freedom.    Depending on the design of the drill bit exchange mechanism, totally automated or a vision based system will be designed to transfer the core to the storage tray. The onboard control system must be able to deal with these complex operations and develop contingency plans for errors.    This can be accomplished by verifying each operation for successful completion using rover’s mast and the onboard vision system.    Currently several operations such as soil sampling goal confirmation are controlled this way on the Rocky 7 Rover.

A prototype mechanism will be produced and demonstrated during FY98.    By end of that year, the prototype would be ready to integrate to the Rocky 8 rover to demonstrate the capability of a rover-mounted rock sampling and inspection system.    This prototype would be integrated with a spectrometer or fiber optical viewer on the Long Range Science Rover in FY 99 for the determination of imaging performance and design of the rover control system.    Also performed in FY99 will be the required integration of the system with the Rover manipulator arm, and the determination of the control issues.      In FY 00, the control system design will be completed for a full-up "end-to-end" demonstration of deploying, sampling, selecting and caching a subsurface sample.

4.0    Participants

Participants at JPL include members of the technical staff of the Mars Exploration Engineering Group, Exploration of Small Bodies Task, and Automation and Control Section, and the Long range Science Rover Task Motor.    Gearbox technology developed by the ESB Task will be further improved and developed for accomplishing this task.    The Long Range Rover Task will participate with this effort to develop the control systems for performing integrated testing and to determine mechanical and electrical interfaces, as well as the development of the fiberoptic IR spectrometer and a small manipulator arm.    JPL is also funding a Small Business Innovative Research grant (SBIR) to Honeybee Robotics for the development of a rover-mounted sample acquisition and transfer mechanism.    The Rock Internal Inspection and Selection task will collaborate with Honeybee in the incorporation of the coring technologies and mechanization.

The cryogenic motor development will continue with American Technology Consortium, a small business in Southern California.

5.0    Deliverables

The first year of this effort will concentrate on the system design that best meets the requirements for internal rock inspection, which consists of comparing the capabilities and complexities of coring/drilling versus fracturing, and the associated sample handling mechanization.    The second and third years will integrate the mechanism with a rover, science instrument, and develop an integrated control system.    Technology development and associated milestones are as follows:

Fiscal 1998

- Design of drilling/coring versus fracturing mechanism and consensus by Mars Science Working Group

- Design, fabricate, and develop a prototype inspection and sample acquisition mechanism

- Design, fabricate, and develop a prototype inspection system deployment mechanism

- Demonstrate the sampling and inspection mechanism on representative rocks.    (Level 1 Milestone)

Fiscal 1999

- Integrate the mechanism with an optical instrument

- Integrate the mechanism with a rover science platform and manipulator arm.

- Develop rover-compatible control system

- Demonstrate system test of rock drilling and inspection from rover      (Level 1 Milestone)

Fiscal 2000

- Integrate the mechanism with a rover control system

- Use fully operational control system to deploy, drill, view samples from a rover platform and acquire samples

- Demonstrate drilling and sample acquisition in at least two rocks (Level 1 Milestone)

6.0    Commercialization    Potential

This task uses technology developed by past and current JPL Telerobotics in order to develop the technology necessary to perform on-board sampling and in-situ optical examination.    It is anticipated that this technology will be valuable to many future in-situ and sample return missions.    Honeybee Robotics is developing drilling mechanisms for the Champollian mission.    Many of the component level technology developments are applicable to other low temperature, low atmosphere, terrestrial, cometary, and asteroid missions.    These include lubrication, motors, gears, drilling and polishing hardware.

7.0    Funding

| YEAR | MILESTONES | COST |
| --- | --- | --- |
| FY 98 | Prototype rock inspection mechanism Demonstrate sampling and selection of representative rocks | $400 K |
| FY 99 | Integrate mechanism with Rover and optical instrument Demonstrate the ability to sample, manipulate and view samples | $500 K |
| FY 00 | Engineering Model of mechanism integrated on Rover with fully operational control system to deploy, sample, select and cache a subsurface sample | $500 K |

8.0    Facilities

The majority of the effort of this task will be performed at the Jet Propulsion Laboratory in Pasadena, California.    Some of the resources to be used at this facility are the comet drill lab, robotics labs, Rocky 7 and Rocky 8 Rovers, machine shops, and temperature/ atmosphere chambers to verify the designs in a relevant environment.

9.0    Principal Investigators

Greg R. Gillis-Smith is a member of technical staff in the Mars Exploration Engineering Group at JPL.    Mr. Gillis-Smith has been a member of the JPL staff since 1987.

(Phone: 818-354-1919; Fax: 818-393-1324; e-mail: Greg.R.Gillis-Smith@jpl.nasa.gov; JPL Mail Stop 301-320)

Samad Hayati is the manager of the Long Range Science Rover Task.    Dr. Hayati is a member of the JPL Section 345 Staff and has led several telerobotics tasks in the last 14 years.

(Phone: 818-354-8273; Fax: 818-393-5007; e-mail: Samad.Hayati@jpl.nasa.gov; JPL Mail Stop 198-219)

Donald R. Sevilla is a Member of Technical Staff in the Mechanical Engineering Section at JPL.    In the past 19 years at JPL, he has been the Technical Manager for the Mechanical Devices Subsystem for both Cassini and Galileo spacecrafts as well as a lead mechanisms expert for numerous flight projects.    Mr. Sevilla was a recipient of the NASA Exceptional Service Medal in 1992.

(Phone:818-354-2136;    Fax:818-393-1324;    mail: Donald.R.Sevilla@jpl.nasa.gov;

JPL Mail Stop 301-320)

The mailing address of the principal investigators is:

Jet Propulsion Laboratory

4800 Oak Grove Drive

Pasadena, CA 91109-8099