# Cooperative Robotic Cliff Exploration

TRIWG FY '98 New Proposal

PIs: Richard Welch, Paolo Fiorini

Co-PIs: Richard Volpe, Brian Wilcox

Jet Propulsion Laboratory

# 1.0 Objective

The goal of this proposal is to develop advanced hardware and software components enabling the exploration of cliffs and other near-vertical surfaces by a cooperating team of micro and nano rovers.    This technology will give access to scientifically interesting terrain, such as craters and cliff faces shown in Figure 1, not reachable by a single rover.    The utilization of micro and nano rover technologies developed under other NASA research programs will allow this task to focus on the following two areas:

1) Tether system development to design, fabricate and integrated a miniature tether system enabling locomotion on steep terrain.

2) Cooperative sensing and control to collect data from different rovers, integrate them as appropriate, and control rover motions.

The tether mechanism will be integrated on a nano    rover base and connected to a micro rover (Pathfinder size) as the mother rover.    The proposed three year effort will demonstrate the deployment of a tethered robotic cliff explorer from the mother rover, and the cooperative descent/ascent of more than 75 meters on near-vertical natural rock while gathering scientific data with representative science instruments.

# 2.0 Justification

This proposal represents a logical evolution of JPL's rover technology and research programs to develop hardware and software enabling the cooperation among rovers and thus improving the capabilities of    Mars exploration missions. This proposal synergistically combines the JPL varied resources into an efficient, complementary arrangement. Furthermore, it closely follows NASA guidelines of developing highly integrated and cost effective systems that can meet the challenge of Mars exploration. The technologies proposed here will provide increased robustness, autonomy, safety and reliability to Mars exploration missions. The use of multiple, cooperating rovers will allow to design missions robust to the failure of a single rover, and with a better resource allocation. With this approach, long-range autonomy of micro rovers will be enhanced by the accessibility to difficult areas provided by nano rovers.

Planetary Scientist Jack Farmer of ARC confirmed the scientific value of robotic cliff exploration.    “ The value would be to examine the finer-scale stratigraphy and microstructures in particularly layered rocks. As a geologist, I can assure you this kind of info could really contribute a lot to understanding the local geology, particularly in settings where beds are mostly flat-lying and where the stratigraphy can only be accessed in vertical exposures.    This would be practical for many areas of Mars”, [personal communication November 1996]. Stratification layers in cliff faces, craters, crevasses and outcroppings are a key source of geologic information. However, accessing cliffs or near-vertical surfaces is not possible with rover and aerobot systems being proposed for Mars exploration.      The ability to gather close-up imagery as well as mineralogical information (such as by near-IR spectroscopy) of exposed layering will enable scientists the gain a much better understanding of the geological history of the area being explored.    The features of exposed stratification layers may provide access to geologic records similar to those recovered by deep core drilling. However, deep drilling is not likely to occur in near term missions because of mass and power constraints. A tethered deployment system, as demonstrated by the CMU Dante projects [4,5], is a viable method for exploring cliffs or near-vertical surfaces.

# 3.0 Approach

The approach of this proposal is to leverage other technology development efforts as much as possible to achieve rapid prototyping and demonstration of the robotic cliff explorer at a reasonable cost.    The JPL nano rover prototype is an ideal starting point for the mobility, electronic, and science sub-systems.    Previous work on rover power/communication and tether system, done by JPL Rover Technology Task and CMU Dante projects, will provide the basis for the tether system. A three-year effort is proposed to integrate these technologies with new algorithms for cooperative control and to demonstrate a realistic experiment of robotic cliff exploration.    In the following we briefly summarize our technology base and outline the new technology developments.

Figure 2    Nano rover prototype

## 3.1 Leveraged Rover Technology

### 3.1.1 Nano Rover

The nano rover shown in Figure 2 provides the ideal base for the robotic cliff explorer because of its small size, posable bogie mobility mechanism, science instruments and flight-like design.    The posable bogie mechanism enables the individual placement of each wheel, as required to achieve stability on irregular surfaces (see nano rover continuation proposal for details).

Furthermore, the same mechanism allows the vehicle chassis and the science instruments to be pointed at the cliff face with an arbitrary orientation to achieve the best scientific results. The nano rover science payload consists of a multi-band imager and near infrared (1-2.5 nm) point reflectance spectrometer. These instruments are the basic suite desired by many Mars scientists to better understand the Martian geology and mineralogy.    They have been included in the nano rover design to ensure an adequate scientific return and expand the primary scientific objective of new Mars Missions.    Furthermore, they provide a mean for validating the data gathering capabilities of the nano rover in terrestrial tests.    Finally, the nano rover electronics uses a flight-like design with low-power, radiation hard components. This design imposes limitation on computational performance, but forces an optimal use of the resources, and will simplify the transition to a true flight system.

3.1.2 Long Range Science Rover

Figure 3 Rocky 7 and nano    rover prototype

Figure 3 provides a size comparison of the Long Range Science Rover (LRSR) Rocky 7 (Pathfinder size) and the nano    rover.    The LRSR provides an ideal mother rover for carrying the proposed cliff explorer, for cooperating with the nano rover during motion across the cliff face, and    for supplying additional computational power.

### 3.1.3 Dynamic Motion Planning

The flexible tether between the micro rover and the nano rover represents a dynamic constraint that must be satisfied when planning and controlling their coordinated motion. An algorithm suitable for motion planning and control in the presence of dynamic and static constraints is presented in [8], and it will provide the basis for developing the coordinated motion needed in this task. Figure 4-a shows an example of constraint satisfaction, and Figure 4-b a simulation result. This algorithm computes a simple geometric representation of the state variables of the robot that do not violate the problem constraints. The vector VA represents a velocity satisfying the dynamic constraints of robot A and avoiding obstacle B.

a						b

Figure 4 Path planning with dynamic constraints.

## 3.2 Tether System Design

### 3.2.1 Tether Mounting on Nano Rover

Side view

Instruments facing cliff

Side view

Top View

~10cm

Figure 5 Tether system mounted on nano rover

Figure 5 shows schematically the tether system attached to nano rover top face via a passive rotary joint with position encoding, replacing the solar panel.    The tether is deployed from the nano rover rather than the mother rover to avoid dragging it across the terrain.    The tether spool is actuated and includes an encoder to determine the amount of tether deployed.    A level-wind feed mechanism is used to evenly wind/unwind the tether from spool.    The tether provides mechanical support as well as supplies power to the nano rover. A slip ring assembly will provide connection of tether power wires from the tether spool to the nano rover. Communication is achieved using the nano rover developed RF transceiver.

The body of the nano rover can rotate to any position between the two shown schematically in the side views of Figure 5, in order to point the science instruments at areas of interest, as shown by the shadowed box. The top view of Figure 5 shows the ability of the nano rover to rotate relative to the tether by using its wheels to turn in place.    This enables the system to apply tractive forces in any direction to achieve vertical motion as well as cross-slope or horizontal motion.

### 3.2.2 Connection to the Mother Rover

The cliff explorer is envisioned as a stand alone bolt on package that communicates with the mother rover via as serial interface.    The    support equipment attached to the mother rover includes:

- Attachment bracket for tethered nano rover deployment.

- Tether connection point with force sensing (tether tension and direction).

- RF transceiver for communication with nano rover.

- Power unit providing high voltage supply for tether power transmission.

- Computer/software for sensing and serial communication with the mother rover.

The equipment is estimated to weigh approximately 500grams, not including additional batteries to supply power during testing and demonstrations.

Figure 6    Mounting on Rocky 7

The tethered nano rover system could be attached to the mother rover, as shown in Figure 6.    To deploy, the nano rover can simply lower itself to the ground by means of the tether and then drive to the area of interest.    Note that the nano rover may actually need to traverse several meters of relatively flat terrain before reaching a cliff edge, to avoid any risk to the mother rover.    This is another important reason motivating the selection of a rover configuration for the robotic cliff explorer. Note also that the planned deployment direction of the mother rover is parallel to the cliff edge.    Provided that rocks are not blocking the motion, this position enables the cooperative exploration of a larger cliff area by moving the mother rover along the cliff edge.

### 3.2.3 Tether System Mobility

Figure 7 Simple cross slope mobility analysis

The tether provides stability for the nano rover traversing steep slopes.    The system is primarily intended for use when the nano rover has tractive contact with slope (< 90 degrees).    Figure 7 shows a simple analysis of cross slope mobility of the system. Figure 8 is a graph of the relationship between the slope and the achievable cross slope angle at several different coefficients of friction.

Figure 8 Graph of cross slope mobility function

For example, Figure 8 shows that at a cliff slope of 75 degrees from horizontal and coefficient of friction of 0.3 the tractive force of the wheeled nano    rover could angle the tether up to ~4 degrees off the vertical fall line.    With 10m of tether length this allows for ~1.4m wide cross slope mobility area.    The ability to move in this area not only increases the science exploration area but also enables the system to maneuver if needed to avoid localized obstacles.

3.2.6 Two Tether Option

Figure 9 Two-tether deployment option

The option to utilize two tethers to control the nano rover position has the potential of achieving a much higher mobility, since the nano rover could move over a much larger cliff face area, as shown in Figure 9. This design will be considered during the first year’s effort to see if it is a viable concept for development in later years.

## 3.3 Cooperative Control of Tethered Robots

Previous work on tethered robots has focused on planning algorithms for multiple robots    tethered to fixed bases [6] and avoiding each other [7]. Research on cooperative control of multiple mobile robots has presented solutions for model generation [1], distributed sensing [2], and position acquisition [3]. In this technology element, we will address coordinated planning and control of tethered robots when the tether base is moving, when forces are applied through the tether between the two rovers, and in the presence of terrain obstacles.

The motion planning for the nano rover will be based on extensions to the case of cooperating rovers of the algorithm presented in [8] for a single robot. The simple geometric representation of the problem computed by this algorithm makes it very efficient for pre-mission planning, and its small memory requirements make it suitable for real-time control of the nano rover motion. The constraints represented by the tether can be easily combined with the dynamic constraints of the rovers and with the constraints due to the obstacle avoidance, to yield the set of feasible controls for the rovers.

Cooperative control algorithms will be developed to detect and compensate for tether snags, and/or maneuver around them. The algorithm will use the asymmetry between the higher maneuverability of the nano rover, and the higher pulling force of the mother rover to achieve overall capabilities better than those of each individual rover. Cooperation between the two rovers will be particularly necessary to aid in freeing the nano rover should it become stuck, as well as enabling cross-slope exploration by moving the mother rover along the cliff edge.    The development of these control methodologies will be a major focus of the task.    Navigation and control algorithms for independent cliff traverse by the tethered nano rover will be explored in parallel.

A model for representing the environment and integrating the sensory data will be developed for pre-exploration planning, and it will be updated in real time during the evolution of the cliff exploration. A variety of sensors will be used to facilitate mobility in this environment and to control the rovers.    The attachment point at the mother rover will contain a force sensor to detect the tension and direction of the tether deployment, and a similar sensor will be used at the nano    rover end.    This will provide critical information on tether snagging.    Other sensors on the nano    rover will include:

- Accelerometers to determine nano    rover body orientation

- Current sensing of wheel and tether drive motors

- Position sensing of wheel struts (bogies)

- Encoding of tether length deployed

- Proximity sensing

# 4.0 Facility Usage

For technology development we plan to use the existing Mars Rover development Laboratory, and to upgrade the Mars Yard with an artificial cliff for outdoor tests and validation. As such, the investment made in the past years for Mars Rover development will continue to be utilized in full, and the new facility cost for this proposal will be minimal. Furthermore, from the outset, close collaboration and coordination will be established between this task and the other Mars Rover tasks at JPL. This approach will facilitate the task progress, ensure an easy transfer of technology among JPL projects, and provide continuous feedback to keep the development focused on Mars exploration requirements. This strategy will prevent any duplication of effort, while minimizing total costs and maximizing total utility.

# 5.0 Deliverables, Schedule & Resource Requirements

| Milestones | FY ‘98 | FY ‘99 | FY ‘00 |
| --- | --- | --- | --- |
| Demonstrate ~1kg mobility system capable of descending/ascending 10m over a near-vertical surface under teleoperation control | ======= |  |  |
| Demonstrate system with integrated science instruments capable of descending/ascending 25m over near-vertical rock surface under supervisory control |  | ======= |  |
| Demonstrate autonomous system operation in 75m outdoor decent/ascent over a near-vertical natural rock surface |  |  | ======= |
| Funding Level | $475K | $475K | $425K |

## 5.1 FY '98 Activities ($475K)

The first year of the task will focus on the design of the tether deployment system and on the development of basic algorithms for cooperative control.    A nano rover mobile base with on-board electronics, but without instruments, will be acquired and equipped with a mechanical tether system (not including power) for testing and evaluation.    The system will be deployed from a mother rover (with fixed position) and controlled under teleoperation to descend/ascend >10 meters over a near-vertical surface (level 2 milestone).    FY ‘98 activities are broken down as:

Design/fabrication of tether mechanism					$150K

Replication of nano    rover and integration of mechanical tether		$150K

Development of navigation and cooperative control algorithms		$100K

System Testing and evaluation						$75K

TOTAL									$475K

## 5.2 FY '99 Activities ($475K)

In FY '99 science instruments (multi-band imager and near-IR spectrometer) will be integrated on the nano    rover and the mother rover mounted support equipment will be developed. Cooperative control methods will be implemented enabling the system to be automatically deployed from the mother rover to gather science images and spectrum.    A >25 meter decent/ascent under supervisory control on near-vertical rock surface will be demonstrated (level 2 milestone).

## 5.3 FY '00 Activities ($425K)

The final year will focus on sensing and control to enable fully autonomous operation of the system as well as the addition of power transmission in the tether system.    System testing will be done in natural outdoor terrain.    Final system demonstration will be an autonomous descent/ascent (with multiple science stops) of >75 meters over a near-vertical natural rock surface.

# 6.0 Dual-use Potential

The outcome of this effort will be directly applicable to tasks requiring the coordinated operation of groups of rovers, such as search and rescue operations and humanitarian de-mining, and inspection of vertical surfaces, such as chemical storage tanks, and mine shafts. The developed technology has potential use in terrestrial scientific investigation of steep terrain such as cliff faces or other difficult to reach areas.    Because the system is so small it could easily be hand carried and deployed at a field site for remote operation.

# 7.0 Participants & Partnerships

Participants will include members of both the nano    rover and Long Range Science Rover Technology tasks as appropriate.    Science input will be sought from Mars scientists who will be invited to participate in field testing of the developed system.

# 8.0 Bibliography

[1] Rao, N.S.V. and Protopopescu, V., and    Manickam, N., “Cooperative Terrain Model Acquisition by a Team of Two or Three Point-Robots”, Proceedings of the IEEE International Conference of Robotics and Automation,    pp. 1427-1433, Minneapolis, MN, April 22-28 1996.

[2] Cai, A. and Fukuda, T. and Arai, F. and Ishihara, H., “Cooperative Path Planning and Navigation Based on Distributed Sensing”, Proceedings of the IEEE International Conference of Robotics and Automation,    pp. 2079-2084,    Minneapolis,    MN, April 22-28 1996.

[3] Karazume, R. and Hirose, S. and Nagata, S. and Sashida,    N., “Study on Cooperative Positioning System”, Proceedings of the IEEE International Conference of Robotics and Automation, pp. 1421-1426, Minneapolis, MN, April 22-28 1996.

[4] Apostolopouls, D. and Bares, J., “Locomotion Configuration of a Robust Rappelling Robot,” Proceedings of the IEEE IROS ‘95 Conference, pp. 280-285,    Pittsburg, PA, 1995.

[5] Krishna, M. Bares, J. and Mutschler, E., “Tethering System Design for Dante II,” Accepted for publication ICRA ‘97.

[6] Sinden, F., “The Tethered Robot Problem”, Int. Journal of    Robotics Research, Vol. 9, N. 1, Feb. 1990, pp. 122-133.

[7] Hert, S. and Lumelsky, V., “The Ties that Bind: Motion Planning for Multiple Tethered Robots”, IEEE Int. Conf. of Robotics and Automation, 1994, pp. 2734-2741.

[8] Fiorini, P. and Shiller, Z., “Motion Planning in Dynamic Environments”,    7th Int. Symposium of Robotics Research, Munich (D), October 1995.

POC: Paolo Fiorini

(818) 354-9061

fiorini@telerobotics.jpl.nasa.gov