Telerobotic Exploration of Europa (TREE)

Jet Propulsion Laboratory

California Institute of Technology

Pasadena, CA

Paul S. Schenker (PI),

E. T. Baumgartner, F. D. Carsey, G. S. Hickey,

J. C. Horvath, J. A. Jones, and R. G. Kern (Co-I’s)

Contact: Paul.S.Schenker@jpl.nasa.gov

phone: (818) 354-2681 (voice), FAX: (818) 393-4057

Spacecraft Galileo’s recent observations of Europa – an icy satellite of Jupiter – suggest that a deep ocean underlies thick water-ice surface layers (1-100 km). Gravity effects from Jupiter and Jupiter’s neighboring moons Io, Europa, and Ganymede have induced active volcanoes in the first and a molten iron core with magnetosphere in the third. Europa, the middle moon, is inferred to possess a warm ocean floor and hydrothermal vents – an environment known to have fostered over 300 chemico-synthetic life-forms in earth-based analogues. The science community therefore has great interest, paralleling its recent focus on Martian exobiology, to explore the Europan ice and water bodies in depth. In addition to Europa, other moons in our solar system, such as Jupiter’s Callisto and Saturn’s Enceladus, are also theorized to have liquid water oceans below their thick ice caps. Several recent workshops, led in part by members of this team, detail this mounting interest, related mission concepts, and science instrumentation options. The emergent mission system paradigm is one of subsurface ice and water-based robotic exploration, enabled by a penetrating “Cryobot” and deployable “Hydrobot.” If such missions are to be undertaken, new robotic technologies in mechanical, thermal, and control design need to be defined, developed, demonstrated, and evaluated. There is opportunity to do so in simulated earth context, with potentially great science value – a terrestrial mission to the newly discovered Lake Vostok: a sub-glacial (~ 3 km deep) pristine Antarctic water body thought to harbor evolutionary-distinct life forms over a million years old.

Objectives

Figure 1: Cryobot/Hydrobot artist’s concept.

The overall project objective is to design, develop, and test within an Earth-relevant environment the technology required to explore in-situ the ice sheets and possible liquid ocean of Europa. We will focus on the development of technologies that will enable the mission to Europa while remaining extremely relevant to the science requirements for Earth-based missions. There are at least two basic parts of the engineering problem in accomplishing this feat: a Cryobot carrier probe to melt down through the ice, performing observations as it goes; and a Hydrobot miniature submersible that fits inside the Cryobot and is released once a liquid water regime is entered. As an aid to understanding and visualizing the system concepts, an artist’s conception of the Cryobot and Hydrobot is shown in Figure 1.

The main focus of this proposed task is the formation of an integrated program of technology development and demonstration for robotic exploration of ice and oceans on Earth and Europa that benefits both planetary and terrestrial communities. Specific technical objectives include:

- Design, develop and fabricate the Cryobot. Critical technology thrusts include addressing structural and thermal design issues, developing smart melting technology, understanding the ice melting/sublimation process, identifying communication techniques through the surface ice, and ensuring the thermal isolation of the heat source from components.

- Design, develop and fabricate the Hydrobot. Critical technology thrusts include the development of robust navigation techniques, development of a buoyancy system for controlling the descent and ascent of the vehicle, addressing structural issues related to operation within a high pressure environment, identifying acoustical communication techniques for data transfer between the Cryobot and Hydrobot.

- Conduct full-scale demonstrations of the Cryobot and Cryobot/Hydrobot systems within accessible Earth environments: sub-glacial lakes in Greenland or Canada and Lake Vostok in Antarctica. The Lake Vostok demonstration promises a significant return for many in the scientific community including glaciologists, biologists, and oceanographers.

Justification

| Characteristic | Value |
| --- | --- |
| Diameter | 3,138 km |
| Mass | 4.8E22 kg |
| Surface Gravity (Earth = 1) | 0.135 |
| Rotational Period | 3.551 days |
| Density | 3040 kg/m^3 |
| Surface Composition | Water Ice |
| Surface Temperature | 100 K |
| Surface Pressure | 10E-11 bar |
| Ice Thickness (best estimate) | 1-100 km |
| Ice/Ocean Interface Pressure (best estimate) | 2 kbar |
| Ocean Temperature (best estimate) | 273 K |

Table 1: Europa characteristics

Recent findings suggestive of life on Mars coupled with high-resolution image data from the Jovian moon Europa indicating water/ice tectonic plate movement have fueled intense interest in the possibility of extra-terrestrial life.    The Europan data in particular has renewed interest in the prospect that life form(s), however primitive, may be present in the ocean postulated to be beneath its icy surface. Tidal heating due to the position of Europa in orbit around Jupiter may be creating volcanoes similar to those seen on neighboring Io. These volcanoes would be venting under the ocean and creating environments similar to those seen in deep ocean vents in Earth’s oceans which are known to harbor entire ecosystems including bacteria, tube worms, crabs, and fish. For Europa, major scientific questions arise: What is Europa’s physical oceanography? Does the ocean have currents? What is the chemistry of the ocean/ice? Are biological processes at work? What might some of the biological habitats be?

On Earth, there are recent and surprising climate-change findings from deep coring of Greenland and Antarctica as well as clear indication of ongoing climate change in the midlatitude glaciers, ice caps, and permafrost domains. In addition, subglacial lakes, such as Lake Vostok which is located between the bedrock and basal ice of the Antarctic ice sheet, may harbor life in unique forms analogous in some respects to the situation on Europa. Vostok is a volcanically-heated freshwater lake formed several kilometers under the surface of the Antarctic ice sheet whose water depth is estimated to be 100 to 500 meters. It is postulated that the lake has been sealed off from the rest of the Earth for a period of one to three million years, thus being a closed, potentially long-lived evolving ecosystem.

Figure 2: Location of Lake Vostok

Lake Vostok in many ways is a “Europa simulator” on Earth – with issues and opportunities similar to those in the Europan ocean. This combination of scientific interests converge in the need to develop advanced probes which can enter ice sheets, melt their way through the ice sheet with minimal disruption and contaminant containment, deploy sensors to explore the fluid ocean underneath the ice, and return data taken to remote observers. The United Nations committee which controls access to Antarctica (SCAR, Scientific Committee on Antarctic Research) has suspended work above Lake Vostok until such time that a plan with acceptable forward and back contamination control can be put in place for actual penetration into the lake. A borehole exists to within hundreds of meters of the water interface at the present time. Contact has been made with SCAR, and feedback has been given that it will be required that the techniques and hardware to explore Vostok will need to be demonstrated somewhere else first – preferably in an ice-covered deep lake. Fortunately, such lakes are known to exist in Greenland and elsewhere. Thus, tests in Greenland will be scheduled before any deployments in Lake Vostok itself.

Approach

The main focus of this task is the development of the Earth-analog Cryobot and Hydrobot for use in the Lake Vostok environment while addressing the critical technologies that will enable the Europan Cryobot/Hydrobot mission. The overall system concept for both missions begins with the surface deployment of the ice-melting Cryobot and ends with the water deployment of small Hydrobot submersibles from the Cryobot. For both the Cryobot and the Hydrobot, science investigations are conducted throughout the mission duration. The Cryobot remains tethered to the surface for communication purposes during the Europan mission (possibly via communication repeater stations dropped during the probe’s descent) and for power and communication purposes during the Earth missions. Several different scenarios are being considered for the operation of the Hydrobot. These include fully tethered operations (power and communication), communication-only tethered operation, and tetherless operation using battery power and acoustical communication techniques. The remainder of this section details the technical issues surrounding the Cryobot and Hydrobot along with the science packages envisioned for each. This section concludes with some remarks regarding planetary protection issues for the Lake Vostok and Europa missions.

The Cryobot:

The Cryobot is similar in many ways to the Philberth probe [Philberth, 1974], developed and used occasionally in terrestrial ice since the 1960s. The probe is characterized by melting its way through the ice and descending due to gravity alone. These probes are powered by a surface electric generator via a wire spooled from the rear of the descending probe. The melted water above the probe is eventually re-frozen as the probe descends. The University of Nebraska, Lincoln is home to the NSF Polar Ice Coring Office (PICO) which has the charge of developing technology for all NSF-funded ice exploration. Glaciologists at PICO have worked extensively with these probes for over thirty years, and they have developed “smart” melting mechanisms that direct the heat in the proper location of the melting head, such that the probe descends almost perfectly vertical.    Furthermore, the melting head cone has been designed to whisk dust and small pebbles to the side, so as to avoid contaminant thermal insulation buildup. The University of Nebraska’s extensive experience with this type of probe is the reason JPL is proposing that they build the first electrically heated Cryobot for testing at both Greenland and Lake Vostok, Antarctica. It is intended that the Cryobots for Earth studies will be fabricated by PICO and initially tested at their facility in Lincoln (see letter of interest/support).

Figure 3: Europan Cryobot schematic.

The actual Europa Cryobot, however, will obtain heating from a series of RTG General Purpose Heat Source (GPHS) modules, which emit 250 watts thermal heat and weight about 1.5 kg each.    A typical cryobot heating section might contain a stack of 16 GPHS modules and be about 0.85 meters long with an outside diameter of about 0.15 meters.    It is anticipated that the resulting 4 kW thermal heat dissipation could descend through ice at the rate of about 1 km per month, although actual detailed thermal modeling is necessary, based on laboratory measurements at a full range of ice temperatures (100K – 273K). In order to maximize energy efficiency on the Europan Cryobot, it will be necessary to switch from the initial sublimation mode to a melt mode as soon as possible since sublimation requires eight times more energy than melting.    This can be done by installing a pressure-containing cap in the hole above the Cryobot after it has descended a few meters into the ice surface.    The cap could be a lightweight, inflatable cap that maintains a lower hole pressure of about 0.01 bar or greater.

The Cryobot/Hydrobot will have to survive a broad range of environmental conditions, particularly in the Europan version. (Surface conditions at Europa are near-vacuum and 100 degrees Kelvin;    ice/water interface pressures might be as much as 2 kbar.) To cover some of the endpoints of this range, probe descent rate experimentation is currently underway for water ice at atmospheric pressure and approximately 268 degrees Kelvin at Caltech, and at approximately 100 degrees Kelvin and near-vacuum at University of Leicester, Leicester, England. Along with the development of the Cryobot, work on the thermal modeling of Europan ice melting/sublimation process will commence.

In support of the Europa Cryobot thermal development, JPL proposes to continue ongoing tests at the University of Leicester, UK and at Caltech for the development of preliminary ice probe melting thermal models.    JPL will also provide additional testing and expand these models for cryogenic and vacuum operation.    An additional necessary work element is the development of a cap, possibly inflatable, that will limit initial loss of penetrator heat due to sublimation.    This cap must be capable of sustaining pressures of about 0.01 bar in the underlying melt cavity.

Another work element that is essential for a Europa Cryobot design is the development of a “smart” heating system which has a configuration similar to a stack of RTG GPHS modules.    Present Philberth glacial probes use electrical heat concentrated in a cone melting head in such a way that a mercury thermal switch directs the heat to the proper location to ensure vertical descent.    A Europa Cryobot probe, however, must have the heat dissipated in a much larger region, e.g., 0.85 meters long ´ 0.15 meter diameter for a 4 kW stack.    The heat must then be collected with a pumped fluid loop such that the hottest portion of the fluid is directed to the melting head (see Figure 3).    The head must have a “smart” sensing system that directs the heat to the proper location on the head to ensure vertical descent.    The probe must also be capable of whisking away Europan ice contaminants, e.g., dust, sulfur, frozen ammonia, etc., as well as small pebbles.    It may also be necessary to include a short range glacial radar/sonar system (~100 m) to avoid larger rocks and meteorites in the Europan ice crust interior. Thus the Europan Cryobot must have a “smart” fluid loop heat transfer system that can ensure vertical descent, while avoiding large rocks.

Finally, thermal isolation of the RTG power sources from the remainder of the Cryobot/Hydrobot package will be critical. This thermal isolation would consist of a combination of low conductivity mechanical structure and multilayer insulation. Multilayer insulation is very effective in vacuum and can be developed to be effective at high temperature. An alternative concept that may be lower mass is opacified carbon aerogel which is an effective high temperature insulator within low pressure and vacuum environments.

The Hydrobot:

Analogous to the rover/lander paradigm, the Hydrobot provides the required mobility for venturing out beyond the confines of the Cryobot thereby increasing the overall science value of the mission. Not unlike planetary rover technologies, the design and development of the Hydrobot represents significant technical challenges in the area of autonomous navigation, communication, packaging, and power. In recent years, some of these issues have been addressed by the Autonomous Underwater Vehicle (AUV) research community.

Specifically, autonomous, deep-submersible vehicles have been developed by researchers at Woods Hole Oceanographic Institute (WHOI) and by researchers at the Massachusetts Institute of Technology (MIT) during the last 5 years. The WHOI-developed vehicle is known as the Autonomous Benthic Explorer (ABE) [Yoerger, et al, 1991] while the MIT vehicle is known as Odyssey [Bellingham, et al, 1994]. Both of these vehicles are capable of reaching depths of 5000 meters and provide a unique capability for exhaustive oceanographic studies through autonomous, deep ocean surveys. While these vehicles represent a breakthrough development in underwater robotic technologies, the Europan mission puts a unique set of requirements on the development of the Hydrobot as distinct from current Earth-based AUV designs. Woods Hole will be a major contributing partner in this task and will provide their many years of experience and technology to the design and development of the Hydrobot (see letter of interest/support).

Figure 4: ABE schematic (from WHOI).

The main technical thrust areas for the design and development of the Hydrobot will focus on the following areas: navigation, acoustic communication, and buoyancy control. The design of the Hydrobot will leverage off of the already successful ABE vehicle. Thus, an initial concept for the Hydrobot will utilize an open-frame design in which the lower pod contains the heavy batteries and electronics while the upper pods contain the buoyancy system. This design is dynamically stable in the roll and pitch planes and does not require any active control (i.e., use of fins) to maintain vehicle stability. Miniaturization of this design will be a major effort in order to fit the entire Hydrobot package within the 15 centimeter diameter constraint associated with the outside diameter of the Cryobot (see Figure 3). Like ABE, the Hydrobot will be maneuvered via three actuators (DC brushless motors) one of which controls the forward thrust of the vehicle and two small thrusters for controlling vertical and horizontal movements. Also critical to the design of the Hydrobot is the development of lightweight mechanical structure and electronic housings that can withstand the high-pressure environments of the deep ocean.

The navigation and control of the Hydrobot will primarily be accomplished through dead-reckoning (localization based on sensed vehicle speed and heading/pose) and an inertial package (e.g., fiber optic gyro). Earth-based AUVs typically utilize transponder nets (multiple sonar transmitters at known locations) for vehicle localization and correction of dead-reckoning errors. Obviously, for the Hydrobot, transponder nets are not available, therefore, the Hydrobot will rely heavily on dead-reckoning and the inertial unit for maintaining accurate position estimates relative to the Cryobot. One focus of this task will be the evaluation and utilization of small, low power gyro units for Hydrobot navigation. Note that this task development could have a significant impact on the use of inertial units for rover navigation which have experienced problems incorporating gyro information into the rover localization problem. In addition to basic localization, obstacle avoidance capabilities will be developed using down-looking sonar so that the Hydrobot does not collide with the ocean bottom.

For Hydrobot communication, two scenarios are being considered: tethered and untethered operations. For untethered operations, recent advances in acoustical modem technology have realized data rates of 9.6 kbps under ideal acoustic conditions [Freitag, 1997]. However, this data rate performance is highly dependent on the following factors: ambient acoustic noise; signal losses due to spreading losses, absorption, and signal blockage; multipath signals arising from reflected acoustic rays; and channel fluctuation. Thus, the use of acoustic communication technologies in current underwater vehicles is limited unless the acoustical environment is well characterized and the required data bandwidth is low. For the Europan Hydrobot, the environment is envisioned to be adverse to acoustic communication due to the expected noise from the movement of the ice sheets and the unknown chemistry of the Europan ocean. However, for the Lake Vostok Hydrobot, the environment will be favorable to the use of acoustic communication technology due to the low expected ambient noise (little ice sheet movement) and well characterized water properties. We will work with acoustic communication experts at Woods Hole to design and build an acoustic modem for the Lake Vostok mission and evaluate the possibility of acoustic communication in the Europan oceans.

To increase the mission life of the Hydrobot and substantially increase the data rate between the Hydrobot and the Cryobot, tethered operations are also being considered. Tethers have been used quite successfully in underwater vehicle technologies (e.g. Woods Hole’s JASON Remotely Operated Vehicle [Yoerger, et al, 1986]). Tethers typically consist of a twisted three copper wire strand and three fiber optic strands wrapped in a steel jacket to prevent breakage. The tether can be spooled out from the Cryobot or from the Hydrobot or from both. By providing external power to the Hydrobot and a dedicated communication link, the scientific return from the Hydrobot missions to both Lake Vostok and Europa will be greatly enhanced. Therefore, it is proposed that, for the Lake Vostok mission, two Hydrobots will be deployed from the Cryobot: one tethered and one untethered. In this way, we will be able to provide the scientists with two unique capabilities: a long term (on the order of months) tethered vehicle platform for Cryobot-local operations and a short term (on the order of days due to limited battery life) tetherless vehicle platform for long-range traverses from the Cryobot. Thus, both Hydrobot vehicles released in Lake Vostok will provide valuable lessons learned that will be utilized for the design of the Europa Hydrobot.

The final technical thrust area deals with the problem of achieving buoyancy control within a limited mass/power budget. One possible solution for achieving buoyancy control is through the use of a thermal generator which is used to heat a water source contained in a ballast tank within the Hydrobot. To conform to mass and volume constraints, the ballast tank will be an inflatable structure which is valved so that the tank can exchange the hot and cold water source. Once heated, the density of the highly pressurized water can be reduced by over 30% thereby providing the necessary buoyancy through the change in mass of the entire Hydrobot vehicle. (At high water pressures, the density of water is reduced by over 30% when the water temperature is increased from 20 to 300 degrees C.) The design and development of the thermal generator will be driven by the overall Hydrobot mass and power constraints. The buoyancy control system is critical to the Europa Hydrobot because of the large number of unknowns associated with the Europa ocean. With this thermally-controlled buoyancy system, we hope to develop a technology that will enable buoyancy control within an unknown hydrodynamic environment.

Instruments:

A JPL-sponsored Earth/Europa workshop was held at JPL on October 28 and 29, 1996 in order to examine the scientific objectives and observational techniques likely to be of interest to terrestrial and planetary science associated with an integrated Earth/Europa program and the development of enabling technologies. A follow-on to the JPL Earth/Europa workshop to determine requirements for an instrumentation package for the Cryobot and Hydrobot, especially with regards to detecting “life”, is being planned for April 9-11, 1997.    Many scientists and engineers have already accepted invitations to this workshop which is expected to give a prioritized set of science requirements and desired instrumentation. Preliminary results from our October workshop, however, can be used to give broad outlines of the instrumentation expected to be required on the Cryobot and Hydrobot.

The Cryobot would carry the In-Situ Chemistry Lab (ISiCL). This package would address particle size distributions, chemical constituent concentrations emphasizing a few key cations and anions, and isotopic distributions. It is likely that the emphasis will differ for Europa and Earth. On earth chemical species will be selected to reflect variables that characterize past climate states. Detection of life will be one of the primary goals of the ISiCL; we will be looking to our April workshop to define the parameters of this search. The funding of the ISiCL development is currently being discussed with various agencies and is not a part of this proposal, however, the resulting science instrument package will be integrated within the Cryobot.

The Hydrobot would have an instrument package that might include: a hydrophone to listen for volcanism; a device to look at backscatter of particulates; and a CTD (conductivity, temperature, density) device. A device to measure ocean currents, if any, and a sonar imager would be highly desirable. A visible imaging system would also be a strong desire, particularly to image near volcanic vents. The Hydrobot would also potentially carry a variation of the ISiCL package developed for the Cryobot, with a strong emphasis on searching for “life we can recognize.” Sampling mechanisms and exploration strategies and options for the Hydrobot are an open issue, and a major thrust of this proposal.

Planetary Protection Issues:

The planetary protection requirements for Lake Vostok and Europa Cryobots/Hydrobots will be in compliance with the COSPAR policy as determined by the NASA Planetary Protection officer. Current NASA requirements for a “life detection mission to Mars” can be summarized as follows: Hardware will be assayed and certified to have a bioburden of less than 300 spore forming microorganisms per square meter prior to sterilization. Subsequently terminal sterilization by dry heat at 112 degrees C for a period of a minimum of 30 hours may be required. Since the presumed aqueous anaerobic environment should possess some level of mineral salts and chemical source of reductant to support a number of known classes of microbes found on the surface, therefore sterilization will be used to prevent forward contamination.

Deliverables

FY98 Level 1: Design, develop and fabricate a neutrally buoyant Hydrobot vehicle and demonstrate the navigation and control capabilities of this vehicle in a constant depth, short range traverse (approx. 10 meters) [Aug 98]. Level 2 tasks include: investigate buoyancy control sub-systems, conduct thermal modeling of the Europan ice melting/sublimation process; complete preliminary ice melting experiments, establish science instrument definition for Cryobot and Hydrobot.

FY99 Level 1: Design, develop and fabricate an Earth-analog Cryobot system and demonstrate the vehicle melting through a simulated ice field to a depth of 10 meters [Aug 99]. Level 2 tasks include: define planetary protection requirements for Lake Vostock and Europan environments; design and implement a buoyancy control sub-system, integrate Cryobot science instruments; design the Europan ice melting/sublimation sub-system.

FY00 Level 1: Demonstrate the Cryobot in the Greenland sub-surface lake environment [Aug 00]. Level 2 tasks include: integrate Hydrobot science instruments; fabricate and demonstrate the Europan ice melting/sublimation sub-system within a cryogenic vacuum chamber; demonstrate the Hydrobot in a multiple-depth, long-range traverse (approx. 1000 meters).

FY01 Level 1: Demonstrate the Cryobot in the Lake Vostok environment [Feb 01] and demonstrate the Cryobot/Hydrobot in the Greenland sub-surface lake environment [Aug 01]. Level 2 tasks include: demonstration of sterilization techniques to satisfy planetary protection requirements, analyze and design thermal isolation concepts for the Europan Hydrobot.

FY02 Level 1: Demonstrate the Cryobot/Hydrobot in the Lake Vostok environment [Feb 02].

Resource Requirements

| Year | FY98 | FY99 | FY00 | FY01 | FY02 |
| --- | --- | --- | --- | --- | --- |
| Budget ($K) | 1000 | 1000 | 1000 | 1250 | 1500 |

Schedule (Level 1 Milestones in Bold)

Buoyancy control system concept	FY98, Q2

Experimental ice melting studies	FY98, Q2

Thermal modeling of ice melting/sublimation process	FY98, Q4

Science instrument definition	FY98, Q4

Hydrobot laboratory demonstration	FY98, Q4

Definition of planetary protection requirements	FY99, Q2

Design and implement buoyancy control system	FY99, Q3

Integrate Cryobot science instruments	FY99, Q3

Design ice melting/sublimation sub-system	FY99, Q4

Cryobot laboratory demonstration	FY99, Q4

Integrate Hydrobot science instruments	FY00, Q3

Demonstrate ice melting/sublimation sub-system	FY00, Q4

Demonstrate long-range Hydrobot traverse	FY00, Q4

Cryobot Greenland demonstration	FY00, Q3

Demonstration of sterilization techniques	FY01, Q1

Analyze and design of thermal/radiation isolation	FY01, Q3

Cryobot Lake Vostok demonstration	FY01, Q2

Cryobot/Hydrobot Greenland demonstration	FY01, Q3

Cryobot/Hydrobot Lake Vostok demonstration	FY02, Q2

Dual Use Potential

The proposed task will have a major impact on the technology used to support future missions to Europa and its interior oceans. The technological issues addressed in this task and the lessons learned from the Lake Vostok mission will enable the development of the Europa Cryobot/Hydrobot. The timetable and schedule proposed in this task mirrors the mission schedule currently under study for the in-situ exploration of the Europan environment. For example, the Advanced Concepts Office is considering two Europan missions. The first mission will carry a derivative of the Cryobot with the ISiCL package which will melt a relatively short distance into the Europan ice and take as many measurements as possible. A full derivative of the Cryobot/Hydrobot would fly on a later mission, and this later mission would then descend through a thin spot in the Europan ice (postulated to be of order 1-30 km) and deploy the Hydrobot to explore the ocean. Studies are underway to determine optimal and realistic phasing of these activities, trajectory opportunities, and so on.

In addition to the Europan missions, the technology developed will benefit terrestrial scientists, including planetary ice scientists, climatologists, and biologists by providing these scientists with the ability to reach to-date unreachable Earth environments. The science return from the proposed Lake Vostok mission will be invaluable and significant. The possibility of the discovery of life forms that are distinct from our evolutionary path is of great interest to the general science community and will provide further indications that life may exist on other solar system bodies, including Europa and Mars, or wherever water is present.

Finally, the component technology developed from this task will benefit future AUV designs by pushing the mass/volume/power constraints associated with the Hydrobot and Cryobot. The AUV community has yet to tackle these problems in support of the design and development of very small, yet capable, autonomous submersibles. The resulting Hydrobot could provide the oceanographic community with an simple, light-weight design that will enable the low-cost study of deep ocean features, such as lava flows, hydrothermal vents, and deep-ocean life forms.

Facilities

We will develop facilities at JPL needed for thermal testing of Cryobot probes including a large walk-in freezer (-20°C) for preliminary testing of Cryobot probe models and a cryogenic vacuum chamber (100K, 2-meter diameter) with high vacuum pumping speed for water vapor.    Facilities needed for testing the deep submersible Hydrobot include a hyperbaric water chamber (~1 kbar). In situ testing of the Cryobot/Hydrobot assembly in Greenland and Lake Vostok, Antarctica will require a mobile electric generator (~5 kW) that is well-insulated from the underlying ice surface in order to prevent the sinkage of the surface equipment into the ice.

Partnerships and Participants

This task proposal has been strongly influenced by an initial concept study conducted as a joint study between JPL’s Advanced Concepts Office, the Center for Excellence in In-Situ Science and Exploration, and the Ocean Science Research Element. This study has been underway for over a year, with significant growth in interest and partnering over the last six months including the strong participation at the October workshop and the expected participation at the April workshop. The initial concept study has provided an suggested key technologies in the process of laying out an overall mission timetable for Earth and Europa.

Industry Partnering And Outreach:

As a result of the initial concept study and the continuing workshops, university and industrial partnering has been quite strong. We have received letters of interest and support from the NSF Polar Ice Coring Office (PICO) at the University of Nebraska, Lincoln and Woods Hole Oceanographic Institution in Woods Hole, Massachusetts. PICO has the charge of developing technology for all NSF-funded ice exploration while Woods Hole is the recognized world-leader in the development of piloted, remotely-operated, and autonomous underwater vehicles. We will partner with PICO for the design and development of the Cryobot based on their extensive knowledge and recent testing of thermal probes in the ice sheets of Greenland. Likewise, we will partner with Woods Hole in the design and development of the Hydrobot by leveraging off of their knowledge of deep ocean submersibles and their current design of AUVs. To this end, E. Baumgartner (JPL) visited Albert Bradley and Dana Yoerger of WHOI on 2/14/97 to discuss technical issues related to the design of the Hydrobot.

There is also strong interest in this project by the pharmaceutical industry. This industry has long experience in instrumentation that works in wet biological environments, which is quite different from the usual aerospace experience base. They also have experience in contamination management. There is also a desire to obtain unique ice and ocean biologicals (e.g., anaerobic, ice-inhabiting, low-pH, high pressure/temperature, etc.). The industry traditionally obtains unusual biologicals to test for possible clinical usefulness, and the possibilities inherent in independently-evolved life forms are limitless. We have been seriously negotiating with this industry regarding a variety of forms of participation in this effort.

Science Interactions:

As stated previously, a Earth/Europa workshop was held in October, 1996 at JPL which was attended by terrestrial ice scientists (including representatives from PICO), planetary scientists, instrumentation/spacecraft engineers, and representatives from the pharmaceutical industry. This workshop has resulted in a white paper [Carsey and Horvath, 1996] which developed specific recommendations for essential scientific goals and instrumentation. Shortly thereafter, in November, 1996, a poster was presented on the overall concept at the Europa Oceans Conference #5 in San Juan Capistrano, California. Reactions to the poster were enthusiastic, and a conference poll of scientists at the end of the meeting showed that exploration of the ocean by a Hydrobot was preferred to a sample return mission by those present. Significant popular and scientific press coverage of the mission concept has been received as a result of this poster. Another paper has been submitted to an SPIE Conference on Instrumentation for Exobiology Research in San Diego in August, 1997.    Also, J. Horvath has been invited to speak about the concept in Trieste, Italy, in September, 1997 at a meeting on exobiology at the UNESCO science center there. Finally, in April, 1997, a second Earth/Europa workshop will be held at JPL to discuss the requirements and recommendations for the Cryobot and Hydrobot science instrument packages.

Cooperating Organizations & External Partners:

- Kenneth Jezek, Byrd Polar Research Center, Ohio State University [glaciology experiments]

- Karl Kuivinen, Polar Ice Coring Office, University of Nebraska Lincoln [ice mechanics]

- Tzyy-Wen Jeng,    Microbiology/Exobiology, Abbott Pharmaceuticals [bio-chemicals]

- Albert Bradley, Deep Submergence Lab, Woods Hole Oceanographic Institute [submersibles]

- JPL ice probe studies to date with investigators at Univ. of Leicester and Caltech

Biographies

Paul S. Schenker (PI) is Staff, Science and Technology Development, JPL/Caltech, leading robotics and mobility R&D for the Mechanical Systems Engineering and Research Division. He received his education at Cornell University (B.S. Engineering Physics ‘71) and Purdue University (M.S.E., Ph.D., Electrical Engineering, ‘72, ‘74). His technology specializations include machine perception, teleoperation & human-machine interfaces, supervisory control, and their applications to both terrestrial and space robotic systems. Dr. Schenker joined JPL in 1984, after serving on the Electrical Sciences faculty of Brown University and leading a Signal Processing Systems group at Honeywell Inc. Paul is active in IEEE and SPIE, and is an elected Fellow and officer of the latter.

Eric T. Baumgartner (Co-I) is Member of Technical Staff in the Science and Technology Development Section. He received the Ph.D. in Mechanical Engineering from the University of Notre Dame in 1992. Dr. Baumgartner currently serves as the systems engineer for advanced rover and manipulator technology tasks. His tenure at JPL began in 1996 and, previous to that, he was a faculty member in the Mechanical Engineering Department at Michigan Technological University, Houghton. During the summer of 1995, Baumgartner conducted research at JPL through the NASA/ASEE Summer Faculty Fellowship Program. He has published over 20 articles in the area of robotics, controls, and estimation theory. Dr. Baumgartner is active in the SPIE and ASME professional societies.

Frank D. Carsey (Co-I) received the Ph.D. in Physics from UCLA in 1971 for ultrasonic studies in transition metals. He then took a position at NOAA Wave Propagation Laboratory studying the probing of the lower atmosphere. In the polar science field, he has used satellite and in-situ data for examining issues in sea ice coverage, dynamics and thermodynamics for the past 19 years with positions at Goddard Space Flight Center, NASA Headquarters, and JPL. He was Task Scientist at JPL for the Alaska SAR Facility (ASF) Task from 1983 to the present, and at present he is chief Scientist at ASF. He also leads the Earth science aspects of in-situ instrument development for a project studying planetary ices and water bodies.

Greg S. Hickey (Co-I) has a B.S. from U.C. Berkeley, and a M.S. in Chemical Engineering from the University of Virginia and Astronautical Engineering from UCLA. He has published over 30 papers in the area of chemical systems and composite materials and co-authored a book on Spacecraft Antenna design. He was the lead engineer for the integrated thermal structures for the Mars Rover and Lightweight Survivable Rover programs. He co-chairs a session for Lunar and Planetary Thermal Control for the SAE/ICES conference.

Joan C. Horvath (Co-I) currently holds a joint appointment between the Technology Affiliates Program and Advanced Concepts Program at JPL. Prior to this appointment, she was deputy manager of software development for the sequencing subsystem of the TOPEX spacecraft; a sequence integration engineer on the Magellan Venus spacecraft; and concurrent engineer on the Cassini Saturn spacecraft. She received the B.S. in Aeronautics and Astronautics from MIT, and a M.S. in Mechanical, Aerospace, and Nuclear Engineering from UCLA. With J. Hanly and E. Koffman, she is co-author of the 1995 Addison-Wesley textbook, C Program Design for Engineers.

Jack A. Jones (Co-I): MSME, Rice University, NSF Fellow, 1973; BSME, Rutgers University, James Slade Scholar, 1970.    Jones is a Member of Technical Staff at JPL since 1979 performing thermal analysis, R&D, and developing engineering solutions for spacecraft thermal systems, planetary balloon systems, and ground-based refrigeration systems. He was a Senior Mechanical Thermodynamics Engineer at Garrett AiResearch from 1975-1979 performing thermal analyses and heat transfer designs for ground-based and spacecraft systems. Jones has written over 80 professional publications and holds 20 patents and 3 major NASA Inventions for refrigeration systems.

Roger G. Kern (Co-I), Ph. D. in Microbiology from U.C. at Davis, is a co-discover of hydrogen chemoautrophic plasmids in bacteria. He is currently a Technical Group Leader in the Chemical and Biological Technologies Group at JPL. At present he is JPL’s senior microbiologist supporting planetary protection activities associated with the Mars exploration. Since arriving at JPL in 1988 he has been task manager on DARPA, DOE sponsored tasks focused on chemoautotrophic polysaccharide production and served as molecular biologist associated with two recent flight radiation biology experiments on board STS - 42 and 76. Dr. Kern has authored over 10 papers in bacterial genetics and biochemistry and currently holds patents in remote biochemical assays and the biochemical genetic methods manipulating bacterial polysaccharides for the production of anti-inflammatory carbohydrate based pharmaceuticals.

References

J. G. Bellingham, C.A. Goudey, T.R. Consi, J.W. Bales, D.K. Atwood, J. J. Leonard, and C. Chryssostomidis, “A Second Generation Survey AUV”, Proceedings of the 1994 Symposium on Autonomous Underwater Vehicle Technology (AUV ’94), Cambridge, MA, July, 1994.

F. A. Carsey and J. C. Horvath, “A Program Concept for Integrated Studies of Ice and Oceans on Earth and Europa,” Earth/Europa Workshop White Paper, October, 1996.

D. L. DeVincenzi, P. Stabekis and J. Barengoltz, “Refinement of Planetary Protection Policy for Mars Missions” Adv. Space Res., Vol. 18, No. 1/2, pp. 311-316, 1995 COSPAR, 1996.

L. Freitag, Personal communication, Woods Hole Oceanographic Institution, Woods, Hole, MA, February 14, 1997.

K. Philberth, “The Thermal Probe Deep Drilling Method by EGIG in 1968 at Station Jarl-Joset, Central Greenland,” in Ice Core Drilling, ed. J. F. Splettstoesser, University of Nebraska Press, 1974.

D. Yoerger, A. Bradley, and B. Walden, “The Autonomous Benthic Explorer (ABE): A Deep Ocean AUV for Scientific Seafloor Survey,” Unmanned Systems Magazine, pp. 17-23, Spring, 1991.

D. Yoerger, J. B. Newman, and J-J. E. Slotine, “Supervisory Control System for the JASON ROV,” IEEE Journal of Oceanic Engineering, Vol. OE-11, No. 3, pp. 392-400, July, 1986.