Reconfigurable Robots for Space Exploration (RRSE)

Paul S. Schenker, Ph. D. (PI), Jet Propulsion Laboratory

Mechanical Systems Engineering and Research Division

paul.s.schenker@jpl.nasa.gov, (818) 354-2681 (voice)

Steven Dubowsky, Ph. D. (Co-I), Massachusetts Institute of Technology

Department of Mechanical Engineering

dubowsky@mit.edu, (617) 253-2144

Dimitrious (D. A.) Apostolopoulos, Ph. D. candidate (Co-I), Carnegie Mellon University

The Robotics Institute, Field Robotics Center

da1v@frc2.frc.ri.cmu.edu, (412) 268-7224

Objectives

At present, the development of NASA rovers is entrenched in single-point design:    a mission context is given, with many assumptions made; a fixed robot concept is developed by heuristics, supporting experimentation/simulation, and available analytic tools; and finally, a flight-targeted implementation is pursued subject to cost-cap and single pass implementation.    Recurring costs are implicitly high with this approach (little legacy), and actual vehicle performance remains subject to significant unknowns due the unstructured environments and limited prior database (terrain geometry, properties, etc.). We propose to define, technically develop, implement and demonstrate in earth-simulated planetary environments examples of robot architectures that can be easily and cost-effectively “reconfigured” for a changing task conditions, different tasks, or unexpected change in the robot state itself during any task.    Roughly speaking, these objectives map architecturally to variable geometry robots, modular robots, and autonomous self-repair and self-organization of same.    We plan a progression of    enabling technology and demonstrations:

- Creation of variable geometry, c. g. and traction conventional-wheeled rovers, and their benchmark evaluation in Mars simulation environments (MarsYard), wherein we have more conventionally run such systems as the JPL Rocky, Go-for, and LSR series of vehicles. Example:    a four wheel rover adapts its stance, weight distribution, and/or cleat surface traction in response to changed terrain

- Creation of a few representative building blocks (chassis/busses, actuators, drive linkages/differentials, wheels, cleats, struts, computing nodes/kernels, sensors, and standardized mechanical & electronic interfaces), controls (servo-loops, kinematics, finite state positioning/primitives, and sensor-driven routines), and their hierarchically structured synthesis-and-integration into different vehicles and operational strategies best suited to a    particular    environmental specification. Example:    it is possible, from the given resources, to a priori assemble a variety of wheel placements, spacing, strut geometrys, drives, etc. – which solution is best for a given terrain specification?

- Creation of    simplest feasible control protocols where the resources of “2” above can be mechanically reorganized in situ by robot supervisory control (under remote user command) to enable new robot functions or robot recovery from failure. Progressive Examples: 1) the wheeled rover has become trapped, tipped, seized an actuator, etc. – what a posteriori modifying mechanical action set gives best chance of recovery for the known vehicle state and executable degrees of freedom?; 2) a conventional rover has four wheels and a strong manipulator. A contingency situation develops in which the arm is to be used as an active and cooperative agent to assist rover c. g. distribution, drive force, traction management – how?; 3) the rover actually carries a discrete tool set of the building blocks (mechanisms, sensors, computing nodes, control kernels) above. It arrives at a scenario in which a particular viewing, manipulation, of modified mobility function is needed (four wheeled rover adds two, arm self-extends degrees of freedom) – how is this implemented, and by what criteria?

Justification

Our motivation for undertaking this work is to increase performance, capability and survivability of future planetary mobile robots. Mission drivers include surface explorations of the densely cratered Mars highlands, planetary moons, and asteroids. These sites are believed to be rich in geologic history and mineralogical detail, but are difficult to robotically access and sample. The terrains are rough and changing, with variable porosity and dust layering; the small bodies present further challenges of low-temperature, micro-gravity environments. Even the more some benign areas of Mars are highly variegated in character (VL2 rock densities or more), presenting significant risk to conventional rovers. This proposal suggests a NASA initiative to develop a new class of small, highly agile, self-reconfiguring robots for extremely difficult environments. The scope of planned work includes a system design methodology, enabling mechanical technology and prototypes, demonstrations in simulated    environments, and system benchmarks. “Challenge tasks” – to prove capability and robustness – will include adaptive mobility over changing terrain, operations in escarpments and fissures, self-recovery from traps/falls/upending, and robot self-repair of system failures.

Approach

Supporting Expertise and Participation

Three essential ingredients are needed to practically and expediently undertake the proposed work:

- JPL: A base of mechanical, sensors, and computing    technology and integration expertise from which to develop the proposed vehicular prototypes (including materials, thermal controls, electronic & mechanical i/f issues ensuring flight relevance). JPL brings this knowledge and hands-on experience, and can apply it in a cost-effective way, largely derived from recent work done in the ongoing Lightweight Survivable Rover (LSR) and Planetary Dexterous Manipulators (PDM) tasks, both under management of the proposing PI, P. Schenker. Each task has taken an intrinsically modular approach development of its mobility mechanization and manipulator configuration/actuation. Of particular importance is the LSR “Sample Retrieval Rover” deliverable at FY97-end, which incorporates the variable angle strut geometry shown below, plus a very strong (~3:1 lift/mass) arm, thus providing a testbed for many of the above mentioned ideas in adaptive pose, cooperative arm-wheel interactions, etc. The vehicle can easily be modified extended to include many further principles of    sliding c. g., alternate modes of wheel drive and steering (skid, Ackerman), variable gearing/drive-coupling, etc.

- CMU: A systematic analysis, synthesis, and set of metrics for the problems of low-level mobility design, incorporating detailed parameterization of vehicle structure, soil/terrain mechanics, and relative importance/trades in generating a desired class of mobility/terrainability performance. Co-I Dimi Apostolopoulos, a principal designer of the CMU Nomad/Dante-II vehicles, brings this expertise and a related software toolset, as gained in his Ph.D. thesis: “Synthesis of Robotic Locomotion Configuration Design (Advisor: Red Whittaker)”

- MIT: Complementary to the above tools for lower level kinematic/quasistatic analysis & synthesis of mobility configurations, there is needed a physically based methodology for systems-level design and task-level sensor-based control planning of modular robotic systems, so as to enable a coherent rapid prototyping in response to particular tasks or environmental situations. Paralleling Apostolopoulos’ work, MIT graduate student Shane Farritor is carrying out development of such techniques for his Ph. D. thesis “The Design of Modular Field Robotic Systems (Advisor: S. Dubowsky)” – cf. June ’96 TRIWG proceedings reports (Stanford Univ. meeting). Professor Dubowksy’s group is currently funded under a continuing NASA Telerobotics Program task contract (monitored through JPL). Per discussions of recent reciprocal site visits of Schenker/Dubowsky, both are agreeable to focus the future MIT statement of work sharply on the topics of this proposed task (MIT has already submitted to JPL a draft SOW to this effect, and S. Farritor spent a month’s visitation with Schenker’s group during summer ’96 in related study).

Design thrusts, avenues of vehicle development, and conceptual demonstrations

We pursue two primary technology paths: Variable Geometry Wheeled Vehicles and Reconfigurable/Modular Robotic Architectures. Per earlier discussion, we list these thrusts in terms of increasing difficulty, e.g., an evolutionary view for the proposed project.

Variable Geometry Mobility for Wheeled Vehicles:

Robotic designs utilized for JPL Mars rovers (e.g. MPF/Sojourner and design concepts now under consideration for the Mars’01 facility rover) are based on conventional configurations whereby single-point failures (wheel hang-up, tipping, etc.) can lead to an overall mission failure. To this end, if the rover possessed the capability to geometrically reconfigure its kinematics and/or mass-force distribution through intelligent deployment of its on-board resources, then mission risk would be substantially reduced. In addition, the adaptive mobility features of such a rover will enable the traverse of very challenging terrain using simple changes in vehicle pose. This task thrust pursues three separate enabling technologies.

The first technology focus is development of reconfigurable running gear in which multi-actuated running gear limbs and bogies are used to modify substantially the pose of the vehicle (e.g. ground-clearance height, distance between wheels). These pose changes would be realized through actuated, telescoping limbs which change the effective distance between the drive wheels. In this way the physical configuration of struts, a rocker-bogie mechanism, etc. can be modified to provide variable terrain mobility control. Likewise, an actuated bogie can be used to modify rover ground clearance height such that vertical location of the vehicle’s center of gravity (c. g.) ameliorates tip-over concerns.    These ideas are illustrated in the LSR Sample Retrieval Rover (SRR-1) FY97-end deliverable:

SRR-1: four wheel lightweight rover (~5 Kg) having

20 cm diameter wheels, DC brushed motor (planetary

gearing) actuation, actively articulated strut members,

and (not shown) a 60 cm. 3 d.o.f. front mounted arm.

The second technology focus is development of a variable horizontal c. g. control system which utilizes either an internal actuated sliding mass or the rover-mounted manipulator to effectively modify the horizontal location of the vehicle’s    mass center. With this capability, the rover can transfer its weight from one wheel to another in an attempt to increase or decrease a given wheel’s ground contact pressure, thereby modifying the tractive forces between the wheel and soil. The use of the already-existing SRR-1 arm is attractive, since its dual purpose role (manipulation and c. g. control) can be enabled by simply kinematically planning/posing the arm in a well-modeled 3-D free-space configuration. The optimal arm configuration for various terrain features will be studied through extensive dynamic simulation trials as well as quasi-static analysis methods. This kinematically-based approach can be viewed as a precursor to more sophisticated planning of    cooperative arm and wheel interactions with the environment.

The final technology focus for variable geometry wheeled vehicles is development of retractable wheel cleats. From extensive field trials in support of rover operations (Sojourner and Rocky 7), we have found that the traction realized from fixed cleats or growsers is, at best, a compromise between wheel/soil interactions and wheel/rock interactions. For example, the coefficient of friction needed for efficient mobility in sandy soil is vastly different than the coefficient of friction needed to climb a rock. We propose to develop a shape-memory alloy cleat that will deploy or retract under proper terrain conditions.

Note that all of the above technology areas rely on the ability of the rover to sense its current pose (yaw, pitch and roll angles), the current state of the wheels (ground contact pressure), and the terrain type (sand, rocks, gravel). JPL will instrument the vehicles accordingly. The MIT SOW includes as a significant WBS element the use of such fused sensory information to plan and control task level mobility, as based on underlying physical models of the vehicle.

Reconfigurable and Modular Robotic Architecture:

Creation of a truly modular design wherein separable/reusable arm links, actuators, wheels, running gear, and chassis are designed so that the interface is generic is an active R&D topic. Researchers have explored this problem for fixed-base manipulators (e.g. [Kelmar and Khosla, 1988 and Paredis, et al., 1996]; however, limited research has been focused on modular rover designs (e.g. [Fukuda et al., 1992, related work of Beni and Hackwood, etc.]). We have begun to address this issue in mobility design of the Lightweight Survivable Rover (LSR) and the use of its minimally-modified running gear and chassis designs to quickly, cheaply develop a new Long Range Science Rover (LRSR) circa end-FY98. We will pursue two basic paradigms here, one derived directly from MIT theoretical work to date, and the other strongly inferred.

I:	we will generate a set of canonical wheels, actuator hubs, struts, joints (including several active members/axles by which the limbs and joins can be geometrically re-posed), and a main chassis (with uniform mechanical    & electrical busses, instrumented with standard gyros, inclinometers, and visual sensing – in essence, a generalized LSR-1 architecture ). We will define several archetypal task test cases, e.g.:

- passage through random obstacles of characteristic mean free path,

- surmounting of obstacles of various characteristic geometry (area, height)

- ascent of an escarpment of given slope and tractability

- traverse of pits/hazards of characteristic dimension

The objective, drawing on CMU and MIT design tools for low and high-level mobility optimization design, will be to a priori plan, demonstrate and experimentally validate rapidly    prototyped vehicular solutions to these test cases.

II	the module inventory above will be extended to enable vehicle mechanical reorganization during actual task execution, e.g., an on-board arm used to add/reposition/remove various system elements, drives/differentials that can engage/disengage alternate torque paths.    We will define various pathological task cases, e.g.:

- traps

- tip-overs/upending

- seized limbs/wheels/arms

- change of task objective (go fast and flat versus climb)

The objective, drawing on MIT design tools for high-level mobility plan synthesis will be to a posteriori plan a modular reconfiguration and demonstrate its execution.

Deliverables & Schedule (Level 1)

FY98:	For simulated Mars traverse over modeled VL1/VL2 terrain, develop, demonstrate and benchmark a variable stance and variable c .g. four wheel rover, showing ability of system to achieve higher mean speed and climbing angles by mobility adaptation    [Sep 98]

FY99:	Develop and demonstrate an approach to cooperative use of a rover arm with mobility, showing first the concept of arm as active c. g. control, and then as means to facilitate rover escapes from traps and hang-ups [Sep 99]

FY00:	Develop a modular implementation of a wheeled rover, including the features of actively controlled struts (telescoping) and active traction control (variable cleats, etc.). Demonstrate rapid prototyping of same for various Mars relevant mission models and constraints (e.g., given a certain power budget, maximize traverse speed, maximize climb angle, … ) [Sep 00]

FY01:	Demonstrate capability of the above modular rover (with appropriate enhancements) to autonomously change its mechanical organization, self-reconfiguring in response to a pathological or opportunistic task condition [Sep 01]

FY02:	Demonstrate autonomous reorganization of a modular robot, effecting self-repair upon detection of mechanical or electrical failures [Sep 02]

Resources

Year		FY98	FY99	FY00	FY01	FY02

JPL ($K)	450	600	750	750	750	includes CMU personnel

MIT ($K)	150	150	250	250	250	existing MIT contract, not new $

Facilities and Partnerships

Facilities (JPL):	1)    MarsYard simulated VL1/2 and Mars nominal short range traverse; 2)    Bldg. 113 adjustable incline sand/Mars simulant (Basalt) testing; 3) Bldg. 158 composites processing & thermal test facilities; 4) Bldg. 107 support electronics/sensor hardware development, and calibrated rover test-board (cf. Rocky series/Go-For); 5) and a new 2400 sq. ft. Planetary Robotics Laboratory scheduled for summer ’97 completion and enabling significant indoor work in cooperative rover-lander tasking.

Responsibilities

JPL:		robot physical architecture, engineering implementation, science

integration, operational demos, and experimental documentation

MIT:		continues existing MIT contract work on physically-based

modeling, design synthesis, task controls, robot actuation

CMU:		new work on locomotion design, RRSE mobility benchmarks analysis

References

D. Apostolopoulos and J. Bares, “Configuration of a Robust Rappelling Robot,” Proceedings of the 1995 IEEE Int. Conf. on Robotics and Automation, Pittsburgh, PA,    April, 1995; and D. Apostolopoulos, “Systematic Configuration of Robot Locomotion,” CMU-RI-TR-96-30, Carnegie Mellon University, Pittsburgh, PA, July, 1996.

C. J. J. Paredis, H. B. Brown, P. K. Khosla, “A Rapidly Deployable Manipulator System,” Proceedings of the 1996 IEEE Int. Conf. on Robotics and Automation, Minneapolis, MN, pp. 1434-1439, April.

L. Kelmar and P. K. Khosla, “Automatic Generation of Kinematics for a Reconfigurable Modular Manipulator System,” Proceedings of the 1988 IEEE Int. Conf. on Robotics and Automation, pp. 663-668.

S. Farritor, S. Dubowsky, N. Rutman, and J. Cole, “A Systems-Level Modular Design Approach to Field Robotics,” Proceedings of the 1996 IEEE Int. Conf. on Robotics and Automation, Minneapolis, MN, April, 1996; and S. Farritor, S. Dubowsky, and N. Rutman, “On the Design of Rapidly Deployable Field Robotic Systems,” Proceedings of    the 1996 ASME Design Engineering Technical Conferences and Computers in Engineering Conference, Irvine, CA, August.

T. Fukuda, T. Ueyama, Y. Kawauchi, and F. Arai, “Concept of Cellular Robotics System (CEBOT) and Basic Strategies for its Realization,” Computers in Electrical Engineering, Vol. 18, No. 1, pp. 11-39, 1992.