NAME

configPM - Confgure Port Module registers and programmable features

SYNOPSIS

C function in pccUtil.c

INTERFACE

STATUS configPM( int slot_num , PM_CONF *confStruct)

slot_num		- Identifies which Port Module need to be configured

confStruct         - A pointer to the PM configuration structure

typedef struct{

UINT16 iepathAllInt; 	/* IEpath All intrrupt Enable/Disable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 loopBack;    	/*Which LoopBack mode */

/* <NO_LOOPBAK,UTOPIA_LOOPBAK,DECODER_LOOPBAK> /

UINT16 ivcParSense; 	/* ivc_par_sense:  when set, writes the the ipath memory will have bad parity

written into the memory. Subsequent reads should cause parity errors.*/

/* <FORCE_PARITY,DISABLE_PARITY> */

UINT16 vciParSense;  	/* vci_par_sense:  when set, writes the the epath memory will have bad parity

written into the memory. Subsequent reads should have

parity errors.*/

/* <FORCE_PARITY,DISABLE_PARITY> */

UINT16 crc32Check;   	/* crc32_enable: enables checking of crc32 on incoming credit cells*/

/* <CRC32_CHECKING_ON,CRC32_CHECKING_OFF> */

UINT16 allowTxIdle; 	/* allow_tx_idle: allows idle cells coming out of the core to be written in

to the txfifo. Otherwise these cells are dropped.*/

/* <ALLOW_IDLE, DROP_IDLE> */

UINT16 matchEnable; 	/* match_enable: enable the IVC comparison between the dequeued cell and the

cell received from the core. Cells which fail the compare

are discarded before leaving the epath.*/

/* <ENABLE,DISABLE> */

UINT16 passThru;    	/*passthru: allow all cells that come into the ipath to proceed without

having their IVC overridden upon detection of an error such

as HEC, or frame error.*/

/* <KEEP_IVC,USE_ERROR_IVC> */

UINT16 errorNull;    	/* Config the Null bit for error cells  */

/*<SET_NULL,CLR_NULL>*/

UINT16 IE_IntrruptMask; 	/*intrmask:mask the generation of interrupts for the appropriate error.*/

/*<IE_MASK_INTRRUPT,IE_UNMASK_INTRRUPT>*/

UINT16 rpiStatus; 		/* Instruct the Downstream to send ideal RPI enable/Disable */

/*<ENABLE,DISABLE>*/

UINT16 dmAllInt; 		/* DM All intrrupt Enable/Disable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 clmDeqInt; 		/* CLM dequeue error interrupt enable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 clmParityInt;  	/* W_IVC and W_CADR parity errors interrupt enable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 clmAlmDeqInt;  	/* ALM dequeue error interrupt enable*/

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 clmOutFIFOint; 	/* CLM output IVC FIFO full error interrupt enable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 clmInFIFOint;  	/* CLM input FIFOs full error interrupt enable */

/*<INT_ENABLE,INT_DISABLE>*/

UINT16 cmAllInt; 		/*All CM intrrupt Enable/Disable */

/* <INT_ENABLE,INT_DISABLE> */

UINT16 cmMembershipCheck; 	/* Internal/External membership check for R_IVC */

/* <INTERNAL,EXTERNAL> */

UINT16 autoCcntReset; 	/* Ccnt automatic reset enable */

/* <AUTO_RESET,NO_AUTO> */

UINT16 ivcFIFOfull; 	/* R_IVC FIFO full error interrupt enable */

/* <INT_ENABLE,INT_DISABLE> */

UINT16 fuseSet; 		/* fuse setting interrupt enable */

/* <INT_ENABLE,INT_DISABLE> */

UINT16 calmDeqInt;		/* CALM dequeue error interrupt enable */

/* <INT_ENABLE,INT_DISABLE> */

UINT16 schStatus; 		/* Scheduling Enable/Disable Flag */

/* <ENABLE,DISABLE> */

} PM_CONF;

DESCRIPTION

The configPM routine takes the setting vaules in the configStruct structure and

and configure accordingly the PM card in slot number slot_num.

////////////////////////////////////////////////////////////////////////////////

NAME

resetScheduler - Resets and Initializes the scheduler processor

SYNOPSIS

C function in pccUtil.c

INTERFACE

resetScheduler()

DESCRIPTION

This routine loads the Scheduler software to the PCC-SCh dual port memory and

resets the Scheduler Processor on the current Port Module. Also fires the communcation interface

to the scheduler.

/////////////////////////////////////////////////////////////////////////////////

NAME	runSchedulerAlgo -  Runs the Cell Scheduling algorithm

SYNOPSIS

C function in pccUtil.c

INTERFACE

STATUS runSchedulerAlgo (int algoNum)

algoNum 	- Identify which Scheduling algorithm.

DESCRIPTION

This routine instructs the Scheduler processor to start running a the scheduling algorithm

and stops the commuincation interface from being running. To stop this algorithm and returnback to

the commuincation Interface mode, use the function resetScheduler to relaod the Scheduler software.

/////////////////////////////////////////////////////////////////////////////////

NAME

setPmBit - set a bit in a PM memory register

SYNOPSIS

C function in pccUtil.c

INTERFACE

STATUS setPmBit(int addr, int bitNum)

addr		- Address of PM memory register net to be set

bitNum		- The bit need to be set

DESCRIPTION

This routine sets a bit in PM memory register. the routine does not check if the

bit is set.

///////////////////////////////////////////////////////////////////////////////

NAME

clearPmBit - clears a bit in a PM memory register

SYNOPSIS

C function in pccUtil.c

INTERFACE

STATUS clearPmBit(int addr, int bitNum)

addr		- Address of PM memory register net to be cleared

bitNum		- The bit need to be cleared

DESCRIPTION

This routine clears a bit in PM memory register. the routine does not check if the

bit is cleared.

///////////////////////////////////////////////////////////////////////////////

NAME

scuInitPortCardTest - Initialize the port card loopback test

SYNOPSIS

C function in scuPortCardTest.c

INTERFACE

STATUS scuInitPortCardTest( IVC_TABLE *ivcTable, UINT16 numIvcs )

ivcTable	- pointer to array of IVC_TABLE structures

numIvcs		- the number of IVCs structures in the table

typedef struct

{

UINT16  ivcIn;		// IVC into the core

UINT16  ivcOut;		// IVC of the cell out of the core

BOOL    ivcFound;	// TRUE if cell was found in the core

} IVC_TABLE;

DESCRIPTION

The scuInitPortCardTest routine resets the SCU (Switch Control Unit),

clears the FLM halt bit, sets the port thresholds to the maximum, initializes

the WIVC lookup table with ivcOut values and fills the FLM with sequential

cell addresses.

scuInitPortCardTest takes an ivc table as a parameter.  The ivc table

must be set up before calling this routine.  The ivcFound field should be

initialized to FALSE.

/////////////////////////////////////////////////////////////////////////////

NAME

scuPutCellInCore - Place a cell into the core

SYNOPSIS

C function in scuPortCardTest.c

INTERFACE

STATUS scuPutCellInCore( UINT16 wivc, CELL_STRUCT *cell )

wivc     - the IVC of the cell to write

cell     - pointer to a CELL_STRUCT or NULL

DESCRIPTION

scuPutCellInCore assigns the wivc to the cell and places it in

core memory.  The FLM determines where the cell is written in the core.

scuPutCellInCore will generate a cell if the cell pointer is NULL.

////////////////////////////////////////////////////////////////////////////

NAME

scuGetCellsFromCore - Retrieve all cells in the WIVC fifo

SYNOPSIS

C function in scuPortCardTest.c

INTERFACE

STATUS scuGetCellsFromCore( IVC_TABLE *ivcTable, UINT16 numIvcs,

CELL_STRUCT *coreCells )

ivcTable        - pointer to array of IVC_TABLE structures

numIvcs         - the number of IVCs structures in the table

coreCells	- pointer to buffer where cells from the core will

be placed

DESCRIPTION

scuGetCellsFromCore walks the high priority WIVC fifo.  For each

entry it reads the ivc and cell address.  The contents of the cell address

are read from the core and added to the coreCells buffer.  Each ivc found

in the fifo is looked up in the ivcTable and its ivcFound value is set to

TRUE.

/////////////////////////////////////////////////////////////////////////////

NAME

pmNonFlowCtrlVCsetup - Set up non flow control VC and IVC on Port Cards

SYNOPSIS

C function in pmVCsetup.c

INTERFACE

STATUS pmNonFlowCtrlVCsetup( pmVCdataType *vcd )

typedef struct

{

int    ivc;               // IVC in concern

int    src_slot;          // slot where the cell is received (from linecard)

int    dst_slot;	    // slot where the cell is sent out (to linecard)

int    src_vpi;           // VPI of the received cell

int    src_vci;           // VCI of received cell

UINT32 src_cr_hdr;        // credit cell header for the receving slot

UINT32 dst_hdr;           // data cell header for the output slot

int    dst_cr_vpi;        // credit cell vpi for slot where credit cell is received

int    dst_cr_vci;        // credit cell vci for slot where credit cell is received

int    class;             // class of the data cell ivc

int    cr_class;          // class of credit cell for ivc in interested

int    ict_threshold;     // ICT threshold, for determining when to generate credit cell

int    ict_offset;        // ICT offset, for determining when to generate credit cell

} pmVCdataType;

DESCRIPTIONS

This routine performs the necessary steps for setting up a a non flow

control VC on a Creditnet Port Module. It first maps src_vci and src_vpi to an

address in the IPATH VC translation table of the src_slot where the ivc is

written to. It then set the CM ownership flag and reset the flow control bit

for that ivc in the same slot.

In the slot where the cell leaves the switch, the outgoing cell header

is copied into a location specified by the ivc in the EPATH translation table.

Note that the write procedure is done in two steps, one for the high 16-bit and

one for the low 16-bit. The DM ownership flag, the class value and the express

bit in the var alid table for the ivc are also set accordingly.

src_cr_hdr, dst_cr_vpi, dst_cr_vci, cr_class, ict_threshold, ict_offset

are not used here.

RETURNS

OK - if the verification of data in I/EPATH trandlation table is OK.

ERROR - if the verification of data in I/EPATH trandlation table fails.

SEE ALSO

pmFlowCtrlVCsetup

INTERNAL

Copyright 1994 Northern Telecom, All Rights Reserved.

///////////////////////////////////////////////////////////////////////////////

NAME

pmFlowCtrlVCsetup - Set up flow control VC and IVC on Port Cards

SYNOPSIS

C function in pmVCsetup.c

INTERFACE

STATUS pmFlowCtrlVCsetup( pmVCdataType *vcd )

typedef struct

{

int    ivc;               // IVC in concern

int    src_slot;          // slot where the cell is received (from linecard)

int    dst_slot;	    // slot where the cell is sent out (to linecard)

int    src_vpi;           // VPI of the received cell

int    src_vci;           // VCI of received cell

UINT32 src_cr_hdr;        // credit cell header for the receving slot

UINT32 dst_hdr;           // data cell header for the output slot

int    dst_cr_vpi;        // credit cell vpi for slot where credit cell is received

int    dst_cr_vci;        // credit cell vci for slot where credit cell is received

int    class;             // class of the data cell ivc

int    cr_class;          // class of credit cell for ivc in interested

int    ict_threshold;     // ICT threshold, for determining when to generate credit cell

int    ict_offset;        // ICT offset, for determining when to generate credit cell

} pmVCdataType;

DESCRIPTIONS

This routine performs the necessary steps for setting up a a flow control

VC on a Creditnet Port Module. It first maps src_vci and src_vpi to an address

in the IPATH VC translation table of the src_slot where the ivc is written to.

And the credit cell header is copied into a location in EPATH translation table

specified by the ivc. It then set the credit cell class, ICT values, CM ownership

flag and the flow control bit for that ivc respectively.

In the slot where the cell leaves the switch, the outgoing cell header is

copied into a location specified by the ivc in the EPATH translation table. Note

that the write procedure is done in two steps, one for the high 16-bit and one for

the low 16-bit. It then maps the vpi and vci of the credit cells to a location in

IPATH translation table where the ivc is placed. The DM ownership flag, the class

value and the express bit in the var alid table for the ivc are also set accordingly.

RETURNS

OK - if the verification of data in I/EPATH trandlation table is OK.

ERROR - if the verification of data in I/EPATH trandlation table fails.

SEE ALSO

pmNonFlowCtrlVCsetup

INTERNAL

Copyright 1994 Northern Telecom, All Rights Reserved.