Multirobot/Multiagent Reading Group Notes

3-2-05

Part 1: RoboCup Rescue Physical League

Overview:

- 3 arenas of various levels of difficulty (yellow = easy, red = hard)

- Mannequin victims with motion, sound, heat, and CO2 emissions

- Any number of/any kind of robots are allowed

- 20 minutes to map environment, discover victims

- Score points for each victim based on accuracy of location, victim state, situation, signs of life, ID tag

- Score divided by (1 + # operators)2

Pros:

- Humanitarian goals

- Simultaneous addresses many problems

  - Challenging terrain

  - SLAM

  - Adjustable autonomy

  - Human-robot interaction

  - Multirobot coordination

  - Sensing

  - Other?

- Standardized metric for comparison for different lines of research

Cons:

- Not applicable for real disaster sites

- Murphy claims 2 operators per robot while RoboRescue encourages 1 operator per n robots.

- Must simultaneously address many problems (large initial time investment before addressing “research” in any single area)

Part 2: CMU/U.Pittsburgh RoboRescue Team

Overview:

- Robots as agents

- Heterogeneous robots: PERs (2), Pioneer P3AT (for mapping), Tarantula (stairs)

- Focus on adjustable autonomy and situational awareness

- High fidelity simulator (may be useful to others—available online): http://usl.sis.pitt.edu/ulab/usarsim_download_page.htm

Strengths:

- Lots of thought in system design and architecture

- “Cheap” heterogenous robots

- Victim identification fairly advanced

Weaknesses:

- Mapping (in process of switching to ActivMedia software)

- Cheap robots lack sensors and limit autonomy

Part 3: Coordination in USAR environment

Overview:

- Set of robots with (binary) heterogeneous capabilities

- Set of tasks with (binary) heterogeneous requirements, duration, reward

- Physical environment that can be represented as graph

- Reward decreases over time (want to find victims as soon as possible)

- Goal: allocation of robots that maximizes utility

Our approach:

- Anytime algorithm for robot/task allocation

- Formulate problem as MILP problem

- Interleave domain specific planning algorithms and general ILP algorithms over various planning horizons

- Distributed centralized approach

- Replanning based on reasoning about constraints

Pros:

- Explicitly considers team utility

- Ability to provide performance bounds (based on relaxed solution)

- Anytime algorithm

- Can represent different travel times for different classes of robots

- Can easily represent task dependencies (Partially ordered plans)

- Decreased communication costs/solution time (compared to DCOP algorithms)

- Reasoning about commitments when replanning

Cons:

- Not scalable for large teams (need to subdivide problem)

- Relationships must be linear

- Goal satisfaction is binary (could be overcome)

- Capabilities are binary (could be overcome)

- All robots need access to all information

Challenges:

- Faulty communication

- Relaxing full observability requirements

Results: Should have been included in notes (sorry!)

- Greedy heuristics are effective due to time varying reward

- Using previous solution to start search improves performance

Other thoughts? Feedback?

Fig. 1  Utility-Time tradeoff for example problem with varying levels of look-ahead.

Fig. 2 Effect of planning horizon on solution convergence on example problem.

Fig. 3 Effectiveness of starting solution on randomly generated test cases (each set was averaged over 3 randomly generated test cases).  If the optimal solution was not found after 3 minutes, the search was halted and the best solution so far was used.