Peer To Peer Guide

How to put your computer into peer to peer mode (for Windows XP):

- Double click the network icon in the System tray.

- Hit Properties to open the Wireless Network Connection Properties.

- On the General tab, select Internet Protocol (TCP/IP) from the list and hit Properties.

- Select the radio button next to “Use the following IP address:”

- In the IP address field enter 192.168.2.x. You do not want to use the same last digits as any of the robots or other computers.

- Enter 255.255.255.0 for the Subnet mask.

- Hit OK.

- On the Wireless Networks tab, click Add.

- On the Association tab enter RoverNet as the Network name (SSID).

- Select the checkbox next to “This is a computer-to-computer (ad hoc) network…”

- Hit OK.

- Make sure that RoverNet is the first network in the list of Preferred networks. You may need to remove other networks.

- Hit OK.

- Select RoverNet in the Available networks and hit Configure.

- Hit OK.

- Hit OK.

- You are now in peer to peer mode on RoverNet.

How to put the rover in peer to peer mode (for rovers running version 2.0.0 or higher of the Stayton code):

- Start with the rover powered off. Turn on the rover.

- After the rover nods its head, cover the camera with your finger to put the rover into self-test mode. The rover’s servos will twitch to indicate that it has entered the self-test mode. Remove your finger from the camera.

- The rover will pan and tilt its head. Then it will begin to turn each of the wheels one at a time, starting with the front left wheel and moving counter clockwise around the rover. Cover the camera while the front two wheels are turning. Do not cover the camera while the back two wheels are turning. This will put the rover in to peer to peer mode on RoverNet with its default IP address (192.168.2.xx). You will know that the rover has switched to peer to peer mode because it will drive to its right in the next step of the self-test (rather than forward as it does normally).

- At the end of the self-test, cover the camera again to return to normal operation.

During the self-test the wheels turn one at a time beginning with the front left wheel and moving counter clockwise around the rover. To switch to peer to peer mode, cover the camera while the front wheels are moving and not while the back wheels are moving.