OmniDepth3D : Reference guide

# Glossary

uint : unsigned int defined in global.h

# Structures

Point3D

Orient3D

ImgCoord

# Definitions

## OMNIDEPTH3D_PI	3.1415926

Definition of PI for the trigonometry calculation.

## MAX_DEPTH		100.0f

Definition of the maximum depth of the field of view. This distance define the far clipping plan.

## DISP_FACTOR		26.0f

Definition of the multiplying factor used in the Disparity <-> Depth conversion.

## DISP_OFFSET		0

Definition of the offset factor used in the Disparity <-> Depth conversion.

FOV_UP=1

FOV_DOWN

FOV_LEFT

FOV_RIGHT

## FOV_FAR

Enumeration of the different faces of the field of view.

# Functions

- Constructor, Destructor

## OmniDepth3D(void)

Do the initialization of the members.

~OmniDepth3D(void)

Free the allocated memory.

- Parameter Setting Functions

void SetStIntensityImg( unsigned char *img, uint width, uint height )

Set the pointer to the 8 bits array containing the grayscale intensity image seen by the stereo imager.

img corresponds to the pointer to the 8 bits array.

width corresponds to the width in pixel of the image stored in the array.

height corresponds to the height in pixel of the image stored in the array.

Note: The intensity image width and height are used other places in the code. They are important value to be set. If no display function will be used the img pointer can be set to NULL, but  width and height have to be defined.

You might want to change that when the integration with the JPL stereo will be done.

void SetStDisparityImg( unsigned char *img, uint width, uint height, uint cropwidth = 0, uint cropheight = 0)

Set the pointer to the 8 bits array containing the disparity image processed by the stereo algorithm.

img corresponds to the pointer to the 8 bits array.

width corresponds to the width in pixel of the array.

height corresponds to the height in pixel of the array.

cropwidth corresponds to the width in pixel of the image stored in the array (the image is smaller than the array in SVM, the origin stay the same at the top left corner).

cropheight corresponds to the height in pixel of the image stored in the array (the image is smaller than the array in SVM, the origin stay the same at the top left corner).

void SetStDepthImg( float *img, uint width, uint height )

CURRENTLY NOT SUPPORTED !!  NOT USED LATER IN THE CODE YET !!!!

Set the pointer to the float array containing the depth image processed by the stereo algorithm.

img corresponds to the pointer to the float array.

width corresponds to the width in pixel of the image stored in the array.

height corresponds to the height in pixel of the image stored in the array.

void SetStDispOffset( char xoffset, char yoffset )

Set the offset between the top left corner of the intensity image and the top left corner of the disparity image in order to align them together. These offsets should be set if the disparity image is smaller than the intensity image.

xoffset corresponds to the horizontal offset in pixels between the intensity and the disparity image.

yoffset corresponds to the vertical offset in pixels between the intensity and the disparity image.

void SetOmniPos(float x, float y, float z)

Set the Omnicam current position in the robot coordinate frame of reference.

x corresponds to the x coordinate of the Omnicam position in meters (right side of the robot for positive values).

y corresponds to the y coordinate of the Omnicam position in meters (in front of the robot for positive values).

z corresponds to the z coordinate of the Omnicam position in meters (on top of the robot for positive values).

void SetOmniPos(Point3D pos)

Set the Omnicam current position in the robot coordinate frame of reference.

pos corresponds to the x, y, z structure of the coordinates of the Omnicam position in meters.

void SetOmniRay(float pan, float tilt)

Set the Omnicam epipolar ray direction in the robot frame of reference, corresponding to an Omnicam pixel.

pan corresponds to the pan angle of the ray in degrees (a positive value turns counterclockwise when seen from top).

tilt correspond to the tilt angle of the ray in degrees (a positive value looks up).

void SetOmniRay(float rx, float ry, float rz)

Set the Omnicam epipolar ray direction in the robot frame of reference, corresponding to an Omnicam pixel.

rx corresponds to x coordinate of a vector in the robot frame of reference, align with the ray.

ry corresponds to y coordinate of a vector in the robot frame of reference, align with the ray.

rz corresponds to z coordinate of a vector in the robot frame of reference, align with the ray.

void SetOmniRay(Point3D direction)

Set the Omnicam epipolar ray direction in the robot frame of reference, corresponding to an Omnicam pixel.

direction corresponds to x, y, z coordinates of a vector in the robot frame of reference, align with the ray.

void SetStPos(float x, float y, float z, float h, float p, float r)

Set the stereo imager position and orientation in the robot coordinate frame of reference.

x corresponds to the x coordinate of the position of the stereo camera in meters.

y corresponds to the y coordinate of the position of the stereo camera in meters.

z corresponds to the z coordinate of the position of the stereo camera in meters.

h corresponds to the heading (yaw) orientation the stereo camera in degrees.

p corresponds to the pitch orientation of the stereo camera in degrees.

r corresponds to the roll orientation of the stereo camera in degrees.

void SetStPos(Point3D pos, Orient3D orient)

Set the stereo imager position and orientation in the robot coordinate frame of reference.

pos corresponds to the x, y, z structure of the coordinates of the position of the stereo camera in meters.

orient corresponds to the heading (yaw), pitch, roll structure of the orientation the stereo camera in degrees.

void SetStFOV(float horiz, float vert)

Set the total field of view corresponding to the intensity image of the stereo cameras and calculates the normal of each face of the FOV by calling ProcessNormal function.

horiz corresponds to the full (not half) horizontal field of view angle in degrees.

vert corresponds to the full (not half) vertical field of view angle in degrees.

- Intersection functions

bool ProcessNormal(Point3D p1, Point3D p2, Point3D p3, Point3D *normal)

Process the normal of the triangle defined by the 3 points to improve the intersection process speed.

This function is called for all the faces of the FOV pyramid in SetStFOV function.

p1 corresponds to the x, y, z coordinates of the first point of the triangle, in meters.

p2 corresponds to the x, y, z coordinates of the second point of the triangle, in meters.

p3 corresponds to the x, y, z coordinates of the third point of the triangle, in meters.

normal corresponds to a pointer to a 3D vector to return the normal coordinates.

Returns TRUE if the normal could be calculated, FALSE otherwise.

Note if one of the point is the origin, a small value will be added to all point to calculate the normal.

Note, the normal is actually the a. b. c coefficients of the plan equation that pass by the triangle.

bool IntersectTriangle(Point3D p1, Point3D p2, Point3D p3, Point3D normal, Point3D *intersec)

Calculate the intersection point with a triangle given the 3 points of the triangle and the precalculated normal.

The intersection algorithm can be optimized by looking at the sign of the vector product projected on the normal.

Ask Martial Hebert for more details.

p1 corresponds to the x, y, z coordinates of the first point of the triangle, in meters.

p2 corresponds to the x, y, z coordinates of the second point of the triangle, in meters.

p3 corresponds to the x, y, z coordinates of the third point of the triangle, in meters.

normal corresponds to the precalculated normal given by the ProcessNormal function.

intersec corresponds to a pointer to a 3D point to return the intersection point.

Returns TRUE if the intersection is inside the triangle, FALSE otherwise.

bool IntersectRectangle(Point3D p1, Point3D p2, Point3D p3, Point3D p4, Point3D normal, Point3D *intersec)

Calculate the intersection point with a rectangle given the 4 points of the rectangle and the precalculated normal.

The function call IntersectTriangle twice to process the two triangles defined by the rectangle.

Note the 4 points needs to be ordered. Can be optimized too.

p1 corresponds to the x, y, z coordinates of the first point of the triangle, in meters.

p2 corresponds to the x, y, z coordinates of the second point of the triangle, in meters.

p3 corresponds to the x, y, z coordinates of the third point of the triangle, in meters.

p4 corresponds to the x, y, z coordinates of the fourth point of the triangle, in meters.

normal corresponds to the precalculated normal given by the ProcessNormal function.

intersec corresponds to a pointer to a 3D point to return the intersection point.

Returns TRUE if the intersection is inside the rectangle, FALSE otherwise.

int  IntersectFOV(void)

Calculate the intersection points between the Omnicam ray and the stereo field of view.

The corresponding members of the class will be assign with the new intersection values.

( m_Intersection1, m_Intersection2, m_NbIntersections, m_Side1, m_Side2 )

Returns the number of intersections ( 0, 1 or 2 )

- 3D transform functions

void RotHPR(Orient3D orient, Point3D *point)

Rotate a 3D point according to an h, p, r orientation.

orient corresponds to an h, p, r orientation in degrees.

point corresponds to a pointer to a 3D point as the input point and to return the rotated point.

void Translate(Point3D trans, Point3D *point)

Translate a 3D point according to an x, y, z translation vector.

trans corresponds to an x, y, z translation vector in meters.

point corresponds to a pointer to a 3D point as the input point and to return the translated point.

void InvRotHPR(Orient3D orient, Point3D *point)

Do an inverse rotation on a 3D point according to an h, p, r direct orientation.

orient corresponds to an h, p, r orientation in degrees (direct, not inverted).

point corresponds to a pointer to a 3D point as the input point and to return the rotated point.

void InvTranslate(Point3D trans, Point3D *point)

Do an inverse translation on a 3D point according to an x, y, z translation vector.

trans corresponds to an x, y, z translation vector in meters (direct, not inverted).

point corresponds to a pointer to a 3D point as the input point and to return the translated point.

- Projection functions

bool IntersectOnImage(void)

Calculate the intersection points between the Omnicam ray and the stereo field of view by calling IntersectFOV, then project the intersections found onto the disparity image plan. If the Omnicam is inside the stereo FOV, the origin of the Omnicam is projected on the disparity image plan.

The corresponding members of the class will be assign with the new intersection values.

( m_NearIntersection, m_FarIntersection, m_NearDepth, m_FarDepth, m_IntersectionDepth  )

Returns TRUE if two projected points could be found, FALSE otherwise.

bool ProjectOnImage(Point3D point, unsigned char side, ImgCoord *pos, float *depth)

Project a 3D point onto the disparity image plan.

point corresponds to the 3D point to be projected.

side corresponds to the FOV side to which belongs the intersection point (to optimized the projection speed)

pos corresponds to a pointer to image coordinate (x, y structure in pixels) to return the projected point.

depth corresponds to a pointer to a float to return the depth of the projected point in meters.

Returns TRUE if the projected point is inside the intensity image array, FALSE otherwise.

bool ProjectOrigin(ImgCoord *pos, float *depth)

Project the Omnicam origin on the disparity image plan.

pos corresponds to a pointer to image coordinate (x, y structure in pixels) to return the projected origin.

depth corresponds to a pointer to a float to return the depth of the projected origin in meters.

Returns TRUE if the projected origin is inside the intensity image array, FALSE otherwise.

bool ProjectInfinity(ImgCoord *pos, float *depth)

NOT USED ANYMORE AS THE RAY CAN INTERSECT THE FAR CLIPPING PLAN

Project the Omnicam infinity point (ray direction) on the disparity image plan.

pos corresponds to a pointer to image coordinate (x, y structure in pixels) to return the projected infinity point.

depth corresponds to a pointer to a float to return the depth of the projected infinity point in meters.

Returns TRUE if the projected infinity point is inside the intensity image array, FALSE otherwise.

bool IsOmniInFOV(void)

Test the geometry to know if the Omnicam is inside the stereo field of view pyramid.

Returns TRUE if the Omnicam is inside the FOV pyramid, FALSE otherwise.

int  IntersectLine(ImgCoord st_nearcoord, ImgCoord st_farcoord,

float near_depth, float far_depth, ImgCoord *int_coord, float *int_depth)

Intersect the projected 3D line (bounded by the 2 projected points that intersects the FOV) with the disparity surface.

st_nearcoord corresponds to the near projected point coordinate (x, y in pixels) that defines the 3D line.

st_farcoord corresponds to the far projected point coordinate (x, y in pixels) that defines  the 3D line.

near_depth corresponds to the depth of the near point in meters.

far_depth corresponds to the depth of the far point in meters.

int_coord corresponds to a pointer to an image coordinate (x, y) to return the intersected point (with disp. surface).

int_depth corresponds to a pointer to a float to return the depth of the intersected point (with the disparity surface).

Return a confidence value between 0 and 255 (255 for high confidence)

bool GetNearIntersectionOnImage(ImgCoord *near_coord, float *near_depth)

Get the near projected intersection (with FOV) after an IntersectOnImage or a GetDepth call for display purpose.

near_coord corresponds to a pointer to a point coordinate (x, y in pixels) to return the near projected intersection.

near_depth corresponds to a pointer to a float to return the depth of the near intersection in meters.

Return TRUE if the near intersection point exists, FALSE otherwise.

bool GetFarIntersectionOnImage(ImgCoord *far_coord, float *far_depth)

Get the far projected intersection (with FOV) after an IntersectOnImage or a GetDepth call for display purpose.

far_coord corresponds to a pointer to a point coordinate (x, y in pixels) to return the far projected intersection.

far_depth corresponds to a pointer to a float to return the depth of the far intersection in meters.

Return TRUE if the far intersection point exists, FALSE otherwise.

bool GetIntersectionOnImage(ImgCoord *coord, float *depth)

Get the intersection between the Omnicam ray and the disparity surface, after an IntersectOnImage or a GetDepth call for display purpose.

coord corresponds to a pointer to a point coordinate (x, y in pixels) to return the intersection with the disp. surface.

depth corresponds to a pointer to a float to return the depth of the intersection in meters with the disparity surface.

Return TRUE if the intersection point exists, FALSE otherwise.

float GetDepth(float pan, float tilt)

float GetDepth(float rx, float ry, float rz)

float GetDepth(Point3D direction)

int   DistToDisp( float dist )

float DispToDist( int disp )

public:

Point3D m_OmniPos;

Point3D m_OmniRay;

Point3D m_StPos;

Orient3D m_StOrient;

unsigned char *mp_StIntensityImg;

unsigned char *mp_S tDisparityImg;

float *mp_StDepthImg;

unsigned int m_StIntensityWidth, m_StIntensityHeight;

unsigned int m_StDisparityWidth, m_StDisparityHeight;

unsigned int m_StDepthWidth, m_StDepthHeight;

unsigned int m_StDisparityCropWidth, m_StDisparityCropHeight;

int m_StDispIntensXoffset, m_StDispIntensYoffset;

Point3D m_Intersection1, m_Intersection2;

unsigned char m_NbIntersections;

unsigned char m_Side1, m_Side2;

Point3D m_StUpLeft, m_StUpRight, m_StDownLeft, m_StDownRight;

float m_StHalfHorizFOV, m_StHalfVertFOV;

Point3D m_NormalU, m_NormalD, m_NormalL, m_NormalR, m_NormalF;

bool m_FOVset, m_NormalProcessed;

ImgCoord m_NearIntersection, m_FarIntersection, m_Intersection;

float m_NearDepth, m_FarDepth, m_IntersectionDepth;