####=========================================================####

####                                                         ####

####                     NETLIST FORMATS                     ####

####                                                         ####

####=========================================================####

(A) NETLIST FORMAT FOR GRID-ROUTER

###################################

The netlist is given by a simple ASCII file. This format is

universal to the SLICE, V4 and V4R router.  The netlist

specifies the connectivity and the pin (terminal) locations.

The file consists of lines in the following format:

<x> <y> <z> <net>

followed by a single line

-1

<x>,<y>,<z> are the coordinates of the terminal. The coordinate

value begins with 0 (.i.e x,y,z >= 0). Note that the z

coordinate value is ignored for the SLICE, V4 and V4R router.

<net> is the (integer) net number of the terminal . All the

terminals of the same net are to be connected together. This

number must be unique for each terminal location. A special net

number of -1 indicates an obstacle. A special net number of 0

indicates a place holder. The place holder is used to define the

size of the routing region (see example below).

Example:

1 1 0 1

1 2 0 1

1 3 0 1

2 2 0 2

2 4 0 2

9 9 0 0

-1

This example shows a net list with 5 terminals. It has 1

3-terminal net and 1 2-terminal net. The routing area is 10x10.

(B) FORMAT OF MCC NETLIST

#########################

The netlist file contains a DEVICES, ELEMENTS, and NETLIST

section. The units are 1000 per micron.  The DEVICES section

gives the footprint of the chips and connectors.  The ELEMENTS

section gives the placement with lower left corner at (0, 0).

The NETLIST section associates a net number and net name with

each pin.  No power or ground pins are identified. It is assumed

that power and ground pins will be directly connected to power

and ground metal layers.

In the ELEMENT section, the last (5th) field indicates the

orientation of the DEVICE:

0 = no rotation

1 = 90 degree clockwise rotation

2 = 180 degree CW

3 = 270 degree CW

4 = reflection y->-y

5 = reflection y->-y followed by 90 degree CW rotation

6 =     "    "         "       180   "

7 =     "    "         "       270   "

(Last updated: 2 March 1993)