# Hardware and Software platform specification

# Aethra

### June 12, 2000

# This document presents an overview of the hardware and software platform for Nespole!. The overall architecture has been designed taking into account the geographical distribution of the four Language Specific HLT Servers, and assuming complete structural symmetry of Agents and Clients. Moreover, monitoring tasks have been distributed among four distinct hosts.

Overall architecture

# The overall architecture is drawn in the diagram below.

##### Fig.1 Overview of the overall architecture

In the proposed scenario, it is intended that Aethra would be in charge of the software module for the Mediator, as well as of the Whiteboard software module.

# The main features of the above platform are:

- Agent and Client are H.323 terminals, namely they are able to make H.323 calls and hence  send audio in G.711 format, video in H.261 format, and data in T.120 format. Examples of H.323 terminals are: videoconference systems by various producers (Aethra, ELSAvision, PictureTel, etc.), PCs equipped with an audio-capture card, a video-capture card, and Microsoft NetMeeting software.

- The Global Nespole! Server is drawn as a unique entity, but the various Language Specific HLT Servers can be arbitrarily distributed.

- The Aethra Software Module runs on a Windows platform.

- All the communications among Agent, Client, Mediator and HLT Server are via sockets. This is particularly appropriate for communications between Mediator and HLT Server, since transmission via sockets is independent on the actual operating systems running at the endpoints.

- Communications between Agent and Mediator, and between Client and Mediator are ruled by the H.323 standard. This imposes audio and video packets be transmitted using RTP (Real Time Protocol) over the UDP (User Datagram Protocol) transport layer, and data packets be transmitted using the T.120 protocol.

- As for the communication between Mediator and HLT Server, RTP over TCP will be used at the levels 4 and 5 of the ISO/OSI stack (Fig.2). The UDP protocol could be used instead of TCP. This last protocol makes communication a bit slower, but guarantees reliability: it implements a recovery  procedure that triggers the retransmission of those packets which got lost or corrupted. Loss and corruption of packets is not recovered by UDP.

###### Fig.2 ISO/OSI Stack

Nespole! Call handling

We now describe the functionality of the proposed architecture,  assuming, for clarity, that the Client speaks a language X  and wants to communicate with an Agent speaking language Y. In the following we denote by MY and SY the Mediator and HLT Server for language Y. Analogous abbreviations do hold for MX, SX, etc.

- The Client makes a H.323 call to MY. The call mode allows the Mediator to identify the source language X (see below for more details on this issue).

- The Mediator, which acts as a bridge between Client and Agent, makes a H.323 call to the Agent for language Y.

- Sockets for audio, video, and data packets are opened between Client and MY, and between MY and Agent, respectively.

- Each video packet received by MY from the Client is directly sent to the Agent.

- Each audio packet received by MY contains a unique sequence number, say Nm for the audio packet Am. Suppose that, correspondingly to the reception of Am from the Client, MY receives a certain data packet Dk. Then Dk is enriched with the timestamp Nm. Also, the audio packet  Am is transformed into AXm: an audio packet in PCM format which contains the time-stamp Nm. The Mediator MY sends AXm to SX.

- Packet AXm is analyzed by SX , transformed into IF keeping track of the time-stamp Nm, and then sent to the Communication Server (C.S. in Fig.1) .

- C.S. sends the IF corresponding to AXm to SY.

- The HLT Server SY elaborates the IF and returns to MY the PCM audio packet AYm which contains the time-stamp Nm.

- The Mediator MY receives AYm, fetches the packet  Dk  from its data buffer, and sends to the Agent the G.711 coding of AYm together with Dk.

- Audio, data, and video packets sent to MY by the Agent are treated similarly.

## General remarks

# Some general remarks about the proposed platform follow.

- As highlighted in the above example, one single Mediator is involved in the interaction between Client and Agent. This allows video and data packets to be kept inside the involved Mediator, so avoiding to send them to the Global Server which, on its side, should then forward them to yet another Mediator. Of course future developments of multi-modal features could imply the transmission of data packets to the Global Server. It is quite important, however, that video packets are sent around following a strictly by-need discipline. The video stream, in fact, is by far wider than both data and audio streams.

- Each Mediator is identified by a distinct IP address, so the Client selects the address to call depending on the foreign language he wants to use.

- In the above example MY is called by a Client speaking language X. It is crucial for MY to recognize the source language X. Different strategies can be adopted to achieve this result. One possibility is to take advantage of the fact that H.323 terminals usually allow the definition of an alias, and to require that Clients only use `well-formed’ aliases. For instance, we could impose that the alias of any German Client starts with the string ‘de’, the alias of any French Client starts with ‘fr’, and so on.

- The interaction between the Client speaking X and the Agent speaking Y is monitored by MY and actively involves both SX and SY. A Mediator can manage only one transaction at a time, so it will accept a H.323 call only if the following requirements are met:

- the Mediator itself is free from other calls;

- the HLT Servers of the calling and the called language do accept a bi-directional connection with the Mediator;

- the Agent is not involved in another communication.

Nevertheless HLT Servers are free to handle more than one transaction at a time (more than one mediator can communicate with HLT Server through the same socket). It is up to every single HLT Server to accept or refuse the communication request by Mediators.

Communication between Mediator and HLT Server: API definition and general remarks

Mediators and HLT Servers communicate via sockets. Each HLT Server keeps waiting on one bi-directional sockets, say the port 3000, for the transmission of linear audio.

Whenever a Client tries to call a certain Agent, two Servers, and hence two sockets, are involved. As we already said, the success of a H.323 call from a Client to a Mediator is intimately related to the possibility to open the sockets between the Mediator and the relevant HLT Servers. If any of the two attempts to open communications with the Servers fails, the very first H.323 call is rejected and fails.

The following issue is of fundamental importance. Each Server can be  either in `Waiting’ or in `Busy’ status. It is in `Waiting’ status if either it did not receive any `Open Socket’ request, or its socket has been closed via a `Close Socket’ request. The Server is in `Busy’ status otherwise.

If the Server is not able to manage more than one transaction at time, it must reject  `Open Socket’ requests when it is in `Busy’ status.

Each socket between Mediator and HLT Server is bi-directional and carries two data streams: one stream per direction. The communication protocol between Mediator and HLT Server must respect a few minimal requirements. The communication protocol will use packets as depicted below:

##### Fig.3 Packet exchanged between Mediator and HLT Server

The Length is the payload’s length.

The Timestamp field has obvious meaning.

The Session Identifier, assigned by the Mediator, is needed in the case that the HLT Server is able to manage more than one transaction at a time: HLT Server can use this identifier to differentiate audio data flow of different transaction.

The Payload contains the actual data, which can be either Status message, PCM audio, or T.120 Whiteboard data, as specified in the Payload Type field; in particular the Payload Type is one of the following:

STATUS_MESSAGE (= 1)  Initial message sent by the Mediator immediately after the Open Socket request.

PCM_FROM_G.711 (=2)  PCM Linear audio derived from a G.711 decoding,

PCM_FROM_G.722 (=3)  PCM Linear audio derived from a G.722 decoding,

PCM_FROM_G.728 (=4)  PCM Linear audio derived from a G.728 decoding,

PCM_FROM_PCM (=5)  PCM Linear audio not derived from a decoding,

WB_DATA (=6)  T.120 Whiteboard Data.

The Payload of a packet with Payload Type = STATUS_MESSAGE is:

##### Fig.4 Payload  structure for Payload Type = STATUS_MESSAGE

Additional information can be added in a second moment in the STATUS_MESSAGE packet: the Lenght field gives us this flexibility.

The transmission of PCM data deserves a more detailed comment.

- Due to structural constraints given by the definition of the H.323 standard, a PCM payload corresponds to about 20 milliseconds of speech. Then, most likely, one single PCM payload is just a small fraction of the minimal input that the HLT Server needs in order to start the analysis process. Hence the HLT Server should be able to buffer as many packets as necessary.

- We already mentioned that the generation of IF data from PCM audio should leave timestamps unaffected. Nonetheless, it is not necessary to keep a strict one-to-one correspondence between timestamps and packets. For instance, assume that the HLT Server generates an IF stream corresponding to the analysis of  audio payloads time-stamped by t1, .., tn. It is not necessary to split such a stream in n distinct portions; the IF can be sent to the other HLT Server in a single packet time-stamped by t1.

## Multimodality issues: main features of the proposed Whiteboard

The Whiteboard we propose can distinguish three different kinds of gestures/actions:

- free hand strokes;

- pointing of an area;

- loading of an image;

- running a browser.

# We agree with the partners about the choice of a partial lexical interpretation for free hand strokes. We propose to interpret a `reasonably long’ release of the mouse button as a separator between complex graphical objects. In particular, the sequence of strokes drawn while keeping the button down are taken all together as parts of a single object. The actual end of the gesture occurs when releasing the mouse button for more than a suitable amount of seconds, say 5.

As for the pointing gesture, two different area shapes can be defined: a rectangular shape, and an elliptic one. The user would select a certain shape, and then point the chosen area. After that, the rectangle (or ellipse) containing the pointed area could be highlighted.

The Whiteboard makes available another functionality particularly important for the first showcase: running a browser opened on a specified Web page and making the remote site running the browser opened on the same page translated in the remote site language.

Correspondingly to the different kinds of gesture, the data packet transmitted by the Whiteboard to the Mediator can be organized as follows:

- (type = freehand, context = “picture.bmp”, strokes_number = n, line_1 = (…), …., line_n = (…))

- (type = pointing, context = “picture.bmp”, pointing_type = rectangle/ellipse, area_data = (…))

- (type = loading, image_data = (…))

- (type = running browser, url_data = (…))

The Mediator is in charge of adding to the above packets a `time-stamp’ field, and of monitoring the synchronization of data and translated audio as already described above.

For the aim of multimodality, requested in the second showcase, Whiteboard data should be transmitted to the relevant HLT Server: this would be done sending a packet as depicted in Fig.3 where Payload Type is WB_DATA and Payload has the following structure:

##### Fig.5 Payload structure for Payload Type = WB_DATA

where Type is one of the following:

FREE_HAND (=1)

POINTING (=2)

LOADING (=3)

and Data has different structure depending on Type (Fig.6).

Data’s structure for Type = FREE_HAND

Data’s structure for Type = POINTING

Data’s structure for Type = LOADING

Fig.6