Carnegie Mellon University

Information Networking Institute

Design, implementation and evaluation of multiple load balancing systems based on a Network Processor architecture

TR 2000-

###### A Thesis Submitted to the

###### Information Networking Institute

in Partial Fulfillment of the Requirements

For the Degree

###### MASTER OF SCIENCE

in

INFORMATION NETWORKING

by

Servio Lima Reina and Suraj Vasanth

Pittsburgh, Pennsylvania

March 2001

# Acknowledgements

Infinite thanks to my wife Dalila and my son Servio Ricardo for being my motivation during this unforgettable experience.

Servio Lima

To our parents because of they were the ignition motor that helps us out to reach our goals.

We wish to thank Professor Peter Steenkiste for its vision and wise guidance not only during our thesis research but in our personal life too.

We are indebted to Dr. Prashant Chandra of Intel and to all the personnel at Intel whose advice and help went always beyond their duties. Especially to Erik Heaton.

Professor Peter Druschel and Mohit Aron from Rice University for their help our design and implementation of the content-based request distribution system.  Eric Nahum from IBM for providing invaluable advise for the implementation of the TCP Handoff.

We are grateful to Professor Raj Rajkumar for accepting to be our reader. To Professors David O’Hallaron and Srini Sheshan for their recommendations.

Last but not least we thank to Joe Kern, Sue Jones and Lisa Currin of the Information Networking Institute for their unconditional support during our days at the INI. And Nancy Miller, Chris Lin and CMCL support staff for providing us the equipment and support for the thesis.

Servio Lima & Suraj Vasanth

Table of Contents

Acknowledgements	2

Abstract	6

1. Introduction	7

2. Background	10

2.1. Load Balancing Techniques	10

2.1.1. Layer 3 Front-End	10

2.1.2. HTTP Redirect	11

2.1.3. Front-end Proxy	12

2.1.4. Front-end Handoff	13

2.1.5. Other Solutions	13

2.2. Scheduling Algorithms	14

2.2.1. Locality Aware Request Distribution (LARD)	15

2.2.2. Weighted Round Robin	19

2.3. TCP Handoff Mechanism	20

2.4. Intel PA-100  Network Processor	22

2.4.1. Brief Description	22

2.4.2. PA100 System Sequence Of Events	23

2.4.3. PA100 Development Environment	25

3. Design and Implementation of Load Balancing Switching Systems	28

3.1 Load Balancing system building blocks	28

3.2 Design considerations for HTTP 1.1 (Persistent HTTP)	31

3.2.1. Back-End Request Forwarding:	31

3.2.2. Multiple Handoff	32

4. Evaluation	33

4.1 Test bed	34

4.2. Evaluation of load balancing system using the testbed	36

4.2.1 Determining practical requests/second rate limit	36

4.2.1 Evaluating the impact of different number of backends	37

4.3 PA100 theoretical limits	39

4.4. PA100 CPU cycles analysis	40

4.5. PA100 memory  analysis	45

4.6. Overall analysis	46

4.7. Influence  of external factors on load balancing systems.	47

4.8. Porting PA100 Load Balancing design to IXP1200	48

5. Related Work	54

6. Conclusions	55

7. References	56

#### List of Figures

Figure 1: Layer 3 front end using Loop-back address	11

Figure 2: HTTP Redirect	11

Figure 3: Front-end Proxy	12

Figure 4: Front-end handoff	13

Figure 5: TCP Handoff mechanism	21

Figure 6: Intel PA100 Network Processor Architecture	22

Figure 7: PA100 Classification Engine architecture	23

Figure 8: Sequence of events for receiving a packet in the PA100 platform	25

Figure 9: Action Classification Engines used in PA100	26

Figure 10: Functional blocks of a load balancing system	29

Figure 11: Backend Request Forwarding	32

Figure 12: Multiple handoff	32

Figure 13: Testbed configuration	35

Figure 14: Analyzing Requests/second rate capacity  supported by each load balancing system	36

Figure 15: Latency for setting up an HTTP session vs number of backend servers	38

Figure 16: IXP1200 architectural diagram	50

Figure 17: IXP1200 Per-packet pseudo-code	51

#### List of Tables

Table 1: Theoretical Mpps given number of clock cycles in PA100	41

Table 2: Maximum number of requests/second supported for each Load balancing system	41

Table 3: Objects used in each Load balancing system	42

Table 4:  CPU cycles consumed for each class/method used in a load balancing system	43

Table 5: Estimated HTTP sessions/sec taking into consideration memory latency	45

Table 6:  Comparing HTTP sessions/second when CPU or memory are the bottleneck	46

Table 7: Comparison between theoretical requests/second and measured requests/second values	46

Table 8: Number of read/writes from/to memory for each Load balancing system	52

Table 9: Comparison of  estimated requests/sec supported in IXP1200 and PA100	52

# Abstract

In this paper, we have studied the implementation of load balancing applications using network processors.  We consider the load balancer as a front-end device to a server cluster that directs incoming requests to the back-end servers.  Specifically, we have compared implementations of three load balancing architectures and the impact of design issues on the performance of network processor applications.

We have used PA100 as a front-end device that directs incoming request to one server in a farm of back-end servers using different load balancing mechanisms.  In this thesis, we have implemented and evaluated the impact that different load balancing algorithms have on the PA100 network processor architecture.  No studies had been done before that designed and implemented multiple load balancing systems using the Intel PA100 network processor and furthermore compare the advantages that Content Based Switching System have over traditional load balancing mechanism (Layer 3).  Locality Aware Request Distribution (LARD) and Weighted Round Robin (WRR) are the load balancing algorithms analyzed.

# 1. Introduction

The growth of Internet business requires mission-critical information and services to be available at all times.  The World Wide Web has grown to be a robust infrastructure that handles mission-critical business traffic.  The number of e-commerce users is forecasted to surpass half a billion by 2003 and increase the commerce on the Internet to over one trillion dollars by then.  To support this growth, web sites of the future must support millions of connections every second.

The ever-increasing computing and bandwidth requirements of these services are generally achieved through clustering.  A server cluster is a set of computer systems connecting together multi-system hardware and software to provide services that was traditionally provided by a single system.  Server clusters are used to achieve the following benefits:

Scalability:  Traditional multiprocessor systems had relatively low scalability due to contention for shared memory and I/O.  Clustered machines scale better as the workload for a particular connection is bound to an individual machine in the cluster and therefore, results in less contention of shared resources.

Availability:  Individual machines in the cluster can fail without adversely affecting the services being provided to the users.  The other operational machines in the cluster take over the extra load of the users who were disconnected.

Easy Growth and Maintenance:  Additional machines can be added to the cluster without affecting the operations of other machines.  A new machine can simply be added to the network and the front-end (or back-end servers) is configured to redirect requests to the new machine.  The new machine, when in operation, reduces the load on the existing machines.

Load Balancing:  Applying load balancing, spreading the load to all servers hence preventing overloading any one machine, results in avoiding wasted capacity on some systems.

Load balancer is a front-end device that is used in distributing requests to a set of back-end servers.  Cluster based servers comprising of workstations connected together by high speed LANs are being deployed extensively on the Internet for critical applications.  Clusters provide the advantage of being cheaper, scalable, reliable and fault tolerant when compared to a centralized approach.  Cluster based network servers need to have a front-end device that distributes the requests to the back-end servers.  This distribution can be done with varying degree of complexity and intelligence at the front-end.

Currently, front-end devices are hardware devices being built using application-specific integrated circuits (ASICs).  Although ASICs work at wire speed and provide the best performance, any changes to existing technologies or implementing new technologies requires redesign of the ASICs, which takes between 12 to 18 months.  Another challenge with traditional ASICs is their inflexibility in supporting customization of an application.  These problems with ASICs make them infeasible for use in an environment that changes frequently and requires high level of intelligence.

Network processor is a programmable communications integrated circuit that offers real-time processing, security, store and forward, switch fabric, IP packet handling and learning capabilities.  Traditional CPUs, like Pentiums, are general-purpose processors that are not optimized for packet processing.  Specialized processors like network processors, allow the provisioning of faster and more intelligent packet processing services at exponentially increasing wire speeds.  As network applications are being developed in software, they can be implemented much quicker and modifying the application is trivial, when compared to traditional ASICs.  These advantages of network processors make them ideal candidates for implementing load balancers.  We have used Intel’s PA 100 network processor in our study to implement load balancers.

The rest of the paper is organized as follows.  Section 2 provides background information about various load balancer architectures, scheduling algorithms used in distributing the request, the handoff mechanism used in content-aware request distribution system and description of the platform, Intel’s PA 100, on which they were implemented.  Section 3 describes the design and implementation for the three load balancing techniques assessed in this paper.  Section 4 provides an evaluation of the test results for the implementations and how they compare against each other.  We demonstrate that among CPU and memory resources in the PA-100 platform, memory appears as the main cause of bottleneck due to the high level of memory contention and we can achieve at least 57% better performance if we increase the speed of DRAM.  This is true for the three load balancing systems implemented and evaluated.  We finally demonstrate that even in the worst-case scenario, IXP1200 is able to perform 43% better than its PA100 counterpart.  Section 5 lists the related work in this field and Section 6 provides the conclusions of this study.

# 2. Background

This section provides background information on the various design issues, algorithms and devices that were used in the implementations of the load balancers.

The first subsection provides brief descriptions of modern schemes used to implement load balancers for server clusters and their relative pros and cons.  Three of these schemes – the Layer 3 Front-end, Front-end Proxy and Front-end Handoff have been implemented and evaluated in this study.  This is followed by a description of the scheduling algorithms, which are used by the front-end to choose which back-end server to direct the request to.  With content-based request distribution (layer 5), the front-end needs to handoff incoming requests to the back-end server in a manner transparent to the client – this is explained in Section 2.3. Since, we have implemented our solution on the PA 100 system, the PA 100 system is described in Section 2.4.

## 2.1. Load Balancing Techniques

Load Balancing is used to distribute the load (requests) to a set of servers.  Load Balancing system usually have a front-end device that directs the request to a selected back-end server.  The complexity and intelligence of the front-end device varies in all the schemes.  The following provides a brief description of each scheme with its advantages and disadvantages.

### 2.1.1. Layer 3 Front-End

In this scheme, the front-end assigns and forwards the requests to an appropriate back-end server irrespective of the requested URL.  The front-end performs a hash of the 5-tuple that identifies each connection and maps the hash to a table that contains the list of all the backend servers.  This operation is performed for all packets that pass through the front-end.  The response from the back-end server is forwarded directly to the client, if the loop-back address is used by the backend.  If the backend servers do not use the loop-back address, then the front-end behaves as a NAT device.  This technique is most primitive in performing load balancing.  All backend servers need to replicate the same information in their hard disk.  The requests being assigned to each backend follows a round robin order and therefore, the backend may need to perform more operations to serve each request as it may need to move the requested information from its hard disk to cache, as the chances for the request being in its cache is lower.  The advantages of this technique are that since the front-end device performs very few tasks, when compared to the other methods, the front-end is simple to implement and has high throughput.  It is also does not need to maintain any state for the connections.

Client

Internet

Layer 3

Front-End

Back-End Servers

Request

Response

Figure 1: Layer 3 front end using Loop-back address

### 2.1.2. HTTP Redirect

Client

Internet

Front-End

Back-End Servers

Request

Response

Another simple technique is to have the front-end send a HTTP redirect message to the client.  After the front-end receives the request from the client, the front-end assigns a backend server for the request by using either a hash function (random) or a more complex selection algorithm (like LARD, WRR) and sends the selected backend server’s IP address to the client in a HTTP redirect message.  And, thereby, having the client send a request to the chosen back-end server directly.  The problem with this approach is that the IP address of the back-end server is exposed to the client, thereby exposing the servers to security vulnerabilities.  Also, some client browsers might not support HTTP redirection.

Figure 2: HTTP Redirect

### 2.1.3. Front-end Proxy

In this technique, the front-end establishes two TCP sessions – first one with the client and second with the back-end server.  All traffic in both ways has to pass through the proxy.  The proxy maintains state about the connection and translates the information for all the traffic passing through it.  The proxy maintains a persistent HTTP connection with each of the back-end servers through which it sends requests and receives replies from the back-end servers.  The front-end proxy can select the back-end server using the URL of the request.  The disadvantage of this scheme is that the proxy here needs to maintain state for each HTTP connection and requires greater complexity.  This also limits the maximum number of requests the proxy can handle at any instance.  The throughput of the proxy is expected to be much lower as the proxy needs to perform more number of operations and needs to maintain state each HTTP connection.  Since the traffic going back to the client passes through the proxy, this adds to the load on the proxy, as web traffic is asynchronous (the traffic going out of the system back to the client is by a large factor greater than the traffic coming from the client into the system).  The advantage of this technique is that the address of the back-end server is hidden from the client, thereby protecting it from security vulnerabilities.

Client

Internet

Front-End

Back-End Servers

Request

Response

Figure 3: Front-end Proxy

### 2.1.4. Front-end Handoff

A more complicated solution is to perform a handoff between the front-end and back-end server.  After the front-end receives the URL, it transfers its end of the TCP connection to the back-end server.  Once the TCP state is transferred to the back-end, in our implementation - by performing the 3-way handshake and sending the sequence number, the back-end servers can directly send packets to the client bypassing the front-end.  The front-end maintains a mapping between the connections and the server the connections have been assigned to.  This is because the ACK packets from the client need to pass through the front-end.  The traffic from the back-end servers does not pass though the front-end as the front-end does not need to perform any operations on the packets.  This reduces the load considerably on the front-end device and is main advantage of this scheme when compared to proxying.  The address of the server is hidden from the client.

The handoff mechanism being used plays an important role for the viability of this technique.  The handoff needs to be transparent to the client and performed with low overhead.

Client

Internet

Front-End

Back-End Servers

Request

Response

Figure 4: Front-end handoff

### 2.1.5. Other Solutions

#### 2.1.5.1.Round Robin DNS

Early solutions to this problem used Domain Name Service (DNS) that did the translation between hostname and IP address.  Instead of replying the same IP address to requests for a particular hostname, a rotated (round robin) list of IP addresses was used.  Therefore, this approach is referred to as Round Robin DNS.  The advantage is that it is transparent to both server and client, and easy to deploy as it was shipped as a standard function in DNS servers.  The disadvantage is that the immediate name servers and clients cache the IP address of the DNS reply.  Subsequent use of the hostname is resolved to the same IP address.  This results in a hot spot.  Another serious disadvantages of this technique are that it does not have any awareness about the availability or load of the servers.

#### 2.1.5.2. Network Address Translation (NAT)

NAT devices work by translating the source and destination IP address for traffic passing through them.  All traffic – inbound and outbound pass though the NAT.  The capacity of NAT should not only be able to support both inbound and outbound traffic, but also large amount of outbound traffic.  All traffic to the servers has to pass through the NAT device, irrespective of them needing to be load balanced.

For an intelligent load-balancing scheme (layer 5 information), like Front-end proxy and Front-end handoff, to be efficient the algorithm used to distribute the load plays an important role.  These algorithms are useful in improving the locality (cache hit ratio) at the back-end servers and play an important role in optimal distribution of the requests.

## 2.2. Scheduling Algorithms

Scheduling algorithms, which run on the front-end device, provide the address of the back-end server to which the request should be directed.  The scheduling algorithms differ on their complexity and intelligence.  The inputs to the scheduling algorithm allow it to decide on the appropriate back-end server.  We have implemented Locality Aware Request Distribution (LARD) and used the Front-end handoff technique for load balancing, as LARD uses the URL of the request to choose the back-end server.  Weighted Round Robin (WRR), in addition to using the URL, can operate at the granularity of the requests without regard for the requested content (URL).  We have implemented WRR for Layer 3 Front-end and Front-end Proxy for our tests.  The following is a brief description of both the algorithms.

### 2.2.1. Locality Aware Request Distribution (LARD)

Locality-aware request distribution algorithm was developed at Rice University as part of the ScalaServer project.  Material in this section of the paper is derived from the following papers published by them: [Aron99], [Gau97], and [Pai98].  Locality-aware request distribution is focused on improving hit rates.

Most cluster server technologies like [IBM00] and [Cisco00], use weighted round robin in the front-end for distributing requests.  The requests are distributed in round robin fashion based on information like the source IP address and source port, and weighed by some measure of the load, like CPU utilization or number of open connections, on the back-end servers.  This strategy produces good load balancing.  The disadvantage of this scheme is that it does not consider the type of request; therefore, all the servers receive similar sets of requests that are quite arbitrary allocated.

To improve the locality in the back-end’s cache, hash functions can be used.  Hash functions can be employed to partition the name space of the database.  In this way, requests for all targets in a particular partition are assigned to a particular back-end.  The cache in each back-end will hence have a higher cache hit rate, as it is responding to only a subset of the working set.  But, a good partitioning for locality may be a bad for load balancing because if a small set of requests in the working set account for a large portion of the requests, then the server partition serving this small set of requests will be more loaded than others.

LARD’s goal is to achieve good load balancing with high locality.  The strategy is to assign one back-end server to serve one target (requested document).  This mapping is maintained by the front-end.  When a first request is received by the front-end, the request is assigned to the most lightly loaded back-end server in the cluster.  Successive requests for the target are directed to the assigned back-end server.  If the back-end server is loaded over a threshold value, then the most lightly loaded back-end server at that instance in the cluster is chosen and the target is assigned to this just chosen back-end server.  A node’s load is measured as the number of connections that are being served by this node – connections that have been handed off to the server, have not been complete and are showing request activity.  The front-end can monitor the relative number of active connections to estimate the relative node on the back-end server.  Therefore, the front-end need not have any explicit communication (management plane) with the back-end servers.

#### 2.2.1.1. Basic LARD Algorithm

Whenever a target (requested document) is requested, according to LARD, the target is allocated to the least loaded server.  This distribution of targets lets to indirect partitioning of the working set (all documents that are served by the cluster of servers).  This is similar to the strategy that is used to achieve locality.  Targets are re-assigned only when a server is heavily loaded and there is imbalance in the loads of the back-end server.

The following is the LARD algorithm proposed in [Pai98]:

while(true)

fetch next request r;

if server[r.target] = null then

n, server[r.target]  {least loaded node};

else

n  server[r.target];

if (n.load > THIGH &&  node with load < TLOW) || n.load  2* THIGH then

n, server[r.target]  {least loaded node};

Send r to n;

Here, THIGH is the load at which the back-end server causes delay and TLOW is the load at which the back-end has ideal resources.  If an instance is detected when one or more back-end servers has a load greater than THIGH and there exists another back-end server with a load less than TLOW, then the target is reassigned to the back-end server with a load less than TLOW.  The other reason a target maybe reassigned is when the load of a back-end server exceeds 2 x THIGH, this is when none of the back-end servers are below TLOW, then the least loaded back-end server is chosen.  If loads of all back-end servers increase to 2 x THIGH, then the algorithm will behave like WRR.  The way to prevent this from happening is to limit the total number of connections that are forwarded to back-end servers.  Setting the total number of connections S = (n-1) * THIGH + TLOW –1, makes sure that at most (n-2) nodes have a load THIGH, while no load is less than TLOW.

TLOW  should be chosen so as to avoid any ideal resources in the back-end servers.  Given TLOW, THIGH needs to be chosen such that (THIGH – TLOW) should be low enough to limit the delay variance among the back-end servers, but high enough to tolerate load imbalances.  Simulations done in [Pai98] show that the maximal delay increases linearly with (THIGH – TLOW) and eventually flattens.  Given a maximal delay of D seconds and average request service time of R seconds, THIGH can be computed as:  THIGH = (TLOW + D/R) / 2.

#### 2.2.1.2. LARD with Replication

The disadvantage of the Basic LARD strategy (explained in the previously) is that at any instance a target is always served by a single back-end server.  If a target has large number of hits, then this will lead to overloading of the back-end server serving that target.  Therefore, we require a set of servers to serve the target, so that the requests can be distributed to many machines.  The front-end now needs to maintain a mapping from a target to a set of back-end servers.  Requests to the target are sent to the least loaded back-end server in the set.  If all the servers in the set are loaded then a lightly loaded server is picked and assigned to the set.  To reduce the set of back-end servers serving the node (whenever there are less requests for the target), if a back-end server has not been added to this set for a specific time, then the front-end removes one server from the server set.  In this way the server set is changed dynamically according to the traffic for the target.

If an additional constraint is added that the file is replicated in a set of servers (rather than throughout the cluster) then an extra table mapping the targets to all the back-end servers that store the target in their hard disk, needs to be maintained.  This table is accessed whenever a server has to be added to the server set.

#### 2.2.1.3. Advantages and Disadvantages of LARD

LARD provides a good combination of load balancing and locality.  The advantages are that there is no need for any extra management plane communication between the front-end and back-end servers.  The front-end need not try to model the cache in the back-end servers and therefore, the back-ends can use their local replacement policies.  Since, the front-end does not have any elaborate state, it is easy for the front-end to add back-end servers and recover from back-end failures or disconnections.  The front-end simply needs to reassign the targets assigned to the failed back-end to the other back-end servers.

The disadvantage with this scheme is the concern about the size of the table that maps targets to back-end servers.  The size of this table is proportional to the number of targets in the system.  One way to reduce this table is to maintain this mapping in a least recently used (LRU) cache.  Removing targets that have not been accessed recently does not cause any major impact as they may have been cleared out of the server’s cache.  Another technique is to use directories.  Targets can be grouped inside directories and the entire directory can be assigned to a back-end server or a set of servers.

As shown in the simulations and graphs in [Pai98], LARD with Replication and Basic LARD have similar throughput and cache miss ratio.  Therefore, we have implemented the Basic LARD strategy in our implementation.

### 2.2.2. Weighted Round Robin

The Weighted Round Robin (WRR) strategy aims at distributing the load by directing the requests to all the cluster nodes.  In a round robin strategy, the front-end maintains a list of all the back-end servers and keeps going through the list one after the other, allocating the requests to the back-end servers whenever a request is received.  In this way all the servers in the cluster get the same number of requests.  In weighted round robin, each server is provided a weight and the number of requests being directed to the server depends on the server’s weight.  Weighted Round robin strategy provides fairness as requests are distributed to all the servers and is less complex than LARD.  Also, since WRR does not require the URL, it can be implemented for Layer 3 Front-end.

With content-based request distribution, the front-end needs to handoff incoming requests to the back-end server in a manner transparent to the client.  We have implemented a simple handoff mechanism that requires minimal modifications to the back-end kernel and is easy to implement keeping in mind that the front-end in the PA 100 does not have a TCP stack in the kernel.

## 2.3. TCP Handoff Mechanism

One question that arises when implementing Content Aware Request Distribution System is how to handoff TCP connections to the back-ends.  We implemented a technique known as delayed binding or TCP splicing, which consist in replaying TCP session parameters from the client-front-end communication to the front-end-back-end communication. Figure 5 shows how this replaying happens and which are the TCP session parameters to be replayed.

In order to handoff the TCP state information from the client-front-end communication to the backend, the following sequence of events is executed:

- Client starts a TCP connection with the front-end using the standard TCP three-way handshake procedure.

- Once the three-way handshake procedure is finished and the URL information is received by the front-end, the front-end starts a new TCP connection with the backend chosen by the front end’s load balancing algorithm (i.e. LARD or WRR). As the front-end and backend use the same initial sequence number (backend receives sequence number information in TCP option field from the front-end), they are able to replay the same TCP session parameters used in the client-front-end three-way handshake communication.

- Once the backend receives the URL information from the front-end, the backend starts sending HTML pages directly to the client without the front-end intervention. (See figure 2)

- Client’s ACK packets still pass through the front-end. Using data plane’s hashing function capabilities the front-end is able of forwarding the ACK packets to the proper backend.

- FIN packet is generated by the backend server

- Client responds with FIN and ACK packets

- TCP session is finished with the ACK packet sent by the backend to the client.

FrontEnd Processing Delay

.

.

.

.

.

ACK

seqno_client+urldatalen,  seqno_be+htmldatalen,

URL,

seqno_client+1,

seqno_be+1

ACK

seqno_client+urldatalen,  seqno_be+htmldatalen,

HTML,

seqno_be+1,

seqno_client+urldatalen

ACK, seqno_client+1,

seqno_be+1

SYN, seqno_client, ack=0

SYN+ACK, seqno_be, seqno_client+1

URL, seqno_client+1,

seqno_be+1

SYN, seqno_client, ack=0

ACK, seqno_client+1,

seqno_be+1

SYN+ACK, seqno_be, seqno_client+1

CLIENT

FRONTEND

BACKEND

CLIENT

FRONTEND

BACKEND

1

2

32

42

52

72

ACK

ACK

FIN

FIN

FIN

FIN

ACK

ACK

62

Figure 5: TCP Handoff mechanism

## 2.4. Intel PA-100  Network Processor

This study was performed on the PA 100 network processor.  The schemes explained in the earlier subsections have been implemented on this platform.  At this moment this is the only network processor from Intel that has high level language  libraries for programming with.  The following is a description of the system.

### 2.4.1. Brief Description

PA100 is a network processor created by Intel Inc. whose purpose is to run network applications at wire speed. It differs from general-purpose processors in that the hardware is specifically designed to handle packets efficiently. We choose the Intel PA100 processor because it provides a programming framework that is used by current and future implementations of Intel's network processors.

All the Load balancing systems were implemented using the Intel PA100 Network Processor depicted in figure 5.

Figure 6: Intel PA100 Network Processor Architecture

The board consist of a PA100 policy accelerator (dotted area), 128 Mb DRAM, a propietary  32 bit x 50 Mhz processor bus, a set of media access controller (MAC) chips implementing 2 ethernet ports (2x100 Mbps). Additionally a 32 bit x 33 Mhz PCI bus interface is included.

Figure 7: PA100 Classification Engine architecture

The PA100 chip itself contains a general-purpose StrongARM processor core and four special-purpose classification-engines (CE) running at 100 Mhz.  Figure 6 shows the components of a single CE. Each CE has an  8 KB instruction store. The StrongARM is responsible for loading these CE instruction stores; actual StrongARM instructions are fetched from DRAM.

The chip has a pair of  Ethernet MACs  used to send/receive packets to/from network ports on the processor bus. These MACs have associated with them a Ring Translation Unit that mantains pointers to a maximum of 1000 packes stored in DRAM. The receive MAC inserts packets  along with the receive status into 2 KB buffers and updates the  ring translation units associated with the MAC. Transmit MAC follows also a ring of buffer pointers.

### 2.4.2. PA100 System Sequence Of Events

For a better understanding of how a packet is handled when it reaches the PA100 platform, we describe step by step which are the sequences of events that a packet must follow through.  This sequence of events is adapted for a Layer 5 switch that takes into consideration TCP session information. The steps to follow are:

- A packet is generated in the Client host, pass through Edge Router (ER) and arrives to the PA100’s port A

- The packet is stored in PA100’s DRAM memory

- A Classification Engine (CE) extracts relevant packet’s fields (Ethernet, IP or TCP/UDP) as specified in the Network Classification Language (NCL) code associated with the CE.

- A Network Classification Language (NCL) program executes NCL’s rules and stores rules’ result in a 512-bit vector.  The vector result allows the invocation of an Action associated with the rule.

- An Action Classification Engine (ACE) associated with the Action is invoked. The name of the ACE as shown in figure 7 is Ccbswitching.

- A TCP Session Hash Table is queried in order to find out if a TCP Session Handler object is associated with the incoming packet. If there is a TCP Session Handler associated with the packet, it is invoked. Otherwise, if the packet is a SYN packet, a new entry in the TCP Session Hash Table is added and a new TCP Session Handler object is created, otherwise it is dropped.

- If a received packet needs to be answered, the TCP Session Handler takes care of it.

- The packet to be sent as response is stored in DRAM and transmitted to the port A (i.e. an ACK packet is sent as response)

- A Classification Engine is used to execute fast lookup of the URL among several packets.

- Once enough packets has been received for assembling the URL, a TCP session is established between the front-end and the backend through port B. This new TCP session replays the parameters used in the TCP session between the client and the front-end.

FROM/TO BACKEND SERVERS

FROM/TO CLIENT HOSTS

ETHERNET

1

uPROCESSOR

DRAM

Map hash table

Pkt

Buffer

Classification Engine

SEARCH

Classification Engine

Classification Engine

PORT A

PORT B

N

Ccbswitching

ACE

....

TCPSessionHandler

1

SINGLE PROCESS

TCPSessionHashTable

FROM/TO EDGE ROUTER

2

3

4

6

5

7

8

10

9

Figure 8: Sequence of events for receiving a packet in the PA100 platform

### 2.4.3. PA100 Development Environment

PA100 system allows the programmer to use C++ as the programming language for the StrongARM platform. In addition it defines a set of libraries called Action Classification Libraries (ACL) and Network Classification Libraries (NCL) useful at the time of designing the Load balancing systems analyzed.

PORT A

PORT B

Ccbswitching ACE

Default pass/drop

port_B_target

Figure 9: Action Classification Engines used in PA100

ACL libraries have the following constrains:

- Mono-threaded

- No floating point support

- No file handling support

On the other hand, NCL libraries allow programmers to use rules, predicates and actions for accessing to fields in packet's header or payload at wire speed.  Its proprietary code runs on the Classification Engines.

All Load balancing Systems implemented are based in the software design described in figure 8. There is one single object (Ccbswitching) that handles all incoming and outgoing packets. Additional constrains that were taken into consideration at the time of designing the Load balancing Systems in PA100 were the following:

  - No write capabilities at the data plane level. This limits the capacity of the data plane. We created a pseudo data plane that uses clock cycles from the control plane (StrongARM 110). A combination of NCL language and ACL code was necessary for implementing the pseudo data plane.

  - No thread support.  The PA100 software environment is neither an Operating System (OS) nor an environment with thread support. We are limited to the use of a single thread of execution.

# 3. Design and Implementation of Load Balancing Switching Systems

This section gives a description of the implementation of the concepts introduced in the previous section.  We have implemented three load-balancing systems – Layer 3 WRR, Layer 5 LARD and an application level proxy with WRR.  In Section 3.1, we introduce the functional blocks of a load-balancing system and how the three implemented systems map to it.  In Section 3.2, we explain Persistent HTTP and how it can be supported in the architectures.

## 3.1 Load Balancing system building blocks

Figure 10 represents all the building blocks for a load balancing switching system. In order to contrast the main features of each load balancing system, we decided to implement three load balancing switching techniques: 1.) Layer 3 switching with WRR (L3WRR), 2.) Layer 5 switching with LARD and TCP splicing (L5LARDTCPS) and 3.) Application Level Proxy with WRR (PROXYWRR).

- Layer 3 switching with WRR (L3WRR) is essentially a data link layer switch that forwards incoming requests using Weighted Round Robin (WRR) algorithm and changes the Media Access Control (MAC) address of the packet. It is called layer 3 because it uses Transport layer information for recognizing new TCP connections and mapping subsequent packets to its proper connection. The logical topology of this architecture is depicted in Figure 1.

- Layer 5 switching with LARD and TCP splicing (L5LARDTCPS) is a Application Layer switch that reads incoming Universal Resource Locator (URL) information, applies LARD algorithm for load balancing and opens an exact replica of the initial TCP session with the back-ends (TCP splicing). The logical topology of this architecture is depicted in Figure 4.

- Application Level Proxy with WRR  (PROXYWRR) is an Application Layer switch that reads incoming URLs and redirects them to the nearest cache server to the user. If the information is not cached, it load balance the request among a farm of web server using WRR. It uses Network Address Translation for hiding the address of back-end servers. The logical topology of this architecture is depicted in Figure 3.

Each one of the systems mentioned use part or all the blocks shown in Figure 10. L3WRR is a MAC layer switch that only uses blocks 1, 2 and 5. L5LARDTCPS uses blocks 1, 2, 3, 4 and 5 and PROXYWRR uses blocks 1, 2, 3, 4 and 5 too. Blocks 6, 7 and 8 are optional and can be implemented by any of the systems.

Figure 10: Functional blocks of a load balancing system

According to [Arrowpoint00], Load balancing Switching system design has the following functional requirements:

- Flow classification: A block should be provided that enable the classification of flows and process a large number of rules. This task is memory intensive.

- Flow Setup : A method for handling HTTP sessions and handing off those sessions to the backends should be provided. The method implemented for L5LARDTCPS system is delayed binding or TCP splicing.The method used for PROXYWRR is Network Address Translation (NAT). L3WRR system does not need to use this block. This process is very processor intensive, depending on the amount of information in the HTTP request header that can be used to classify the content request. Flow setup requires a substantial processing “engine” .

- Flow forwarding: A block that handles packets at wire speed should be provided.  All the load balancing systems use this block.

- Support for high number of concurrent connections: capacity to “store” state for hundreds of thousands of simultaneous visitors. The number of concurrent flows in a web site is a function of the transaction lifetime and the rate of new flow arrival.

- Flow management: Functions such as management, configuration and logging should also be considered in the system.

In the design of the load balancing systems studied all these functional requirements have been taken into account.

## 3.2 Design considerations for HTTP 1.1 (Persistent HTTP)

Persistent HTTP (P-HTTP) connections allow the user to send multiple GET commands on a single TCP connection.  This is very useful as this reduces network traffic, client latency and server overhead [Mog95][Pad94].  However, having multiple requests on a single TCP connection introduces complications in clusters that use content-based request distribution.  This is because more than one back-end server might be assigned for responding to the multiple HTTP requests of a single TCP connection.

Requesting a HTML document can involve several HTTP requests, for example, embedded images.  In HTTP 1.0 [RFC1945], each request requires a new TCP connection to be setup.  In HTTP 1.1 [RFC2068], the client browsers are able to send multiple HTTP requests on a single TCP connection.  The servers keep the connection open for some amount of time (15 seconds), in anticipation of receiving more requests from the clients.  Sending multiple server responses on a single TCP connection avoids multiple TCP slow-starts, thereby increasing network utilization and effective bandwidth perceived by the client [Ste94].

The problem is that the mechanisms for content-based distribution operate at the granularity of TCP connections.  Hence, when each HTTP request arrives on a single TCP connection, the TCP connection can be redirected to the appropriate server for serving the request.  In the case where multiple HTTP requests arrive on a single TCP connection, as in HTTP/1.1, distribution of the request based on the granularity of TCP connection constraints the distribution policies.  This is because, when operating at the granularity of the TCP connection, requests on a single TCP connection must be served by one back-end server.

The following is a brief description of techniques that can be used to support P-HTTP.

### 3.2.1. Back-End Request Forwarding:

This mechanism studied in [Aron99], combines the single handoff mechanism with forwarding of responses and requests among the back-end nodes.  Here, the front-end hands off the connection to a back-end server, along with a list of other back-end servers that need to be contacted.  The back-end server to which the connection was handed off to then requests the other back-end servers either through a P-HTTP connection between them or through a network file system.  Only one back-end server is used in responding to the request.  The disadvantage of this mechanism is the overhead of forwarding responses on the back-end network.  Therefore, this mechanism is appropriate for requests that produce responses with small amounts of data.

Client

Internet

Front-End

Back-End Servers

Request

Response

Figure 11: Backend Request Forwarding

### 3.2.2. Multiple Handoff

A more complicated solution is to perform multiple handoffs between the front-end and back-end servers.  The front-end transfers its end of the TCP connection to servers sequentially among the appropriate back-end servers.  Once the TCP state is transferred to the back-end, in our implementation - by performing the 3-way handshake in our case and sending the sequence number, the back-end servers can directly send packets to the client bypassing the front-end.  After the response by the back-end server, the TCP state needs to the passed back to the front-end, so that the front-end can pass the TCP state to the next appropriate server.

Client

Internet

Front-End

Back-End Servers

Request

Response

Figure 12: Multiple handoff

A single handoff, like the one described in Section 2.1.4, can support persistent connections, but only one back-end server serves all requests.  This is because the connection is handed off only once.  The implementation of the front-end can be extended to support multiple handoffs to different servers, per TCP connection.  The advantage of having multiple handoffs is that it supports content-based request distribution at the granularity of the individual HTTP requests and not TCP connections.  To preserve the advantages of multiple HTTP requests per TCP connection - lower latency and server loads, the overhead of the handoff between the front-end and back-end servers should be low.

As shown in [Aron99], back-end forwarding mechanism trades off a per-byte response forwarding cost for a per-request handoff overhead.  The trade-off between the two methods depends on the relative costs of handoff (used in multiple hand-off) versus data forwarding (in back-end forwarding and the platform that is used at the front-end.

# 4. Evaluation

The purpose of this section is to determine the performance of each load balancing system studied. We define performance as the number of requests/second supported by the system. In addition we define a request as a complete TCP session that involves HTTP information exchange: starting from the SYN packet and ending in the FIN packet.

We follow two approaches for determining the performance: First we use a test bed (described in the next section) specifically designed for testing each one of the systems and collect practical information (section 4.2) and second we theorize about the performance threshold that these systems should achieve given the resource constraints of the PA100 system (section 4.4 and 4.5). Then, we compare the results from the two approaches and see how well they match. In addition we include a section (section 4.3) that describe the theoretical limits of PA100 platform in terms of memory bandwidth and forwarding speed that sets the upper bound of our back of the envelope calculations. In addition we include an analysis of how external factors (external to the front end) could help to alleviate the workload at the front end (section 4.7). For the sake of completeness, we finish with an analysis of how to implement the load balancing systems studied in the Intel IXP1200 Network Processor Architecture.

## 4.1 Test bed

We setup a testbed with the following  characteristics:

- SCLIENT: This is the HTTP request generator tool described in [Gau97] that allows to control the rate at which we send the requests. With SCLIENT there is no need of having a farm of client machines for testing the load balancing systems. SCLIENT is capable of simulating the behaviour of this farm of machines, but using only a single computer. It is capable of generating requests at client’s processor clock speed.

- FLASH web server: This is the web server used in the backends. We prefer this web server to Apache because  in our practical testings we determined that FLASH was capable of handling  higher requests/second rates than Apache. In addition FLASH logging capabilities make it suitable for collecting and representing useful data about the behaviour of the web server software.

- A client computer running FreeBSD 3.4 and SCLIENT for packet generation. This machine is a Pentium II 333Mhz , 128 Mb RAM with a 10 Mbps Ethernet card. According to our testing SCLIENT was capable of generating a maximum of 1024 requests/second due to limited socket buffer resources.

- A frontend computer running Windows NT 4.0 SP6 and hosting one PA100 card in a 33 Mhz PCI slot. This machine is a Pentium III 800 Mhz, 512 Mb RAM .

- Several backend machines running FreeBSD 4.1 and FLASH web server. These machines are Pentium II 266 Mhz 128 Mb RAM with a 10 Mbps Ethernet card each. According to our testings, each machine was capable of handling a maximum of 512 requests/second due to limited CPU processing capacity  (More than 512 requests/second were just dropped silently by the OS stack).

- 10BT Ethernet hubs for interconnecting all the machines.

Having said this, we were able to generate a maximum of 1024 requests/second  in the client and being capable of handling an aggregate of 2048 requests/second (with 4 backend servers).

Figure 13: Testbed configuration

## 4.2. Evaluation of load balancing system using the testbed

We start our evaluation with the analysis of the practical implementation of the load balancing systems. Our goal is to determine how well the load balancing systems analyzed perform under different scenarios. In the next subsections we explain how the systems behaves under increasing requests/second rates and under increasing number back end servers.

### 4.2.1 Determining practical requests/second rate limit

The purpose of this experiment is to determine the requests/second rate supported for each of the load balancings systems studied.

For this experiment we used the testbed described in section 4.1 and  we use SCLIENT as the request generator tool. SCLIENT has the ability to use the clock counter of the client machine for generating the desired requests/second rate.

Figure 14: Analyzing Requests/second rate capacity  supported by each load balancing system

Analyzing Figure 14 we observe that L5LARDTCPS is less tolerant to high requests/second values than PROXYWRR and L3WRR. We attribute this result to the fact that L5LARDTCPS is more complex than the other load balancing systems. L5LARDTCPS has to extract Layer 5 information from a packet and apply a more complex load-balancing algorithm (LARD). This has shown to reduce severely its capacity of handling large number of requests per second.  PROXYWRR follows in complexity. It also uses Layer 5 information for redirecting requests but uses a faster load-balancing algorithm (WRR). Finally, L3WRR is the best performer due to its shorter processing overhead over a packet (only Layer 2/3 information is extracted).

The previous results explain in coarse detail the causes of bottleneck in the systems studied. In order to confirm these results, we do in section 4.4 and 4.5 a fine grain analysis of the systems taking in consideration two factors: cpu cycles consumed and memory instructions used for each load balancing system.  We consider these two factors as CPU and memory are generally the most intense resources used in a Network Processor architecture [Bux01].

### 4.2.1 Evaluating the impact of different number of backends

The purpose of this experiment is to determine which is the impact of increasing number of backends in the load balancing systems. This testing is provided as a supplement to the previous testing for the sake of completeness.

For this experiment we used the testbed described in section 4.1 and  SCLIENT as the request generator tool. In addition we fix the request/second value to a constant of 100 requests/second. In this way packet loss will not interfere with our measurements (see Figure 14). The requested file size is 512 bytes and the number of SCLIENT requests is 4.

Figure 15: Latency for setting up an HTTP session vs number of backend servers

In Figure 15 we observe that  in general terms,  the effect of adding more backends is to reduce HTTP request latency (time spend from the beginning of an HTTP request until its completion). This is true for L3WRR and PROXYWRR. However in the case of L5LARDTCPS  this latency remains the same. This is because all the incoming requests hit one single server despite we increase the number of backend servers. The reason for this is that LARD directs all incoming requests to a single node if the number of requests is less than TLOW. In our case the number of requests is 4, lower than the value of TLOW (defined as 5). This gives us an idea of how sensible is  the L5LARDTCPS system to the values of TLOW and THIGH. This is why we decided to change the values of THIGH and TLOW  to being closer to each other (THIGH=3, TLOW=2), and this improved the performance of L5LARDTCPS because the load was smoothly divided among the backends. This confirms what is said in [Pai98]: LARD performance is closely related to the values chosen  for THIGH and TLOW.

The performance of L5LARDTCPS is closer to L3WRR than PROXYWRR. This is because L5LARDTCPS and L3WRR logical topology (see figure 2.1.4 and 2.1.1 respectively) tries to minimize the number of packets handled by the PA100 platform (10-11 packets per session, meanwhile PROXYWRR topology does not do this (15 packets per session). This has a considerable impact in the PA100 platform and produces the higher latency that we observe for PROXYWRR.  Improved cache hit ratios another factor that  helps to reduce the observed latency in L5LARDTCPS.

In the next sections we take the approach of analyzing the load balancing system theorizing about the performance threshold that these systems should achieve given the resource constraints of the PA100 system

## 4.3 PA100 theoretical limits

The purpose of this section is to set up the limits of the PA100 platform in terms of memory bandwidth and forwarding speed.  This section describes the upper bounds of the PA100 platform to which all our back of the envelope calculations are constrained.

We start our analysis with memory. The only kind of memory that PA100 has is DRAM. The most natural use of DRAM memory is to buffer packets, but in PA-100 DRAM is also used for  storing code and data structures from the StrongARM,  as a staging area for Classification Engine’s microcode loading and for buffers used in communicating  with the host and other PCI programs. The DRAM is connected to the processor by a  64 bit x 100 Mhz data path, implying a potential to move packets into and out of DRAM at  6.4 Gbps. In addition, the processor bus peak capacity is 1.6 Gbps (32bits x 50 Mhz). In theory, this is more than enough to support the 2 x 100 Mbps = 0.2 Gbps total send/receive bandwidth of the network ports available on the PA100 system.

In the PA100 system, there is no partition of the received data packet as in the IXP1200 case  (where a packet is divided in 64 bytes chunks called MPs). This would cause that long packets  take longer to be read/write  from/to  memory than  short packets,  causing a variable delay in memory access time for each packet.

For simplifying our calculations, we assume an average packet size of 64 bytes (minimum sized ethernet packet) , it will take 64 x 8 / 64/100Mhz = 80 ns to read/write a packet from/to DRAM memory. We should add to this time, the time that takes to classify a packet which involves the moving of all or some part of the packet from DRAM to the Classification Engine’s memory space. Assuming that a full packet is moved (this is true when UDP or TCP checksums are calculated) it will take an extra 80 ns to move the packet (the same value is used because CEs also use DRAM memory for storing information). This yields a total of 80 ns +  80 ns + 80 ns =240 ns to write an incoming packet,  classify it and read it at the output. This corresponds to a maximum forwarding rate of  4.1 Mpps. If no packet classification is considered, the maximum forwarding rate raises to 6.2 Mpps. In general the forwarding rate is decreased if we  run more sofisticated forwarding functions. The question then, is how much computation can we expect to perform on each packet, given some fixed packet rate.  In the next sections we tackle this question considering cpu and memory limitations.

## 4.4. PA100 CPU cycles analysis

The purpose of this section is to evaluate how much cpu cycles are consumed for each one of the load balancing systems and how it affects the number of requests/second supported.  In the PA100 system the only source of information about CPU cycles is a 32 bit register that increments its value every 1 usec. Therefore, every value read from this register is an integer that represents a multiple of 1 usec.  Table 1 shows the results obtained.

| LOAD BALANCING SYSTEM | Value read from clock register per request (multiple of 1 usec) (clock_register) | Packets per request (pkts_per_request) | Estimated Mpps (1000000/(clock_register/pkts_per_request)) (Mpps) |
| --- | --- | --- | --- |
| No load balancing | 2 | 10 | 5 |
| L3WRR | 55 | 10 | 0.182 |
| L5LARDTCPS | 257 | 11 | 0.043 |
| PROXYWRR | 245 | 15 | 0.061 |

Table 1: Theoretical Mpps given number of clock cycles in PA100

In addition we can calculate the number of requests/second that can be handled by each load balancing system, using the estimated Mpps and the number of packets per request from Table 1. Table 2 shows the calculated values.

| LOAD BALANCING SYSTEM | Estimated Requests/second CPU analysis (Mpps/pkts_per_request) |
| --- | --- |
| No load balancing | 500000 |
| L3WRR | 18200 |
| L5LARDTCPS | 3909 |
| PROXYWRR | 4066 |

Table 2: Maximum number of requests/second supported for each Load balancing system

One obvious conclusion from Table 2 is that the more the CPU cycles used by any given load balancing system, the less the requests/second supported.  Table 2 represents the maximum requests/second supported  assuming that memory  does not impose performance limitations and only cpu is considered.

In order to have a better understanding of where in the load balancing system’s code the cpu bottleneck originates, we can do a more detailed analysis about the number of cpu cycles consumed by the most relevant classes and methods used.

Table 3 shown the classes/methods used for each one of the load balancing systems studied and Table 4 shows how long it takes for each one to be executed along with its frequency of use and its purpose. Names of each class/method are self descriptive, but a short description is provided in Table 4.

| MOST RELEVANT CLASS/METHODS | No load balancing | L3WRR | L5LARDTCPS | PROXYWRR |
| --- | --- | --- | --- | --- |
| TCPSessionHandler |  |  |  |  |
| TCPSHashTable |  |  |  |  |
| EthernetHashTable |  |  |  |  |
| LARD_HashTable |  |  |  |  |
| Packet_template |  |  |  |  |
| TCP session deletion |  |  |  |  |

Table 3: Objects used in each Load balancing system

| MOST RELEVANT CLASS/METHODS | Value read from clock register per request (multiple of 1 usec) | Frequency of use | Purpose/type |
| --- | --- | --- | --- |
| TCPSessionHandler | 11 | Every non duplicated SYN pkt | Keeps TCP session’s state information and is destroyed when session ends. Non persistent object |
| TCPSHashTable | 2 | Any arrival of packet | Hash table that keeps pointers to TCPSessionHandlers for fast lookup. Persistent object |
| EthernetHashTable | 2 | Any arrival of packet | Hash table that keeps pointers to MAC addresses for fast lookup. Persistent object |
| LARD_Table | 9 | After receiving URL packet | Hash table that keeps mapping between URL and backends for fast lookup. Persistent object |
| Packet_template | 18 | Every SYN and ACK+URL packet sent to backend | Generates a packet to be sent as response to backend servers. Non persistent object |
| TCP session deletion | 10 | After receiving a FIN packet from client | Frees memory resources used by Objects. Method |

Table 4:  CPU cycles consumed for each class/method used in a load balancing system

TCPSHashTable and EthernetHashTable are used for every single incoming packet during an HTTP request. TCPSessionHandler, LARD_Table and TCP session deletion are used once for each HTTP request. Packet_template is used twice during an HTTP request. Therefore we can easily determine that Packet_template jointly with all the classess/methods used once during an HTTP request are the main bottlenecks of  those load balancing system that use them. Lets analyze each one of the main bottlenecks in further detail.

Packet_template  is a class used for responding to certain classes of incoming packets. The main idea is to read an arbitrarily pre-defined packet stored in DRAM, change the proper fields on it and send it as a reply to an incoming packet.  Another alternative analyzed and also used in our code is to receive an incoming packet in memory, change the proper fields of it and send it back as a response. The latter method seems to be more efficient in terms of accessing to memory (one access as opossed to almost twice the number of  accesses in the former method) but it was no possible to implement it in all cases. As an example of cases where it was not possible, we cite when a new SYN packet  is  created from scratch or when more than one packet is needed to be generated as response (ACK +URL).  Both cases happens in the three way handshake communication between the frontend and the backend (when using L5LARDTCPS or PROXYWRR)

TCPSessionHandler is a repository of HTTP session information that should be created at the beginning of a session. There is a considerable ammount of information that should be written to memory, such as TCP  states, TCP seqno, TCP client’s address , selected backend server, etc.  but this only happens whenever a new HTTP session is created. As more HTTP sessions are  created and kept in memory (such as in HTTP 1.1, where HTTP sessions stays longer in DRAM memory), this object becomes a non trivial source of memory consumption and contention. Memory contention will be discussed in section 4.5

LARD_Table handles a hash table for mapping URL to backend servers, similar in functionality to TCPHashTable or EthernetHashTable. However, LARD_Table amounts for a higher number of clock cycles (almost 5 times the number of clock cycles used in the latter classes – see Table 4)  because URL strings needs to be converted to a hash index representation before being inserted in an associative array that maps hashed URLs to backends.

TCP session deletion is a subroutine used for deleting all the objects associated with an HTTP session. Despite this subroutine is called only once during the life of an HTTP session, to erase and free memory is not a trivial task considering that a complete TCPSessionHandler object and an TCPHashTable/EthernetHashTable entry should be deleted.

These four classes/methods are the main source of cpu cycles consumption and also has an active role in memory contention. Next section analyze in further detail memory as a source of bottleneck in PA100 system.

## 4.5. PA100 memory  analysis

In this section, our goal is to measure how many memory access each load balancing system execute and which is its impact in the overall performance in terms of requests/second supported.

For calculating requests/second supported when only memory is considered, we count the number of memory read and write instructions used for each load balancing system. This number is the average number of accesses to memory per packet, as shown in Table 5.

| LOAD BALANCING SYSTEM | Number of read instructions per packet | Number of write instructions per packet | TOTAL Read+write Instructions per packet (total_instructions) | TOTAL execution time for instructions (usec) (total_instructions/100Mhz) (exec_time) | Number of packet per HTTP session (pkts_per_session) | Estimated requests/second DRAM analysis 1000000/(exec_time x pkts_per_session) |
| --- | --- | --- | --- | --- | --- | --- |
| DIRECT | 34 | 21 | 55 | 0.55 | 10 | 181810 |
| L3WRR | 1264 | 574 | 1838 | 18.38 | 11 | 4946 |
| L5LARDTCPS | 2701 | 1223 | 3924 | 39.24 | 11 | 2317 |
| PROXYWRR | 2414 | 1095 | 3509 | 35.09 | 15 | 1890 |

Table 5: Estimated HTTP sessions/sec taking into consideration memory latency

The results shown in Table 5 does not take into consideration pipelining of instructions and cache access in StrongARM whose effect should decrease the estimated execution time of the assembler instructions. What we are providing are the values for  the worst case scenario (i.e. no instructions in processor’s cache and sequential execution of memory access commands) for accessing to memory in the StrongARM platform, therefore the values estimated in Table 5 for  requests/second are the minimum values that in theory the PA100 can support simultaneously before starting to lose requests.

| LOAD BALANCING SYSTEM | Estimated requests/second CPU cycles analysis (values from Table 2) | Estimated requests/second DRAM analysis (values from Table 5) | % difference |
| --- | --- | --- | --- |
| DIRECT | 500000 | 181810 | 63 |
| L3WRR | 18200 | 4946 | 73 |
| L5LARDTCPS | 3909 | 2317 | 41 |
| PROXYWRR | 4066 | 1890 | 54 |
| Average % 57 |  |  |  |

Table 6:  Comparing HTTP sessions/second when CPU or memory are the bottleneck

If we compare estimated requests/second supported  when CPU is the bottleneck (Table 2) and  estimated requests/second when memory is the botleneck (Table 5) we get Table 6.  From Table 6 we can conclude that memory (DRAM) is the  main source bottleneck in PA100 reducing in an average of 57% the number requests/second supported. Furthermore we can say that with faster DRAM memory , the number of requests/second supported will increase in at least 57 %.

## 4.6. Overall analysis

The purpose of this section is to compare the practical and theoretical values gotten from section 4.2 and 4.5 and obtain useful conclusions.

| LOAD BALANCING SYSTEM | Estimated requests/second DRAM analysis (values from Table 5) | Measured requests/second at which packet loss start (see figure 14) | % error |
| --- | --- | --- | --- |
| DIRECT | 181810 | Not measured | Not measured |
| L3WRR | 4946 | 4957 | 0.2 |
| L5LARDTCPS | 2317 | 300 | 87 |
| PROXYWRR | 1890 | 1136 | 40 |

Table 7: Comparison between theoretical requests/second and measured requests/second values

Table 7 shows that, for the L3WRR case, there is almost a perfect match for the number of request/second calculated from theory and requests/second measured. This is not the case for the others load balancing systems. L5LARDTCPS calculated value (2317) is too far from the measured value (300). This means that the impact of the load balancing algorithm (LARD) is higher than we forecasted, that is LARD algorithm cause more memory contention than we were expecting.  For the PROXYWRR case we can say that the impact of the topology used (see section 2.1)  is more than we expected, causing the system to reach the limit of request/second rate faster than we though. These results at least validate our theoretical calculations for the L3WRR case. For the L5LARDTCPS and PROXYWRR case we believe that the use of a high level language and Standard C++ libraries has hidden the real number of memory access when LARD or WRR algorithms are used. This has biased our theoretical calculations.

## 4.7. Influence  of external factors on load balancing systems.

The purpose of this section is to analyze how external factors could help to reduce memory contention or cpu cycles consumed at the frontend.  We define external factor as any mechanism external to the frontend that is used as part of the load balancing system and affects the behaviour of the overall system.

We have seen so far that one of the main reasons why the Load balancing systems have not reached higher performance is because of PA100 memory limitations, that is PA100  have a high degree of memory contention when input and output ports are used  intensively (as shown in Table 5), when the complexity of the system (in terms to access memory or cpu cycles – see Table 2) is high or just simple when we are dealing with a high number of packets in the network.  A smart design of the  Load balancing system could help to alleviate the workload on the PA100 platform. Techniques such as asymetric logical topologies for redirecting high volume of traffic (as shown in figure 1)  helps to deviate the load through different paths.

Techniques such as  TCP handoff proposed in [Hunt97] , even though is simple and does not violate TCP semantics at the backend, can be a source of bottleneck due to the use of  a higher number of  packets than a simple TCP three way handshake. [Pai98] suggest a technique for TCP handoff that eliminates the need of replaying the TCP session and starts the TCP session since the ESTABLISHED state in the backend. This technique will definitely alleviate the workload at the frontend. The drawback of this technique is that it violates TCP semantic and modifies the TCP stack of the backends (adding a kernel loadable module), making it not transparent for the backend. Improving cache locality  at the backends is another technique that helps to reduce memory contention because, if the information is found in the backend’s cache the HTTP session will stay shorter in frontend’s memory (because of the faster response of the backend) causing less memory contention.  We can extrapolate this result to HTTP 1.1 and predict that PA100 performance may decrease if we implement the technique suggested in 3.2.2 called Multiple handoff   because this technique have to handle an HTTP request for longer time (due to the  opening of new TCP connections for every GET received in a single request), causing more memory contention at the frontend. The alternative technique proposed in 3.2.1 called  Back-end request forwarding is the technique that we suggest to be implemented as the HTTP 1.1 solution because  it keeps memory contention low at the backend (due to its use of a single request handoff to the backend and smart way of moving the frontend’s workload to the backends)

## 4.8. Porting PA100 Load Balancing design to IXP1200

After our analysis of implementing load balancing systems under PA100, we want to provide some insights about how  to port the same systems to the IXP1200 Network Processor Architecture. This section is provided for the sake of completeness of our analysis about Network Processors.

IXP1200 is a more powerful Network Processor system developed by Intel. Porting a Load balancing system from PA100 to IXP1200 is not a trivial task because of the architectural differences among them. IXP1200 is aimed to handle speeds up to 2.5 Gbps. It has been demonstrated by [Spalink00] that IXP1200 is capable support 8x100 Mbps ports with enough headroom to access up to 224 bytes of state information for each minimum-sized IP packet.

The building blocks of IXP1200 are: A StrongARM SA-110 233 Mhz processor, a Real Time Operating System (RTOS) called VxWorks running on StrongARM, 64bit DRAM and 32 bit SRAM memory, 6 microengines (uengines) running at 177 Mhz and each one handling 4 threads, a proprietary 64-bit, 66 Mhz IX Bus, a set of media access controllers (MAC) chips implementing ten Ethernet Ports (8x100Mbps+2x1Gbps), a scratch memory area used for synchronization and control of the uengines and a pair of FIFOs used for send/receive packets  to/from the network ports. The DRAM is connected to the processor by a 64 bit x 88 Mhz data path. SRAM data path is 32x88Mhz. Each uengines has associated a 4 KB instruction store.

We can use the same design guidelines of section 3.1 to distribute the different functional units (blocks) among the hardware components of IXP1200. Flow forwarding and classification should be handled at wire speed, therefore we can use the six uengines for handling this task.  In IXP1200 we can be fine grained and implement all the hash lookup functionality in SRAM and packet storage, hash tables, routing tables and any other piece of information in DRAM.

Flow setup that is a processor intensive task, should be handled by the StrongARM. Furthermore, with the RTOS we can assign priorities to the different task running in Flow Setup (i.e. higher priority to flow creation rather than flow deletion). In addition we can use the TCP/IP stack that comes with VxWorks in order to do the TCP handoff and avoid to program it from scratch (as in the PA100 platform). Finally, an external General Purpose Processor such as a Pentium processor could  handle the Flow management module.

Figure 16: IXP1200 architectural diagram

This is in general terms the way we can map the functional units of a load balancing system to the IXP1200 platform. Companies such as Arrowpoint [Arrowpoint00] have built their Load balancing systems from scratch: using their own hardware and software and following the guidelines of section 3.1.

A more interesting question is which is the expected number of requests/second that an IXP1200 platform could handle. We can extrapolate some of the results of section 4 for the PA100 platform and predict which will be the performance of IXP1200.

It has been demonstrated by [Spalink00] that memory bandwidth limits the IP packet-forwarding rate of IXP1200 to 2.71 Mpps with the total number of accesses to memory shown in figure 17.

Figure 17: IXP1200 Per-packet pseudo-code

annotated with the number of actual instructions (I),

DRAM accesses (D), SRAM accesses (S),

and scratch (local) memory accesses (L) [Spalink00]

The function Reg_Entry.func() includes  all protocol specific packet header or content modifications. This function could execute a vanilla IP forwarding function or a more complex function such as  load balancing or algorithms such as LARD or WRR.  If we consider the number of memory read/writes that we used in the implementation of the Load balancing systems studied under the PA100 architecture as if they were the number of read/writes we need for accessing memory in IXP1200, we get the results shown in Table 8. Note that achievable memory bandwidth for IXP1200 is 5.01 Gbps according to [Spalink00].

| LOAD BALANCING SYSTEM | TOTAL reads+writes in PA100 (from table 5) | TOTAL DRAM memory access IXP1200 (+ 5 due to extra DRAM access in IXP1200) (dram_access) | Total bits transferred to/from memory (dram_access x 32 bits) (bits_transferred) | Total expected forwarding rate IXP1200 in Mpps 1000000/(bits_transferred/5.01 Gbps) (Mpps) | Estimated request/second supported IXP1200 (Mpps/pkts_per_request) |
| --- | --- | --- | --- | --- | --- |
| DIRECT | 55 | 60 | 1920 | 2.6 | 260000 |
| L3WRR | 1838 | 1843 | 58976 | 0.085 | 8500 |
| L5LARDTCPS | 3924 | 3929 | 125728 | 0.040 | 4000 |
| PROXYWRR | 3509 | 3514 | 112448 | 0.045 | 4500 |

Table 8: Number of read/writes from/to memory for each Load balancing system

The requests/second rate supported is more for the IXP1200 than for the PA100 (compare against Table 5 or 6). Table 9 shows a comparison of each platform in terms of requests/second supported.

| LOAD BALANCING SYSTEM | Estimated requests/second supported IXP1200 (see table 8) | Estimated request/second DRAM analysis (values from Table 5) | % difference |
| --- | --- | --- | --- |
| DIRECT | 260000 | 181810 | 30 |
| L3WRR | 8500 | 4946 | 42 |
| L5LARDTCPS | 4000 | 2317 | 42 |
| PROXYWRR | 4500 | 1890 | 58 |
| Average % 43 |  |  |  |

Table 9: Comparison of  estimated requests/sec supported in IXP1200 and PA100

We still have to remember that we can improve the value of requests/sec for the IXP1200 platform. Recall that we are assuming the same number of instructions in PA100 and IXP1200, which in practice could be much less (because of the use of  proprietary micro code language in the IXP1200 platform). In addition, we are assuming that all the accesses of our load balancing systems when ported to IXP1200 are made in DRAM. This is also not accurate because most packet handling and hash lookup of these systems could be made in SRAM (faster memory). Therefore, Table 9 gives us the lower bound of expected value to be supported in the IXP1200. But even in the worst-case scenario, IXP1200 is able to perform an average of 43% better than PA100.  A more accurate result could be gotten if the load balancing systems are actually implemented in the IXP1200 platform.

# 5. Related Work

Rice University:  Research in load balancing is being pursued for the past few years by Prof. Peter Druschel’s team at Rice University [Pai98][Pai99][Aron99][Aron00].  In addition to their load balancing algorithm – LARD, they have developed a HTTP client (Sclient) and HTTP server (Flash).  We have used Sclient and Flash [Pai99] for performing our tests.  Prof. Druschel’s team has developed load balancing techniques, which they have proven to show better results than our implementation.  Mostly they have used a Linux machine at their front-end.

Princeton University:  A team at Princeton has been working on the IXP 1200.  Their understanding and study of the IXP 1200 has been documented in a paper recently published by them [Spalink00].  Their research is focused on the IXP 1200 and not on load balancers.

IBM T.J. Watson:  The research staff at IBM T.J. Watson has been trying to design simple load balancers [Goldszmidt97] [IBM00].  They have proposed a few techniques in performing the hand-off between the front-end and the back-end servers [Hunt97].  We have implemented one of the techniques proposed by them.

There are several commercial vendors who sell load balancers.  Due to the increased use of server clusters and the need to distribute the traffic, the load balancer market is growing at a very fast rate.  Major network equipment vendors – Cisco [Cisco00] and Nortel purchased two load balancer makers – Arrowpoint Communications [Arrowpoint00] and Alteon WebSystems, respectively.  There are many newer entrants developing both layer 3 and layer 5 load balancers.  Some of the vendors include – Hydraweb. Resonate, Cisco’s Local Director (Layer 3), IBM, Foundry Networks and BigIP Networks.

Commercial vendors use customized hardware and software, and are therefore able to process more number of packets and handle more number of TCP connections.  They also implement a management plane – that keeps track of the performance and availability of the back-end servers and also provide a user interface.

# 6. Conclusions

We demonstrate that among CPU and memory resources in the PA-100 platform, memory appears as the main cause of bottleneck due to the high level of memory contention and we can achieve at least 57% of better performance if we increase the speed of DRAM.  This is true for all the load balancing systems implemented and evaluated. Techniques such as paralelism are commonly employ to hide memory latency. For example Intel IXP1200  includes six micro-engines, each supporting four hardware context. The IXP1200 automatically switches  to a new context when the current context  stalls on a memory operation.

Complex memory interleaving techniques that pipeline memory access and distribute individual packets over multiple parallel DRAM  chips can  is the technique suggested by [Bux01] to minimize memory latency in Network Processors.

We demonstrate that even in the worst-case scenario, IXP1200 is able to perform 43% better than its PA100 counterpart. This demonstrates the advantages that implies the use of techniques such as parallelism, multithreaded micro engines and micro code programming.

In order to alleviate the workload at the frontend we have used techniques such as asymetric logical topology (as shown in figure 4) for the Load balancing system that redirects  backends’ responses through an alternate path, bypassing the frontend. Other techniques include the use of loadable kernel modules for starting the TCP session since the ESTABLISHED state at the backends and using LARD for improving cache locality at the backend. In general, the deployment of  complex systems with Network Processors that yields a good performance should consider not only the software design of the frontend but the design of the overall system. Any Network Processor would be alleviated if with a smart system design its workload is reduced.

# 7. References

[Apache00] Apache. http://www.apache.org

[Arl96] M.F. Arlitt and C.L. Williamson. Web Sever Workload Characterization: the Search for Invariants.  In Proceedings of the ACM SIGMETRICS `96 Conference, Philadelphia, PA, Apr. 1996.

[ARM7500] ARM Processor instruction set. ARM Corporation. http://www.arm.com

[Aron99] M. Aron, P. Druschel, W. Zwaenepoel. Efficient Support for PHTTP in Cluster Based Web Servers. In Proceedings of the 1999 Annual Unix Technical Conference, Monterey, CA, June 1999.

[Aron00] M. Aron, D. Sanders, P. Druschel, W. Zwaenepoel. Scalable Contentaware Request Distribution in Clusterbased Network Servers. In Proceedings of the 2000 Annual Usenix Technical Conference, San Diego, CA, June 2000

[Arrowpoint00] A comparative Analysis of Web Switching Architectures.  Arrowpoint Communications.  (http://www.arrowpoint.com)

[Bux01] Technologies and building blocks for Fast Packet forwarding. Werner Bux, Wolfgang E. Denzel, Ton Engbersen, Andreas Herkersorf, and Ronald P. Luijten. IBM research.  IEEE Communications Magazine. January 2001

[Cisco00] Cisco System Inc. Cisco LocalDirector. http://www.cisco.com

[Fox] Cluster based Scalable Network Services. Armando Fox, Steven D. Gribble, yatin Chawathe, Eric A. Brewer, Paul Gauthier. University of California at Berkeley.

[Gau97] Gaurav Banga, Peter Druschel. Measuring the Capacity of a Web Server.  USENIX Symposium on Internet Technologies and Systems (USITS). Monterrey, CA, Dec 1997. Winner of Best Paper and Best Student Paper Awards.

[Goldberg] The Ninja Jukebox, Ian Goldberg, Steven D. Gribble, David Wagner and Eric A. Brewer, The University of California at Berkeley, http://ninja.cs.berkeley.edu

[Goldszmidt97] NetDISPATCHER: A TCP connection router. G. Goldszmidt, G. Hunt. IBM Research Division T.J. Watson Research Center. May 1997.

[Hunt97] G. Hunt, E. Nahum, and J. Tracey. Enabling contentbased load distribution for scalable services. Technical report, IBM T.J. Watson Research Center, May 1997

[IBM00] IBM Corporation. IBM Interactive Network Dispatcher.

http://www.ics.raleigh.ibm.com/ics/isslearn.htm

[Iyengar97] A. Iyengar and J. Challenger. Improving web server performance by caching dynamic data. In Proceedings of the USENIX Symposium on Internet Technologies and Systems (USITS), Monterey, CA, Dec. 1997

[Mog95] J.C. Mogul. The Case for Persistent-Connection HTTP. In Proceedings of the ACM SIGCOMM `95 Symposium, 1995.

[Pad94] V.N. Padmanabhan and J.C. Mogul. Improving HTTP Latency. In Procedings of the Second International WWW Conference, Chicago, IL, Oct 1994.

[Pai98] V. Pai, M. Aron, G. Bana, M. Svendsen, P. Druschel, W. Zwaenepoel, E. Nahum. LocalityAware Request Distribution in Clusterbased Network Servers. In Proceedings of the ACEM Eight International Conference on Architectural Support for Programming Languages and Operating Systems, San Jose, CA, Oct 1998.

[Pai99] Flash: An efficient and portable web server. Vivek S. Pai, Peter Druschel, Willy Zwaenepoel. Department of Electrical and Computer Engineering Rice University. Proceedings of the 1999 Annual Usenix Technical Conference, Monterey CA, June 1999

[Peterson00]  Computer Networks: A System Approach. Larry  L. Peterson, Bruce S. Davie. Morgan Kaufman press. Second Edition

[Resonate00] Resonate Inc. Resonate dispatch. http://www.resonateinc.com

[RFC793] TRANSMISSION CONTROL PROTOCOL, DARPA Internet Program Protocol Specification. University of Southern California. September 1981

[RFC1945] T. Berners-Lee, R. Fielding, and H. Frystyk. RFC 1945: Hypertext Transfer Protocol - HTTP/1.0, May 1996.

[RFC2068] R. Fielding, J. Gettys, . Mogul, H. Nielsen, and T. Berners-Lee. RFC 2068: Hypertext Transfer Protocol - HTTP/1.1, Jan 1997.

[SA-110-I] StrongARM SA-110 Microprocessor Instruction Timing. Application Note.Intel Corporation. September 1998

[SA-110-MEM] Memory Management on the StrongARM SA-110. Application Note. Intel Corporation. September 1998

[SA-110-uP] SA-110 Microprocessor Technical Reference Manual.  Intel Corporation. September 1998.

[Sing] Efficient Support for Content-Based Routing in Web server Clusters. Chu-Sing Yang and Mon-Yen Luo. Department of Computer Science and Engineering National Sun Yat-Sen University. Kaohsiung, Taiwan.

[Spalink00] Evaluating Network Processors in IP Forwarding, Tammo Spalink, Scott Karlin, Larry Peterson, Princeton University, Technical Report TR-626-00, November 15,2000

[Ste94] W. Stevens. TCP/IP Illustrated Volume 1 : The Protocols.  Addison-Wesley, Reading, MA, 1994.

[Yates96] D.J. Yates, E. M. Nahum, J.F. Kurose, and D. Towsley. Networking support for large scale multiprocessor servers. In Proceedings of the ACM Sigmetrics Conference on Measurement and Modeling of Computer Systems, Philadelphia, Pennsylvania, May 1996.

[Zhang] X. Zhang, M. Barrientos, J. Bradley Chen, M. Seltzer. HACC: An Architecture for Clusterbased Web Servers. In 3 rd USENIX Windows NT Symposium.

APPENDIX

TCPSessionHandler

Tcpsessionkey

NBSearchEngine

NBSearchContext

Etherheader

Ipheader

TCPheader

Urlstring

SetupSearch()

Replypkt()

Apply_LARD()

Backend_tcpsession()

Cksum()

Ccbswitching

Ace

Set_tcpsset

Target

Pkthandler()

Lookupset()

NBSearchEngine

NBSearchContext

NBStringMatchReport

TCPSHashTable

TcpSessionHandler

Map

Elt_tcpsset

key_[0]

key_[1]

key_[2]

key_[3]

Set_tcpsset

Search()

port_B_target

Buffer

b

NCL (interface)

Action_drop()

action_exist_tcpsession_synack_backend()

action_exist_tcpsession_ack_backend()

action_new_tcpsession_syn()

action_exist_tcpsession_ack()

AfterBuffer()

GotString()

LARD_table

Map

Totalsumconnections

existlessTLOW

Leastloadednode()

LARD_checksumallloads

Backend_server

Search()

CONTENT AWARE REQUEST DISTRIBUTION USING LARD: CLASS DIAGRAM

Content Aware Request Distribution is a technique used for switching client's requests based on the request's content information in addition to information about the load on the server nodes (backend nodes). There are several techniques used for implementing Content Aware Distributor systems.

Abstract

Load balancing has traditionally being used as the way of share the workload among a set of available resources. In a web server farm, load balancing allows the distribution of user requests among the web servers in the farm.

Content Aware Request Distribution is a load balancing technique used for switching client's requests based on the request's content information in addition to information about the load on the server nodes (backend nodes).

Content Aware Request Distribution has several advantages over current low-level layer switching techniques used in state-of-the-art commercial products. It can improve locality in the backend servers' main memory caches, increase secondary storage scalability by partitioning the server's database, and provide the ability to employ backend server nodes that are specialized for certain types of request (e.g. audio, video)

Intel PA100 is a network processor created for the purpose of running network applications at wire speed. It differs from general-purpose processors in that the hardware is specifically designed to handle packets efficiently. We choose the Intel PA100 processor as it provides a programming framework that is being used by current and future implementations of Intel's network processors.

No studies have been done before that design and implement multiple load balancing systems using the Intel PA100 network processor and furthermore compare the advantages that Content Based Switching System have over traditional load balancing mechanism. Our purpose is to use PA100 as a front-end device that directs incoming request to one server in a farm of back-end servers using different load balancing mechanisms.

In this thesis, we also implement and evaluate the impact that different load balancing algorithms have on the PA100 network processor architecture.  Locality Aware Request Distribution (LARD) and Weighted Round Robin (WRR) are the load balancing algorithms analyzed. LARD achieves high cache hit rates and good load balancing in a cluster server according to [Pai98]. In addition, it has been confirmed by [Zhang] that focusing on locality can lead to significant improvements in cluster throughput. WRR is attractive because of its simplicity and speed.

We also implement a TCP handoff protocol proposed in [Hunt97], in order to hand-off incoming request to a backend in a manner transparent to the client, after the front end has inspected the content of the request.

We demonstrate that among CPU and memory resources in the PA-100 platform, memory appears as the main cause of bottleneck due to the high level of memory contention and we can achieve at least 57% of better performance if we increase the speed of DRAM.  This is true for all the load balancing systems implemented and evaluated.

We finally demonstrate that even in the worst-case scenario, IXP1200 is able to perform 43% better than its PA100 counterpart.

--------------------------------------------------------------------------------------------------------------

Request has the meaning explained at the beginning of section 4

These values are valid assuming an HTML request of 512 bytes. The bigger the HTML file requested, the more  packets are sent in a request. For simplifying our analysis we stick with the 512-byte file size in the rest of our evaluation unless a different file size is explicitly specified.

This is the finest granularity that can be read in StrongARM SA110 registers in terms of CPU clock cycles, but it  is still coarse. Recall that actual duration of a clock cycle in StrongARM SA110 is 1/233 Mhz = 4 ns

Request represent a complete HTTP session as explained at the beginning of section 4

Packets/requests value is obtained from a 512-byte HTML file request. This value increases as the requested HTML file size increases, but for  keeping our calculations simple we have stick with this value in the whole paper, if not otherwise specified.

This value fits in the range specified  in section 4.3 in terms of forwarding speed.  6.2 Mpps was  found as the upper bound limit in PA100 if no packet classification is considered.

Request represent a complete HTTP session as explained at the beginning of section 4

HTTP 1.1 is characterized for sending more than one HTTP request through the same TCP session, thus extending the life of a TCP session handler in DRAM memory.

100 Mhz is  because of memory bandwidth limitation

VxWORKS is a RTOS developed by WindRiver (http:/www.windriver.com)

Recall that both platforms use the same Strong ARM SA110 processor at the same clock speed

Assuming that we work with 64 byte-size packet transfer

Packets per request values can be found in Table 5.

This technique is used by [Pai98]. Other techniques include the use of pre-established long live TCP connections between front-end and backend as described in [Sing]