###### Virtual Private Network with IPsec Tunneling

To: 	Professor Peter Steenkiste

By: 	Terence Law

Xiaofeng Wang

Chi Tong

Date: 	Dec 18th, 2000

CONTENTS

Contents……………..………….………….……………………………………...	2

Introduction.…………………..….……….……………………………………….	3

#### Design Goals………………………………………………………………………	5

Architectural Design………………………………………………………………	6

Implementation……………………………………………………………………	9

Evaluation…………………………………………………………………………	13

Conclusion…………………………………………………………………………	15

Reference…….…………………………….…………………………..…………..	16

Appendix: Evaluation Data…..………….…………………………………….….	17

Abstract

In this report, we presented our works on design, implementation and evaluation of a Virtual Private Network system with IPsec techniques using Intel IXP1200 Network Processors.

Introduction

As the Internet grows and expands, communications over a wide area network (WAN) becomes cheap and easy.  Virtually everywhere in the world can get connected through the Internet.  Instead of spending huge amount of money building private communication channel through lease lines, companies can make full use of the Internet to enjoy cheap connections among their worldwide branches.

However, the Internet is a public inter-network and transmission of private data is a big concern. As a shared communication medium, the Internet is vulnerable to interception, corruption and interruption from malicious parties. For example, a spy in commerce can “sniff” packets between two branches of a company on the public routers to steal the commercial information (interception), modify the packets (interruption) or even pretend to be one of the two parties to cheat the other (corruption). These problems greatly impair the benefits that the public inter-network brings to users. On the other hand, though private networks have no similar security problems, as mentioned in the first paragraph, dear expenses limit its extensions in comparison to the Internet.

A compromise between security (private networks) and cost (public inter-network) is Virtual Private Network (VPN). A VPN connects components and resources of a private network over a public network. It accomplishes this by allowing the user to tunnel through the Internet in a manner that lets the tunnel participants enjoy the same security and features formerly available only in private networks (see Figure 1) [1].

Figure 1 Virtual Private Networks

VPN

VPNs allow telecommuters, remote employees like salespeople, or even branch offices to connect in a secure fashion to a corporate server located at the edge of the corporate Local Area Network (LAN) using the routing infrastructure provided by the Internet. From the user's perspective, the VPN is a point-to-point connection between the user's computer and a corporate server. The nature of the intermediate inter-network is irrelevant to the user because it appears as if the data is being sent over a dedicated private link.

As mentioned, VPN technologies also allow a corporation connecting branch offices or other companies (Extranets) over the Internet, while maintaining secure communications. The VPN connection across the Internet logically operates as a Wide Area Network link between the sites.

In both of these cases, the secure connection across the inter-network appears to the user as a private network communication -- despite the fact that this communication occurs over a public inter-network -- hence the name Virtual Private Network. Such an approach achieves the security of private network while keeps the balance amounting to the public inter-network.

##### Tunneling

The key technique inside VPN is tunneling. Tunneling is a method of using an inter-network infrastructure to transfer data from one network over another network. The data to be transferred (or payload) can be the frames (or packets) of another protocol. Instead of sending a frame as it is produced by the originating node, the tunneling protocol encapsulates the frame in an additional header. The additional header provides routing information so that the encapsulated payload can traverse the intermediate inter-network.

The encapsulated packets are then routed between tunnel endpoints over the inter-network. The logical path

which the encapsulated packets travel through the internetwork is called a tunnel. Once the encapsulated frames reach their destination on the internetwork, the frame is unencapsulated and forwarded to its final destination. Note that tunneling includes this entire process (encapsulation, transmission, and unencapsulation of packets).

Figure 2 Tunneling

Tunneling can be implemented in layer 2 (data link layer) or layer 3 (networking layer) according to the OSI reference model. Over the Internet, a widely used implementation of tunneling is through IP (network layer) by using IPsec [2][3]. IPsec is a security standard defined by IETF for security in IP. It allows end-to-end encryption and authentication, and thus makes TCP/IP communications secure for use in both public and private networks [2].

## Project

In this project, we designed a VPN architecture by using IPsec in tunnel mode.  Implementation is done on the Intel IXP1200 platform and finally measured the performance of the system. Our objective is to get hands-on experience of secure network design and performance evaluations. In the rest part of the report, a detailed description of our work is presented.

The report is organized as follows: Next section introduces the goals of system design; followed by the architecture design, implementation and evaluation results.  We conclude the project in the last section.

#### Design Goals

The primary goal of our project is to implement router support for VPN with IPSec tunnel, test the robustness and integrity of our design and evaluate the system performance.

In our design, end users are those who want to send traffic securely, the secured transmission is provided by our VPN design. Network administrators manage the VPN.

There are two types of design goals: 1. goals for providing VPN service to end users and, 2. goals for managing VPN service by network administrators.

For End Users:

- User Transparency
The existence of the VPN is transparent to the users. In other words, the users on both ends of the transmission enjoy data security without any requirements from the end hosts.

- Network Traffic Independency
The VPN is set up by using IP in IP tunneling.  (In the real world, the original packet could be an IPX or AppleTalk packet while the tunnel is an IP tunnel.) The users enjoy the secured data transmission regardless of the underline network technologies.

For Network Administrators:

- Policy management
Network administrators can decide which packets should use the VPN and which packets should not depending on security policy of the sites.

- Tunnel management
Tunnels can be setup between any routers that supports the VPN implementation.  In addition, multiple tunnels can be set up between a pair of routers.  The tunnel management is therefore able to scale.  IP packets flow inside different tunnels with different security associations.

- Centralized VPN management
Different routers in the VPN can be managed centrally.  A management host sends out control packets to different routers in the VPN.

- Dynamic tunnels management
The network administrators should be able to update information inside the VPN routers without turning off the routers or interrupting the network, because network downtimes are very expensive.  Two minutes downtime per month is considered unacceptable.

#### Architectural Design

The general architecture of the targeted system can be divided into three planes: Application, Control and Data.  The application plane provides application-oriented services to end user and network administrators. The control plane contains the functional modules which control the data flow in the data plane. The data plane implements the control instructions from the control plane.

In our design, we divide the functional modules insides these three planes as follows:

User Interface

Key exchange        Security association management    Policy management

Classifier                            Tunneling                  IPsec packet processing

Figure 3 General architecture

Application plane

Control plane

Data plane

User interface in the application plane enables the network administrator to control the system. Policy management regulates the security policies (who should use the tunnel) in the system. Security association management manages the data required in the tunnel setup (for example, tunnel no., encryption key, etc.,). Key exchange protocol is used to establish a secret communication key between ingress and egress routers. Classifier categorizes the packets according to the security policy. Tunneling module establishes tunnel by referring to the security association data. The IPsec packet processing module encodes and decodes the packets by using the secret communication key between ingress and egress routers.

The design details and  functionalities of each modules are presented as follows:

## User Interface

The user interface provides network administrator a channel to interact with the system. He/she can initialize or change the parameter for the establishment of the tunneling.  These commands will be passed to the modules in the control plane. The network administrator can further monitor the system status through the information provided by the user interface.

##### Policy Management

Policy management manages the security policy about which packet is required to use tunnel. There are three types of service to all IP packets.  When a packet comes, it can be discarded, passed on in tunnel mode and passed on without tunnel.  All the source IP addresses that are required by policies to use IPsec tunnel are kept in a policy database (PD).  These are used in the classification process.

##### Security Association Management

Security association management manages the data for the establishment of tunnel. These data are stored in a security association database (SAD). Each entry in the SAD represents one security association which corresponds to a tunnel. The fields of SAD are: Destination host address, Security parameter index (SPI), Destination router address, Sequence Number, Security communication key.

##### Key Exchange Protocol

Key exchange protocol is used to establish a secret communication key between two tunnel end routers (ingress and egress routers).

In our design, we use a revised Diffie-Hellman protocol [5] as our key exchange protocol. The reason to use such protocol is that most key exchange protocol proposed for IPsec i.e. ISAKMP, Oakley, SKIP, Photuris, etc., are based on this protocol.

We assume there is a trusted third party: certificate authority (CA), which can conduct the authentication on users’ behalf.  The CA has a pair of public-private key (KCA.KCA−1). A party can obtain a certificate from CA in the following format: C(A,s)=(ID(A),s,KCA−1(ID(A),s))where, ID(A) stands for A’s identity; s for the information A wants to convey. The KCA is publicly known.

We also assume that every SG has already obtained its own certificate from the CA before any key exchange happens.

All the parties know two numbers: p and q, where p is a prime and q is a primitive of Zp.

If a source SG (SSG) wants to communicate with a destination SG (DSG), they can exchange keys as follows:

- SSG chooses a random number 0≤s≤p−2, and then computes ssg=qsmodp.

- DSG chooses a random number 0≤d≤p−2, and then computes dsg=qdmodp.

- SSG sends C(SSG, ssg) to DSG and DSG sends C(DSG, dsg) to SSG.
SSG and DSG are the identity certificate of SSG and DSG respectively.

- SSG computes K=dsgsmodp  
DSG computes K=ssgdmodp

K is stored in the security association database along with other necessary information.  After that, both parties can communicate with each other by using K.

To guarantee the reliable communication during secret exchange, we design a simple hand-shaking secret exchange protocol.

Sender

Receiver

Tunnel setup request and secrets

Acknowledgement and replying secrets

Timeout

Figure 4 Secret exchange

The sender sends out tunnel setup request along with the key secrets. The receiver once receives the data, responses with an acknowledgement with replying secrets. The sender waits for the reply from receiver for a period of time. Once timeout, the sender assumes the packet has lost. Thus, it will re-transmit the request.

##### Classifier

Classifier conducts classification on the incoming packets. Such classifications are based on the information from the PD and SAD.  The classifier classifies the packets into three categories: normal packets (those are not required to use tunnel), outbound packets (those required to use tunnel) and inbound packets (the packets coming from tunnel).

##### Tunneling

The establishment of tunnel is based on the data stored in security association database.  For outbound packets, an encapsulating security payload (ESP) header is inserted in between the outer IP header (which contains source and destination addresses of the tunnel entry and exit points) and the inner IP header (original header) of the original packet. The ESP packet contains security parameters index in the corresponding SAD entry, the encrypted payload data (by using secret communication key), next header and padding. For inbound packets, the tunneling module can decapsulate the packets by removing the outer IP header and ESP header.

##### IPsec Processing

An outbound packet is encrypted using the key specified by the security association in SAD.  An inbound packet is decrypted similarly.

The interactions among the functional modules are shown in the following figure:

Application

Control

Data

User Interface

KE

SAM

PM

Tunnel

C

IPsec

Figure 5 Interactions among the modules

PD

SAD

Administrator

System administrator takes control of all the modules in control plane through user interface. Key exchange module passes the secret key to the security association management module which controls the tunnel setup module in the data plane.  Policy management module gives source IP addresses required to use IP tunnel to the classifier.  A packet is classified by the classifier and is passed to tunnel module if it is inbound or outbound. Tunnel module processes the packet according to its nature (normal, inbound or outbound) and passes the packet to the IPsec module.

#### Implementation

## IXP1200 Network Processor

The design is implemented on Intel IXP1200 Network Processor.  The network processor is a fully programmable device which has been specifically designed to handle the high speed data manipulation requirements of networking equipment.  It implements a symmetric array of six RISC data processors (Microengines) and a StrongARM processor, two memory interfaces (SRAM and SDRAM), a PCI Interface, and an IX Bus Interface on a single chip.  Up to four thread-level tasks can be allocated per Microengine for a total of twenty-four threads in a single IXP1200.  In LAN switching applications, the six Microengines are capable of packet forwarding of over 3 million Ethernet packets per second at Layer 3.  The StrongARM processor can then be used for more complex tasks such as address learning, building and maintaining forwarding tables, and network management. [4]

## Development Environment

Control plane functions for controlling the security gateways of our VPN design are developed under Tornado, which is an integrated development platform with a built-in C/C++ compiler runs on Microsoft Windows operating system.  Tornado also provides communications between the development platform and the IXP1200.  Source codes for the control functions are compiled into object files, and the object files are loaded to the VxWorks real time operating system running on the StrongARM processor.

The IXP1200 Developer Workbench is another integrated development environment that runs on Microsoft Windows operating system for developing microcodes for microengines.  It provides microcode debugging in both simulation and hardware modes.  Although the Developer Workbench has a lot of functions such as setting breakpoints, data watch, memory watch etc., the software seems to have some memory leakage or memory management problems that limits its usefulness for developing microcodes.  We have to close and reopen the microcode project every time we start running the microcode in either the simulation or hardware mode.  If we stop debugging and start debugging again without closing the microcode project first, the Developer Workbench dies.

## Control Plane

We implemented the following policy management functions:

policy_init()			- initialize policy table in SRAM

policy_add( entry_no, src_ip )	- add an entry with src_ip

policy_del( entry_no )		- remove the entry_no entry from the table

policy_lookup()			- print out the whole table on screen

The security association management functions we implemented are:

sad_init()			- initialize security association table in SRAM

sad_add( entry_no, dest_ip, spi, tunnel_ip, seq_no, sa_key )	- add an entry into the table

sad_del( entry_no )		- remove an entry from the table

sad_lookup()			- print out the whole table on screen

In this configuration:

H1 --- R1 === R2 --- H2

When we want to setup an IPSec tunnel from H1 to H2 going through R1 and R2, R1 and R2 will store the policy and security associations to the tables after they agree with a security key.

In this case, R1 will do:

policy_add( H1_IP )

sad_add( next_entry, H2_IP, spi, R2_IP, 0, key )   // for packets going into tunnel destined to H2

sad_add( next_entry, R1_IP, spi, R2_IP, 0, key )   // for packets coming from tunnel destined to R1

Similarly, R2 will do:

policy_add( H2_IP )

sad_add( next_entry, H1_IP, spi, R1_IP, 0, key )

sad_add( next_entry, R2_IP, spi, R1_IP, 0, key )

Let us look at the scenario when a packet going through the tunnel from H1 to H2.

The packet arrives in R1.  R1 recognizes that it can provide security service for packets coming from H1.  R1 then tries to look for a security association for packets going to H2. Once a security association is found, R1 uses the security key to encrypt the packet flowing from H1 to H2, increments the sequence number of the security association in the table, includes the SPI in the outgoing packet (inside the ESP header), and forwards the encrypted packet to R2.  (In case of the security association not found, i.e. not going to H2, the packet is forwarded as usual.)

The encrypted packet arrives at R2. R2 recognizes that the incoming packet is an encrypted packet.  It extracts the SPI from the packet and tries to find a security association for the packet going to R2 with the specific SPI.  If the security association is found, R2 increments the sequence number in the table, decrypts the packet using the security key found, and realizes the packet is actually going to H2.  The decrypted packet is then forwarded to H2.  (In case of the security association not found, i.e. the encrypted packet is not destined to R2 or the SPI is not matched, the packet is forwarded as usual. This is because other routers may know the security association.)

Let us look at a scenario when multiple security associations are stored in the routers.

In this configuration:

H1

\

R1 === R2 --- H2

/

H3

In this case, an IPSec tunnel from H1 to H2 is setup first with SPI A and a tunnel from H3 to H2 is setup next with SPI B in both R1 and R2.

A packet going from H1 to H2 uses the security association with SPI A, and a packet going from H2 to H1 uses the same security association with SPI A.  It is the same as the case with one security association described above.

A packet going from H3 to H2 uses the security association with SPI A because R1 realizes it can provide security service for H3 and it tries to find a security association going to H2.  It finds the security association with SPI A because the sequential lookup returns the first match.  A packet going from H2 to H3 uses the security association with SPI B since the table in R2 indicates packets going to H3 should use the security association with SPI B.  It may look strange.  If we consider an organization using the VPN, H1, H3, and R1 are within a local domain that is protected from the outside network.  Similarly, R2 and H2 are within a local domain.  It does not matter which security association to use as long as it is encrypted between R1 and R2.  There may have security concern when H1 and H2 belong to one VPN while H3 and H2 belongs to another VPN since packets going from H1 to H2 and from H3 to H2 share the same key.  This problem only happens when two different VPNs have a common member.  The solution is to use both source and destination IP addresses for security association.  If we use the ingress and egress router IP for security association, only one security association can be used between a pair of ingress and egress routers.  The SPI can be used to determine security association only at the egress router, since a normal packet arriving at the ingress router does not contain SPI.  We have to use the source address, destination address, or other means (e.g. the protocol field in IP header) available at the ingress router if multiple security associations between the same end points of a tunnel are desired.

The key exchange protocol in the design has not been implemented in the system.  This is because we find difficulty in communication between control and data plane, especially sending packets from control plane to peer control plane in another router.  The works in this part are going to carry out in the future works.

## Data Plane

We implement our data plane functions for VPN based on the pfwd8_1f_rx16_tx6 reference design project.

The pfwd8_1f_rx16_tx6 project uses 4 Microengines for receiving packets and 2 Microengines for transmitting packets.  One of the four threads in the transmission Microengines is used for transmit scheduling, therefore, there are a total of 4x4 = 16 threads doing receiving and 2x3 = 6 threads doing transmission.  8_1f in the name of the project probably means 8 ports, and one of the eight ports is a “fast port”.

We made a number of changes to the original reference design:

- Change of flow of packet processing
The original packet processing flow is:

receive packet  verify header  modify header  route lookup  store header  receive rest of packet  enqueue packet  transmit packet

The new packet processing flow is:

receive packet  verify header  modify header  classify packet  store header  receive rest of packet  load header  create new header (optional, depends on classification)  route lookup  store header  enqueue packet  transmit packet

- Packet Classifier
Three new macros are implemented for classifying packets:

lookup_policy – looks up policy table sequentially, returns first match
lookup_sa – looks up security association depending on destination IP address at ingress router sequentially, returns first match
lookup_sa_by_spi – looks up security association depending on destination IP address and SPI at egress router sequentially, returns first match

A pkt_label is created to indicate the type of packet after classification.  pkt_label has 3 possible values indicating normal packet, outbound packet, or inbound packet.  Three macros are copied and modified from existing macros to carry the value of the pkt_label in the duration after classification and before the packet is transmitted:

Rx_SaveSlowState_w_pkt_label – modified from Rx_SaveSlowState, saves pkt_label during receive rest of packet
Rx_RestoreSlowState_w_pkt_label – modified from Rx_RestoreSlowState, retores pkt_label saved
Rx_WriteDescriptor_w_pkt_label – modified from Rx_WriteDescriptor, write packet descriptor along with pkt_label after packet enqueue, transmit thread can get back the pkt_label from the packet descriptor.  2 bits inside the packet descriptor is used to store pkt_label.

- Change of memory location for storing packets
32 bytes need to be reserved for outer IP header and ESP header for outbound packets.
Rx_SetPktBufAddr_w_base -- modified from Rx_SetPktBufAddr returns the starting location of a packet.
A piece of code is added to the transmit thread to change status_byte and ele_remaining according to pkt_label to reflect the correct number of bytes to transmit.

- Create new header

A piece of code is added to create new header for outbound packets.  It includes a macro ip_checksum_create_be to calculate the new checksum of header, the macro is modified from ip_checksum_verify_be.

A location in SRAM is used to store the IP address of the router, and a macro get_router_ip is used to read from the memory location and return the IP address of the router.  The router IP address is put into the source IP address field of the outer IP header.  The destination IP address field is the egress router IP address returned by the security association table lookup.

esp_spi_extract is used to extract the SPI field from the ESP header for an inbound packet.

- Encryption/Decryption
A macro endecrypt is used to perform XOR operations for both inbound and outbound packets.

In the above list,  2  belongs to classifier module in our design; 3 and 4 belong  to tunneling module and 5 belongs to IPsec processing module.

#### Evaluation

In the evaluation, we want to know the end-to-end performance impact of our implementation of VPN and IPSec tunnels.

## Round Trip Time

Firstly, we compare round trip delays of traffic going through the tunnel with traffic going through without tunneling.  We wrote a script to send ping packets with variable sizes and the results are logged.  Each ping command sends 10 ping packets to get the average of the round trip time.  We test the first run with normal packets and the second run with tunnel packets.  The setup is as follows:

192.168.1.169 --- R1 --- R2 --- 192.168.1.162

The 192.168.1.169 is the host sending ping packets and collecting results.  The results are included in the appendix and are summarized in the following table.

Size of Ping packets vs. RRT

| Size (bytes) | Normal (avg. 10 times in ms) | Tunneled (avg. 10 times in ms) |
| --- | --- | --- |
| 64 | 0.83 | 0.83 |
| 128 | 0.84 | 0.82 |
| 256 | 0.83 | 0.83 |
| 512 | 0.85 | 0.87 |
| 640 | 0.91 | 0.91 |
| 1024 | 1.10 | 1.10 |
| 1280 | 1.21 | 1.20 |
| 1408 | 1.30 | 1.30 |

A few exceptionally large results i.e. 2.6 ms are rejected.  The reason of the large RTT is due to our interference with the operating system during collection.

The results show that there is virtually no difference in forwarding packets with or without tunneling.  It is also possible that there is not enough traffic to affect the forwarding capability of the network processor.  If this tunneling model is put on a large-scale network, the processing of the tunneled packets only happens at the ingress and egress routers, and the packets travel through many routers in between.  Therefore, the conclusion that no impact in end-to-end round trip delay of tunneled packet is reasonable.

## Processing cost introduced into packet processor

Secondly, we want to know how much extra processing is required at the ingress and egress routers.  We ran the microcode for forwarding packets in simulation mode and track the number of clock cycles required for processing.  One test packet is sent every time to eliminate the variation of multi-threads processing different packets at the same time.  The results collected are as follows:

Classification Cost

| No. of entry | Policy Lookup (clock cycles) | SA Lookup (clock cycles) |
| --- | --- | --- |
| 1 | 32 | 90 |
| 2 | 71 | 175 |
| 3 | 132 | 212 |
| 4 | 145 | 201 |

The header processing cost of outbound packets at the ingress router is around 233 clock cycles, including 1 SDRAM read operation and 2 write SDRAM operations for reading and writing headers plus 1 read and 1 write SDRAM operation for checksum calculation.

The total processing cost for an outbound packet requires a policy lookup hits + a SA lookup hits + header processing.  Assuming 200 clock cycles for policy lookup, 200 clock cycles for SA lookup and another 200 clock cycles for header processing.  600 clock cycles for Microengine running at 166MHz comes to 3.6 μs for every outbound packet at the ingress router.  Processing 1M outbound packets takes an extra 3.6 s.  Intel claims that six Microengines can forward 3M Ethernet packets in 1 sec, therefore each Microengine can forward 0.5M packets in 1 sec, or 1M packets in 2 sec.  Adding our 3.6 sec extra cost for processing 1M outbound packets gives 1M packets in 5.6 s, or 0.18M packets in 1 sec.  Around 1M packets per sec for six Microengines.  Therefore, around 2/3 or 67% throughput reduction for the packet processor.

## Encryption cost

Lastly, we want to know the impact of encryption to end-to-end delay and the processing cost required in the router.  The data portion of a packet is first read into transfer registers and is encrypted/decrypted before storing into SDRAM.  When not using encryption, the data portion of a packet is transferred to SDRAM in parallel with other processing, it is because transferring from the receive buffer to SDRAM does not require interaction with the Microengines.  A signal is sent to the particular thread when the transmission is done.  The clock cycle elapsed during the “receive rest of packet” phase is collected for both normal and encrypted packets with variable sizes in simulation mode.  Again, only one test packet is sent every time.

Size vs. Processing Time during “receive rest of packet”

| Size (bytes) | Normal (clock cycles / μs) | Encrypted Outbound (clock cycles / μs) | Extra Time (clock cycles / μs ) |
| --- | --- | --- | --- |
| 128 | 659 / 3.97 | 765 / 4.61 | 106 / 0.64 |
| 256 | 2635 / 15.87 | 2803 / 16.89 | 168 / 1.01 |
| 512 | 6795 / 40.93 | 6847 / 41.25 | 52 / 0.31 |
| 1024 | 14827 / 89.32 | 14934 / 89.96 | 107 / 0.64 |

The results show that there is only slightly difference in processing time between processing packet with and without encryption.  This is probably due to the SDRAM access time is much larger than the time required for doing encryption, for the same packet size, it requires similar time for accessing the SDRAM.  Therefore, it will have very little impact on end-to-end round trip delay.

Also, it seems that it is very unlikely that we can implement a complex encryption algorithm in microcode.  When we increase the complexity of encryption (3 XOR operations instead of originally 1 XOR operation).  The microcode fails to compile with the error [(0): error : Page, common_code, exceeds the 1024 address limit! ********].

Assuming it takes 100 extra clock cycles for encryption, we can follow the same calculation from the previous section.  It takes an extra 0.6 s for processing 1M packets.  It’s around 2.3M packets per sec for only encryption. There is 0.7/3 or 23% throughput reduction of the packet processor.

If we have both encryption and tunneling, a total of 90% throughput reduction of the packet processor is introduced.  The original 3M packets per sec packet processor can only forward 0.3M packets per sec.  The end-to-end round trip delay remains the same (if no congestion).

Conclusion

We have designed and implemented a Virtual Private Network system with IPsec on Intel IXP1200 Network Processors. The evaluation suggests interesting results:  firstly, it justifies the viability of VPN IPsec tunnel over the network processors and secondly, it implies certain performance concerns (the throughput could be reduced up to 90%).  A practical implementation should take both the functionalities and performance into consideration.

Through the project, not only had we got more understanding about VPN, IPsec and network processors, but also had hands-on experience on network design, implementation and evaluation.  However, with the time limitation, not enough resource and long learning curve for the IXP1200 platform, we have not implemented the key exchange protocol.  Future works should be carried out to integrate the module to our system to complete the design.

Moreover, our design reflects only one of the many ways to implement VPNs.  There are some future works that we could do to make our design better. For example, we could implement a VPN containing multiple protocols such as: 1. IPsec , 2. L2TP running under IPsec, and 3. PPTP.

Of these, IPsec is expected to be dominant in the near future. L2TP running under IPsec is still in development, and PPTP is a proprietary protocol that has been implemented by many companies, most notably Microsoft.

We could also use more sophisticated encryption methods such as RSA Public-Key Algorithm as far as operation difficulty inside the network processor can be solved, and different VPN topologies such as a Firewall/VPN-to-client topology in which users can set up VPN connection to the firewall from their laptop computers.

Reference

[1] Virtual Private Networking: An Overview, http://msdn.microsoft.com/workshop/server/feature/vpnovw.asp

[2] CyLAN IPSec White Paper.  http://www.cylan.com/files/whpaper.htm

[3] Security Architecture for the Internet Protocol, http://www.ietf.org/rfc/rfc1825.txt

[4] Intel IXP1200 Network Processor Datasheet, Product Description and Introduction.

[5] Applied Cryptography

[6] Implementing Virtual Private Networks, by Steven Brown

[7] IPsec, by Doraswamy and Harkins

Appendix

Results of ping packets going through without tunneling:

PING 192.168.1.169 (192.168.1.169): 16 data bytes
24 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.7 ms
24 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.6 ms
24 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=2.6 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.6/0.9/2.6 ms
PING 192.168.1.169 (192.168.1.169): 32 data bytes
40 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
40 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=2.6 ms
40 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=2.7 ms
40 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=2.6 ms
40 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
40 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/1.3/2.7 ms
PING 192.168.1.169 (192.168.1.169): 64 data bytes
72 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
72 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
72 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 128 data bytes
136 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
136 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
136 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 256 data bytes
264 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
264 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
264 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 384 data bytes
392 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
392 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
392 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.9 ms
392 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.9 ms
392 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 512 data bytes
520 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
520 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 640 data bytes
648 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
648 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.9/0.9/1.0 ms
PING 192.168.1.169 (192.168.1.169): 768 data bytes
776 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
776 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.0 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.9/0.9/1.0 ms
PING 192.168.1.169 (192.168.1.169): 896 data bytes
904 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.1 ms
904 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.0 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.0/1.0/1.1 ms
PING 192.168.1.169 (192.168.1.169): 1024 data bytes
1032 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.1 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.1/1.1/1.1 ms
PING 192.168.1.169 (192.168.1.169): 1152 data bytes
1160 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.2 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.2/1.2/1.2 ms
PING 192.168.1.169 (192.168.1.169): 1280 data bytes
1288 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.3 ms
1288 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.2 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.2/1.2/1.3 ms
PING 192.168.1.169 (192.168.1.169): 1408 data bytes
1416 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.3 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.3/1.3/1.3 ms

Results of ping packets going through with tunneling:

PING 192.168.1.169 (192.168.1.169): 16 data bytes
24 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
24 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.9 ms
24 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.9 ms
24 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
24 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
24 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.7 ms
24 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.7/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 32 data bytes
40 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
40 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
40 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
40 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
40 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 64 data bytes
72 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
72 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
72 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
72 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 128 data bytes
136 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
136 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=2.6 ms
136 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
136 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/1.0/2.6 ms
PING 192.168.1.169 (192.168.1.169): 256 data bytes
264 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
264 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
264 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
264 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.8 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 384 data bytes
392 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
392 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.9 ms
392 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.8 ms
392 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 512 data bytes
520 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
520 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.8 ms
520 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
520 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.8/0.8/1.0 ms
PING 192.168.1.169 (192.168.1.169): 640 data bytes
648 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
648 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=0.9 ms
648 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=0.9 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 0.9/0.9/1.0 ms
PING 192.168.1.169 (192.168.1.169): 768 data bytes
776 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.0 ms
776 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.0 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.0/1.0/1.0 ms
PING 192.168.1.169 (192.168.1.169): 896 data bytes
904 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.1 ms
904 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.0 ms
904 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.0 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.0/1.0/1.1 ms
PING 192.168.1.169 (192.168.1.169): 1024 data bytes
1032 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.1 ms
1032 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.1 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.1/1.1/1.1 ms
PING 192.168.1.169 (192.168.1.169): 1152 data bytes
1160 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.2 ms
1160 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.2 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.2/1.2/1.2 ms
PING 192.168.1.169 (192.168.1.169): 1280 data bytes
1288 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.2 ms
1288 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.2 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.2/1.2/1.2 ms
PING 192.168.1.169 (192.168.1.169): 1408 data bytes
1416 bytes from 192.168.1.169: icmp_seq=0 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=1 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=2 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=3 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=4 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=5 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=6 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=7 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=8 ttl=64 time=1.3 ms
1416 bytes from 192.168.1.169: icmp_seq=9 ttl=64 time=1.3 ms

--- 192.168.1.169 ping statistics ---
10 packets transmitted, 10 packets received, 0% packet loss
round-trip min/avg/max = 1.3/1.3/1.3 ms