ATM Validation Program

User’s Guide

Mark Snesrud

# Overview

ATMVAL is a Windows program for validating the PCI ATM Adapter Board being developed by the ATM group.  ATMVAL allows access to low level features of the board such as register reading/writing, as well as higher level functions such as scheduling and PDU sending/receiving.

# Installation

The validation program consists of the following files:

WINATM.EXE

PHYSMEM.386

WINDLL.DLL

WINATM.INI

To install the program, copy WINATM.EXE, PHYSMEM.386, and WINDLL.DLL to a subdirectory such as C:\ATM_SV.  Copy WINATM.INI to the C:\WINDOWS directory (or appropriate Windows directory.)  Add the following line to the SYSTEM.INI file under [386Enh]

device=c:\atm_sv\physmem.386

You need to restart Windows at this point for the program to work.  After Windows has been restarted, run WINATM.EXE from file manager.

# Menus and Buttons

## Windows

The validation program consists of a number of windows corresponding to different functions and features of the adapter board.  The windows can be accessed from both the Window menu and the row of buttons below the menu bar.  Each window in the Window menu has a corresponding button on the button bar.  To open a window just click on the button or select it from the menu.

The numeric fields in all of the windows are displayed in hex, and all numbers entered by the user should be in hex (without an 0x or h hex signifier.)

## Options

The following options can be selected from the Options menu.  To choose an option, open the Options menu and click on the desired option.  Most options use a check-mark to signify whether they are on or off.

### Interrupt/Polling

Choose either Interrupt or Polling.  When Interrupt is selected, the adapter board will generate interrupts when certain events occur to alert the validation program.  When Polling is selected the validation program will periodically read the Interrupt Status Registers to find out if certain events have occurred.  The polling frequency can be specified by the user.

### SAR On/SAR Off

Select SAR On and SAR Off to turn the SAR on and off.  Turning the SAR off stops the sending and receiving of cells.  The SAR should be turned off when modifying When the SAR is turned on all freelists that have the Auto Fill option set will be filled up to their high water mark (see 4.7).

### Reset Board

Click on Reset Board to reset and initialize the adapter board and host memory.  This also resets the schedulers and freelists so they will have to be reconfigured after resetting the board.

### UTOPIA Idle Cell Generation

Choose UTOPIA Idle Cell Generation to have the UTOPIA interface generate idle cells when there are no cells to send.  If UTOPIA idle cell generation is turned off, the SONET interface will generate idle cells when there are no cells to send.

### Receive Logging

Turn on Receive Logging to have the validation program log all PDUs received to a file on the hard disk.  For each VC, the PDUs received on that VC will be saved to a file named PDUvc#.LOG (where vc# is the four hex digit VC number, such as PDU0001.LOG.)  This option should only be enabled when there is a low volume of PDUs being received.  During performance testing this should be turned off.

## Slave

The Slave menu allows reading from and writing to ASIC RAM, ASIC registers, SONET registers, PCI registers, and Flash registers.  When an item is selected from this menu, a window will pop up and allow the user to enter the address to read to/write from.  This window is described below in the Window Functionality section.

## Tests

A number of tests have been prewritten to run automatically when selected from the Tests menu.  To run a test just click on it in the menu.  When the test is finished a window will popup to notify whether or not the test was successful.

# Window Functionality

## Transmit PDU

The Transmit PDU window is used to define a PDU and add it to the Transmit VC Table.  Once added to the transmit table the adapter board will eventually send the PDU out on the appropriate VC.  Set the cell type to either AAL5, non-AAL5 or RAW.  Next set the VC number to send the PDU on.

The PDU to be sent can be loaded from a file on disk by clicking on the Load button.  This PDU file needs to be created ahead of time.  When a PDU is loaded from disk, the PDU Length field is set to the length of the file.  If no PDU is loaded, the payload defaults to be the letters in the alphabet repeated over and over in bytes 3 through 48 of  each cell’s payload.  The first two bytes in each payload default to be the number of the cell within the PDU (ie. the first two bytes of the first cell are 0x0000, the first two bytes of the second cell are 0x0001, etc.)

The user has some control over the buffers used to store the PDU before transmission.  Select Host Memory to use buffers in host RAM, or select Local Memory to use buffers in the adapter board memory.  Specify the size of buffers to use.  Any size buffer from 1 byte to 64K-1 can be used.  If the buffer size is smaller than the PDU size, multiple buffers will be linked together (although they won’t necessarily be contiguous) and the chain of buffers will be added to the Transmit VC Table.  Buffer alignment can also be specified.  Enter the lower 12 bits of the starting address of the buffer to align the tranmit buffers anywhere in a 4K range.

To send the PDU, click on the Send PDU button.  The PDU will be added to the Transmit VC Table and the corresponding bit in the scheduler bit vector will be set.  This does not guarrentee that the PDU will be sent by the scheduler.  The VC must be scheduled in the static scheduler (Schedulersee 4.2).

The PDU can also be scheduled dynamically by checking the Schedule Dynamically box before sending the PDU, and entering the list number and burst size to schedule.  A VC can be scheduled dynamically without sending a PDU by clicking on Add VC, and a dynamic list can be cleared by clicking on Clear List.  The dynamic scheduling functionality is repeated in the Scheduler window (see 4.2).

To send more than one PDU (the same PDU over and over), enter the number of PDUs to send and check the Repeat box.  The sending of PDUs can be stopped by clicking on the Stop button.

## Scheduler

The static schedule is a table of VC numbers that the static scheduler cycles through continuously.  Enable the static scheduler by checking the Enable box, and set the size of the table by setting the # Entries field.  The size of the table must be a multiple of 2, and must be 4 or greater.  The table can be set in three ways.  The table can be loaded from disk.  It can be manually modified by entering VC numbers and using the scroll bars.  Or VC numbers can be scheduled into the table by specifying slot numbers and burst sizes.  A static table can be saved to disk by clicking on Save To File.

The dynamic scheduler lists can be set up as well.  Enter the contract length and priority, and enable the list for each list being used.  Unused lists should be left unenabled.  VCs can be scheduled into each list by specifying the VC number and burst size, and clicking on Schedule VC.

## ASIC SAR Registers

This window displays some of the ASIC registers and allows the user to modify their values.  To modify the values click on the Modify button, make the desired changes, and click on Accept Changes.  To cancel any changes made click on Cancel.

## Transmit VC Table

Use the scroll bar to select a VC number or enter it directly.  The fields in the transmit VC table for that VC number will be displayed.  The Cell Type, Mailbox ID, and VPI/VCI fields can be modified.  The VC table fields can be saved on a per VC basis by clicking on the Save button.  Next time the program is run the values will remain.  Pressing the Cancel button will restore the last saved values.  Any changes to the VC table that aren’t saved will not appear next time the validation program is run.

## Receive VC Table

Use the scroll bar to select a VC number or enter it directly.  The fields in the receive VC table for that VC number will be displayed.  The Cell Type, Mailbox ID, Freelist A and Freelist B fields can be modified.  The VC table fields can be saved on a per VC basis by clicking on the Save button.  Next time the program is run the values will remain. Pressing the Cancel button will restore the last saved values.  Any changes to the VC table that aren’t saved will not appear next time the validation program is run.

## Mailbox

Use the arrow buttons to select a mailbox number.  The fields of the first VR in the mailbox will be displayed in the window.  To view other VRs in the mailbox use the Previous VR and Next VR buttons.  The buffer pointed to by the VR can be viewed by clicking on See Buffer.

The first VR in a mailbox is actually just a dummy VR used to prevent a race condition with the SAR and isn’t really in the mailbox.

## Freelist Manager

The freelist manager controls the setup of the 16 local VR freelists.  Before modifying freelist parameters the SAR should be turned off.  For each list specify the buffer size, high and low water marks, freelist marker spacing, and the location of the buffers (either in host memory or local memory.)  To accept the any changes made press the Accept button.  Pressing Cancel will restore the fields to the last accepted values.  Use the Save button to save the values to disk and use them the next time the program is run.

The Current Size field shows the number of VRs currently in the freelist.  VRs can be manually added by entering the number of VRs to add and clicking the Add button.  The list can also be automatically filled to the high water mark by checking the Auto Fill box.  If this option is chosen the list will be filled when the SAR is turned on.

## PDU Log

All PDUs sent and received are logged to this window.  There are two different modes used to display the PDUs.  For low volume traffic the List All PDUs mode can be selected.  In this mode the VC number and PDU length are displayed for every PDU sent and received.  For higher volume traffic us the Count PDUs for Each VC mode.  In this mode the number of PDUs received for each VC is displayed.  To clear the log press the Clear button.  For performance testing this window should be closed.

## SONET

The states of a number of SONET alarms are displayed using red boxes to represent alarms that are on, and green boxes to represent alarms that are off.   The SONET counters are also shown.  To reset all the counters to zero press the Clear button.  SONET Diagnostic Loopback Mode can be turned on to loop all cells transmitted through the SONET back into the receive part of the SONET.  This mode is useful for loopback testing because the loopback occurs outside of the ASIC.

## Slave Accesses

These windows allow testing the ASIC as a PCI bus slave.

### Static RAM Read

Enter the address of the 32-bit value to read.  Valid addresses are 0 - FFFFF with 4MB of SRAM installed.   Click on OK to read the value and display it in the Data field.  Checking the Continuous box before pressing OK causes the program to continually read from the address until the Continuous box is unchecked.  The Random box can be checked to read from random memory locations within the static RAM.

### Static RAM Write

Enter the address and 32-bit data to write.  Click on OK to write the value.  If the Verify Write box is checked, the address is read after the data is written to verify that the write was successful.

### ASIC Register Read

Enter the address of the register to read and click on OK. Valid addresses are 0 - 3FC in multiples of 4.  The value read will be displayed in the data field.  Check the Continuous box before pressing OK to repeatedly read from the register.

### ASIC Register Write

Enter the address and data to write to the register and click on OK.  Check the Verify Write box to have the program read from the register after writing to verify that the write worked correctly.

### PCI Configuration Register Read

Enter the address of the PCI config register to read and click on OK.  Valid addresses are 0 - 4C in multiples of 4.  The value read will be displayed in the data field.  Check the Continuous box before pressing OK to repeatedly read from the register.

### PCI Configuration Register Write

Enter the address and data to write to the PCI config register and click on OK.  Check the Verify Write box to have the program read from the register after writing to verify that the write worked correctly.

### SONET Register Read

Enter the address of the SONET register to read and click on OK.  Valid addresses are 0 - FF.  The 8-bit value read will be displayed in the data field. Check the Continuous box before pressing OK to repeatedly read from the register.

### SONET Register Write

Enter the address and 8-bit data to write to the SONET register and click on OK.  Check the Verify Write box to have the program read from the register after writing to verify that the write worked correctly.

### Flash Memory Read

Enter the address of the Flash to read and click on OK.  Valid addresses are 0 - 7FF.  The value read will be displayed in the data field.  Check the Continuous box before pressing OK to repeatedly read from the address.